3GPP TSG-RAN WG2 Meeting #128	R2-2410718

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:	8.7.3

Source:	Samsung

Title:	Discussion on RRM measurement gaps/restrictions enhancements for Rel-19 XR

Document for:	Discussion & Decision

Introduction

RAN1 has reached a working assumption [1] for enabling transmission/reception in gaps/restrictions, as excerpted below.

Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI.

Besides, in the last RAN2 meeting, we have reached the agreements as follows.

Agreements for RRM measurement gap skipping - RAN2 assumes that at least some impact on DSR from MG skipping can be avoided by NW implementation. FFS whether there is an impact which would require some specification changes/enhancements. - No need to have delay-aware LCP enhancements specific for MG skipping, i.e. MG skipping and delay-aware LCP are designed as independent features - RAN2 can further evaluate whether there is any impact on DRX from MG skipping. For the moment, the issue is unclear. - RAN2 will focus its work on supporting the solution chosen by RAN1/RAN4 - RAN2 can discuss whether there is a need to additionally have other solution (e.g. RRC-based) which can be decided after RAN1/RAN4 evaluation and knowing more details of DCI-based solution

In this paper, we discuss the essential aspects of the operations regarding transmission/reception in gaps/restrictions, taking into account the relevant RAN1 progress.

Discussion

NW-Initiated Solution

RAN1 has agreed with the working assumption [1] that a one-bit indication in DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI.

From the MAC perspective, TS 38.321 specified that during an activated measurement gap, the MAC entity is not allowed to perform transmission/reception/PDCCH monitoring, on the serving cell(s) in the corresponding frequency range.  Since we have one more state, i.e., not skipped, of an activated measurement gap occasion to be applied for, we should reflect the condition accordingly in the TS 38.321 for the conditions of the UE operations conducted previously during an activated measurement gap.

Proposal 1: RAN2 to reflect the newly introduced state, i.e., not skipped, for an activated measurement gap in TS 38.321, as the additional condition of the UE operations conducted previously during an activated measurement gap.

On the other hand, new UE capability signalling should be introduced to indicate the support of the measurement gap occasion skipping based on DCI indication.

Proposal 2: RAN2 to introduce a new UE capability signalling, indicating the support of the DCI-based measurement gap occasion skipping.

UE-Initiated Solutions

Besides the NW-initiated solution, a question worth discussing is, whether or not the UE-initiated solution is essential for serving delay-critical UL data.

First, we should consider whether network has sufficient information/capability to handle the delay-critical UL data in terms of:

- Case 1: if the UL data is perfectly periodic or the jitter is not that intense in terms of its arrival time and traffic amount, network is able to properly make the decision on whether to skip a certain gap/restriction to prioritize the urgent uplink traffic based on NW-initiated solution.

- Case 2: if the uplink traffic is not periodic or with intense jitter in terms of its arrival time and/or traffic amount, network is not able to make a decent decision on whether a gap/restriction should be skipped or not, if UE does not provide relevant assistant information, e.g., BSR/DSR/SR, in a timely manner. Since UE is aware of the whole situation for UL data, it is reasonable for UE to proactively decide whether to skip gap(s)/restriction(s) or not, in this case.

Since it is likely XR UL traffic is pertaining to Case 2, we should introduce UE-initiated solution(s), to deal with the uncertainties in packet arrival time/size in UL.

Proposal 3: RAN2 to consider introducing UE-initiated solution(s) for skipping gaps/restrictions caused by RRM measurements.

In developing UE-initiated solution(s), the target issues worth being addressed are:

- Issue 1: an SR triggered by LCH with delay-critical data is delayed by overlapping gaps/restrictions. Before receiving the SR, network has no knowledge on the existence of the delay-critical uplink data, thus relying solely on network-initiated solution is not feasible in this case.

- Issue 2: a CG PUSCH transmission multiplexed with DSR is delayed by overlapping gaps/restrictions. Note that if CG resource is available for accommodating DSR MAC CE plus its subheader, the corresponding SR for DSR will not be triggered/transmitted (based on SR for DSR triggering condition), and hence, network is not aware of the existence of the delay-critical data before receiving the CG PUSCH with DSR, thus relying solely on network-initiated solution is not feasible in this case.

- Issue 3: a CG PUSCH transmission multiplexed with delay-critical data is delayed by overlapping gaps/restrictions. Note that, DSR/SR may not be transmitted by or before the CG PUSCH, if the CG PUSCH includes all the PDCP SDUs associated with all pending DSR(s) (based on current DSR inclusion rule), and hence, network is not aware of the existence of the delay-critical data before receiving the CG PUSCH with the delay-critical data.

It is better for RAN2 to first discuss the target issues to be addressed by the UE-initiated solution(s), before delving into the solution(s) itself.

Proposal 4: RAN2 to confirm the issues to be addressed by UE-initiated solution(s), if supported, by considering:

- Issue-1: an SR triggered by LCH with delay-critical data is delayed by overlapping gaps/restrictions caused by RRM measurements.

- Issue-2: a CG PUSCH transmission multiplexed with DSR is delayed by overlapping gaps/restrictions caused by RRM measurements.

- Issue-3: a CG PUSCH transmission multiplexed with delay-critical data is delayed by overlapping gaps/restrictions caused by RRM measurements.

On the other hand, since the skipping of a measurement gap occasion may delay the measurement operation/reporting, and hence, the UE mobility performance can be affected. It means that NW needs a way to bound the side effect caused by UE-initiated skipping of the measurement gap occasion, e.g., by configuring which measurement gap or measurement gap occasion(s) can or cannot be skipped. We can consider letting NW configure the candidate skippable measurement gap or measurement gap occasions semi-statically (e.g., via RRC) in advance, to bound the scope, within which, the UE-initiated solution, e.g., a measurement gap occasion is skipped if a certain condition at UE side is satisfied, can take place.

Proposal 5: NW can configure candidate measurement gap occasions, within which, measurement gap occasions are allowed to be skipped by UE-initiated solution.

Conclusion

Based on the discussion above, we have the following observations and proposals:

Proposal 1: RAN2 to reflect the newly introduced state, i.e., not skipped, for an activated measurement gap in TS 38.321, as the additional condition of the UE operations conducted previously during an activated measurement gap.

Proposal 2: RAN2 to introduce a new UE capability signalling, indicating the support of the DCI-based measurement gap occasion skipping.

Proposal 3: RAN2 to consider introducing UE-initiated solution(s) for skipping gaps/restrictions caused by RRM measurements.

Proposal 4: RAN2 to confirm the issues to be addressed by UE-initiated solution(s), if supported, by considering:

- Issue-1: an SR triggered by LCH with delay-critical data is delayed by overlapping gaps/restrictions caused by RRM measurements.

- Issue-2: a CG PUSCH transmission multiplexed with DSR is delayed by overlapping gaps/restrictions caused by RRM measurements.

- Issue-3: a CG PUSCH transmission multiplexed with delay-critical data is delayed by overlapping gaps/restrictions caused by RRM measurements.

Proposal 5: NW can configure candidate measurement gap occasions, within which, measurement gap occasions are allowed to be skipped by UE-initiated solution.

References

[1] 3GPP TSG RAN WG1 #118, RAN1 Chair’s Notes.