3GPP TSG-RAN WG2 Meeting #128	R2-2410740

Orlando, USA, Nov 18-22, 2024

Source:	III

Title:	Discussion on XR RRM measurement gaps/restrictions related enhancements

Agenda item:	8.7.3

Document for:	Discussion

- Introduction

During the RAN#127bis meeting [1], the following agreements were noted:

Agreements for RRM measurement gap skipping - RAN2 assumes that at least some impact on DSR from MG skipping can be avoided by NW implementation. FFS whether there is an impact which would require some specification changes/enhancements. - No need to have delay-aware LCP enhancements specific for MG skipping, i.e. MG skipping and delay-aware LCP are designed as independent features - RAN2 can further evaluate whether there is any impact on DRX from MG skipping. For the moment, the issue is unclear. - RAN2 will focus its work on supporting the solution chosen by RAN1/RAN4 - RAN2 can discuss whether there is a need to additionally have other solution (e.g. RRC-based) which can be decided after RAN1/RAN4 evaluation and knowing more details of DCI-based solution

Topics relative to XR-specific capacity issues was raised as mentioned above. In this contribution,” The Conflict between Measurement Gaps and Configured Grants” and “Remaining Time Management for UEs in MG Scenarios” are addressed.

- Discussion

To support enhancement for capacity, the following issues need to be further considered:

- The Conflict between Measurement Gaps and Configured Grants,

- Remaining Time Management for UEs in MG Scenarios.

  - The Conflict between Measurement Gaps and Configured Grants

As the demand for Extended Reality (XR) applications continues to rise, mobile networks are facing unprecedented challenges in managing resource allocation. XR technologies, including augmented reality (AR) and virtual reality (VR), require high bandwidth and low latency to deliver immersive experiences. Consequently, this increasing demand has prompted a reevaluation of traditional mechanisms in mobile communication, particularly the management of measurement gaps (MG) and configured grants (CG).

Measurement gaps are crucial for maintaining connection quality, allowing User Equipment (UE) to assess the radio environment. In the past, the conflict between MG and CG (both operating periodically) was solved by granting a higher priority to MG. This decision aimed to ensure stable connectivity but often resulted in reduced data transmission capacity, especially detrimental in XR scenarios where high data rates are essential.

To enhance data throughput, there are two key approaches to consider:

- Granting a higher priority to CG: By prioritizing CG over MG, the network can improve data transmission capabilities, ensuring that XR applications operate smoothly even under varying conditions. However, this approach has a significant drawback—it lacks flexibility. In scenarios where measurement requires MG, the rigid prioritization of CG may cause issues.

- Creating Adaptive Mechanisms for Balancing MG and CG: This approach focuses on developing mechanisms that dynamically adjust the balance between MG and CG based on real-time network conditions and application requirements. While this can greatly enhance resource utilization and allow the network to adapt to varying demands, a potential downside is the complexity of the mechanisms involved.

Both methods offer distinct advantages and disadvantages, highlighting the need for careful consideration when optimizing network performance.

Observation 1: In the past, the conflict between MG and CG (both operating periodically) was solved by granting a higher priority to MG that is too conservative and leads to throughput reduction.

Proposal 1: To enhance data throughput, there are two key approaches should be considered: granting a higher priority to CG and/or creating adaptive mechanisms for balancing MG and CG.

  - Remaining Time Management for UEs in MG Scenarios

In addition to rethinking the priority of MG and CG, there is a critical issue surrounding the remaining time for PDU(s) or PDU set(s). If the resources allocated by the gNB do not meet the available remaining time, UEs risk exceeding their transmission deadlines when they face insufficient time to transmit data, at which point MG can be used to resolve this issue, though it may complicate resource management strategies.

To address this challenge, RAN2 should explore the development of new mechanisms that allow UEs to effectively use MG for uplink data transmission when necessary. However, this solution will not be without its challenges. One significant concern is the potential for resource competition among UEs, which could lead to inefficiencies and increased latency. Furthermore, timely resource allocation by the gNB is crucial; delays in this process could undermine the benefits of using MG for data transmission.

Another key aspect of this issue is the synchronization of information between the UE and the gNB. For UEs to successfully utilize MG for data transmission, both the UE and the gNB must be aware of the UE's intention to use MG. Ensuring that this information is accurately communicated and synchronized is essential for effective resource management and for preventing potential conflicts.

Proposal 2: RAN2 should explore the development of new mechanisms that allow UEs to effectively use MG for uplink data transmission when necessary.

Observation 2: The solution will face challenges such as resource competition among UEs, ensure timely resource allocation by the gNB, and enable effective synchronization of information between the UEs and the gNB.

- Conclusion

The related proposals and observations from above discussions are below:

Observation 1: In the past, the conflict between MG and CG (both operating periodically) was solved by granting a higher priority to MG that is too conservative and leads to throughput reduction.

Proposal 1: To enhance data throughput, there are two key approaches should be considered: granting a higher priority to CG and/or creating adaptive mechanisms for balancing MG and CG.

Proposal 2: RAN2 should explore the development of new mechanisms that allow UEs to effectively use MG for uplink data transmission when necessary.

Observation 2: The solution will face challenges such as resource competition among UEs, ensure timely resource allocation by the gNB, and enable effective synchronization of information between the UEs and the gNB.

References

[1] 	Report of 3GPP TSG RAN WG2 #127bis