3GPP TSG-RAN WG2 Meeting #128											        R2-2410765

Orlando, 18-22 Nov. 2024

Agenda item:			8.9.2

Title:						Further considerations on S&F operations

Source:					Continental Automotive

Document for:		Discussion and Decision

# Introduction

During RAN2#127bis [1] the following agreements were made:

Agreements:

1.	The dynamic indication that “the cell is operating in S&F mode” is called “S&F operation” indication (we can come back on the exact name when putting this in the spec, if needed)

2.	RAN2 assumes that if an indication that “the cell is operating in S&F mode” is not provided in a NTN cell, the UE should assume that the NTN cell is operating in real-time mode (i.e. default/normal/”not-S&F” mode). RAN2 assumes there should be no distinction between the case that the UE is served by a NTN cell that do not support S&F capability and the case that the UE is served by a NTN cell that does support S&F capability but indication that “the cell is operating in S&F mode” is not provided.

3.	An S&F explicit capability indication by the serving cell, conceived as a static indication of whether the S&F capability is supported or not by a specific satellite/NTN-cell, in addition to the indication that “the cell is operating in S&F mode”, is not needed

4.	If the “S&F operation” indication is not broadcast, a Rel-19 UE (regardless whether supporting S&F or not) shall follow the legacy barring procedure

5.	When present, the “S&F operation” indication has two possible settings:

‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell)

‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

In this paper, we share further considerations on S&F operations.

# 2. Discussion

In NGSO scenarios involving earth-moving cells, e.g., where service is provided by LEO satellites, feeder link may only be intermittently available. Consequently, user data needs to be stored on the satellite until feeder link becomes available again and data can be forwarded to the ground segment (e.g., packet data gateway).

In the RAN2#127bis meeting, an explicit “S&F operation” indication was agreed [1]. Further, potential impacts on MO data delivery as well as different delivery time indications were also discussed in the RAN2#127bis meeting, where the UE’s transmission behaviour during S&F operations is left to UE implementation. For example, a UE with S&F capability will only transfer data, if its application data is delay tolerant. Otherwise, if the application data is non-delay tolerant, the UE will not transfer the data. However, for managing quality of service at the network level, further enhancements may be needed.

Observation 1: An explicit “S&F operation” indication was agreed, but UE’s transmission behaviour during S&F operations is left to UE implementation.

To enhance quality of service at the network level, the network should provide further guidance to the UE regarding its transmission behaviour when operating in S&F mode. For example, the network could differentiate between delay tolerant and non-delay tolerant services per logical channel and configure S&F-capable UEs to transmit data during S&F operations accordingly.

Proposal 1: RAN2 should further discuss enhancing quality of service during S&F mode on network level. For example, the network could differentiate between delay tolerant and non-delay tolerant services per logical channel and configure S&F-capable UEs accordingly.

# Conclusions

The following proposals have been made in this contribution:

Observation 1: An explicit “S&F operation” indication was agreed, but UE’s transmission behaviour during S&F operations is left to UE implementation.

Proposal 1: RAN2 should further discuss enhancing quality of service during S&F mode on network level. For example, the network could differentiate between delay tolerant and non-delay tolerant services per logical channel and configure S&F-capable UEs accordingly.

# References

- R2-2409213, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2#127bis, Hefei, China, Oct. 14th-18th, 2024.