3GPP TSG-RAN WG2 Meeting #128	R2-2410797

Orlando, USA, 18 – 22 November 2024

Agenda item:	8.3.5

Source:	Nokia, Nokia Shanghai Bell

Title:	On the remaining simulation assumptions and model generalization aspects

WID/SID:	FS_NR_AIML_Mob- Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#127bis the simulation assumptions for the different use-cases were discussed at length. The simulation assumptions were further consolidated in a post meeting e-mail discussion. In this contribution, we present our views on the topics that were raised during the e-mail discussion and generalization aspects.

# 2	Discussion

## 2.1 	Follow-Up on the Email Discussion

In the sequel, we present our views on the remaining aspects of the e-mail discussion [1]. Specifically, we focus on the direct prediction output for event and failure prediction use-case, as well as on the need to consider ETD in the reported simulations.

### 2.1.1	Direct Prediction Output

In the email discussion [1], a probability threshold for the direct measurement event and RLF prediction is proposed. However, in our view, this is not necessarily needed. The direct prediction method for measurement event prediction or RLF prediction, is implemented as a classification ML model, which will be trained to minimize the loss function based on labels of the associated events, either measurement event or RLF, occurring in the PW or not. As a starting point, the direct prediction should be evaluated with classification metrics such as F1-score and AUC, as trained by the selected algorithm and the loss function.

Observation 1: The direct prediction method for measurement event prediction or RLF prediction can be implemented as a classification ML model, which will be trained to minimize the loss function based on labels of the associated events, either measurement event or RLF, occurring in the specified PW or not.

Depending on the selected algorithm in the direct method, it may be possible to derive during inference confidence or probability estimates for the binary classes (the event predicted to occur or not) from the classification model. For example, the following methods can be used when handling a classification task in direct event or RLF prediction.

- Random Forest: each decision tree in the ensemble independently makes a classification prediction. The probability of a class is calculated as the fraction of trees that vote for that class.

- Gradient Boosting: the model often produces log-odds for binary classification, which are then converted to probabilities using a sigmoid function.

- CNN or LSTM: it is used when used for classification tasks in sequence data, a softmax or sigmoid activation is applied to the final layer, converting outputs to probabilities.

When the probability estimate is available, it may potentially be utilized to configure the true and false positive detection rate trade-off at inference time, without requiring re-training or fine tuning the model. In this case, a sweep of the classification probability threshold may be evaluated with classification metrics. The optimal classification threshold depends on decisions and actions made based on the decisions and consequent cost of false negative vs. false positive predictions.

Observation 2: The optimal classification threshold depends on decisions and actions made based on the decisions and consequent cost of false negative vs. false positive predictions.

Proposal 1: The selection of the probability threshold that is used in the classification evaluation is left for each company to choose.

## 2.1.2 	Measurement Event Prediction

Figure 2.1.2-1 Example timeline for FR2 temporal domain case A based on direct prediction

In the figure above, from the ongoing email discussion [1], measurement event prediction interaction with the handover is presented. In our understanding, the t0 refers to a point in time, when a positive measurement event prediction is made in the PW. However, it is not clear, what is triggering making the prediction. Are the predictions made periodically after every measurement, when the UE is configured with an A3 measurement event in its measurement configuration or are there additional trigger conditions for making the measurement event predictions? Periodic measurement event predictions without additional prediction trigger conditions have a significant class imbalance between positive and negative predictions, introduce overhead and may be susceptible to false positive predictions.

Observation 3: Periodic measurement event predictions without additional prediction trigger conditions have a significant class imbalance between positive and negative predictions, introduce overhead and are susceptible to false positive predictions.

Proposal 2: RAN2 to discuss the trigger conditions for making the measurement event predictions.

In our understanding, one potential advantage of the discussed approach is to allow for an early preparation of a handover and thus reducing the handover duration after handover decision is made. In our understanding, the state-of-the-art A3 measurement event can be assumed to be configured in addition to configuring to report predicted A3 measurement events. In the email discussion, it is also proposed to potentially trigger the handover based on the predicted measurement event time, if it occurs before the “real A3 event” is reported. Triggering handovers based on predicted A3 events requires high prediction accuracy and robustness, because false predictions may lead to a HOF or an RLF, causing service interruptions. RAN2 should also study and evaluate deciding the HO preparation based on the prediction but triggering the HO only based on the real/measured A3. This is a more robust approach, because it does not lead to a HOF/RLF if the measurement event prediction was wrong, while still saving the HO preparation time. In this method, both indirect and direct measurement event predictions can be handled the same way. In any case, it is always up to the network to decide on handover preparation and handover execution based on the A3 measurement event predictions and/or “real A3 measurement events” reported by the UE, as configured to report by the network.

Observation 4: The state-of-the-art A3 measurement event can be configured in addition to configuring to report predicted A3 measurement events.

Observation 5: A3 measurement event prediction can be used in handover preparation and/or execution decisions.

Proposal 3: RAN2 to also study and evaluate deciding the HO preparation based on measurement event prediction but triggering the HO only based on the real/measured A3.

## 2.1.3	RLF Prediction Discussion

In ongoing email discussion [1], the following definition for indirect RLF prediction is given:

In indirect RLF event prediction, future L1 SINR measurement result(s) of serving cell is predicted by a RRM measurement prediction model in temporal domain at first, based on which and optional historical actual L1 SINR measurement result(s) a RLF event at one future time instance is derived without further AI/ML model.

In RAN#126 it was agreed that the study should focus on RLF due to T310 expiry (i.e. in-synch/out-of-synch case) as the representative RLF case for direct and indirect prediction. However, predicting serving cell L1 SINR measurements is alone not sufficient to predict actual RLFs due to T310 expiry that may occur in the network, which would need to consider also if a handover is possible, when it is executed, if traffic steering etc. are available, and so on. Our understanding of the indirect RLF prediction is that it is predicting when the serving cell L1 SINR measurement(s) are predicted to fall below a specified threshold, rather than actual RLFs observed in the network due to T310 expiry during training data collection.

Observation 6: Predicting serving cell L1 SINR measurements is alone not sufficient to predict actual RLFs due to T310 expiry in a real network.

Related to the observation above, it should be clarified, how the labels or target values used in the indirect RLF prediction evaluation are collected. In a simulation scenario, where handovers are disabled, we may simulate when the UE declares an RLF due to T310 expiry as the serving cell signal strength degrades. However, RAN2 should clarify how the labels are collected in a real network, where handovers are not disabled and the UE in most cases performs a handover before an RLF occurs. Is the labelling in that case done based on applying a threshold on collected serving cell L1 SINR measurements rather than actual RLFs due to T310 expiry observed in the network during training data collection?

Observation 7: In a simulation scenario, where handovers are disabled, we may simulate when the UE declares an RLF due to T310 expiry as the serving cell signal strength degrades. However, in a real network the UE in most cases performs a handover before an RLF occurs due to the servicing cell SINR degrading.

Proposal 4: RAN2 to clarify, how the labels for indirect RLF prediction are collected in a real network, where handovers are not disabled.

## 2.1.4 	Discussion on the Event Timing Distance (ETD)

In the ongoing email discussion [1] Section 2.1.2, evaluation metrics for the measurement event and RLF prediction use cases are discussed. For indirect prediction, the “time difference of true time event and predicted time event”, i.e., Event Timing Difference (ETD) has been agreed. However, computing the ETD only makes sense for true positive predictions, i.e., when a measurement event or an RLF is correctly predicted to be occurring in the PW of the RRM measurement prediction model. Therefore, in our view, computing additional classification metrics for the true positive predictions based on the ETD is unnecessarily complicated, but rather by meaningfully configuring the measurement event or RLF prediction PW, computing the classification metrics (F1-score, AUC) of correctly predicting the event occurrence in the PW is better. Computing the ETD and using it for further classification of true positive predictions should be optional.

Observation 8: Computing the ETD only makes sense for true positive predictions, i.e., when a measurement event or an RLF is correctly predicted to be occurring in the PW of the RRM measurement prediction model.

Proposal 5: Computing the ETD and using it for further classification of true positive predictions should be optional.

## 2.2	Generalization Aspects and Simulation Assumptions

When machine learning models are trained to learn a function that maps an input to an output, there are two underlying assumptions:

1. The input data has the same distribution in both the training and the test datasets.

2. The relevant relationship or function between the input data and the target values or labels is the same in the training and test dataset.

Data drift occurs when the first assumption is invalidated, and concept drift when the second. Some ways in which data or concept drift can happen are:

- Training a model on a certain frequency layer and testing it on another frequency layer.

- Training a model on multiple cells and testing it on a cell that is different from the ones trained on.

- Training a model on a certain propagation environment and testing it on a significantly different propagation environment.

If we focus at the two last items, we can identify at least two levels of generalization in simulations in the RRM measurement prediction use case:

- General model, which is trained to cover any network or any part of a network and not aware of specific cells(s) or beam(s) as input

  - Can only learn to do generic extrapolation with limited gains expected over non-ML solutions

- Cell-, site- or area-specific models

  - Training data from one cell or from a cluster of cells

  - Model inputs may be specific to specific cell(s) or beam(s)

  - Can learn the local radio environment and UE trajectories

To evaluate the broadest generalization in case 1, the random seed of, for example, the LOS/NLOS model may be changed between the drops used in the training and test datasets and the UEs trajectory model based on random walk. The training and test datasets can be split based on the simulation drops, where the random seeds are changed.

To evaluate case 2, the stationary features of a network scope, such as the LOS/NLOS and UE trajectory patterns, should remain the same between the drops and only the transient features are changed. UEs should follow defined trajectory patterns, such as graph-based mobility with limited “fuzziness” to avoid identical trajectories.

One scenario between cases 1 and 2 is, where the training and test datasets are split, for example, based on the UE (to avoid having overlapping time series in the two datasets) from all the simulation drops.

Observation 9:  We can identify at least two levels of generalization in simulations in the RRM measurement prediction use case:

- General model, which is trained to cover any network or any part of a network and not aware of specific cells(s) or beam(s) as input

- Cell-, site- or area-specific models

Observation 10: To evaluate case 1, the random seed of, for example, the LOS/NLOS model may be changed between the drops used in the training and test datasets and the UEs trajectory model based on random walk.

Observation 11: To evaluate case 2, the stationary features of a network scope, such as the LOS/NLOS and UE trajectory patterns, should remain the same between the drops and only the transient features are changed.

Proposal 6: RAN2 to agree on the simulation assumptions for different studied generalization options in the RRM measurement prediction use case.

# 3	Conclusion

This document has made the following observations:

Observation 1: The direct prediction method for measurement event prediction or RLF prediction can be implemented as a classification ML model, which will be trained to minimize the loss function based on labels of the associated events, either measurement event or RLF, occurring in the specified PW or not.

Observation 2: The optimal classification threshold depends on decisions and actions made based on the decisions and consequent cost of false negative vs. false positive predictions.

Observation 3: Periodic measurement event predictions without additional prediction trigger conditions have a significant class imbalance between positive and negative predictions, introduce overhead and are susceptible to false positive predictions.

Observation 4: The state-of-the-art A3 measurement event can be configured in addition to configuring to report predicted A3 measurement events.

Observation 5: A3 measurement event prediction can be used in handover preparation and/or execution decisions.

Observation 6: Predicting serving cell L1 SINR measurements is alone not sufficient to predict actual RLFs due to T310 expiry in a real network.

Observation 7: In a simulation scenario, where handovers are disabled, we may simulate when the UE declares an RLF due to T310 expiry as the serving cell signal strength degrades. However, in a real network the UE in most cases performs a handover before an RLF occurs due to the servicing cell SINR degrading.

Observation 8: Computing the ETD only makes sense for true positive predictions, i.e., when a measurement event or an RLF is correctly predicted to be occurring in the PW of the RRM measurement prediction model.

Observation 9:  We can identify at least two levels of generalization in simulations in the RRM measurement prediction use case:

- General model, which is trained to cover any network or any part of a network and not aware of specific cells(s) or beam(s) as input

- Cell-, site- or area-specific models

Observation 10: To evaluate case 1, the random seed of, for example, the LOS/NLOS model may be changed between the drops used in the training and test datasets and the UEs trajectory model based on random walk.

Observation 11: To evaluate case 2, the stationary features of a network scope, such as the LOS/NLOS and UE trajectory patterns, should remain the same between the drops and only the transient features are changed.

And proposed the following:

Proposal 1: The selection of the probability threshold that is used in the classification evaluation is left for each company to choose.

Proposal 2: RAN2 to discuss the trigger conditions for making the measurement event predictions.

Proposal 3: RAN2 to also study and evaluate deciding the HO preparation based on measurement event prediction but triggering the HO only based on the real/measured A3.

Proposal 4: RAN2 to clarify, how the labels for indirect RLF prediction are collected in a real network, where handovers are not disabled.

Proposal 5: Computing the ETD and using it for further classification of true positive predictions should be optional.

Proposal 6: RAN2 to agree on the simulation assumptions for different studied generalization options in the RRM measurement prediction use case.

# 4	References

[1] R2-2410190, Summary of [POST127bis][022][AI mobility] Simulation assumptions (OPPO), RAN2#128, Orlando, Florida, November, 2024.