3GPP TSG-RAN WG2 #128	R2-2410848

Orlando, US, 18th – 22nd, November 2024

Source:	NTT DOCOMO, INC., Nokia, Samsung

Title:	Discussion on applicable BWP for Multi-cell scheduling

Document for:	Discussion and decision

Agenda Item:	7.8.2

- Introduction

In the last RAN2 meeting, we discussed the CR [1] of TS 38.331 for multi-cell scheduling and found that the changes are technically valid, but we postponed it to find a way to capture minimal change, i.e., avoid duplicated description of RAN1 spec.

This contribution provides the summary of unofficial offline discussion on the postponed CR [1].

- Discussion

  - Change on applicable BWP

The original CR suggested changes on the parameters, RateMatchDCI-1-3, TCI-DCI-1-3, ZP-CSI-DCI-1-3, SRS-OffsetCombo, and SRS-RequestCombo. The changes are suggested to clarify that if a code point in DCI format 0_3/1_3 does not indicate a BWP configured for a scheduled cell, the BWP to be applied for each table is not that indicated in the code point in DCI format 0_3/1_3 but the current active BWP, based on following RAN1 agreements [2].

Agreement - BWP indicator in a DCI format 0_3/1_3 applies only to the scheduled cell(s) with valid FDRA value(s). - For a cell scheduled by DCI format 0_3/1_3 with valid FDRA value, if the BWP indicator indicates a code point that does not correspond to a configured BWP for the cell, the UE does not perform dynamic BWP switching based on the BWP indicator and transmits/receives data on the current active BWP of the cell.

During offline discussion, companies have the same view on introducing references to the corresponding RAN1 specification, e.g.;

RateMatchDCI-1-3 Configure each row of the joint rate matching indication table for DL scheduling via DCI format 1_3, where bitmap for a cell points to a corresponding rate matching indication applicable for DCI format 1_1 (i.e., MSB and LSB of bitmap refer rateMatchPatternGroup1 and rateMatchPatternGroup2 for a cell, respectively), the order of rate matching indication bitmap in each row refers the order of cells in ScheduledCellListDCI-1-3, that are configured with rateMatchPatternGroup1 or rateMatchPatternGroup2 on at least one DL BWP (i.e., first bitmap is for the first cell in ScheduledCellListDCI-1-X, that are configured with rateMatchPatternGroup1 or rateMatchPatternGroup2 on at least one DL BWP and so on), the number of entries in a row of rateMatchDCI-1-3 should be the same as the number of cells, that are configured with rateMatchPatternGroup1 or rateMatchPatternGroup2 on at least one DL BWP, included in ScheduledCellListDCI-1-3, and entries for co-scheduled cells in a row of rateMatchDCI-1-3 are interpreted based on the BWPs of co-scheduled cells on which the UE operates based on the BWP indicator field of DCI format 1_3 (see TS 38.212 [17], clause 7.3.1.2.4 and TS 38.213 [13], clause 12).

  - Change on applicable TCI state

To the parameter, TCI-DCI-1-3, a different approach from other parameters is needed because the entries of the table indicated by the parameter include TCI states associated with scheduled cells and even non-scheduled cell from scheduledCellListDCI-1-3 (see RAN1 CR [3]). However, RAN1 has not made clear agreement on applicable BWP for non-scheduled cell while RAN1 has several agreements below which implicitly suggest that current active BWP is applied for non-scheduled cell.

RAN1#114-bis [4] - Single joint table is configured per set of cells for each of Type-1B fields other than TDRA (i.e., rateMatchListDCI-1-3, zp-CSI-RSListDCI-1-3, tci-ListDCI-1-3, srs-RequestListDCI-1-3, srs-OffsetListDCI-1-3, srs-RequestListDCI-0-3, srs-OffsetListDCI-0-3). - Entries for each CC are interpreted based on the new/target BWPs per cell that is indicated by the BWP indicator field of DCI 0_3/1_3. RAN1#115 [2] BWP indicator in a DCI format 0_3/1_3 applies only to the scheduled cell(s) with valid FDRA value(s).

After offline discussion, companies share a similar direction, i.e., the field description of TCI-DCI-1-3 should allow to indicate cells regardless of they are scheduled or not like below;

TCI-DCI-1-3 Configure each row of the joint TCI table for DL scheduling via DCI format 1_3, where index for a cell points to a corresponding TCI applicable for DCI format 1_1, and the order of TCI index in each row refers the order of cells in scheduledCellListDCI-1-3 (i.e., first index is for the first cell in scheduledCellListDCI-1-3 that configured with tci-StatesToAddModList and so on), the number of entries in a row of TCI-DCI-1-3 should be the same as the number of cells that configured with tci-StatesToAddModList on at least one DL BWP, included in scheduledCellListDCI-1-3, and entries for cells in a row of TCI-DCI-1-3 are interpreted based on the BWPs of cells in scheduledCellListDCI-1-3 on which the UE operates based on the BWP indicator field of DCI format 1_3 (see TS 38.212 [17], clause 7.3.1.2.4, and TS 38.213 [13], clause 12).

and RAN2 should not capture more specific descriptions than RAN1 spec, therefore we can just make an agreement on our understanding that current active BWPs are applied when non-scheduled cells are indicated in TCI-DCI-1-3 but no more change is needed to TS 38.331.

We have provided a revised CR in R2-2410845 [5].

Proposal 1.	Apply changes in R2-2410845.

Proposal 2.	RAN2 understands the entries for non-scheduled cells in a row of TCI-DCI-1-3 are interpreted based on the respective current active BWPs.

- Summary

Proposal 1.	Apply changes in R2-2410845.

Proposal 2.	RAN2 understands the entries for non-scheduled cells in a row of TCI-DCI-1-3 are interpreted based on the respective current active BWPs.

- References

[1]	R2-2409100, “Correction on the applicable BWP for certain RRC parameters for DCI format 0_3/1_3,” Samsung, RAN2#127-bis.

[2]	Final Report of 3GPP TSG RAN WG1 #115 v1.0.0 (Chicago, USA, 13th – 17th November 2023).

[3]	R1-2405734, “Correction on indicated unified TCI states for multi-cell scheduling,” RAN1#117.

[4]	Final Report of 3GPP TSG RAN WG1 #114bis v1.0.0 (Xiamen, China, 9th – 13th October 2023).

[5]	R2-2410845, “Correction on applicable BWP for multi-cell scheduling,” NTT DOCOMO, INC., Nokia, Samsung, Huawei.