3GPP TSG-RAN WG2 Meeting #128	R2-2410856

Orlando, USA, Nov. 18th – 22nd, 2024

Source:                    	DENSO CORPORATION

Title:  	Discussion on inter-CU LTM

Document for:        	Discussion and decision

Agenda Item:         	8.6.2	Inter-CU LTM

- Introduction

In RAN2-127bis meeting [1], RAN2-127 meeting[2] and RAN2-125bis[3], RAN2 discussed the detail of security update procedure for inter-CU LTM, and the following was agreed.

Agreements on inter-CU LTM in RAN2-127bis

1.	The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs).

2.	For non-DC case, if the new Rel-19 IDs of the serving cell and the target cell have same values, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18.

3.	The SCPAC-similar security update configuration is introduced for inter-CU SCG LTM, i.e. similar to IEs sk-CounterConfiguration, servingSecurityCellSetId and securityCellSetId. The names of the new IEs are to be discussed in stage3.

Agreements on inter-CU LTM in RAN2-127

RLC and PDCP re-establishment in inter-CU LTM:

- If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. As baseline introduce a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update, however dependent on SA3 response, we can revisit it.

Agreements on LTM cell switch execution phase: in RAN2-125bis

1.	Upon inter-CU LTM execution, UE performs

- MAC reset

- RLC re-establishment

- PDCP re-establishment

- Security key update

In this contribution, we provide our understanding for inter-CU LTM procedure, such as the procedure for security update.

- Discussion

In Rel-19 LTM, RAN2 agreed that the UE performs security key update upon inter-CU LTM execution, and Rel-19 LTM introduces a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update. As the detail of security key update procedure, RAN2 provided 4 options for indicating the NCC value to derive security key used on the UE and send LS to SA3 for asking which option could be supported. We think that RAN2 can proceed the discussion that how to deliver the security information by MAC CE or RRC signaling in consideration of the security threat.

Using new information in MAC CE to deliver the security information may have a security threat. Furthermore, in conditional LTM discussed in RAN2, the source CU does not send cell switch command MAC CE to indicate cell switch. Therefore, considering the conditional LTM, delivering the security information without the use of MAC CE allows the flexibility.

Furthermore, sending NCC value via RRC message during LTM preparation phase can indicate the NCC value to the UE in protected RRC signaling. Since the UE updates the security key using the NCC value included in conditional reconfiguration in CHO, Rel-19 LTM can perform the security key update procedure by applying same procedure of CHO. Therefore, we prefer that the NCC value is included in RRC signalling during LTM Preparation.

Proposal 1: The NCC value is included in RRC signalling during LTM Preparation.

As the sending NCC value via RRC message during LTM preparation phase, RAN2 proposed 3 options (Option 2, 3a,3b). In Option 2, the UE is preconfigured from the source gNB with a list of NCC values per CU using RRC signalling, and the UE chooses the NCC value from the NCC list corresponding to target CU upon inter-CU LTM. In Option 3a, the UE determines the following NCC value to use by itself (e.g., increase by 1) after subsequent inter-CU LTM execution. In Option 3b, UE is pre-configured by the CN (via source gNB RRC signalling) with a list of NCC values and the UE chooses the first unused NCC value as the next NCC value. In Option 2 and Option 3b, the UE is provided the list of the NCC value, similar to sk-counter in SCPAC. However, while the range of sk-counter is 0 to 65535, the range of NCC has the very small, 0 to 7. In addition, the current specifications require the UE to use the same or larger NCC value compared to the stored NCC value during security updates. Then, by using the list of NCC value, the UE may select smaller NCC value than the stored one. However, the current specification does not support the case of the NCC value decreasing. From these reasons, we think it is inefficient to manage in a list.

In our understanding, the CN increases the NCC value by 1 upon every inter-CU mobility in Xn mobility. From this principle, in inter-CU LTM, the CN may increase the NCC value by 1 regardless target CU upon inter-CU LTM. Therefore, we think Option 3a is the simplest way to apply the same security key between the UE and the target CU upon security update. By Option 3a, the same NCC value can be configured between the UE and the target CU without additional signalling by increasing the NCC value by 1 at the UE upon inter-CU LTM execution.

Proposal 2: The UE increases the NCC value by 1 upon inter-CU LTM.

- Summary and proposal

This paper discussed the procedure for security aspects of inter-CU LTM. In summary, the followings were proposed:

Proposal 1: The NCC value is included in RRC signalling during LTM Preparation.

Proposal 2: The UE increases the NCC value by 1 upon inter-CU LTM.

- References

[1] R2-2409502, “RAN2#127bis Meeting Report,” ETSI MCC.

[2] R2-2407902, “RAN2#127 Meeting Report,” ETSI MCC.

[2] R2-2404102, “RAN2#125bis Meeting Report,” ETSI MCC.