3GPP TSG-RAN WG2 Meeting #128	R2-2410888

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:	8.6.3

Source:	Samsung

Title:	Support of L1 Event Triggered Measurement Report

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

Introduction

During RAN2#126, RNA2#127 and RAN2#127bis meetings, RAN2 reached a lot of agreements for event-triggered L1 measurement enhancements for LTM. See the Annex in section 4.

In this contribution, we will further address event triggered L1 measurement reporting in detail.

Discussion

Issue 1: TTT operation per beam or per cell (TTT reset)

In the last RAN2 meeting, there were two different proposals regarding how TTT is operated based on the L1 measurements. We summarize the two options below.

- Option 1: Based on the beam level L1 measurement, UE report the beam level measurement results i.e. beam(s) trigger the event is reported.

  - Trigger event is configured/evaluated for single beam.

  - TTT is reset if the event condition does not meet e.g. quality of this event triggered beam is not satisfy the execution condition, or current beam change case.

- Option 2: Based on the beam level L1 measurement, UE report the cell level measurement results i.e. report single or multiple beams in the CSI resource set.

- First event is triggered by the single beam in a CSI resource set

- TTT is not reset if there are other beams in the CSI resource set meet the execution condition.

First, we think this event-triggered L1 measurement should be enhanced on top of the Rel-18 LTM measurement report framework. Each LTM CSI resource set could include multiple beams in a candidate cell and if this CSI resource set is associated with the certain CSI report configuration, UE report all measured beams in the CSI resource set. So, concept of the Rel-18 CSI measurement report is per CSI resource set (not per beam).

LTM-CSI-ResourceConfig-r18 ::=      SEQUENCE {

ltm-CSI-ResourceConfigId-r18        LTM-CSI-ResourceConfigId-r18,

ltm-CSI-SSB-ResourceSet-r18         LTM-CSI-SSB-ResourceSet-r18,

...

}

LTM-CSI-SSB-ResourceSet-r18 ::=     SEQUENCE {

ltm-CSI-SSB-ResourceList-r18        SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF SSB-Index,

ltm-CandidateIdList-r18             SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF LTM-CandidateId-r18,

...

}

Therefore, we think it is natural to extend this logic to the Rel-19 event triggered measurement report because RAN2 already agreed that the Rel-18 measurement resource/report configuration is reused for the L1 event triggered measurement report. It means UE consider all CSI resources within a CSI resource set if the Event LTM X refers some CSI resource configuration (i.e. CSI resource set). For the TTT reset, we think Option 2 should be the correct way to keep the structure of Rel-18 LTM CSI measurement report.

Proposal 1: Based on the beam level L1 measurement, UE reports single or multiple beams in the CSI resource set.

Proposal 2: After LTM event is triggered by the single beam in a CSI resource set, TTT is not reset if there are other beams in the CSI resource set meet the execution condition.

Another issue for TTT operation is the case if the current beam is changed during the event evaluation for the beams in the CSI resource set. In this case, we think it is reasonable to reset TTT and evaluate the event based on the new current beam.

Proposal 3: If the current beam is changed by NW during the event evaluation for the beams in the CSI resource set, UE reset the TTT and evaluate the event based on the new current beam.

Issue 2: Details of MR MAC CE

If the LTM event is evaluated completely by UE i.e. execution condition is met and TTT is satisfied, the UE should make the event triggered measurement report MAC CE. Based on the last RAN2 meeting, the beam information, beam quantity and the triggered event information (e.g. report ID) are determined. One remaining issue is that how many beams are reported in a MR MAC CE.

From our understanding, the basic concept is that UE just follow the CSI resource/report configuration which is configured by NW. As we explained in above, if NW configures the CSI resource set including multiple beams and this CSI resource configuration is associated with the CSI report configuration which is for the event triggered MR, then UE report all configured beams in the CSI resource set after TTT is satisfied if we follows the above TTT operation concept. It is also possible UE just include the best beam which TTT is satisfied in the end but it is good to know all beams in the CSI resource set from NW perspective. If the multiple beams are reported in a MR MAC CE, it is also good to indicate the beam quality order to the NW, the easiest way is that UE include the multiple beams in order of beam quality.

One additional enhancement is that NW configures the number of reported beams in a MR MAC CE as we have in Rel-18 LTM measurement report (i.e. nrOfReportedRS-PerCell-r18). For example, the CSI resource set #1 includes 3 beams and this CSI resource set is associated with the CSI report configuration #2. In this CSI report configuration, NW configures the N (the number of reported beams) as 2, then UE should report the best two beams among 3 candidate beams in the MR MAC CE.

Proposal 4: If multiple beams are configured in the CSI resource set which is associated with the triggered CSI report configuration for LTM event, UE report all beams in the CSI resource set in the event triggered measurement report MAC CE.

Proposal 5: The number of Reported RSs per cell (same as nrOfReportedRS-PerCell-r18 in LTM-ReportContent-r18) is configured by NW for the L1 event triggered measurement report.

Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: Based on the beam level L1 measurement, UE reports single or multiple beams in the CSI resource set.

Proposal 2: After LTM event is triggered by the single beam in a CSI resource set, TTT is not reset if there are other beams in the CSI resource set meet the execution condition.

Proposal 3: If the current beam is changed by NW during the event evaluation for the beams in the CSI resource set, UE reset the TTT and evaluate the event based on the new current beam.

Proposal 4: If multiple beams are configured in the CSI resource set which is associated with the triggered CSI report configuration for LTM event, UE report all beams in the CSI resource set in the event triggered measurement report MAC CE.

Proposal 5: The number of Reported RSs per cell (same as nrOfReportedRS-PerCell-r18 in LTM-ReportContent-r18) is configured by NW for the L1 event triggered measurement report.

Annex: RAN2 agreements on L1 event triggered MR

RAN2#126 Agreements on measurement enhancements for LTM:

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

RAN2#127 Agreements on L1 MR event evaluation

- Event LTM1 is not defined.

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

- Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

- R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

6. MAC layer handles the event evaluation and measurement report triggering.

RAN2#127 Agreements on L1 measurement reporting

- Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

Agreements#127bis on L1 event triggered MR

- MR can be sent when the leaving condition is met, based on NW configuration.

- Event triggered periodic MR can be supported, based on NW configuration.

- For measurement resource configuration, R18 LTM CSI resource configuration is reused if possible. If CSI-RS resource only IE needs to be defined, we can revisit it in the stage 3.

- For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused if possible. We can revisit it in the stage 3 if needed.

- For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible. We can revisit it in the stage 3 if needed.

- The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1.

- TTT operates only based on a timer (like TTT used in L3 event triggered MR).

- Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5).

- Basic information included in MR MAC CE:

- Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id)

- Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams

- Triggered event information (e.g., ReportConfigID)

MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not).

N is configurable by NW.

- Additional information included in MR MAC CE:

- The information and quantity of current beam, based on NW configuration.

11.  The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE.

12.	NW can configure a dedicated SR configuration for MR MAC CE transmission.