3GPP TSG-RAN WG2 Meeting #128                                                                   R2-2409541

Orlando, USA, November 18th – 22nd, 2024

Agenda item:	8.9.3

Source:	vivo

Title:	Discussion on CB-Msg3 Mechanism

Document for:	Discussion and Decision

1 Introduction

In the last RAN2 meetings, RAN2 had discussed several general aspects on the CB-Msg3 mechanism and had achieved the following agreements [1]:

RAN2#128 Agreements: - RAN2 will introduce support for DSA (i.e. the possibility to transmit more than one replica of CB-msg3, if configured by the NW). RAN2 does not intend to work on CRDSA in Rel-19. - RAN2 will continue their work on CB-msg3 assuming that OCC2 might also apply to CB-msg3 transmission (“CB-NPUSCH”) (final decision whether this is feasible is up to RAN1). - At least system Information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. FFS on signalling details. - CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level (FFS whether we have a CE level specific configuration for DSA). - RAN2 assumes that CB-msg3 EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission, but this has to be confirmed by RAN1. FFS if there is an RSRP threshold that determines whether CB-msg3 EDT cannot be used (the UE will have to use 4-step RA). - At least in the cases confirmed by RAN1/RAN4, a running TAT is not needed to initiate a CB-msg3 EDT transmission. - The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA). - CB-msg3 EDT procedures and any Msg4 enhancement are only introduced for IoT NTN.

In this contribution, we will further discuss the CB-Msg3 mechanism used for IoT-NTN.

2 Discussion

Figure 1: CB-Msg3

Based on our understanding of the agreements, an exemplary overall procedure for CB-Msg3 mechanism is shown in Figure 1 above. Specifically, the network will provide CB-Msg3 PUSCH resources via broadcast singalling (e.g. SIB2) for several UEs that may have a small amount of burst data. For the UEs that trigger CB-Msg3, they will select a CB-Msg3 PUSCH occasion for CB-Msg3 PUSCH transmission (including the RRC message and UL UP data) via SA/DSA, potentially combined with OCC2. Different UEs may select the same occasion (thus leading to collision) or different occasions. At the network side, once the MAC PDU transmitted in a CB-Msg3 PUSCH occasion is successfully decoded, the network can respond the UE with contention resolution identification and a corresponding RRC message, potentially multiplexing the DL data together within the same MAC PDU.

In the following, we will give our analysis of the RAN1/RAN2 spec impacts for supporting CB-Msg3.

  - 2.1 CB-Msg3 configuration

Firstly, we think it is sufficient for the network to only use SIB to deliver the cell-specific CB-Msg3 PUSCH configuration. Some may argue that dedicated configuration should be considered as well so that the network can have control over the total number of UEs that can use the CB-Msg3 PUSCH resources in the cell. We agree with the intention but such kind of control is not efficient at all. Now as the RSRP threshold per CE level is agreed for CB-Msg3 PUSCH resource, the NW can dynamically adjust the RSRP threshold to control the CB-Msg3 PUSCH resource load. For example, once the load is higher than expected, the network can turn up the value of the RSRP threshold. Once the load goes down subsequently, the network can reset the value of that RSRP threshold to the default value.

Proposal 1: Only system Information is used to provide cell-specific CB-Msg3 PUSCH resources.

Further, the eNB should configure the CB-Msg3 PUSCH in a periodic manner (similar way as NPRACH/DPUR) so that there soon will be a next available CB-Msg3 PUSCH for the UE with unpredictable UL bursty data. Another option is that the CB-Msg3 PUSCH resources are only valid for one shot (similar to one-shot DPUR) or for N shots. As the CB Msg3 PUSCH is shared amongst multiple UEs, resource waste might be not a big issue. So we may not consider one-shot or N-shot configuration for Rel-19 CB-Msg3 PUSCH resources. Aside from time domain resource allocation, configuration in other aspects, such as frequency and reference signal for PUSCH, should be considered as well. In our understanding, from PHY transmission point of view, the point-to-point uplink transmission condition and DL response reception are similar to the DPUR UL/DL transmission. Thus we assume it is simplest to reuse DPUR allocation configuration for CB-Msg3 configuration as a baseline. Meanwhile, for the coverage enhancement aspect, we think how to associate the maximum supported TBS with the RSRP threshold per CE level is totally up to NW implementation. And for simplexity, RAN2 should assume the existing TBS framework from DPUR is reused for CB-Msg3. That is it is up to network implementation to configure a suitable TBS for a CB-Msg3 resource linked with a CE level configuration. The configurable parameters should be inherited from the existing parameters designed for DPUR (e.g. possible TBS size) or EDT (e.g. RSRP threshold per CE level). Anyway, RAN2 should check the assumption with RAN1.

Proposal 2: Taking the UL/DL resources related configuration fields in PUR-Config-r16 as a baseline reference for CB-Msg3resources configuration.

Proposal 3: Send an LS to RAN1 to check RAN2’s assumption that the existing TBS size for PUR and CE level/RSRP threshold configuration applicable to EDT can be reused for CB-Msg3.

  - 2.2 CB-Msg3 triggering:

Regardless of the detailed configuration method outcome, from the UE point of view, a UE aware of CB-Msg3 PUSCH configuration anyway has to determine whether it is suitable to use that configuration. Considering the WI scope is still related to EDT, we think the data volume initiating condition can be reused for the CB-Msg3 transmission procedure. Specifically, at the RRC level, the UE shall check whether the size of pending UL data is smaller than TBS configured for the CB-Msg3 resource per selected CE level. By the way, we think the existing CE level selection method can be reused for CB-Msg3 as quoted below.

TS 36.321 sub-clause 5.1.1 - if the RSRP threshold of enhanced coverage level 3 is configured by upper layers in rsrp-ThresholdsPrachInfoList and the measured RSRP is less than the RSRP threshold of enhanced coverage level 3 and the UE is capable of enhanced coverage level 3 then: - the MAC entity considers to be in enhanced coverage level 3; - else if the RSRP threshold of enhanced coverage level 2 configured by upper layers in rsrp-ThresholdsPrachInfoList and the measured RSRP is less than the RSRP threshold of enhanced coverage level 2 and the UE is capable of enhanced coverage level 2 then: - the MAC entity considers to be in enhanced coverage level 2; - else if the measured RSRP is less than the RSRP threshold of enhanced coverage level 1 as configured by upper layers in rsrp-ThresholdsPrachInfoList then: - the MAC entity considers to be in enhanced coverage level 1; - else: - the MAC entity considers to be in enhanced coverage level 0;

Further, there is an FFS on whether there is a need to have a separate RSRP threshold is needed to determine whether CB-Msg3 can be used. We don’t think this threshold is needed. As long as the CE level RSRP configuration is determined, then the UE anyway can determine the proper CE level. If there are associated CB-Msg3 resources and the pending UL data is smaller than the corresponding configured TBS, then the UE can trigger CB-Msg3 and then use the CB-Msg3 resource. If there is no available resource for the selected CE level, with doubt, the UE should trigger the EDT or RA procedure. In other words, the resources corresponding to the other CE levels cannot be used by this UE at this time, considering the resource configuration does not match the UE radio link condition.

Based on the above, we propose,

Proposal 4: Reuse the existing CE level selection procedure for CB-Msg3.

Proposal 5: The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured TBS for the selected CE level.

Proposal 6: Except for the RSRP thresholds corresponding to CE levels, do not introduce an RSRP threshold that determines whether CB-Msg3 can be used.

Proposal 7: It is up to network implementation to associate CB-Msg3 resources with a specific CE level.

Proposal 8: If there are no CB-Msg3 resources available for the selected CE level, the UE cannot use CB-Msg3 regardless of whether there are other CB-Msg3 resources available other CE levels.

  - 2.3 DSA modeling:

In the previous RAN2 meeting, Diversity Slotted ALOHA (DSA) was agreed for CB-Msg3. Since the concept of replicas is totally new for 3GPP, we would like to investigate how to model DSA transmission.

The first issue that comes to us is which value should be the maximum number of replica transmission. Based on the performance results from Figure 4 in [3] and DSA solution is generally used for capacity improvement for cases where the PLR is lower than 10% (i.e. the throughput gain between DSA2 and DSA3 is small), then we think DSA2 can be considered as the maximum. Further, regarding the DSA enabling, as CB-Msg3 resources of different CE levels can be shared by different amounts of UEs (e.g. the total number of UEs in CE3 (e.g. 5% cell edge UEs) might be much less than the number of UEs in CE1), it seems a spontaneous logic that we should guarantee the network flexibility to control whether the CB-Msg3 resources of a CE level should be operated with DSA or not.

Proposal 9: DSA with at most 2 replicas is considered in Rel-19 IoT-NTN.

Proposal 10: DSA can be enabled on a per CE level basis.

DSA allows UE to transmit multiple replicas within a specific time duration (e.g. the superframe used in CRDSA of DVB-RCS2 system [2]). So from the UE point of view, the time duration boundary is necessary for UE to randomly select multiple different CB-Msg3 PUSCH occasions, as shown in Figure 2. With the window defined, the UE can randomly select N DSA occasion for CB-Msg3 transmission.

Figure 2: DSA occasion selection

Proposal 11: RAN2 to define a configurable or pre-defined time window for DSA CB-Msg3 occasion selection.

After CB-Msg3 occasion selection, the UE should do UL-SCH data transfer and the actual transport block transmission via the physical layer. The main preparation steps for the PUSCH transmission are given as follows:

- The MAC entity receives the UL grants;

- The MAC entity delivers the UL grants and the associated HARQ information to the HARQ entity;

- The HARQ entity instructs the multiplexing and assembly entity to generate a MAC PDU;

- The HARQ entity obtains the MAC PDU to transmit from the multiplexing and assembly entity;

- The HARQ entity triggers the corresponding HARQ process to perform a new transmission/retransmission;

- The corresponding HARQ process	instructs the PHY layer to generate a transmission;

- The PHY layer transmits the PUSCH for the obtained MAC PDU with PHY procedure (e.g. power control).

To minimize the potential RAN1 impacts with the introduction, we think each replica should be modeled as a new transmission. On one hand, since the network side cannot perform soft combining for DSA replicas due to the lack of information on which occasions the same UE selects, there is no need to model it as a retransmission. On the other hand, if we treat the replica as a retransmission, we have to consult with RAN1 whether the same transmit power can be reused for this retransmission and what RV index should be considered for it, and so on.

Further, by treating the replica transmission as a new transmission, then the MAC should generate a MAC PDU for each replica transmission. As the content in each replica should be the same, a possible and simple modeling is to obtain the MAC PDU from the HARQ buffer for the 2nd replica. At the physical layer, it can treat each replica transmission as the new transmission as the legacy case (e.g. independently doing the power control, resource mapping, etc), as depicted in Figure 3 below. Moreover, we think that HARQ process id 0 and RV0 can be reused, similar to Msg3 transmission.

Figure 3: DSA replica transmission

Proposal 12: MAC considers each replica transmission as a new transmission.

Proposal 13: HARQ entity obtains the MAC PDU from the HARQ process buffer for the secondary replica.

Proposal 14: HARQ process id 0 and RV 0 are used for CB-Msg3.

  - 2.4 Response reception:

After the Cb-Msg3 transmission, the UE needs to start a PDCCH monitoring window for response reception. Considering that only 1 transmission layer is enabled for IoT-NTN without OCC, as a result, the network can only at most successfully decode 1 MAC PDU within a CB-Msg3 occasion. Moreover, as it was agreed that RNTI for Msg4 is calculated in per occasion granularity, it means the Msg4 can at most include one contention resolution id and RRC-related response for a specific UE. So, the existing MAC PDU format can be reused. And we think the Msg4 monitoring behavior can be reused. The UE montiors a common-RNTI PDSCH, waiting for the matched contention resolution, which can multiplexed with RRC RRCEarlyDataComplete or RRCConnectionRelease, and potentially with DL UP data. The monitoring behavior should be the same as the contention resolution window when the SA access mode is used. For DSA case, for simplicity, one response window is preferable (i.e. the window can be started after the end of the DSA time window, and in return, the network should only deliver the Msg4 after that window). If multiple parallel response window is supported, we may have to discuss early termination and the UE behavior when the windows are overlapping in the time domain.

Proposal 15: For SA case, after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account.

Proposal 16: For DSA case, only one PDCCH monitoring window is started for response reception.

  - 2.5 Backoff:

As analyzed in our paper RR2-2408018, the PLA performance gain difference between SA and SA-BI is quite obvious. Thus the BI-reattempt (like PRACH) solution should be adopted for collision randomization. Regardless of which access mode (e.g. DSA, SA, OCC2) is adopted, a common backoff mechanism should be supported.

Proposal 17: Support re-attempt after backoff time (like PRACH) for CB-Msg3.

3 Conclusion

In this contribution, we have provided our understanding of CB-Msg3 procedure. And we made the following proposals:

CB-Msg3 configuration:

Proposal 1: Only system Information is used to provide cell-specific CB-Msg3 PUSCH resources.

Proposal 2: Taking the UL/DL resources related configuration fields in PUR-Config-r16 as a baseline reference for CB-Msg3resources configuration.

Proposal 3: Send an LS to RAN1 to check RAN2’s assumption that the existing TBS size for PUR and CE level/RSRP threshold configuration applicable to EDT can be reused for CB-Msg3.

CB-Msg3 triggering:

Proposal 4: Reuse the existing CE level selection procedure for CB-Msg3.

Proposal 5: The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured TBS for the selected CE level.

Proposal 6: Except for the RSRP thresholds corresponding to CE levels, do not introduce an RSRP threshold that determines whether CB-Msg3 can be used.

Proposal 7: It is up to network implementation to associate CB-Msg3 resources with a specific CE level.

Proposal 8: If there are no CB-Msg3 resources available for the selected CE level, the UE cannot use CB-Msg3 regardless of whether there are other CB-Msg3 resources available other CE levels.

DSA modelling:

Proposal 9: DSA with at most 2 replicas is considered in Rel-19 IoT-NTN.

Proposal 10: DSA can be enabled on a per CE level basis.

Proposal 11: RAN2 to define a configurable or pre-defined time window for DSA CB-Msg3 occasion selection.

Proposal 12: MAC considers each replica transmission as a new transmission.

Proposal 13: HARQ entity obtains the MAC PDU from the HARQ process buffer for the secondary replica.

Proposal 14: HARQ process id 0 and RV 0 are used for CB-Msg3.

Response reception:

Proposal 15: For SA case, after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account.

Proposal 16: For DSA case, only one PDCCH monitoring window is started for response reception.

Response reception:

Proposal 17: Support re-attempt after backoff time (like PRACH) for CB-Msg3.

4 Reference

[1] Draft_RAN2_127b_Meeting_Report_v3.

[2] DVB document A 155-2 Rev.5, Second generation DVB interactive satellite system; Part 2: lower layers for satellite standard.

[3] R2-2408109, Discussion on the mechanisms for UL capacity enhancement based on simulation results.