3GPP TSG-RAN WG2 Meeting #128	                                                                                  R2-2409594

Orlando, USA, Nov.  18th – 22nd , 2024

Source:                 CATT

Title:	                L1 event triggered measurement reporting

Agenda Item:	8.6.3

Document for:	Discussion and Decision

Introduction

In this paper, we further explore the open issues regarding L1 event-triggered measurement reporting.

The organization of this contribution is as follows. In Section 2, we discuss the following aspects.

- Node to generate the measurement report configuration

- TTT operation

- Measurement RS type alignment

- Measurement report MAC CE

- CSI-RS measurements

Our proposals are summarized in Section 3.

Discussion

Node to generate the measurement report configuration

RAN2 has reached an agreement that for measurement reporting configuration, the R18 LTM-CSI-ReportConfig will be reused. In R18 LTM, the LTM-CSI-ReportConfig is located within the ServingCellConfig of each candidate configuration and is provided by each candidate. Therefore, it is easy to clarify that the same principle applies to measurement reporting configuration. That is, the Source DU determines the initial measurement report configuration of the candidates, and each candidate DU determines the measurement report configuration for other candidates, which will be used when the candidate becomes the new serving cell.

Therefore, it is proposed to clarify that:

Proposal 1a: The Source DU determines the initial measurement report configuration of the candidates.

Proposal 1b: Each candidate DU determines the measurement report configuration for other candidates, which will be used when the candidate becomes the new serving cell.

TTT operation

Granularity of TTT operation for a candidate cell

The granularity of TTT (Time to Trigger) operation for a candidate cell was discussed in the last meeting. However, no conclusion was reached regarding the choice among the options (namely, per beam, per cell, or per CSI-RS resource set).

RAN2 has reached an agreement that,

- Any beam in candidate RS configuration can be used for LTM event evaluation.

According to this agreement, any new beam that fulfills its associated event during TTT should be reported to NW. Thus, it should be sufficient for the TTT operation of a candidate cell to be on a per-beam basis. Therefore, it is proposed to clarify that:

Proposal 2: TTT is operated on a per beam basis.

TTT timer handling when current beam changes

It is possible that the serving cell's current beam may change due to the newly indicated TCI state during event evaluation. It is essential to clarify the UE's behavior in such a case, specifically whether the UE should reset the TTT timer.

If the TTT timer is reset when the serving cell's current beam changes, it will increase the difficulty of fulfilling the event. As a result, the network may not be able to obtain the L1 measurement result in a timely manner, and thus may not be able to trigger the LTM cell switch when necessary. Moreover, the intention of event triggering is to send the measurement results to the network when the beam quality of the serving cell deteriorates and/or the beam quality of the candidate cell is better than that of the serving cell. It does not matter which beam is the current beam of the serving cell. Therefore, there is no need to reset the TTT timer when the serving cell's current beam changes. In any case, the consequence of continuing the evaluation upon a change in the best beam is that more/earlier L1 measurement results may be triggered. This does not have a negative impact on the network and could provide more information to the network. Hence, we suggest continuing the TTT when the serving cell's current beam changes.

Therefore, it is proposed that:

Proposal 3: For events LTM2/LTM3/LTM5, the TTT timer is not reset when the serving cell's current beam changes.

Value range of TTT

In L3 measurement,the value range of the TTT is defined as follows,

-- ASN1START -- TAG-TIMETOTRIGGER-START TimeToTrigger ::= ENUMERATED { ms0, ms40, ms64, ms80, ms100, ms128, ms160, ms256, ms320, ms480, ms512, ms640, ms1024, ms1280, ms2560, ms5120}

Regarding the value range of TTT in L1 event-triggered measurement reporting, reusing these time values should be sufficient. Therefore, it is proposed to confirm the following:

Proposal 4: The TTT value range defined in L3 measurement is reused.

Measurement RS type alignment

We have agreed to support the beam configuration of both SSB and CSI-RS in the L1 measurement resource configuration. However, it remains unclear whether both SSB and CSI-RS are always configured for a long-term monitoring (LTM) CSI resource or if one of them can be configured independently. Consequently, it is also not clear how to determine the RS type used for event evaluation.

In L3 measurement, either SSB or CSI-RS, or both can be included in a measurement object.

ReferenceSignalConfig::=            SEQUENCE {

ssb-ConfigMobility                  SSB-ConfigMobility                                              OPTIONAL,   -- Need M

csi-rs-ResourceConfigMobility       SetupRelease { CSI-RS-ResourceConfigMobility }                  OPTIONAL    -- Need M

}

It is straightforward to follow the same principle for LTM event triggered L1 measurement,

Proposal 5a: In one L1 measurement resource configuration, either SSB, CSI-RS, or both SSB and CSI-RS can be included.

RAN2 agreed that current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation,

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

Since the RS type of a new indicated TCI state for the serving cell can be changed dynamically, the RS types of the serving beam and candidate beam may become misaligned at certain moments. That is, the RS type of the serving beam is different from the RS type indicated in the corresponding event-triggered report configuration. The Measurement RS type alignment issue were discussed last meeting and the following options was still on the table,

- Option 1: NW configures the both types of RS (SSB and CSI-RS) for the new beam(s), UE could select the same type of beam when UE evaluate the L1 event measurement. - Option 2: Only one type of RS is configured for the new beam as a NW restriction, and UE perform the L1 measurement when RS type of the current beam is same with the new beam for event L1 measurement. - Option 3: NW provides the offset which can be applied one type of RS to evaluate the different type of RS. For example, If the current beam is SSB and the measured beam for the candidate cell is CSI-RS, the UE applies the configured the offset to the measured CSI-RS value and then compare it with the current beam quality i.e. SSB measurement. - Option 4: Rely on network configuration, i.e. same RS type is configured. - Option 5 (new option proposed by CATT): Which type to be used in measurement will be configured by NW. Then the UE only consider the configured RS type in the measurement. - Option6 (copied from P1, ZTE): The RS for current beam of serving cell is same as or associated with QCL RS provided in the indicated TCI-state, i.e. using the same type of RS as the candidate beam.

In our opinion, if the NW can configure the RS type, similar to L3 measurement, it will be clearer. Then, to ensure that the same RS type is used for both the serving beam and candidate beams in event LTM3 and event LTM5, it is necessary to clarify that the UE performs event evaluation only when the RS type of the current beam (i.e., a beam corresponding to the indicated TCI state) is identical to the RS type indicated in the corresponding event-triggered report configuration. Otherwise, UE needs to stop the event evaluation.

Proposal 5b: For Measurement RS type alignment, Option 5 is adopted (i.e., the network will configure which type to be used in measurement. Then the UE only considers the configured RS type in the measurement).

Proposal 5c: For event LTM3 or LTM5, when the RS type of the current beam (i.e., a beam corresponding to the indicated TCI state) becomes different from the configured RS type (e.g., due to a change in the indicated TCI state), the UE stops the event evaluation for the new beam.

Measurement report MAC CE

There are still some FFS on the MR MAC CE as follows,

- Basic information included in MR MAC CE:

- Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id)

- Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams

- Triggered event information (e.g., ReportConfigID)

MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not).

N is configurable by NW.

Beam information

Regarding beam information in the MR MAC CE, it is still to be determined which format (SSBRI/CRI or LTM configuration id + SSB/CSI-RS id) is used. In our view, SSBRI/CRI along with ReportConfigID (which is included in the MR MAC CE as per the agreement) would be sufficient to identify a reported beam. Since SSBRI/CRI requires far fewer bits than LTM configuration id plus SSB/CSI-RS id, it is preferable to use SSBRI/CRI.

Proposal 6: SSBRI/CRI is used to identify a beam in the measurement report MAC CE.

Whether the reported beam should satisfy the event

We have reached an agreement that up to N beams can be included in the MR MAC CE. It is FFS whether the beam should satisfy the event or not. NW can configure an appropriate event to select cell/beam for different purposes (i.e., Select the candidate beam/cell to trigger early synchronization, Select the target beam/cell and trigger LTM cell switch procedure). Thus, only the beams that satisfy the event are meaningful to network.

Therefore, in our understanding any beam included in the measurement report MAC CE should satisfy the associated event.

Proposal 7a: The beams included in the measurement report MAC CE should have satisfied the associated event.

In L3 measurement, the cells that have triggered the event condition at any time are retained in the cellsTriggeredList for a specific measure ID. The measurement results of the cells in the cellsTriggeredList can be included in one measurement report in accordance with the limitation on the maximum number of non-serving cells. The same mechanism can be employed for LTM event triggering measurement reporting. More specifically, for beams associated with the same  ltm-CSI-ReportConfigId, the UE maintains a list of all the beams that have satisfied the event condition. When the measurement reporting is triggered, some or all of them are incorporated into the measurement report based on the limitation on the maximum number of candidate beams configured by the gNB.

Therefore, it is proposed that:

Proposal 7b: In addition to the beam that triggers the measurement reporting, other candidate beams associated with the same ltm-CSI-ReportConfigId that have previously satisfied the event can be included in the MAC CE.

CSI-RS measurements

RAN1 has assumed that semi-persistent CSI-RS is supported as follows,

Working Assumption In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective - Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload). Support of semi-persistent CSI-RS is subject to UE capability. Final LS in R1-2409283.

Although in the LS RAN1 only requests RAN3 to examine the feasibility and potential RAN3 workload, there are also impacts on the RAN2 specifications. For example, a MAC CE is required for the activation/deactivation of the semi-persistent CSI-RS.

Observation 1: To support the semi-persistent CSI-RS of a candidate cell, there are also RAN2 impacts. For instance, a MAC CE is required for the activation/deactivation of the semi-persistent CSI-RS.

Proposal 8: Discuss the RAN2 spec impacts (e.g., the MAC CE for activating/deactivating the semi-persistent CSI-RS) of supporting semi-persistent CSI-RS.

Conclusion

According to the previous analysis in section 2, our observations and proposals are summarized as follows:

Observation 1: To support the semi-persistent CSI-RS of a candidate cell, there are also RAN2 impacts. For instance, a MAC CE is required for the activation/deactivation of the semi-persistent CSI-RS.

Node to generate the measurement report configuration

Proposal 1a: The Source DU determines the initial measurement report configuration of the candidates.

Proposal 1b: Each candidate DU determines the measurement report configuration for other candidates, which will be used when the candidate becomes the new serving cell.

TTT operation

Proposal 2: TTT is operated on a per beam basis.

Proposal 3: For events LTM2/LTM3/LTM5, the TTT timer is not reset when the serving cell's current beam changes.

Proposal 4: The TTT value range defined in L3 measurement is reused.

Measurement RS type alignment

Proposal 5a: In one L1 measurement resource configuration, either SSB, CSI-RS, or both SSB and CSI-RS can be included.

Proposal 5b: For Measurement RS type alignment, Option 5 is adopted (i.e., the network will configure which type to be used in measurement. Then the UE only considers the configured RS type in the measurement).

Proposal 5c: For event LTM3 or LTM5, when the RS type of the current beam (i.e., a beam corresponding to the indicated TCI state) becomes different from the configured RS type (e.g., due to a change in the indicated TCI state), the UE stops the event evaluation for the new beam.

Measurement report MAC CE

Proposal 6: SSBRI/CRI is used to identify a beam in the measurement report MAC CE.

Proposal 7a: The beams included in the measurement report MAC CE should have satisfied the associated event.

Proposal 7b: In addition to the beam that triggers the measurement reporting, other candidate beams associated with the same ltm-CSI-ReportConfigId that have previously satisfied the event can be included in the MAC CE.

CSI-RS measurements

Proposal 8: Discuss the RAN2 spec impacts (e.g., the MAC CE for activating/deactivating the semi-persistent CSI-RS) of supporting semi-persistent CSI-RS.

Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4

- Draft_RAN2_127b_Meeting_Report,https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_127bis/Report