3GPP TSG-RAN WG2 Meeting #128                                                                   R2-2409598

Orlando, USA, 18th – 22nd Nov, 2024

Source:	CATT

Title:	Adaptation of Common signal channel transmissions

Agenda Item:	8.5.4

Document for:	Discussion and Decision

Introduction

In RAN2#127bis meeting, paging adaptation was discussed and the following agreements were achieved:

Agreements on paging adaptation

- Select option-b as baseline for R19 NES paging enhancement.

- R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values)

- Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration.

- Legacy UE is not barred.

- Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.

In this contribution, remaining issues on paging adaptation are discussed. In addition, considerations on PRACH adaptation and SSB adaptation are provided.

Discussion

Adaptation of Paging

- Extended value for paging adaptation

In RAN2#127bis, option b, i.e. extending the values of N and Ns, was selected as baseline for R19 NES paging enhancement. And during the discussion on paging adaptation, companies have concern on the signaling overhead of option b due to the extension of PMO (i.e. firstPDCCH-MonitoringOccasionOfPO) configuration.

As shown below, the signaling overhead for PMO configuration = maximum value of Ns*bits for each PMO. With legacy paging configurations, the maximum value of Ns is 4 and the maximum number of bits for PMO is 17, where the maximum offset between the first symbol of PF and PMO is 16 radio frames. The maximum signaling overhead for PMO configuration is 68(4*17).

[[ firstPDCCH-MonitoringOccasionOfPO CHOICE { sCS15KHZoneT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139), sCS30KHZoneT-SCS15KHZhalfT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279), sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559), sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119), sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239), sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479), sCS120KHZoneEighthT-SCS60KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959), sCS120KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919) } OPTIONAL -- Cond OtherBWP ]], //skip firstPDCCH-MonitoringOccasionOfPO-v1710 CHOICE { sCS480KHZoneEighthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..35839), sCS480KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..71679) } OPTIONAL, -- Need R

With option b, N and Ns are extended. If the value of N is extended, e.g. T/32, T/64 and so on, the maximum signaling overhead for each PMO configuration is increased as well as it is the possible location of PMO within a paging cycle. For example: If N is extended to T/32, then the maximum signaling overhead for each PMO configuration is increased to 18bits.

When the value of N is extended, if R19 paging adaptation mechanism is expected to provide the similar capacity of PO as legacy paging mechanism, the maximum number of Ns needs to be extended correspondingly. For example: If N is extended to 64/T, the maximum number of Ns needs to be extended to 16 to achieve the same PO capacity as legacy paging mechanism. If the value of Ns is extended as well, e.g. 8, 16, 32 and so on, the total signaling overhead for total PMO configuration is increased rapidly as the total signaling overhead for PMO configuration = maximum value of Ns*bits for each PMO. For example: If N is extended to T/64 while Ns is extended to 16, the signaling overhead for PMO configuration is increased to 304(16*19).

Observation 1: The total signaling overhead for PMO configuration = maximum value of Ns*bits for each PMO. If the value of N is extended, the maximum signaling overhead for each PMO configuration is increased. If the value of Ns is extended, the total signaling overhead for total PMO configuration is increased rapidly.

Table 1: signaling overhead for PMO with possible N and Ns

| Extended N | Extended Ns | Signaling overhead for PMO configuration(bits) |
| --- | --- | --- |
| T/32 | 8 | 144 |
| T/64 | 8 | 152 |
| T/64 | 16 | 304 |
| T/128 | 8 | 160 |
| T/128 | 16 | 320 |
| T/128 | 32 | 640 |
| T/256 | 8 | 168 |
| T/256 | 16 | 336 |
| T/256 | 32 | 672 |
| T/256 | 64 | 1344 |
| T/512 | 8 | 176 |
| T/512 | 16 | 352 |
| T/512 | 32 | 704 |
| T/512 | 64 | 1408 |
| T/512 | 128 | 2816 |

The signaling overhead for PMO with possible N and Ns are summarized in Table 1. The maximum SIB1 or SI message size is 2976 bits. Considering paging adaptation is target for low loading scenario, we think it is enough to extend Ns to 8 and 16. If the loading is high, NW can change to legacy paging mechanism.

And if the maximum number of Ns is extended to 16, N can be extended to T/32, T/64, T/128, T/256 and T/512. But if N is extended to T/64, T/128, T/256 and T/512, N may be less than 1 when DRX paging cycle in UE is smaller. For example, T is 64 radio frames while N is T/64 is broadcasting system information. The T of the UE is 32 radio frames as T in the UE is determined by the shortest of the UE specific DRX value configured by RRC (if any), the UE specific DRX value configured by upper layers (if any), and a default DRX value broadcast in system information. Then N is 1/2 for the UE according to current calculation, which means there may be no PF in T. And it was agreed the UE will monitor one PEI/PO every paging DRX cycle in RAN2#125 meeting. Hence, the UE cannot use the N directly in this case.

Observation 2: In case N is extended to T/64, T/128, T/256 and T/512, N may be less than 1. This is an issue that needs to be addressed.

If N is extended to T/64, T/128, T/256 and T/512, RAN2 need to discuss how to handle N<1. In this case, the UE/NG-RAN node can:

- Option 1: Switch to legacy paging mechanism.

- Option 2: Set N = 1.

RAN2 discuss which option is adopted if N value is extended to T/64, T/128, T/256 and T/512.

Proposal 1a: The value of Ns is extended to 8 and 16 but not to any value greater than these.

Proposal 1b: The value of N is extended to at least T/32.

Proposal 1c: To address N<1 issue in case N is extended to T/64, T/128, T/256 and T/512, down selects between the following options:

- Option 1: Switch to legacy paging mechanism.

- Option 2: Set N = 1.

- Mechanism to activate/deactivate the paging adaptation resources

Assume there are legacy paging mechanism and Rel-19 paging mechanism (e.g. option b)) in a cell. There are two possible solutions for to activate/deactivate the paging adaptation resources

- Solution 1:  Via system information update.

With the solution, the paging adaptation parameters are provided in system information if NW wants to activate the paging adaptation. The Rel-19 NES UE applies Rel-19 paging mechanism immediately upon receiving corresponding parameters in system information. The paging adaptation parameters are removed from system information if NW wants to deactivate the paging adaptation. The UE is notified of this via the legacy system information modification mechanism. And when paging adaptation parameters are absent in system information, the Rel-19 NES UE applies legacy paging mechanism with received legacy paging parameters.

- Solution 2: Via Short Message in paging DCI

With the solution, parameters for Rel-19 paging mechanism (e.g. option b)) are broadcasted in SI but not activated immediately. The Rel-19 NES UE applies legacy paging mechanism with received legacy paging parameters if Rel-19 paging mechanism is not activated. If the network wants to activate Rel-19 paging mechanism, Short Messages with paging adaptation indication is sent in one modification period. Then the UE can apply Rel-19 paging mechanism in the next modification period without acquiring system information for parameters of Rel-19 paging mechanism.

Both solution 1 and solution 2 are adapted Rel-19 paging mechanism in a semi-static way. But solution 2 needs to use the reserved bits in Short message although it may reduce the UE power saving without acquiring system information for parameters of Rel-19 paging mechanism. There are only 4 bits not used in Short message. Considering the limited reserved bits in Short message and the limit benefit of solution 2, solution 1 is preferred.

Proposal 2: The legacy System information update procedure is used to activate/deactivate paging adaptation mechanism.

Proposal 3: R19 NES UE follows legacy paging mechanism if paging adaptation configuration is absent in the R19 NES cell.

- PEI with paging adaptation

In order to reduce UE power consumption due to paging reception, PEI is introduced in R17. In PEI, the number of PO(s) associated with one PEI monitoring occasion, i.e. po-NumPerPEI, is related Ns, as shown below.

PEI-Config-r17 ::= SEQUENCE { po-NumPerPEI-r17 ENUMERATED {po1, po2, po4, po8}, payloadSizeDCI-2-7-r17 INTEGER (1..maxDCI-2-7-Size-r17), pei-FrameOffset-r17 INTEGER (0..16), subgroupConfig-r17 SubgroupConfig-r17, lastUsedCellOnly-r17 ENUMERATED {true} OPTIONAL, -- Need R ... } //skip po-NumPerPEI The number of PO(s) associated with one PEI monitoring occasion. It is a factor of the total PO number in a paging cycle, i.e N x Ns, as specified in TS 38.304 [20]. The maximum number of PF associated with one PEI monitoring occasion is 2. The number of PO mapping to one PEI should be multiple of Ns when po-NumPerPEI is larger than Ns.

And similar to PMO (i.e. firstPDCCH-MonitoringOccasionOfPO), the total signaling overhead of firstPDCCH-MonitoringOccasionOfPEI-O is related to Ns and N as well.

Observation 3: The value of po-NumPerPEI in PEI is related to the value of Ns.

Observation 4: The total signaling overhead of firstPDCCH-MonitoringOccasionOfPEI-O is related to the value of Ns and N.

With exiting agreement, R19 paging adaptation needs to extend the value of Ns and N. Considering the RRC impacts, we propose RAN2 confirm to apply paging adaptation with PEI.

Proposal 4a: Paging adaptation with PEI is supported.

Proposal 4b: Discuss the RRC signaling impacts to PEI configuration.

Adaptation of PRACH in time domain

RAN1 had already achieved many agreements on adaptation of PRACH in time domain, including:

- The adaptation mechanism for additional PRACH resources is applied to both CONNECTED mode UEs and IDLE/INACTIVE mode UEs.

- At least DCI based adaptation is supported. No introduction of new DCI format.

- SSB-RO mapping rules for additional PRACH resources support:

- Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs

- Case 2: time-domain overlap but no overlap in frequency domain between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs

- Case 3: additional PRACH resources for NES-capable UEs and legacy PRACH resources overlap neither in time nor frequency domains

Based on RAN1 progress, RAN2 can start discuss scenarios from RAN2 point of view.

- CBRA or CFRA

The adaptation mechanism for additional PRACH resources is applied to both CONNECTED mode UEs and IDLE/INACTIVE mode UEs. It is straightforward to support CBRA for adaptation mechanism for additional PRACH resources. And CFRA is dedicated resource, the benefit of PRACH adaptation is not quite clear.

Proposal 5: Support CBRA for adaptation mechanism for additional PRACH resources.

- RA type

RAN1 discussions mainly focus on PRACH resource. If 2-step RA type is supported for PRACH adaptation, it is unclear whether additional PUSCH resources are also considered. Hence, RAN2 can focus on 4-step RA at first. And RAN2 can wait for RAN1 discussion whether 2-step RA can be also supported.

Proposal 6: RAN2 focus on 4-step RA for adaptation mechanism for additional PRACH resources.

- Whether to support RACH adaptation to RACH resources of R17 RACH partitions

In Rel-17, RACH partitioning was introduced. A list of RACH partitions (i.e. AdditionalRACH-Config) can be configured in SIB1, and the network can configure different RACH partitions (i.e. AdditionalRACH-Config) associated with different features (e.g., SDT) or feature combination (e.g., SDT+redcap). We need to clarify whether the RACH resource adaptation is supported for R15 RACH resources configuration only or for both R15 RACH resources configuration and R17 RACH partition resources configuration. In our understanding, it would be simpler if RACH resource adaptation is supported for R15 RACH resources configuration only on R19 NES cell. And when RACH adaptation is configured in the NES cell, there is no need to configure R17 RACH partitions in the NES cell for the sake of energy saving.

Therefore, it is proposed that,

Proposal 7: At least support the case when RACH resource adaptation is not restricted to any certain features. FFS whether to support it only for one or more features including (e)RedCap and/or Slicing and/or SDT and/or MSG3 repetition and/or MSG1 repetition.

Adaptation of SSB in time domain

In the LS on SSB adaptation from RAN4 [1], RAN4 asks RAN2 to answer the SMTC impact question due to SSB adaptation.

RAN2 has not discussed SSB adaptation yet. And the following are agreements made in RAN1.

RAN1#116bis: Agreement For indication of adaptation of SSB in time-domain, - Support at least SSB adaptation provided by gNB without UE trigger RAN1#117: Agreement For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB RAN1#118: Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’) RAN1#118bis: Agreement For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select Option 3, i.e. Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior). - No spec impact

According to RAN1’s agreements, different from on-demand SSB SCell, SSB adaptation only focuses on adaptation of SSB burst periodicity. And RAN1 confirms it is applicable to NCD-SSB on SCell for SSB adaptation. Currently NCD-SSB can be used for L3 measurement. Although there is no agreement if SSB adaptation is applied to L3 measurement in RAN1, it is straightforward that SSB adaptation can be used for L3 measurement as well.

Proposal 8a: SSB adaptation is used in L3 measurement.

STMC impact due to on-demand SSB SCell is discussing in RAN2, including:

- Option1: Based on different measurement configuration when OD-SSB is transmitted.

- Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted.

The signaling for SSB adaptation is unclear. But both two options for SMTC impact with on-demand SSB SCell can be applied to SSB adaptation. If SSB adaptation is applied to L3 measurement, the similar mechanism can be considered as a baseline.

Proposal 8b: If P8a is agreed, the mechanism in on-demand SSB SCell for SMTC impact with L3 measurement can be considered as a baseline.

Conclusion

According to the analysis in section 2, we have the following observations and proposals:

Observation 1: The total signaling overhead for PMO configuration = maximum value of Ns*bits for each PMO. If the value of N is extended, the maximum signaling overhead for each PMO configuration is increased. If the value of Ns is extended, the total signaling overhead for total PMO configuration is increased rapidly.

Observation 2: In case N is extended to T/64, T/128, T/256 and T/512, N may be less than 1. This is an issue that needs to be addressed.

Observation 3: The value of po-NumPerPEI in PEI is related to the value of Ns.

Observation 4: The total signaling overhead of firstPDCCH-MonitoringOccasionOfPEI-O is related to the value of Ns and N.

Adaptation of paging

Proposal 1a: The value of Ns is extended to 8 and 16 but not to any value greater than these.

Proposal 1b: The value of N is extended to at least T/32.

Proposal 1c: To address N<1 issue in case N is extended to T/64, T/128, T/256 and T/512, down selects between the following options:

- Option 1: Switch to legacy paging mechanism.

- Option 2: Set N = 1.

Proposal 2: The legacy System information update procedure is used to activate/deactivate paging adaptation mechanism.

Proposal 3: R19 NES UE follows legacy paging mechanism if paging adaptation configuration is absent in the R19 NES cell.

Proposal 4a: Paging adaptation with PEI is supported.

Proposal 4b: Discuss the RRC signaling impacts to PEI configuration.

Adaptation of PRACH in time domain

Proposal 5: Support CBRA for adaptation mechanism for additional PRACH resources.

Proposal 6: RAN2 focus on 4-step RA for adaptation mechanism for additional PRACH resources.

Proposal 7: At least support the case when RACH resource adaptation is not restricted to any certain features. FFS whether to support it only for one or more features including (e)RedCap and/or Slicing and/or SDT and/or MSG3 repetition and/or MSG1 repetition.

Adaptation of SSB in time domain

Proposal 8a: SSB adaptation is used in L3 measurement.

Proposal 8b: If P8a is agreed, the mechanism in on-demand SSB SCell for SMTC impact with L3 measurement can be considered as a baseline.

Reference

- R2-2409520, LS on SSB adaptation (R4-2416911; contact: Apple)