3GPP TSG RAN WG2 #128                                                                                  R2-2409652

Orlando, US, Oct 18th – 22nd, 2024

Source:	CATT, Turkcell

Title:	Consideration on generalization for RRM predication

Agenda Item:	8.3.5

Document for:	Discussion and Decision

- Introduction

In previous RAN2 meetings, various agreements were made for model generalization [1]-[2].

RAN2#125bis:

12 AI/ML model generalization could be addressed after sufficient performance gains for different use cases are found.

RAN2#127bis:

2: Discussion on what (type of information)/how generalization study can take place in Nov. meeting

In this document, we further discuss what type of information can be considered for model generalization.

- Discussion

In AI/ML, generalization is the ability of a trained model to perform effective transformations and inference on data other than the data on which it was trained. It can be applied in AI due to the algorithm's effective capability of utilizing the knowledge acquired from training data on new data. For RRM prediction in AI mobility, diverse scenarios, such as Uma, UMi, and different cell number for model training, such as cluster based or cell based, different frequencies, different UE states including low mobility, high mobility and etc., may require different models. Hence, model generalization is important to simply our simulation work.

  - UE mobility for inter-frequency case

For inter-frequency, RAN2 has performed the simulation on RSRP prediction of frequencies with 2G and 4G and found that:

- Higher-to-lower and lower-to-higher frequency prediction is comparable

- For co-located scenario,  the UE speed in the inter-frequency case has minor impact on prediction accuracy

Based on agreements above, it can be seen that UE speed in the inter-frequency case has minor impacts on prediction accuracy. Hence, different speed can be used as one dimension for AI/ML generalization for inter-frequency case. For example, the model trained for stable UEs can be used to predict the RSRP of UE with high mobility. This is helpful to reduce the work load of training models. Hence, it is proposed that:

Proposal 1:  Study AI/ML generalization from speed perspective for inter-frequency RRM measurement prediction, i.e., whether the model trained based on training data with one speed can also be applied to inter-frequency RRM measurement prediction for other speeds.

  - UE mobility for intra-frequency case

For intra-frequency cases, it has been observed that, the RSRP difference increases as the speed increases which have been agreed in RAN2#127bis meeting [2].

- For intra-frequency temporal domain, higher UE speeds result in larger prediction errors

However, for UEs with different speeds but within an acceptable speed range, the model can be generalized to reduce the training complexity. For example, the UEs with low mobility can be divided into a group using the model trained with speed 20Km/h, the UE with high mobility can use the model trained with 120Km/h and etc. Similarly, the training work can be greatly reduced.

Hence, it is proposed that:

Proposal 2: Study AI/ML generalization from speed range perspective for intra-frequency RRM measurement prediction, i.e., whether the model trained based on training data with one speed can also be applied to intra-frequency RRM measurement prediction for another speed within a specific range.

  - Per cell model vs generalized model

In RAN2#127 meeting, the following RAN2 agreements were made in the aspect of cluster based model:

=> it is up to companies to select the number of cells for input and output (companies should clarify what they are using for cluster-based approach). Cluster-based approach evaluation is optional and lower priority for now.

If cluster based model is not applied in AI mobility, it means the model has to be trained and applied for each cell, while cluster based model can be applied for a group of cells. Due to the complexity of model training, it’s worth studying the model generalization from cell perspective, i.e., whether the model trained based on one cell’s data can also be applied to another cell’s RRM measurement prediction. Then the generalization performance can be compared with that of cluster based model.

Proposal 3: Study AI/ML generalization from cell perspective for intra-frequency RRM measurement prediction, i.e., whether the model trained based on one cell’s data can also be applied to another cell’s intra-frequency RRM measurement prediction.

- Conclusion

In this contribution, we provide our views on generalization aspects and we propose that:

Proposal 1:  Study AI/ML generalization from speed perspective for inter-frequency RRM measurement prediction, i.e., whether the model trained based on training data with one speed can also be applied to inter-frequency RRM measurement prediction for other speeds.

Proposal 2: Study AI/ML generalization from speed range perspective for intra-frequency RRM measurement prediction, i.e., whether the model trained based on training data with one speed can also be applied to intra-frequency RRM measurement prediction for another speed within a specific range.

Proposal 3: Study AI/ML generalization from cell perspective for intra-frequency RRM measurement prediction, i.e., whether the model trained based on one cell’s data can also be applied to another cell’s intra-frequency RRM measurement prediction.

- References

- R2-2404102, RAN2#126bis chairman notes;

- RAN2_127b_Meeting_Report