3GPP TSG-RAN WG2 Meeting #128	 R2-2409675

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:	8.9.4

Source: 	CATT

Title: 	Support of PWS for NB-IoT NTN UE

Document for: 	Discussion and Decision

Introduction

Below conclusions were reached in RAN2 #127bis on the support of PWS [1]:

Agreements:

1.	RAN2 confirms the understanding that this WID objective covers all PWS services, including ETWS and CMAS

2.	The support for PWS introduced for NB-IoT NTN can also be made applicable for NB-IoT in TN, if this does not require additional NB-IoT TN specific changes.

3.	For NB-IoT NTN PWS, introduce the following new SIBs, taking the content within the corresponding LTE SIBs as a baseline (but also checking whether we can have some optimization/ skip some unnecessary fields):

- SystemInformationBlockType10-NB for primary ETWS notification;

- SystemInformationBlockType11-NB for secondary ETWS notification;

- SystemInformationBlockType12-NB for CMAS notification.

4.	Add the following PWS indication in direct indication information for NB-IoT (Come back in the next meeting on whether this is also added in Paging-NB):

- etws-Indication;

- cmas-Indication.

5.	Upon receiving the PWS notification from NB-IoT cell, the PWS-capable NB-IoT UE acquires the corresponding PWS message immediately (can come back to clarify further what immediately means)

6.	Clarify in the spec (and at least in 36.300) that for NB-IoT the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g. when the UE is in eDRX (can come back to further clarify the specific cases)

This contribution will make further analysis on the remaining issues on the support of PWS for NB-IoT NTN UE.

Discussion

PWS indication in Paging-NB

It was agreed to add the indication of the ETWS/CMAS in direct indication information for NB-IoT, but it is FFS on whether this is also added in Paging-NB.

According to current TS 36.331[2], it is specified that, the Direct Indication information is transmitted on NPDCCH using P-RNTI but without associated Paging-NB message, as follows:

- 6.7.5 Direct Indication Information Direct Indication information is transmitted on NPDCCH using P-RNTI but without associated Paging-NB message. Table 6.7.5-1 defines the Direct Indication information, see TS 36.212 [22], clause 6.4.3.3. When bit n is set to 1, the UE shall behave as if the corresponding field is set in the Paging-NB message, see 5.3.2.3. Bit 1 is the least significant bit. Table 6.7.5-1: Direct Indication information Bit; Field in Direct Indication information. 1; systemInfoModification. 2; systemInfoModification-eDRX. 3, 4, 5, 6, 7, 8; Not used, and shall be ignored by UE if received.

Additionally, from the below description excerpted from TS 36.212 [3] we can find that, when the DCI is used to transmit the direct indication information (i.e. Flag = 0), it does not include Resource assignment information to transmit PDSCH, and direct indication information is included in the DCI itself.

36.212 - 6.4.3.3 DCI Format N2 DCI format N2 is used for paging, direct indication, scheduling of one NPDSCH codeword carrying SC-MCCH in one cell, and notifying SC-MCCH change. The following information is transmitted by means of the DCI format N2: - If the format N2 CRC is scrambled by P-RNTI: - Flag for paging/direct indication differentiation – 1 bit, with value 0 for direct indication and value 1 for paging - Else if the format N2 CRC is scrambled by a SC-RNTI: - Information for SC-MCCH change notification – 1 bit as defined in clause 5.8a of [6] - If the format N2 CRC is scrambled by P-RNTI and Flag=0: - Direct Indication information – 8 bits provide direct indication of system information update and other fields, as defined in [6] - Reserved information bits are added until the size is equal to that of format N2 with Flag=1 - If the format N2 CRC is scrambled by P-RNTI and Flag=1, or if the format N2 CRC is scrambled by SC-RNTI: - Resource assignment – 3 bits as defined in clause 16.4.1.3 of [3] - Modulation and coding scheme – 4 bits as defined in clause 16.4.1.5 of [3] - Repetition number – 4 bits as defined in clause 16.4.1.3 of [3] - DCI subframe repetition number – 3 bits as defined in clause 16.6 of [3]

Considering that the "Flag" field differentiates whether the DCI is for paging or for directly indication, that means if the indication of PWS were not introduced in the Paging-NB, there would be no way for the NW to send the two notifications, i.e. indication of PWS and the Paging-NB message, at the same time in one PO, when there is the need for the NW to page any UE while indicating the PWS. Furthermore, as eMTC also supports to indicate the idle UE with the Paging message, the NB-IoT can follow the same mechanism.

Observation 1: If the indication of PWS were not included in the Paging-NB message, the NW could not perform paging UE and PWS indication at the same time, as the direct indication information cannot be transmitted with associated Paging-NB message.

Hence we suggest including the indication of PWS in Paging-NB as well.

Proposal 1: Add the following PWS indication in Paging-NB:

- etws-Indication;

- cmas-Indication

Content of the SIB10-NB/SIB11-NB/SIB12-NB

It was agreed in the last meeting to take the content within the corresponding LTE SIBs as a baseline, but to also check whether we can have some optimization/skip some unnecessary fields.

Based on the content of existing SIB10/11/12, most of the fields refer to other specs, which are not within the RRC scope. Take SIB10 as an example: we can find that the messageIdentifier/serialNumber/warningType refer to TS 36.413; hence, it seems not possible to determine at the RRC level which of these fields are unnecessary. The possibility of skipping some of the fields (if really any) should be determined and specified in TS36.413.

SystemInformationBlockType10 ::= SEQUENCE { messageIdentifier BIT STRING (SIZE (16)), serialNumber BIT STRING (SIZE (16)), warningType OCTET STRING (SIZE (2)), dummy OCTET STRING (SIZE (50)) OPTIONAL, -- Need OP ..., lateNonCriticalExtension OCTET STRING OPTIONAL }

As for those RRC-defined fields, e.g. warningMessageSegmentNumber/warningMessageSegmentNumber in SIB11, they are mainly used for segmentation. Considering the limited size for SI transmission in NB-IoT which is even smaller than that for LTE, the segmentation function should still be supported for NB-IoT, so that these fields cannot be skipped either.

SystemInformationBlockType12-r9 ::=	SEQUENCE {

messageIdentifier-r9				BIT STRING (SIZE (16)),

serialNumber-r9						BIT STRING (SIZE (16)),

warningMessageSegmentType-r9		ENUMERATED {notLastSegment, lastSegment},

warningMessageSegmentNumber-r9		INTEGER (0..63),

warningMessageSegment-r9			OCTET STRING,

dataCodingScheme-r9					OCTET STRING (SIZE (1))		OPTIONAL,	-- Cond Segment1

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

...,

[[	warningAreaCoordinatesSegment-r15		OCTET STRING	OPTIONAL	-- Need OR

]]

}

As for warningAreaCoordinatesSegment defined in SIB12, it is used to indicate the geographical area where the CMAS warning message is valid, which is needed especially for NTN cell. Also, its definition refers to TS 23.041 [4].

Observation 2: RAN2 cannot determine to skip the parameters in SIB10/11/12 whose contents are defined by other WGs' Specs. As for the parameters defined by RRC in SIB10/11/12, they are used for segmentation and thus cannot be skipped.

Considering the above situation, we think there is no need to further pursue the optimization on potential field skipping, in order to simplify the discussion and avoid further inter-WG coordination. So we have Proposal 2 as follows:

Proposal 2: From RAN2 perspective, the content within the corresponding LTE SIBs can be reused for SIB10-NB/SIB11-NB/SIB12-NB (except the dummy field in SIB10). Optimization on field skipping is not pursued.

Acquisition on PWS immediately

It was agreed that upon reception of the PWS notification from an NB-IoT cell, the PWS-capable NB-IoT UE acquires the corresponding PWS message immediately, but what "immediately" really means is FFS.

The word "immediately" is actually used against the SI update procedure on next modification period. That is, upon receiving the PWS indication, UE does not need to first wait for the arrival of next modification period before really acquiring the PWS notification. It should start receiving the corresponding PWS specific SIBs according to the SI scheduling window right upon receiving the PWS indication.

Proposal 3: If the UE receives a PWS indication, it shall start receiving the corresponding notification SIB according to schedulingInfoList contained in SystemInformationBlockType1-NB, without waiting for the next modification period.

Support of PWS for NB-IoT NTN UE in Connected mode

Whether to support PWS for NB-IoT NTN UEs in Connected mode was discussed in last meeting without consensus. Currently, NB-IoT UEs in connected mode are not required to acquire system information when T311 is not running. In Rel-16, it was already supported for a non-BL UE in CE to receive ETWS/CMAS information, when the UE is in connected mode, based on the following agreements made during RAN2#103bis ~ RAN2#108:

- RAN2 intends to support CMAS/ETWS for non-BL UEs in CE mode in connected mode. - SIBs for ETWS and CMAS are not provided via dedicated signaling. - In connected mode, non-BL UEs in CE monitor MPDCCH to receive ETWS and/or CMAS notification using Type 0 CSS in the same narrowband where unicast transmission can be received. - UE stays in RRC_CONNECTED after receiving the indication for ETWS/CMAS notification. - It is up to UE implementation how ETWS/CMAS notification is acquired. - RAN2 does not intend to introduce any signalling optimization regarding MIB, SIB1-BR and SIB10/11/12 acquisition. - RAN2 understands that UEs in CE Mode B may not be able to acquire ETWS notification within 4 seconds requirement. - Non-BL UE supporting ETWS/CMAS in CE mode is required to monitor the same subframes as in legacy, i.e. non-BL UEs in CE, to receive ETWS/CMAS notification when in connected mode. - Introduce capability for support of ETWS/CMAS indication reception in RRC CONNECTED by UE supporting CE Mode A. - Introduce capability for support of ETWS/CMAS indication reception in RRC CONNECTED by UE supporting CE Mode B. - Introduce in dedicated signaling an indication whether ETWS/CMAS notification is enabled for UE in connected mode. - Use SI-RNTI for ETWS/CMAS notification. - UE is enabled to monitor for ETWS/CMAS notification via dedicated signalling. - The reception types for SI-RNTI for ETWS/CMAS notification is captured in TS 36.302.

The same mechanism can be taken as the baseline for NB-IoT UE in connected mode to receive ETWS/CMAS, if supported. Hence we propose that RAN2 can consider supporting PWS for NB-IoT NTN UEs in connected mode as well.

Proposal 4: Support CMAS/ETWS for NB-IoT NTN UEs in connected mode, reusing the following agreements which were made for support of CMAS/ETWS for non-BL UEs in CE mode in connected mode in Rel-16:

- SIBs for ETWS and CMAS are not provided via dedicated signaling.

- In connected mode, NB-IoT UEs monitor NPDCCH to receive ETWS and/or CMAS notification using Type 0 CSS in the same narrowband where unicast transmission can be received.

- UE stays in RRC_CONNECTED after receiving the indication for ETWS/CMAS notification.

- It is up to UE implementation how ETWS/CMAS notification is acquired.

- NB-IoT UE supporting ETWS/CMAS is required to monitor the same subframes as in legacy, i.e. NB-IoT UEs, to receive ETWS/CMAS notification when in connected mode.

- Introduce capability for support of ETWS/CMAS indication reception in RRC CONNECTED by NB-IoT UE.

- Introduce in dedicated signaling an indication whether ETWS/CMAS notification is enabled for UE in connected mode.

- Use SI-RNTI for ETWS/CMAS notification.

- UE is enabled to monitor for ETWS/CMAS notification via dedicated signaling.

- The reception types for SI-RNTI for ETWS/CMAS notification is captured in TS 36.302.

As the above agreements involve something related to PDCCH monitoring, RNTI handing, etc., we think it is necessary to inform RAN1, if above proposal is agreed by RAN2.

Proposal 4a: Send LS to RAN1 informing the agreements made by RAN2 based on above Proposal 4.

Clarification on when PWS requirement cannot be met

It was agreed to clarify in the spec (and at least in 36.300) that for NB-IoT the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g. when the UE is in eDRX, but it was FFS on the specific cases for which this clarification needs to be made.

Currently, it was specified in TS 36.300 [5] that for UEs in idle mode, the ETWS, CMAS, PWS requirement may not be met when a UE is in eDRX.

- 10.1.4 Paging and C-plane establishment Paging groups (where multiple UEs can be addressed) are used on PDCCH: - Precise UE identity is found on PCH; - DRX configurable via BCCH and NAS; - Only one subframe allocated per paging interval per UE; - The network may divide UEs to different paging occasions in time; - There is no grouping within paging occasion; - One paging RNTI for PCH. When extended DRX (eDRX) is used in idle mode, the following are applicable: - The DRX cycle is extended up to and beyond 10.24s in idle mode, with a maximum value of 2621.44 seconds (43.69 minutes); For NB-IoT, the maximum value of the DRX cycle is 10485.76 seconds (2.91 hours); …//omit - ETWS, CMAS, PWS requirement may not be met when a UE is in eDRX. For EAB, if the UE supports SIB14, when in extended DRX, it acquires SIB14 before establishing the RRC connection; - When the eDRX cycle is longer than the system information modification period, the UE verifies that stored system information remains valid before establishing an RRC connection. Paging message can be used for system information change notification, when including systemInfoModification-eDRX, for a UE configured with eDRX cycle longer than the system information modification period. …//omit

According to the current TS36.300, the scenarios where NB-IoT UE in idle mode may be unable to meet the PWS requirement should be clarified in the Spec. Considering that for a non-NB-IoT UE the default paging cycle cannot be exceeded 4s (as shown below in cyan [2], the current Spec hence only captures that, the ETWS, CMAS, PWS requirement may not be met in the case of eDRX for non-NB-IoT UE.

PCCH-Config ::= SEQUENCE { defaultPagingCycle ENUMERATED { rf32, rf64, rf128, rf256}, nB ENUMERATED { fourT, twoT, oneT, halfT, quarterT, oneEighthT, oneSixteenthT, oneThirtySecondT} }

However, for NB-IoT UEs, the default paging cycle could be configured with rf512 or rf1024, as shown below in yellow. That is, besides the eDRX, the default paging cycle for NB-IoT UE may also exceed 4s, which will make the PWS requirement unable to be met either.

PCCH-Config-NB-r13 ::= SEQUENCE { defaultPagingCycle-r13 ENUMERATED {rf128, rf256, rf512, rf1024}, nB-r13 ENUMERATED { fourT, twoT, oneT, halfT, quarterT, one8thT, one16thT, one32ndT, one64thT, one128thT, one256thT, one512thT, one1024thT, spare3, spare2, spare1}, npdcch-NumRepetitionPaging-r13 ENUMERATED { r1, r2, r4, r8, r16, r32, r64, r128, r256, r512, r1024, r2048, spare4, spare3, spare2, spare1} }

Hence we suggest clarifying in TS 36.300 that for NB-IoT UE in idle mode, in addition to UE in eDRX, the ETWS, CMAC, PWS requirement may not be met when a UE with the DRX cycle is 5.12 second or 10.24 second.

Proposal 5: Clarify in TS36.300 that for NB-IoT UE in idle mode, the ETWS, CMAC, PWS requirement may not be met, when a UE is in eDRX or with the DRX cycle is 5.12 second or 10.24 second.

Conclusion

In this contribution, the remaining issues on support of PWS for NB-IoT UE are discussed with the following observations and proposals:

Observation 1: If the indication of PWS were not included in the Paging-NB message, the NW could not perform paging UE and PWS indication at the same time, as the direct indication information cannot be transmitted with associated Paging-NB message.

Observation 2: RAN2 cannot determine to skip the parameters in SIB10/11/12 whose contents are defined by other WGs' Specs. As for the parameters defined by RRC in SIB10/11/12, they are used for segmentation and thus cannot be skipped.

Proposal 1: Add the following PWS indication in Paging-NB:

- etws-Indication;

- cmas-Indication

Proposal 2: From RAN2 perspective, the content within the corresponding LTE SIBs can be reused for SIB10-NB/SIB11-NB/SIB12-NB (except the dummy field in SIB10). Optimization on field skipping is not pursued.

Proposal 3: If the UE receives a PWS indication, it shall start receiving the corresponding notification SIB according to schedulingInfoList contained in SystemInformationBlockType1-NB, without waiting for the next modification period.

Proposal 4: Support CMAS/ETWS for NB-IoT NTN UEs in connected mode, reusing the following agreements which were made for support of CMAS/ETWS for non-BL UEs in CE mode in connected mode in Rel-16:

- SIBs for ETWS and CMAS are not provided via dedicated signaling.

- In connected mode, NB-IoT UEs monitor NPDCCH to receive ETWS and/or CMAS notification using Type 0 CSS in the same narrowband where unicast transmission can be received.

- UE stays in RRC_CONNECTED after receiving the indication for ETWS/CMAS notification.

- It is up to UE implementation how ETWS/CMAS notification is acquired.

- NB-IoT UE supporting ETWS/CMAS is required to monitor the same subframes as in legacy, i.e. NB-IoT UEs, to receive ETWS/CMAS notification when in connected mode.

- Introduce capability for support of ETWS/CMAS indication reception in RRC CONNECTED by NB-IoT UE.

- Introduce in dedicated signaling an indication whether ETWS/CMAS notification is enabled for UE in connected mode.

- Use SI-RNTI for ETWS/CMAS notification.

- UE is enabled to monitor for ETWS/CMAS notification via dedicated signaling.

- The reception types for SI-RNTI for ETWS/CMAS notification is captured in TS 36.302.

Proposal 4a: Send LS to RAN1 informing the agreements made by RAN2 based on above Proposal 4.

Proposal 5: Clarify in TS36.300 that for NB-IoT UE in idle mode, the ETWS, CMAC, PWS requirement may not be met, when a UE is in eDRX or with the DRX cycle is 5.12 second or 10.24 second.

Reference

- Draft_RAN2_127b_Meeting_Report_v1

- 3GPP TS 36.331: "Universal Terrestrial Radio Access (UTRAN); Radio Resource Control (RRC)"

- 3GPP TS 36.212: "Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding"

- 3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)"

- 3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRAN); Overall description; Stage 2"

- 3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures"