3GPP TSG-RAN WG2 Meeting #128	R2-2409678

Orlando, United States, 18th – 22nd November, 2024

Agenda Item:	8.7.4.2

Source: 	OPPO

Title:  	Discussion on DSR enhancements for XR

Document for:	Discussion and Decision

# Introduction

In Rel-19 XR WID [1] revised in RANP#105 meeting, the objectives of scheduling enhancements are updated as follows.

- Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

This contribution would like to provide our considerations on DSR enhancements for scheduling.

# Discussion on DSR enhancements

In RAN2#127bis meeting, the following agreements were made. Here we further discuss the leftover issues as highlighted below.

Agreements on DSR enhancements - We do not change the definition of delay-critical data - For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s) - Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc. - For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - Buffer size of data volume in each portion - Shortest remaining time among PDCP SDUs buffered in each portion. - There is no need to include PSI in the enhanced DSR MAC CE. - A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. - FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

In legacy DSR, for each LCG only one remaining time threshold is used for both triggering threshold and reporting threshold. For enhanced DSR in Rel-19 XR, caused by the introduced multiple reporting thresholds, the relationship between the single triggering threshold and mutiple reporting thresholds is not clear, for example,

- Case-1: triggering threshold is higher than or equal to all reporting thresholds, e.g., ThreshReport1 < ThreshReport2 < ThreshReport3 < ThreshReport4 <= ThreshTrigger,

- Case-2: triggering threshold is lower than largest reporting threshold, e.g., ThreshReport1 < ThreshReport2 <= ThreshTrigger < ThreshReport3 < ThreshReport4.

In our understanding, no need to have any constraints on how the network configures triggering threshold and reporting threshold(s). In this way, for Case-1 network can rely on the multiple reporting thresholds to obtain finer delay status information for the data that has already been delay-critical. For Case-2, on top of Case-1, network can also obtain finer delay status information for the data that is going to be delay-critical. Therefore, up to network implementation, network could be flexible to obtain the delay status information it would like to know to assist the scheduling better.

- Flexible configuration of triggering threshold and reporting thresholds allows network to obtain finer delay status information it would like to know, including both delay critical data (i.e., the data that has already been delay-critical) and non-delay critical data (i.e., the data that is going to be delay-critical).

For Case-2 (i.e., setting the triggering threshold to value lower than largest reporting threshold), companies have concern on the impact on DSR triggering and cancellation. In legacy DSR procedure, the triggering and cancellation mechanisms of a DSR are specified in 38321 as shown below.

DSR triggering: If an LCG is configured for delay status reporting, the MAC entity shall: 1> if the smallest remaining value of the running PDCP discardTimers among all the SDUs buffered for the LCG that has not been transmitted in any MAC PDU and has not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and 1> if there is no DSR pending for the LCG: 2> trigger a DSR for the LCG.

DSR cancellation: An SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and it is associated with the LCG which triggered the DSR and the remaining value of its PDCP discardTimer is below remainingTimeThreshold. A MAC PDU shall contain at most one DSR MAC CE. The MAC entity shall not include a DSR MAC CE in a MAC PDU if the MAC PDU can accommodate the SDUs associated with all the pending DSRs. After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, either when all the SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the SDUs associated with the DSR (as described in the clause 6.1.3.72). The MAC entity may cancel a pending DSR when a MAC PDU is transmitted and this MAC PDU includes all the SDUs associated with the DSR but is not sufficient to include the DSR MAC CE and its subheader.

Regarding DSR triggering in Case-2, if a SDU buffered for the LCG triggers DSR, in the same LCG there would be some SDUs with remaining time higher than triggering threshold to be reported as data volume in the new DSR MAC CE (i.e., SDUs that is going to be delay-critical). If the DSR MAC CE has been reported successfully, these SDUs that is going to be delay-critical will not trigger further DSR when they become delay-critical. It seems no harm for network scheduling since anyway the DSR info of these SDUs has been reported once before.

Regarding DSR cancellation in Case-2, the understanding of “SDUs associated with the DSR” would be impacted. Based on the current DSR cancellation mechanism, an SDU is considered to be associated with a DSR if the remaining value of its PDCP discardTimer is below the triggering threshold. Therefore, when determining DSR cancellation, only the data with remaining time lower than triggering threshold (i.e., the data that has already been delay-critical) is considered, but for Case-2 a DSR MAC CE may also include delay status information for the data with remaining time higher than triggering threshold (i.e., the data that is going to be delay-critical). We understand even though a DSR is cancelled based on the data with remaining time lower than triggering threshold, since the delay status information has not been reported, the data with remaining time higher than triggering threshold still has chance to trigger DSR when it becomes lower than trigger threshold in the future. Therefore, no essential issue is foreseen.

- No essential impact on DSR triggering and cancellation in case of setting triggering threshold to value lower than largest reporting threshold.

- No need to restrict how network configures triggering threshold and reporting thresholds.

For the new DSR MAC CE design, it is agreed that a one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. It is still FFS on the choice between “a certain/further pair”.

For one bit indicating a certain pair, it is actually a bitmap on presence of each pair for each LCG. It is hard to determine a fixed length of the bitmap at this moment, since the number of pairs depends on the maximum number of reporting thresholds allowed to configure for each LCG.

Therefore, we tend to the later way, i.e., one bit indicating further pair. It would be flexible on the number of pairs, since this one-bit indication is piggyback within each pair. For the format of new DSR MAC CE, as shown in the figure below, the format of legacy DSR MAC CE could be taken as baseline. And R bit in the legacy DSR MAC CE could be used for this one-bit indication, so no additional signalling overhead is introduced compared to the way of bitmap. To differentiate the pairs from each LCG, in the last pair of each LCG it always needs to indicate “no further pair existing for this LCG” as an end maker.

- RAN2 confirms that a one-bit indication may indicate whether further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG.

Another issue is the co-existence with legacy DSR MAC CE. We understand that for it is not necessary for network to configure both old DSR and new DSR at the same time. If networks configure them at the same time, considering the new DSR MAC CE is also compatible to legacy DSR MAC CE, it is straightforward that in case there is at least one LCG configured with new DSR, UE always uses a new DSR MAC CE for all LCGs.

- UE always uses the new DSR MAC CE for all LCGs in case there is at least one LCG configured with new DSR.

Regarding the issue whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR. In our understanding, changing the buffer size calculation for DSR to include non-delay critical data ahead of delay critical data in the buffer is one kind of solutions on DSR enhancement to include information of non-delay critical data. But, based on the proper configuration of DSR trigger threshold and multiple reporting thresholds, to some extent the information of non-delay critical data has already been included by reporting multiple pairs of remaining time and buffer sizes, networks can collect sufficient information to perform proper scheduling for UE. Therefore, we can live without any optimization for on-delay critical data ahead of delay critical data in the buffer.

- Thanks to the enhancement of multiple reporting thresholds, to some extent the information of non-delay critical data has already been captured by reporting multiple pairs of remaining time and buffer sizes.

- RAN2 not pursue optimization for non-delay critical data ahead of delay critical data in the buffer.

Another issue needed to be started to discuss is the capability for DSR enhancement. The DSR enhancement introduced in Rel-19 XR should be an optional feature for a Rel-19 UE to support, thus needing an optional capability reported to network to keep a common understanding between UE and network. R2 can confirm the introduction of the optional capability with signalling.

- Introduce a new capability on whether the UE supports DSR enhancement for Rel-19 XR, which is an optional capability with signalling.

# 3. Conclusion

Based on the discussion above, we made the following observations and proposals:

- Flexible configuration of triggering threshold and reporting thresholds allows network to obtain finer delay status information it would like to know, including both delay critical data (i.e., the data that has already been delay-critical) and non-delay critical data (i.e., the data that is going to be delay-critical).

- No essential impact on DSR triggering and cancellation in case of setting triggering threshold to value lower than largest reporting threshold.

- No need to restrict how network configures triggering threshold and reporting thresholds.

- RAN2 confirms that a one-bit indication may indicate whether further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG.

- UE always uses the new DSR MAC CE for all LCGs in case there is at least one LCG configured with new DSR.

- Thanks to the enhancement of multiple reporting thresholds, to some extent the information of non-delay critical data has already been captured by reporting multiple pairs of remaining time and buffer sizes.

- RAN2 not pursue optimization for non-delay critical data ahead of delay critical data in the buffer.

- Introduce a new capability on whether the UE supports DSR enhancement for Rel-19 XR, which is an optional capability with signalling.

# 4. Reference

[1] RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia