3GPP TSG-RAN WG2#128	R2-2409702

Orlando, Florida, USA, 18-22 November 2024

Agenda Item:	8.2.2

Source: 	MediaTek Inc.

Title:  	Ambient IoT coverage, energy, TB size, and all sorts of other things

Document for:	Discussion, decision

# 1	Introduction

This document discusses a rather broad variety of interconnected issues.  The subject of transport block size, as invoked in the LS in [1], relates in a natural way to coverage on the AIoT interface.  We examine the elements that determine what TB size can be supported at a particular moment, which leads to some consideration of energy storage at the device and the already-discussed energy indication, and eventually to the topics of segmentation and the device’s implementation in terms of memory management.

# 2	Discussion

## 2.1	TB size in general

RAN1 indicated in [1] that we can expect a maximum transport block size of ~1000 bits, and no lower bound on the minimum size.  There are some grounds for concern about the latter point—e.g., it was pointed out in discussion that we would like to avoid segmenting a message containing only an ID—but it is not clear either from the LS or from discussions in RAN1 that the “minimum” TB size is the same as the “maximum always available” TB size.

A device in sufficiently bad coverage may be able to send/receive a TB of only (for example) 100 bits, but it seems unlikely that coverage would dictate a TB size of 3 bits.  At that point the device should simply be considered not in coverage of the reader.  We assume that the smallest TB sizes are intended for transport of correspondingly small data, to avoid the need for excessive padding.  Also, the supportable TB size may be affected by both coverage and energy conditions at the device, as discussed in the following sections.  Because of these interacting effects, the TB size may be the wrong framing for understanding what can be sent; rather, the coverage (and reader Tx power in the R2D case) will dictate a data rate, the device’s energy storage and reader considerations on radio resource allocation will dictate an availability time, and the product of the data rate and the availability time is the maximum supportable TB size at the moment.  We will expand on this relationship in the following sections.

Proposal 1: RAN2 assume that there is no fixed “maximum always available” TB size in either the D2R or R2D direction.  Rather, the maximum supportable TB size at a given moment is the product of the achievable data rate and the device availability time.

With this understanding, some burden falls on the reader to decide when a transmission is feasible.  For instance, if a device cannot support a TB large enough to carry its upper-layer ID unsegmented, the reader may choose to delay inventorying that device (if the reader knows the device limitations).  This seems to be a reader implementation issue with no spec impact.

## 2.2	TB size in relation to coverage

A device in poor coverage (wrt the reader) may be unable to sustain a high data rate (in Tx or Rx directions), and it may thus be constrained to using a small TB size to avoid monopolizing radio resources.  However, this effect is hard to quantify because there is no clear definition of “monopolizing”; it may be acceptable for a device in one deployment situation to occupy a set of resources for several seconds, while a device in a different deployment may be limited to milliseconds.  The device cannot judge these conditions; in general, only the reader can determine what an acceptable occupancy time for a single device is.

The interacting influences determining the usable TB size are as shown in figure 1.  Note that this analysis assumes that the device has no energy constraint, i.e., there is enough energy to sustain operation at the achievable data rate for the allowable occupancy time.

Figure 1: Influences on TB size without energy constraints

In the R2D direction, the reader may be able to transmit at high power to compensate for coverage limitations (though a UE reader may not be able to do this as extensively as a BS reader).  Thus there is some flexibility in determining the data rate in this direction.  In the D2R direction, the device is unlikely to have high power flexibility, and it may well have fixed transmission power, so the D2R data rate should be understood as representing a hard limit.  (The device also probably does not know its own coverage/channel conditions and is limited to transmitting within parameters set by the reader.)

Proposal 2: In the absence of a device energy constraint, the supportable TB size is determined by the achievable data rate and the occupancy time allowed by the reader.

## 2.3	TB size in relation to energy

The analysis in the foregoing section ignores energy reserves at the device, which can obviously limit the occupancy time.  There has been some discussion of whether a device can always estimate its own energy reserves; our understanding is that a device should be able to predict its availability time (e.g., at slot granularity), but this issue is out of RAN2 scope.  However, the transmitting side has to assume something about how long the device will be available; we understand that this assumption would actually be controlled by the reader, which generates R2D transport blocks and schedules D2R transmissions, and therefore it is useful for the reader to have some measure of the device’s energy.  This is aligned with the agreement already taken at RAN2#127bis to have an energy indication from the device.  The existing agreement confirms a 1-bit indication for capture in the TR; however, a 1-bit indication does not seem sufficient to make a real estimate of the availability time, and we propose to capture the applicability of the indication to this case (without fixing a specific granularity).

Proposal 3: Confirm the intention to include an energy status indication from the device, with sufficient granularity for the reader to make an estimate of the device’s availability time.

Some discussion of how the reader can make such an estimate may be necessary.  It is not clear what the right units would be; the device may or may not be able to measure its energy directly, but even if it can, a direct measure of energy may not be helpful for the reader.  We identify three main options:

- The device indicates a level of energy directly (joule units);

- The device indicates an anticipated activity time that it can support (second units);

- The device indicates a number or set of operations that it can support (units need discussion).

Alternative 1 would require the reader to know something about the device’s energy consumption rates, which seems not entirely practical when considering different device implementations.  Alternative 2 requires the device to guess at its own energy consumption rate over time when asked to engage in some AIoT operations, which may be unpredictable; this also seems a bit impractical in terms of forecasting the details of future activity, although perhaps the device could estimate a triple (transmit time, receive time, idle time).  Alternative 3 may be realistic, and would encompass an energy indication that could signify “I have enough energy for the next expected operation” or “I have enough energy to support a Tx/Rx exchange up to M+N bits”.  However, considering the need to complete the study item, we suggest capturing these alternatives and examining the implications further during normative work.

Note that the already discussed 1-bit indication can be seen as a minimal case of alternative 3: The device indicates “I can do something/I cannot do something”, where the reader is supposed to be able to understand in context what the “something” is.  We have some doubt if a single bit carries enough information to allow the reader to make a good judgement of what can be sent and received on the air, but further discussion is possible.

Proposal 4: Capture the following alternatives for an energy status indication in the TR, with the intent to investigate them in normative work:

- The device indicates a level of energy directly (joule units);

- The device indicates an anticipated activity time that it can support (second units);

- The device indicates a number or set of operations that it can support (units need discussion).

The availability time acts as a cap on the occupancy time, of course, as shown in figure 2.

Figure 2: Influences on TB size with energy constraints

This set of interacting constraints implies that the reader really needs to know the device’s energy status to make a good decision about the D2R radio resources; otherwise strange conditions like resource underflow will arise, in which the device cannot use all of the resources allocated to it and sends a shorter TB than the reader expects.

Proposal 5: In the presence of a device energy constraint, the device’s availability time acts as a cap on the occupancy time, and the usable TB size is determined by the data rate and the occupancy time considering both reader and device limitations.

## 2.4	Implications for segmentation and use of NVRAM

The foregoing analysis might be summarised in a sentence: The usable TB size over a period of time is unpredictable.  As a result, it seems very difficult to inform upper layers, especially at the network side, of what increments are available as TBs in the lower layers.  This argues that R2D segmentation will probably be needed in MAC: The controller/application may provide a large block of data to the reader when only a small R2D TB size is available considering the combination of coverage/Tx power/allowable occupancy time/device availability time.  It does not seem possible to provide real-time information to the controller to manage the block size provided to the reader; the reader implementation has some available measures to increase the block size, such as transmitting at higher power to increase the data rate and delaying transmission to wait for good channel conditions, but these techniques cannot be considered as a guarantee of fitting the entire data block in one MAC PDU.  We are forced to the conclusion that the upper-layer data block provided to MAC may be larger than the available TB size.

Proposal 6: Segmentation in MAC is needed in the R2D direction.

RAN2 agreed at RAN2#127bis that AS layer buffering in the device is not supported for segmentation, so the storage of segments while a data block is reassembled will have to take place in some upper layer, resulting in asymmetric management of blocks and segments between the network/reader side and the device side (figure 3).  We understand that this is primarily an R2D issue as it affects RAN2; in the D2R direction, it should be possible to move segments out of an upper-layer buffer in the device more or less transparently to the specs.

Figure 3: Segmentation at reader and reassembly in device upper layer

It would be theoretically possible for reassembly to take place in NVRAM, so that a partially assembled block persists through a loss of power.  However, we understand from RAN1 evaluations that writing to NVRAM is power-expensive, and furthermore, the NVRAM may be rated for a comparatively low number of write cycles over the device’s lifetime (e.g., tens of thousands), so the use of NVRAM for routine MAC PDU processing seems not practical.  While implementations may vary, it is prudent to assume that a device loss of power will result in loss of any partially assembled data block.

Proposal 7: The device implementation of the segment reassembly process is not standardized, but in particular is it not assumed to take place in NVRAM.

If a segment is lost—e.g., if segment 3 does not arrive at the device MAC—it seems possible for the MAC to send a NACK (or not send a positive ACK) and so trigger retransmission, while the partially reassembled block waits in the upper-layer buffer.  The reader’s transmitting MAC entity would presumably delete the concerned segment when it received a positive ACK.  However, if the device loses power while the reassembly is in progress, the stored segments may be lost as noted above, and the whole large block needs to be retransmitted in the upper layer.

Proposal 8: RAN2 consider that the MAC layer cannot guarantee successful delivery of a complete upper-layer data block in the R2D direction at least in the case that the device loses power during reassembly of a segmented block.

# 3	Conclusion

This document promulgated the following proposals:

Proposal 1: RAN2 assume that there is no fixed “maximum always available” TB size in either the D2R or R2D direction.  Rather, the maximum supportable TB size at a given moment is the product of the achievable data rate and the device availability time.

Proposal 2: In the absence of a device energy constraint, the supportable TB size is determined by the achievable data rate and the occupancy time allowed by the reader.

Proposal 3: Confirm the intention to include an energy status indication from the device, with sufficient granularity for the reader to make an estimate of the device’s availability time.

Proposal 4: Capture the following alternatives for an energy status indication in the TR, with the intent to investigate them in normative work:

- The device indicates a level of energy directly (joule units);

- The device indicates an anticipated activity time that it can support (second units);

- The device indicates a number or set of operations that it can support (units need discussion).

Proposal 5: In the presence of a device energy constraint, the device’s availability time acts as a cap on the occupancy time, and the usable TB size is determined by the data rate and the occupancy time considering both reader and device limitations.

Proposal 6: Segmentation in MAC is needed in the R2D direction.

Proposal 7: The device implementation of the segment reassembly process is not standardized, but in particular is it not assumed to take place in NVRAM.

Proposal 8: RAN2 consider that the MAC layer cannot guarantee successful delivery of a complete upper-layer data block in the R2D direction at least in the case that the device loses power during reassembly of a segmented block.

# 4	References

[1]	R1-2409250 / R2-2409405: “Reply LS to RAN2 on data block sizes for Ambient IoT”, LS from RAN1 to RAN2, RAN2#127bis