3GPP TSG-RAN WG2 Meeting #128                                                                    R2-2409718

Orlando, U.S.A, Novermber 18th – 22nd, 2024

Source:	CATT

Title:	LP-WUS in RRC_IDLE/INACTIVE

Agenda Item:	8.4.2

Document for:	Discussion and Decision

Introduction

The following agreements were achieved for LP-WUS procedure in RRC_IDLE/INACTIVE:

If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). The LPWUS monitoring exit condition does not include MR measurements. For CN assigned LP-WUS subgrouping, RAN2 assumes similar procedure for PEI will be used for LP-WUS subgrouping. Final design is up to SA2/CT1/RAN3 discussion. For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping. RAN2 inform this conclusion to SA2/CT1/RAN3. This LS is approved in R2-2409225

In this contribution, further considerations are provided, including:

- RAN2 impact on RAN1 LS.

- UE_ID based LP-WUS Subgrouping.

- Configurations for derivation of serving cell measurement quality from LR.

Discussion

RAN2 impact on RAN1 LS

In RAN1#118 meeting, RAN1 discussed the issue of LR and MR on different carriers/frequencies. RAN1 sent the LS [1] to RAN2/RAN4 as follows:

In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. RAN1 respectfully asks RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs.

In RAN2#127 meeting, RAN2 discussed the possible impacts but no conclusion was achieved. The following options can be considered if UE may not support LP-WUS reception on all the bands supported by the UE:

Option 1: LR and MR are allowed to operate in different bands/carriers.

If LR and MR can operate in different bands/carriers, considering the coverage of LR may be different from that of MR, RRM measurements based on LR may not be applicable to evaluate the channel condition on MR and vice versa.

Observation 1: RRM measurements based on both MR and LR may be needed to evaluate the channel condition on MR and LR respectively, if LR and MR can operate in different bands/carriers.

Currently, serving cell measurement result based on LR is an optional configuration for entry condition for LP-WUS monitoring. Taking Observation 1 into consideration, the serving cell measurement result based on LR should be a mandatory configuration for entry condition for LP-WUS monitoring.

In RAN2#127 meeting, agreed conditions for serving cell measurement offloading are similar to entry/exit conditions for LP-WUS monitoring.

For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. - The exit condition is based on the LR measurement results.

Similarly, RRM measurements based on MR are needed when LR and MR are operating in different bands/carriers according to Observation 1. If so, serving cell measurement offloading cannot be achieved in this case. And considering Observation 1, it is questionable if serving cell measurement based on MR is allowed to be relaxed. The power saving gain of LP-WUS is reduced.

In addition, if LR and MR can operate in different bands/carriers, additional UE capabilities are required for LP-WUS operation.

Furthermore, with option 1, if it is allowed to receive paging messages on LR band and reselects to another band to perform random access, there is a big impact on UE behaviors in RRC_IDLE/INACTIVE as currently the UE is only allowed to camp on one cell and perform random access on the same cell.

Observation 2: When LR and MR are allowed to operate in different bands/carriers:

- Serving cell measurement result based on LR is required to be a mandatory configuration for exit condition for LP-WUS monitoring;

- Serving cell measurement offloading is not allowed. Furthermore, it is questionable whether serving cell measurement based on MR is allowed to be relaxed. These bring negative impacts on the power saving gain of LP-WUS;

- Additional UE capabilities may be required for LP-WUS operation;

- Great impact on UE behaviors in RRC_IDLE/INACTIVE will be introduced, if it is allowed to receive paging messages on LR band and reselects to another band to perform random access.

Option 2: LR and MR are operating in the same band/carrier. LP-WUS UEs consider the band supporting LP-WUS with higher priority.

With Option 2, LP-WUS UEs consider the band supporting LP-WUS with higher priority during cell reselection procedure. This brings impacts on cell reselection procedure. Furthermore, considering LP-WUS UEs tend to camp on the band(s) supporting LP-WUS, the load of cell(s) supporting LP-WUS will be high.

Observation 3: When LR and MR are operating in the same band/carrier and LP-WUS UE considers the band supporting LP-WUS with higher priority, cell reselection procedure is impacted and the load of cell(s) supporting LP-WUS is increased.

Option 3: LR and MR are operating in the same band/carrier. LP-WUS UEs don’t consider the band supporting LP-WUS with higher priority.

With Option 3, LP-WUS UEs perform cell reselection procedure as legacy. No further RAN2 impact is foreseen. But the possibility that LP-WUS UEs perform LP-WUS monitoring is lower than Option 1 and Option 2.

Observation 4: When LR and MR are operating in the same band/carrier and LP-WUS UEs don’t consider the band supporting LP-WUS with higher priority, no further RAN2 impact is foreseen while the possibility that LP-WUS UEs perform LP-WUS monitoring is lower than other options.

In RAN4#112bis meeting, the following agreements were achieved.

Agreement: - RAN4 will further discuss whether any technical issues regarding the LS from RAN1. - From Rel-19 RAN4 RRM requirement of MR offloading and relaxation perspective, RAN4 assumed LR and MR are operating on the same carrier frequency as baseline. FFS for the case of MR and LR working on different carrier frequencies if it is supported in RAN1/2.

Considering the limited time budget, RAN2 assume LR and MR are operating on the same band/carrier as baseline and reply the RAN1 LS with the above observations.

Proposal 1: RAN2 assume LR and MR are operating on the same band/carrier as baseline.

Proposal 2: Reply RAN1 LS with Observation 2, 3 and 4:

- When LR and MR are allowed to operate in different bands/carriers:

    - Serving cell measurement result based on LR is required to be a mandatory configuration for exit condition for LP-WUS monitoring;

    - Serving cell measurement offloading is not allowed. Furthermore, it is questionable whether serving cell measurement based on MR is allowed to be relaxed. These bring negative impacts on the power saving gain of LP-WUS;

    - Additional UE capabilities may be required for LP-WUS operation;

    - Great impact on UE behaviors in RRC_IDLE/INACTIVE will be introduced, if it is allowed to receive paging messages on LR band and reselects to another band to perform random access.

- When LR and MR are operating in the same band/carrier and LP-WUS UE considers the band supporting LP-WUS with higher priority, cell reselection procedure is impacted and the load of cell(s) supporting LP-WUS is increased.

- When LR and MR are operating in the same band/carrier and LP-WUS UEs don’t consider the band supporting LP-WUS with higher priority, no further RAN2 impact is foreseen while the possibility that LP-WUS UEs perform LP-WUS monitoring is lower than other options.

UE_ID based LP-WUS Subgrouping

In RAN2#127bis meeting, it was agreed similar formula defined for PEI subgrouping is reused for UE_ID based LP-WUS subgrouping. The following is the formula for UE_ID based PEI subgrouping in TS 38.304 [2]:

SubgroupID = (floor(UE_ID/(N*Ns)) mod subgroupsNumForUEID) + (subgroupsNumPerPO - subgroupsNumForUEID), where: N: number of total paging frames in T, which is the DRX cycle of RRC_IDLE state as specified in clause 7.1 Ns: number of paging occasions for a PF UE_ID: 5G-S-TMSI mod X, where X is 32768, if eDRX is applied; otherwise, X is 8192 subgroupsNumForUEID: number of subgroups for UE_ID based subgrouping in a PO, which is broadcasted in system information

According to previous discussions, there are two possible options for the formula for UE_ID based LP-WUS subgrouping.

Option 1: The subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping.

For example, SubgroupID_LP = (floor (UE_ID/(N*Ns*k)) mod subgroupsNumForUEID_LP) + (subgroupsNumPerPO_LP – subgroupsNumForUEID_LP), where k is equal to the subgrouping number for UE_ID based PEI subgrouping.

Option 2: The formula for UE_ID based PEI subgrouping is reused.

SubgroupID_LP = (floor (UE_ID/(N*Ns)) mod subgroupsNumForUEID_LP) + (subgroupsNumPerPO_LP – subgroupsNumForUEID_LP)

1) Whether UEs belonging to the same LP-WUS subgrouping can be assigned to different PEI subgroupings as much as possible

In Option 1, bits within UE_ID for UE_ID based PEI subgrouping are excluded when the subgroup UE_ID of LP-WUS is determined. As different bits within UE_ID are used for subgroup UE_ID of PEI and LP-WUS, UEs belong to the same LP-WUS subgrouping can be assigned to different PEI subgrouping as much as possible.

With the Option 2, if the maximum number for UE_ID based LP-WUS subgrouping is not equal to that for UE_ID based PEI subgrouping, different IDs for UE_ID based PEI subgrouping may be assigned for the same ID of UE_ID based LP-WUS subgrouping. But we should avoid subgroupsNumForUEID_LP (i.e. the maximum number of subgroups for LP-WUS UE_ID based subgrouping) is an integer multiple of subgroupsNumForUEID (i.e. the maximum number of subgroups for PEI UE_ID based subgrouping), otherwise UEs belong to the same LP-WUS UE_ID based subgrouping are assigned to the same PEI UE_ID based subgrouping. Considering the formula in Option 2 is similar as that for UE_ID based PEI subgrouping, the rate that UEs belong to the same LP-WUS subgrouping are assigned to different PEI subgrouping with Option 2 is lower than that with Option 1.

Observation 5: Both Options, including Option 1 the subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping and Option 2 the formula for UE_ID based PEI subgrouping is reused, can achieve that UEs belonging to the same LP-WUS subgrouping can be assigned to different PEI subgroupings as much as possible.

2) Whether it need to extend the extended IE for RAN-originated paging in RAN node.

For CN paging, the RAN is provided with the entire UE ID, but for RAN paging, RAN node is provided with an extended IE of 16 LSBs as shown below [3].

#### 9.3.3.52 Extended UE Identity Index Value This IE is used by the NG-RAN node to calculate the Paging Frame and Paging Occasion as specified in TS 36.304 [29], the Paging Frame and Paging Occasion for eDRX and the UE_ID based subgroup ID as specified in TS 38.304 [12]. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Extended UE Identity Index Value; M; BIT STRING (SIZE(16)).

For DRX, the maximum number of Ns is 4 while the maximum number of N is 256. The part of UE ID which determines the PO is the up to 10 LSBs (least significant bits), and part of UE ID used for PEI is the up to 3 LSBs next to the ones used to determine the PO. That is to say, up to 13 LSBs is used if a UE is configured with PEI. For eDRX, the maximum number of Ns is 4 while the maximum number of N is 1024. The part of UE ID which determines the PO is the up to 12 LSBs (least significant bits) and up to 15 LSBs is used if a UE is configured with PEI.

With the extended IE for RAN-originated paging, the left bit(s) for LP-WUS with the above two options are shown in Figure 1.

Figure 1: left bits for LP-WUS with different options for the extended IE for RAN-originated paging

In RAN1#118bis meeting, the following work assumption was made. It can be seen that 32 subgrouping has been assumed by RAN1. With this assumption, up to 5 bit within UE_ID will be used for UE_ID bases LP-WUS subgrouping.

Working Assumption The maximum number of subgroups per PO supported in Rel-19 is 32.

Observation 6: Up to 5 bits in UE_ID will be used for UE_ID based LP-WUS subgrouping when the maximum number of subgroups per PO is 32 which is assumed by RAN1.

With the work assumption for LP_WUS subgroups, the UE-ID in RAN node for RAN-originated paging, may be not enough for LP-WUS subgrouping with the above two options.

With Option 1, only 3 bits are left for LP-WUS subgrouping for the extended IE for RAN-originated paging in RAN node for DRX case. That means for RAN-originated paging, the maximum number for UE_ID based LP-WUS subgrouping is 8 which is far less than the maximum number of subgroups for LP-WUS assumed by RAN1, i.e. 32. The left bit in extended IE for RAN-originated paging in RAN node is not enough for LP-WUS which consumes 5 bits at most. In order to support UE_ID based LP-WUS with Option 1, the extended IE for RAN-originated paging in RAN node needs to be extended to at least 18bits for DRX and 20bits for eDRX.

With Option 2 for DRX, the extended IE for RAN-originated paging in RAN node is enough as there are 6 bits left for LP-WUS subgrouping. But for eDRX with option 2, the extended IE for RAN-originated paging in RAN node is still not enough and need to be extended to at least 17bits.

Observation 7: If the subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping, the extended IE for RAN-originated paging in RAN node needs to be extended to at least 18bits for DRX case and at least 20bits if eDRX with LP-WUS is supported.

Observation 8: If the formula for UE_ID based PEI subgrouping is reused, the extended IE for RAN-originated paging in RAN node needs to be extended to at least 17bits if eDRX with LP-WUS is supported while the existing extended IE for RAN-originated paging in RAN node is enough if eDRX with LP-WUS is not supported.

RAN2 discuss which option is adopted and send LS to SA2/RAN3/CT1 for the extended bits for extended IE for RAN-originated paging in RAN node if needed.

Proposal 3: RAN2 discuss which option is adopted:

- Option 1: The subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping.

- Option 2: The formula for UE_ID based PEI subgrouping is reused.

Proposal 4: RAN2 discuss to send LS to SA2/RAN3/CT1 to ask whether the bits for extended UE ID for RAN paging-originated can be extended.

Configurations for derivation of serving cell measurement quality from LR

In RAN2#126 meeting, it was agreed:

- The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE: - - LP-SS configuration - - LP-WUS configuration - - Entry/exit condition for LP-WUS monitoring (FFS if it always configured)

According to the agreements for entry/exit condition for LP-WUS monitoring, serving cell quality from LR needs to be defined. And multi-beam operation is supported for LP-WUR. Therefore, RAN2 needs to discuss how to derive serving cell measurement quantity from LR.

For cell reselection, how to derive a cell measurement quantity of MR in multi-beam operations is captured in TS 38.304 [2] as follows:

For cell selection in multi-beam operations, measurement quantity of a cell is up to UE implementation. For cell reselection in multi-beam operations, including inter-RAT reselection from E-UTRA to NR, the measurement quantity of this cell is derived amongst the beams corresponding to the same cell based on SS/PBCH block as follows: - if nrofSS-BlocksToAverage (maxRS-IndexCellQual in E-UTRA) is not configured in SIB2/SIB4 (SIB24 in E-UTRA); or - if absThreshSS-BlocksConsolidation (threshRS-Index in E-UTRA) is not configured in SIB2/SIB4 (SIB24 in E-UTRA); or - if the highest beam measurement quantity value is below or equal to absThreshSS-BlocksConsolidation (threshRS-Index in E-UTRA): - derive a cell measurement quantity as the highest beam measurement quantity value, where each beam measurement quantity is described in TS 38.215 [11]. - else: - derive a cell measurement quantity as the linear average of the power values of up to nrofSS-BlocksToAverage (maxRS-IndexCellQual in E-UTRA) of highest beam measurement quantity values above absThreshSS-BlocksConsolidation (threshRS-Index in E-UTRA).

The derivation of cell measurement quantity from MR with multi-beam operation can be reused for LR, e.g. derivation of cell measurement quality from LR as the linear average of the power values of up to N highest beam measurement quantity values above a threshold. And similar to derivation of cell measurement quantity from MR, parameters for derivation of serving cell from LR, e.g. the threshold for beams used for derivation of cell measurement quantity and the number of beams (e.g. N ) used for derivation of cell measurement quantity, need to be included in SIB for LP-WUS monitoring in RRC_IDLE/INACTIVE.

Proposal 5: Cell measurement quality from LR is derived from the linear average of the power values of up to N highest beam measurement quantity values above a threshold. Parameters for derivation of serving cell from LR in SIB for LP-WUS monitoring in RRC_IDLE/INACTIVE, including:

- the threshold for beams used for derivation of cell measurement quantity;

- N, the number of beams to average for cell measurement derivation.

Conclusion

According to the analysis in section 2, we propose:

RAN2 impact on RAN1 LS:

Observation 1: RRM measurements based on both MR and LR may be needed to evaluate the channel condition on MR and LR respectively, if LR and MR can operate in different bands/carriers.

Observation 2: When LR and MR are allowed to operate in different bands/carriers:

- Serving cell measurement result based on LR is required to be a mandatory configuration for exit condition for LP-WUS monitoring;

- Serving cell measurement offloading is not allowed. Furthermore, it is questionable whether serving cell measurement based on MR is allowed to be relaxed. These bring negative impacts on the power saving gain of LP-WUS;

- Additional UE capabilities may be required for LP-WUS operation;

- Great impact on UE behaviors in RRC_IDLE/INACTIVE will be introduced, if it is allowed to receive paging messages on LR band and reselects to another band to perform random access.

Observation 3: When LR and MR are operating in the same band/carrier and LP-WUS UE considers the band supporting LP-WUS with higher priority, cell reselection procedure is impacted and the load of cell(s) supporting LP-WUS is increased.

Observation 4: When LR and MR are operating in the same band/carrier and LP-WUS UEs don’t consider the band supporting LP-WUS with higher priority, no further RAN2 impact is foreseen while the possibility that LP-WUS UEs perform LP-WUS monitoring is lower than other options.

Proposal 1: RAN2 assume LR and MR are operating on the same band/carrier as baseline.

Proposal 2: Reply RAN1 LS with Observation 2, 3 and 4:

- When LR and MR are allowed to operate in different bands/carriers:

    - Serving cell measurement result based on LR is required to be a mandatory configuration for exit condition for LP-WUS monitoring;

    - Serving cell measurement offloading is not allowed. Furthermore, it is questionable whether serving cell measurement based on MR is allowed to be relaxed. These bring negative impacts on the power saving gain of LP-WUS;

    - Additional UE capabilities may be required for LP-WUS operation;

    - Great impact on UE behaviors in RRC_IDLE/INACTIVE will be introduced, if it is allowed to receive paging messages on LR band and reselects to another band to perform random access.

- When LR and MR are operating in the same band/carrier and LP-WUS UE considers the band supporting LP-WUS with higher priority, cell reselection procedure is impacted and the load of cell(s) supporting LP-WUS is increased.

- When LR and MR are operating in the same band/carrier and LP-WUS UEs don’t consider the band supporting LP-WUS with higher priority, no further RAN2 impact is foreseen while the possibility that LP-WUS UEs perform LP-WUS monitoring is lower than other options.

UE_ID based LP-WUS Subgrouping:

Observation 5: Both Options, including Option 1 the subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping and Option 2 the formula for UE_ID based PEI subgrouping is reused, can achieve that UEs belonging to the same LP-WUS subgrouping can be assigned to different PEI subgroupings as much as possible.

Observation 6: Up to 5 bits in UE_ID will be used for UE_ID based LP-WUS subgrouping when the maximum number of subgroups per PO is 32 which is assumed by RAN1.

Observation 7: If the subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping, the extended IE for RAN-originated paging in RAN node needs to be extended to at least 18bits for DRX case and at least 20bits if eDRX with LP-WUS is supported.

Observation 8: If the formula for UE_ID based PEI subgrouping is reused, the extended IE for RAN-originated paging in RAN node needs to be extended to at least 17bits if eDRX with LP-WUS is supported while the existing extended IE for RAN-originated paging in RAN node is enough if eDRX with LP-WUS is not supported.

Proposal 3: RAN2 discuss which option is adopted:

- Option 1: The subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping.

- Option 2: The formula for UE_ID based PEI subgrouping is reused.

Proposal 4: RAN2 discuss to send LS to SA2/RAN3/CT1 to ask whether the bits for extended UE ID for RAN paging-originated can be extended.

Configurations for derivation of serving cell measurement quality from LR:

Proposal 5: Cell measurement quality from LR is derived from the linear average of the power values of up to N highest beam measurement quantity values above a threshold. Parameters for derivation of serving cell from LR in SIB for LP-WUS monitoring in RRC_IDLE/INACTIVE, including:

- the threshold for beams used for derivation of cell measurement quantity;

- N, the number of beams to average for cell measurement derivation.

Reference

- R2-2407921, LS on LP-WUS operation in IDLE/INACTIVE mode.

- 3GPP TS 38.304 V18.2.0, User Equipment (UE) procedures in Idle mode and RRC Inactive state.

- 3GPP TS 38.413 V18.2.0, NG-RAN, NG Application Protocol (NGAP).