# 3GPP TSG-RAN WG2 Meeting #128	                               R2-2409739
Orlando, US, 18 – 22 Nov, 2024

Agenda Item	: 8.2.3 (FS_Ambient_IoWT_solutions)

Source	: LG Electronics Inc.

Title	: Discussion on ambient IoT paging

Document for	: Discussion and Decision

Introduction

In this contribution, we show our views on the A-IOT paging.

Discussion

2.1 Indication for avoiding duplicated response from the A-IOT devices

In RAN2#127, it is agreed that reader can trigger multiple/subsequent A-IoT paging messages that are associated with the same request from the CN. Since the reader trigger the multiple A-IoT paging messages for the same request, the duplicated response from the multiple A-IOT devices may happen. To prevent this, in RAN2#127bis, it was agreed that the information should be included to prevent the duplication of the response from the A-IOT devices as follows.

1 The A-IoT paging message can include information to avoid duplicate response from the device to a reader. This information should be short and simple. FFS how to indicate. Wait for further information and requirements from other WGs and make this decision in normative phase.

During the discussion in RAN2#127bis, two options were proposed as follows.

- Option 1. The transaction ID is contained in an A-IOT paging message, i.e., multi-bit indication.

  - The transaction ID is increased when the reader receives a new service request for a same service from the CN. The A-IOT device can perform the re-access procedure if the transaction ID is not changed in the A-IOT paging message.

- Option 2. The NDI-like indication is contained in an A-IOT paging message, i.e., one-bit indication.

  - The NDI-like indication is toggled when a new service request is received from the CN. In this option, even if the subsequent A-IOT paging message is transmitted, the NDI-like indication is not toggled. The A-IOT device can perform the re-access procedure if the NDI-like indication is not toggled in the A-IOT paging message.

The main difference between option 1 and option 2 is whether to use the one-bit indication or multi-bit indication. In our view, both options may cause an error case where the A-IOT device considers the new A-IOT paging message as the subsequent A-IOT paging message for the re-access procedure. The following figure shows the problematic case for each option.

As shown above figure, both options cannot always avoid the error case. However, the multi-bit indication can significantly reduce the probability of occurrence for error case compared to the one-bit indication. In addition, even if the multi-bit indication causes the signalling overhead for A-IOT paging message, the signalling overhead for A-IOT paging message is not a big problem because only two-bit indication may be sufficient for the multi-bit indication. Thus, we prefer to introduce the multi-bit indication to avoid duplicated responses from the A-IOT devices.

Proposal 1. Introduce the multi-bit indication, e.g., transaction ID, in the A-IOT paging message to avoid duplicated responses from the A-IOT devices.

2.2 FDM for A-IOT paging message

In RAN1, RAN1 decided that the frequency-domain multiple access of D2R transmission will be studied. In addition, RAN1 made an agreement on that the frequency-domain multiple access is not supported for R2D transmissions.

Agreement – RAN1#116bis - Study frequency-domain multiple access of D2R transmissions, at least by utilizing a small frequency-shift in baseband. Further details, including pros/cons, are FFS Agreement – RAN2#118bis Capture in the TR that for R2D: - Time-domain multiplexing is the baseline. - Code-domain multiplexing is not considered for device 1/2a/2b. - Frequency-domain multiplexing is not considered for the devices with RF-ED receiver, and studied for Device 2b with IF-ED or ZIF receiver considering the technical aspects in Table 6.1.1.x-1.

Based on the above agreements, the following scenario can be considered.

As shown above figure, if the FDM is supported in A-IOT, the A-IOT device needs to support the transmission of the D2R transmission, i.e., Msg1 transmission, among the indicated frequency. In other words, the reader should indicate the frequency for D2R transmission to the A-IOT device. For this, we think that the simple way is that the list of the frequency information for D2R transmission is indicated in an A-IOT paging message. Then, if the A-IOT device receives the A-IOT paging message, the A-IOT device randomly selects an access occasion and the frequency to transmit the Msg1.

Proposal 2: If FDM is supported for A-IOT in RAN1, the list of the frequency information for R2D transmission, i.e., Msg1 transmission, is indicated in an A-IOT paging message.

Conclusion

Based on the above discussion, we made following proposals and observations.

Proposal 1. Introduce the multi-bit indication, e.g., transaction ID, in the A-IOT paging message to avoid duplicated responses from the A-IOT devices.

Proposal 2: If FDM is supported for A-IOT in RAN1, the list of the frequency information for R2D transmission, i.e., Msg1 transmission, is indicated in an A-IOT paging message.