3GPP TSG-RAN WG2 Meeting #128		R2- 2409762

Orlando, USA, 18th – 22nd Nov. 2024

Source:	vivo

Title:	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

Agenda Item:	8.4.3

Document for:	Discussion and Decision

- Introduction

In RAN#105 meeting, the revised WID for R19 LP-WUS was agreed [1]. In the WID [1], the objective of the RRC_IDLE/INACTIVE modes is as follows:

- For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

In previous RAN2 meetings, the following agreements for RRM measurement relaxation and offloading in RRC_IDLE and INACTIVE have been achieved [2] [3] [4].

RAN2#126: - For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell. - RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise. - RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’. RAN2#127: - RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs. - For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. - The exit condition is based on the LR measurement results. RAN2#127bis: Working assumption For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

In this contribution, we will further discuss the issues for MR serving/neighboring cell RRM measurements relaxation and serving cell RRM measurement offloading from MR to LP-WUR for RRC_IDLE/INACTIVE mode.

- Discussion

Regarding the specific criteria for MR serving cell RRM measurement relaxation/offloading and MR neighboring cell measurement relaxation, it could simply refer to the relaxation criteria defined for neighboring cell measurement decided in Rel-16/17, which include “serving cell quality” and “low mobility”.

For “serving cell quality” criteria, considering RAN2 already made the following agreements [3] for the entry/exit conditions for LP-WUS monitoring and serving cell measurement offloading, in our view, the “serving cell quality” criteria for serving cell RRM measurement relaxation could also use the same principle, i.e., based on serving cell quality measurement via MR and LR measurement could also be considered.

Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. - The exit condition is based on the LR measurement results.

Regarding whether the “low mobility” criteria could be used for serving cell measurement relaxation/offloading, it should be postponed since the issue may also be related to whether the relaxation criteria for serving cell relaxation/offloading is the same as or separate from the entry condition for LP-WUS monitoring.

Proposal 1: The criteria for serving cell measurement relaxation include good serving cell quality, i.e. the serving cell quality via MR is better than a threshold, and LR measurement could also be considered. FFS whether other criteria, e.g. low mobility.

According to the RAN2 agreement above, for the entry condition of serving cell measurement offloading, the MR-based threshold is always configured and the LR-based threshold is optionally configured.

Observation 1: For the entry condition of serving cell measurement offloading, the MR-based threshold is always configured and the LR-based threshold is optionally configured.

In case the LR-based threshold is also configured together with the MR-based threshold for the entry condition, how UE determines the entry condition for serving cell measurement offloading is fulfilled should be decided, i.e. when either condition or both conditions are fulfilled. Considering the MR-based entry condition has been agreed as the baseline, and in some case, MR-based measurement can not reflect the channel condition of LR measurement, then, LR-based measurement should be considered as the complimentary on top of MR-based measurement for the entry condition. Thus, it is more reasonable that the entry condition is fulfilled when both the MR-based threshold and LR-based threshold are met. Besides, the decision on the entry condition for LP-WUS monitoring has been made as below. Similar conclusion should be made for the condition of serving cell RRM measurement relaxation or offloading.

If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s).

Proposal 2: If NW configures thresholds for both MR and LR measurements for the entry condition of serving cell RRM measurement relaxation or offloading, then the entry condition is met when all the measured results are above the configured threshold(s).

As usual, RAN2 should work on the criteria, while the detailed RRM relaxation/offloading mechanism, e.g. relaxation factor or relaxed requirement would be discussed and determined by RAN4. In RAN2#126 meeting, RAN2 agreed to specify the offloading criterion for serving cell in RAN2, and assume RAN4 will define the specific requirements for serving cell offloading. Similarly, for serving cell measurement relaxation, the serving cell MR measurement relaxation criteria should also be discussed in RAN2, and the specific relaxation factors and relaxed requirements for serving cell measurement relaxation should also be discussed by RAN4.

Besides, in RAN2#127bis meeting, RAN2 made the following agreements for neighboring cell measurement relaxation.

- Working assumption For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

For the specific relaxation factors and relaxed requirements, investigation on the power saving gain when MR enters RRM relaxation mode compared with I-DRX with or without PEI has been carried out at SI phase for LP-WUS duty-cycled monitoring and continuously monitoring. The results are captured in the section 8.1.1.5 and 8.1.1.6 in TR [5] and summarized in the following table:

Table 1 Power saving gain against MR relaxation times (both serving and neighboring cell measurement)

| MR RRM relaxation times | mean power saving gain average (LP-WUR duty-cycled mode) | mean power saving gain average (LP-WUR continuously mode) |
| --- | --- | --- |
| MR relaxed < 8 times | average: 4% | average: 22% |
| 8 times<= MR relaxed <=16 times | average: 40% | average: 32% |
| MR relaxed > 16 times | average: 60% | average: 57% |
| MR offload RRM to LR | average: 83% | average: 74% |

From the above outcome, it is observed a significant power saving gain increase happens when the relaxation times of MR changes from less than 8 to between 8 and 16 for the LP-WUR duty-cycled mode. In general, at least relaxation times of 8 is needed to achieve a reasonable 40% power saving gain based on summary in Table 1. So we think further relaxation for neighboring cell measurement (i.e. MR relaxed >= 8 times) is needed. However, it is also up to RAN4 to discuss and determine whether legacy relaxation or further relaxation is used for neighboring cell measurement.

Proposal 3: It is up to RAN4 to discuss/determine the detailed relaxation requirements for serving/neighboring cell MR measurement, including:

- Serving cell relaxation factor and relaxed requirements;

- Neighboring cell relaxation factor and relaxed requirements, including whether/how to introduce further relaxation.

Regarding whether the relaxation criteria/threshold for serving cell measurement relaxation/offloading and neighbor cell measurement relaxation are the same as or separate from the entry condition/threshold for LP-WUS monitoring, which is described as the following directions, RAN2 needs further discussion.

- Direction 1: the criteria/threshold for RRM relaxation/offloading are the same as the entry condition for LP-WUS monitoring.

- Direction 2: the criteria/ threshold for RRM relaxation/offloading are separate from the entry condition for LP-WUS monitoring.

After the study in Rel-18, the following conclusion was made in TR 38.869 [5]:

UE power saving gain cannot be observed if the existing Rel-18 MR RRM measurement periodicity for serving and neighbour cells are applied and UE MR enters in ultra-deep sleep during LP LP-WUS monitoring, therefore MR serving and neighbour cell measurement with further time domain relaxation than that is allowed in Rel-18 specification for IDLE/INACTIVE and/or at least serving cell RRM offloaded from MR to LR are beneficial.

It could be observed that there is no power saving gain if UE starts to monitor LP-WUS while the serving cell measurement relaxation/offloading or neighbor cell measurement relaxation has not been performed. In this way, if the criteria/threshold for RRM relaxation/offloading are separate from the entry condition for LP-WUS monitoring, i.e. Direction 2, there may be a risk that the entry condition for LP-WUS monitoring is fulfilled, but the RRM relaxation/offloading criteria have not been fulfilled, which is not expected.

Observation 2: There is no power saving gain that UE monitors LP-WUS without RRM measurement relaxation/offloading.

Thus, network configuration on the specific threshold of the RRM relaxation/offloading criteria and entry condition for LP-WUS monitoring should avoid such case without any power saving gain. In order to guarantee serving/neighboring cell measurement is relaxed when UE monitors LP-WUS, thresholds of the criteria for the serving/neighboring cell measurement relaxation should be lower than or equal to the threshold of the entry condition for LP-WUS monitoring. But for simplicity, it is better to adopt the same threshold.

Proposal 4: The threshold of the criteria for serving/neighboring cell measurement relaxation should be at least equal to the threshold of the entry condition for LP-WUS monitoring.

As for serving cell measurement offloading, considering the entry conditions of LP-WUS monitoring include at least serving cell quality via MR, serving cell measurement via MR can not be offloaded to LR before UE enters the LP-WUS monitoring state. Thus thresholds of the criteria for the serving cell measurement offloading should be higher than or equal to the threshold of the entry condition for LP-WUS monitoring.

Proposal 5: The threshold of the criteria for serving cell measurement offloading should be higher than or equal to the threshold of the entry condition for LP-WUS monitoring.

Based on the analysis above, we provide two examples to illustrate the thresholds and UE behaviors for the serving cell measurement relaxation/offloading and neighboring cell measurement relaxation comparing to the threshold of entry condition on LP-WUS monitoring in the following figures.

Figure 1: Thresholds and UE behaviors for serving cell offloading/neighboring cell measurement relaxation and LP-WUS monitoring

Figure 2: Thresholds and UE behaviors for serving/neighboring cell measurement relaxation and LP-WUS monitoring

For simplicity, the serving cell measurement offloading and serving cell measurement relaxation are provided in Figure 1 and Figure 2, respectively. All the criteria for serving cell measurement relaxation/offloading and neighboring cell measurement relaxation and the entry condition for LP-WUS monitoring are based on the serving cell quality via MR.

For neighbor cell measurement relaxation, it refers to the neighboring cell measurement for intra-frequency/inter-frequency with equal or lower priority frequency. As for the inter-frequency with higher priority, it will be more complicated and can be further discussed later. When the serving cell quality based on the measurement via MR exceeds the legacy S-measure threshold, there is no neighboring cell measurement to be performed according to the current specification. Besides, it is plain and obvious that the threshold of the legacy S-measure threshold is higher than or equal to the threshold for the neighboring cell measurement relaxation (i.e. further relaxation is used to differentiate the RRM relaxation in Rel-16/17), since the neighboring cell measurement can be further relaxed only in case UE needs to perform neighboring cell measurement.

In Figure 1, there are two alternatives for the threshold of entry condition for LP-WUS monitoring and the threshold of serving cell offloading criteria with respect to the S-measure threshold, which could either be higher (i.e. Case 1) or lower (i.e. Case 2) than legacy S-measure threshold with the further details as below. Besides, as the analysis above, the threshold of the criteria for the serving cell measurement offloading should be higher than or equal to the threshold of the entry condition for LP-WUS monitoring. For the green arrow, it represents that UE performs serving cell offloading if the serving cell offloading criteria are fulfilled.

- For Case 1, i.e. the threshold of serving cell offloading criteria and the threshold of criteria for LP-WUS monitoring is higher than legacy S-measure threshold, when the entry condition for LP-WUS monitoring is fulfilled and the serving cell offloading criteria is fulfilled, UE will monitor LP-WUS and perform serving cell measurement offloading from MR to LR and UE won’t perform neighboring cell measurement.

- For Case 2, i.e. the threshold of serving cell offloading criteria and the threshold of criteria for LP-WUS monitoring is lower than legacy S-measure threshold, UE will monitor LP-WUS and perform serving cell measurement offloading from MR to LR, and UE performs relaxed neighboring cell measurement via MR.

In Figure 2, similar to Figure 1, there are two alternatives for the threshold of entry condition for LP-WUS monitoring and serving cell measurement relaxation criteria with respect to the S-measure threshold, i.e., Case 1 and Case 2 with the further details as below.

- For Case 1, i.e. the threshold of serving cell relaxation criteria and the threshold of criteria for LP-WUS monitoring is higher than legacy S-measure threshold, when the entry condition for LP-WUS monitoring is fulfilled and the serving cell measurement relaxation is fulfilled, UE will monitor LP-WUS and perform serving cell measurement relaxation and no neighboring cell measurement.

- For Case 2, i.e. the threshold of serving cell relaxation criteria and the threshold of criteria for LP-WUS monitoring is lower than legacy S-measure threshold, UE will monitor LP-WUS and perform serving cell measurement relaxation, and MR neighboring cell measurement relaxation.

For relaxation on neighboring cell measurement, according to the WID [1], the LR cannot perform neighboring cell measurement and only MR is concerned, which is the same as R16/17 neighboring cell measurement relaxation. In our view, the only difference is that the measurement relaxation for neighboring cell could be further relaxed than Rel-16/17.  If RAN4 decides to introduce further relaxation for neighboring cell measurement, the thresholds of R19 neighboring cell measurement relaxation should be stricter than that of R16/17 neighboring cell measurement relaxation.

Proposal 6: The corresponding threshold of R19 neighboring cell measurement relaxation should be stricter than that of R16/17 neighboring cell measurement relaxation if RAN4 decides to introduce further relaxation for neighboring cell measurement.

Regarding the specific metrics for RRM measurement offloading/relaxation criteria, in our view, the metrics for legacy RRM relaxation criteria and entry/exit condition for LP-WUS monitoring can be considered as baseline., i.e., the measurement metric is based on RSRP and optionally on RSRQ. In detail, in some cases, RSRP is enough to reflect the cell quality, while it is up to network to configure whether RSRQ is needed.

Proposal 7: The metrics for RRM measurement offloading/relaxation criteria include (LP-)RSRP and optional (LP-)RSRQ.

Except for the criteria, the specific signalling to carry the RRM offloading/relaxation conditions and parameters should be also discussed. For R16/17 neighboring cell measurement relaxation, the corresponding conditions and parameters are configured in SIB2. We think it is straightforward to follow the same design for R19 RRM offloading/relaxation conditions and parameters.

Proposal 8: The R19 RRM offloading/relaxation conditions and parameters are configured in SIB2.

- Conclusion

In this contribution, we discuss the RRM relaxation of UE MR for both serving and neighboring cell measurements, and UE serving cell RRM measurement offloading from MR to LP-WUR. Based on the discussion, we have the following observations and proposals.

Observation 1: For the entry condition of serving cell measurement offloading, the MR-based threshold is always configured and the LR-based threshold is optionally configured.

Observation 2: There is no power saving gain that UE monitors LP-WUS without RRM measurement relaxation/offloading.

Proposal 1: The criteria for serving cell measurement relaxation include good serving cell quality, i.e. the serving cell quality via MR is better than a threshold, and LR measurement could also be considered. FFS whether other criteria, e.g. low mobility.

Proposal 2: If NW configures thresholds for both MR and LR measurements for the entry condition of serving cell RRM measurement relaxation or offloading, then the entry condition is met when all the measured results are above the configured threshold(s).

Proposal 3: It is up to RAN4 to discuss/determine the detailed relaxation requirements for serving/neighboring cell MR measurement, including:

- Serving cell relaxation factor and relaxed requirements;

- Neighboring cell relaxation factor and relaxed requirements, including whether/how to introduce further relaxation.

Proposal 4: The threshold of the criteria for serving/neighboring cell measurement relaxation should be at least equal to the threshold of the entry condition for LP-WUS monitoring.

Proposal 5: The threshold of the criteria for serving cell measurement offloading should be higher than or equal to the threshold of the entry condition for LP-WUS monitoring.

Proposal 6: The corresponding threshold of R19 neighboring cell measurement relaxation should be stricter than that of R16/17 neighboring cell measurement relaxation if RAN4 decides to introduce further relaxation for neighboring cell measurement.

Proposal 7: The metrics for RRM measurement offloading/relaxation criteria include (LP-)RSRP and optional (LP-)RSRQ.

Proposal 8: The R19 RRM offloading/relaxation conditions and parameters are configured in SIB2.

- References

- RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO

- R2-2405702, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS

- R2-2407572, Report from session on R18 MIMOevo, R18 MUSIM, R19 LP-WUS and R19 SBFD

- R2-2409212, Report from session on Rel-18 MIMO and MUSIM, Rel-19 MIMO, LPWUS, and SBFD

- 3GPP TR 38.869 v18.0.0