3GPP TSG-RAN2#128									           R2-2409786

Orlando, USA, 14 - 18 Nov, 2024

Source: 			ZTE Corporation, Sanechips

Title: 	LCP enhancements for XR

Agenda item:	8.7.4.1

Document for: 	Discussion and Decision

# Introduction

Rel-19 XR WI in RP-240791 has an objective for RAN2 to study and if justified specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity.

In RAN2#125bis meeting, the following related agreements has been made [2]:

- RAN2 will study whether/how to resolve the issue of data with low remaining time being delayed due to other data from LCHs with higher LCH priority when using the existing LCP procedure. At least the following alternatives will be studied: - Alternative 1: Enhance LCP restrictions/LCH selection. - Alternative 2: Enhance LCH prioritization. - RAN2 should consider potential impact on traffic from SRBs.

In RAN2#126 meeting, the following related agreements has been made [3]:

LCP enhancements – LCH prioritization - Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR. - The solution should consider impact on UE complexity (as already indicated in SI objective description) - For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - Use additional priority configured to LCHs in case of these LCHs with delay-critical data - FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH - We try to avoid RAN1 impacts. - RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way. - For LCP restrictions based solutions, RAN2 will not discuss solutions requiring RAN1 work. FFS whether other LCP restrictions based approaches are needed/beneficial - The solutions should not disallow non-delay critical data from using an UL grant. - LCP prioritization within a logical channel will not be considered in RAN2 discussions - FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR). DSR enhancements – multiple thresholds and grouping of data - Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG. - FFS whether DSR triggering is impacted - FFS whether PDU set importance needs to be included

In RAN2#127 meeting, the following related agreements has been made [4]:

LCP Enhancement - RAN2 to no longer consider the enhancement of the LCP restriction, as one of the candidate solutions for LCP enhancements in Rel-19 XR. - FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3). Enhanced DSR contents - Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG. - For enhanced DSR: - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

In RAN2#127bis meeting, the following related agreements has been made [5]:

- As a baseline, additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round. - FFS if we can still change the priority for the 2nd round to ensure fairness, but we need to consider tight timeline of LCP procedure and UE complexity. Companies can also check whether we can leave this to UE implementation - Introduce an independent per-LCH remaining time threshold for applying delay-critical priority. - We do not introduce any setting restrictions of this new remaining time threshold with relation to DSR triggering threshold. For now, we will use “LCH priority-adjusted data”.

In this contribution we will further investigate the possible scheduling enhancements per this objective.

# Discussion

It has been agreed to introduce an independent per-LCH remaining time threshold for applying delay-critical priority in RAN2, and LCP prioritization within a logical channel will not be considered in RAN2 discussions, based on the threshold, “LCH priority-adjusted data” can be determined per LCH, e.g. if there is data in one logical channel with remaining time less than or equal to the threshold, all the data in the logical channel will be prioritized for transmission, and the data in the logical channel will be called “LCH priority-adjusted data”.

Proposal 1: RAN2 confirms that “LCH priority-adjusted data” is all data in a logical channel with data having the remaining time less than or equal to the remaining time threshold.

When studying modifications to LCP, it is important to ensure that this doesn’t result in unforeseen consequences to other services. Specifically, it should be noted that LCP is used also to ensure that SRBs are prioritised over DRBs and certain MAC CEs are sent with higher priority than data. It should be kept in mind that during poor radio conditions there may be RAN level signalling data and/or MAC CEs which may be triggered (e.g. measurements/handover signalling etc). Allowing this signalling to go through unimpeded is important because this can resolve the poor radio conditions which actually cause the problems in the first place. Any delay to such signalling will prolong the adverse conditions at radio level and will only have overall negative consequences. In order to avoid impacts to signalling connection between UE and network it is important to ensure that any enhancements to LCP do not impact the priorities of SRBs over DRBs

Observation 1: During temporary deterioration of radio conditions, additional RAN level signalling may be generated (e.g. measurements/HO signalling) and this signalling is essential to quickly alleviate the problems in the first place.

In RAN2#127 meeting, it has been agreed that:

- RAN2 to no longer consider the enhancement of the LCP restriction, as one of the candidate solutions for LCP enhancements in Rel-19 XR. - FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3).

To improve the transmission for LCH priority-adjusted data, enhancements can be considered for the resource allocation stage of LCP: e.g. treat the priority of the logical channel with LCH priority-adjusted data is higher than the priority of the logical channel without LCH priority-adjusted data, and then allocate the resource based on LCH priority. e.g. the TP for TS 38.321.

##### 5.4.3.1.3 Allocation of resources ... Logical channels shall be prioritised in accordance with the following order (highest priority listed first): - MAC CE for C-RNTI, or data from UL-CCCH; - MAC CE for (Enhanced) BFR, or MAC CE for Configured Grant Confirmation, or MAC CE for Multiple Entry Configured Grant Confirmation; - MAC CE for Sidelink Configured Grant Confirmation; - MAC CE for LBT failure; - MAC CE for SL LBT failure according to clause 5.31.2; - MAC CE for Timing Advance Report; - MAC CE for Delay Status Report; - MAC CE for SL-BSR prioritized according to clause 5.22.1.6; - MAC CE for SL-PRS Resource Request; - MAC CE for (Extended) BSR, with exception of BSR included for padding; - MAC CE for (Enhanced) Single Entry PHR, or MAC CE for (Enhanced) Multiple Entry PHR or MAC CE for Single Entry PHR with assumed PUSCH, or MAC CE for Multiple Entry PHR with assumed PUSCH, or MAC CE for Enhanced Single Entry PHR for multiple TRP or MAC CE for Enhanced Multiple Entry PHR for multiple TRP, or MAC CE for Enhanced Single Entry PHR for multiple TRP STx2P or MAC CE for Enhanced Multiple Entry PHR for multiple TRP STx2P; - MAC CE for Positioning Measurement Gap Activation/Deactivation Request; - MAC CE for the number of Desired Guard Symbols; - MAC CE for Case-6 Timing Request; - MAC CE for (Extended) Pre-emptive BSR; - MAC CE for SL-BSR, with exception of SL-BSR prioritized according to clause 5.22.1.6 and SL-BSR included for padding; - MAC CE for IAB-MT Recommended Beam Indication, or MAC CE for Desired IAB-MT PSD range, or MAC CE for Desired DL Tx Power Adjustment; - Logical Channel priority-adjusted data; - data from any Logical Channel, except Logical Channel priority-adjusted data and data from UL-CCCH; - MAC CE for Recommended bit rate query; - MAC CE for BSR included for padding; - MAC CE for SL-BSR included for padding. NOTE 2: Prioritization among MAC CEs of same priority is up to UE implementation. NOTE 3: “logical channel priority-adjusted data” is all data in one logical channel with data that the remaining time is less than or equal to the remaining time threshold. NOTE 4: Prioritization among logical channel priority-adjusted data is further based on the logical channel priority. ...

This impacts specification less and does not impact the scheduling priority for non delay critical data.

Proposal 2: At the stage for allocation of resources in LCP, treat the priority of “LCH priority-adjusted data” (e.g. the data that the remaining time is below the remaining time threshold) is higher than the priority of the logical channel without “LCH priority-adjusted data”; and then allocate the resource based on LCH priority.

# Conclusion and proposals

The following observations/proposals are made:

Proposal 1: RAN2 confirms that “LCH priority-adjusted data” is all data in a logical channel with data having the remaining time less than or equal to the remaining time threshold.

Proposal 2: At the stage for allocation of resources in LCP, treat the priority of “LCH priority-adjusted data” (e.g. the data with the remaining time below the remaining time threshold) is higher than the priority of the logical channel without “LCH priority-adjusted data”; and then allocate the resource based on LCH priority.

# References

- RP-240791, Revised WID on XR (eXtended Reality) for NR Phase 3, RAN#103

- Draft_R2_125bis_Meeting_Report_v2

- Draft_R2_126_Meeting_Report_v2

- Draft_RAN2_127_Meeting_Report_v1

- Draft_RAN2_127b_Meeting_Report_v1