3GPP TSG-RAN WG2 Meeting#128	R2-2409826

Orlando, USA, 18th – 22nd Nov 2024

Source:	vivo

Title:	Clarification on SLPP session ID existence in SLPP message between target UE and LMF

Agenda Item:	7.1.3

Document for:	Discussion and Decision

- Introduction

According to TS 38.305 [3] and TS 38.355 [4], SLPP Session ID (i.e., the sessionID field) is assigned by target UE and contained in the SLPP messages used for communication between UEs; and SLPP session ID may be optionally included in the SLPP message for the communication between a SL target UE and the LMF. However, it’s not clear under what conditions the session ID is included or not in the SLPP message between target UE and LMF.

TS 38.305: ### 6.2.5 Sidelink Positioning Protocol (SLPP) […] An SLPP Session is used between UEs or between a UE and an LMF to fulfil a ranging/sidelink positioning service request. A SLPP Session comprises one or more SLPP transactions. A UE may simultaneously participate in multiple SLPP sessions. For UE-only scenarios, a SLPP Session ID is used to identify all SLPP transactions belonging to an SLPP session which enables UEs to uniquely distinguish SLPP messages for one session from SLPP messages for other sessions. NOTE 1: For LMF involved cases, the SLPP Session ID is assigned by the SL Target UE and used for SLPP messages exchanged between the group of UEs. NOTE 2: For LMF involved cases, the SLPP Session ID is optionally included in SLPP messages transferred between SL Target UE and LMF. TS 38.355: SLPP-Message ::= SEQUENCE { applicationLayerID OCTET STRING OPTIONAL, transactionID INTEGER (0..255) OPTIONAL, endTransaction BOOLEAN, sequenceNumber SequenceNumber OPTIONAL, sessionID SessionID OPTIONAL, acknowledgement Acknowledgement OPTIONAL, slpp-MessageBody SLPP-MessageBody OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } ### 4.1.2 SLPP Sessions and Transactions An SLPP session is used between UEs or a Location Server and a UE in order to obtain location related measurements based on NR PC5 radio signals, a location estimate or to transfer assistance data. A single SLPP session is used to support a single location request (e.g., for a single SL-MT-LR, or SL-MO-LR). Multiple SLPP sessions can be used between the same endpoints to support multiple location requests (as required by TS 23.273 [5]). For UE-only Operation, the instigator of an SLPP session which is the Endpoint who receives the LCS request, initiates an SLPP session by sending an SLPP message containing an assigned session ID (session identifier) to the other endpoint(s). All constituent messages within a session shall contain the same session ID. For LMF involved Operation, the session ID is assigned by target UE and contained in the SLPP messages used for communication between UEs. The session ID may be included in the SLPP message for the communication between a UE and the LMF.

In this contribution, we investigate the presence of the sessionID field in SLPP message transferred between target UE and LMF.

- Discussion

In terms of CN interaction of sidelink positioning, according to the TS23.273 [1], if an LMF is involved in ranging/sidelink positioning procedures of either SL-MO-LR or SL-MT-LR, AMF would allocate a LCS Correlation identifier and send the Location Request with the allocated LCS Correlation identifier to the LMF invoked by GMLC.

### 6.20.1 Procedures of SL-MO-LR involving LMF […] 9. The serving AMF selects an LMF serving UE1 (e.g. an LMF that supports Ranging/Sidelink positioning) and sends an Nlmf_Location_DetermineLocation service operation towards the LMF with the information from the SL-MO-LR Request. The service operation includes a LCS Correlation identifier. AMF may include its stored information from step 21, the sidelink positioning capabilities of UE1 in the service operation. ### 6.20.3 Procedures of SL-MT-LR involving LMF […] 9. The serving AMF selects an LMF serving UE1 (e.g. an LMF that supports Ranging/Sidelink Positioning) and sends an Nlmf_Location_DetermineLocation service operation towards the LMF with the information received at step 5 e.g. required location results (e.g. relative locations i.e. distances and directions between pairs of UEs velocities and relative velocities), SL reference UE(s) in case of relative locations, Located UE(s) in the case of absolute location, Application layer IDs of the UEs if received in step 5 and whether UE1 supports RSPP. The service operation includes a LCS Correlation identifier. The AMF may include its stored sidelink positioning capabilities of UE1 provided in step 17.

According to TS24.501 [2], for SLPP messages between target UE and LMF, the routing information in NAS messages (i.e., LCS correlation identifier) is used to identify the SLPP session, like the way to identify the LPP session in NAS.

##### 5.4.5.3.2 Network-initiated NAS transport procedure initiation […] In case c1) in subclause 5.4.5.3.1 i.e. upon reception from an LMF of an SLPP message payload, the AMF shall: a) set the Payload container type IE to "SLPP message container"; b) set the Payload container IE to the SLPP message payload received from the LMF; and c) set the Additional information IE to an LCS correlation identifier received from the LMF from which the SLPP message was received. […] ##### 5.4.5.3.3 Network-initiated NAS transport of messages accepted by the UE Upon reception of a DL NAS TRANSPORT message, the UE shall stop the timer T3346 if running. Upon reception of a DL NAS TRANSPORT message, if the Payload container type IE is set to: a) "N1 SM information" and the 5GMM cause IE is not included in the DL NAS TRANSPORT message, the 5GSM message in the Payload container IE and the PDU session ID are handled in the 5GSM procedures specified in clause 6; b) "SMS", the UE shall forward the content of the Payload container IE to the SMS stack entity; c) "LTE Positioning Protocol (LPP) message container", the UE shall forward the payload container type, the content of the Payload container IE and the routing information included in the Additional information IE to the upper layer location services application; c1) "SLPP message container", the UE shall forward the payload container type, the content of the Payload container IE and the routing information included in the Additional information IE to the upper layer location services application; […] ##### 5.4.5.2.2 UE-initiated NAS transport procedure initiation […] In case c1) in subclause 5.4.5.2.1, the UE shall: - set the Payload container type IE to "SLPP message container"; - set the Payload container IE to the SLPP message payload; and - set the Additional information IE to the routing information provided by the upper layer location services application.

Observation 1: According to SA2 specifications, the routing information in NAS messages set as LCS correlation identifier is used to identify the SLPP session to which a SLPP message between target UE and LMF belongs.

From our understanding, if SLPP session ID is only used to identify SLPP session between SL Target UE and LMF, it can be unnecessary, since the routing information i.e., LCS correlation identifier is already contained in NAS TRANSPORT message for both UE and LMF to identify. And it is up to SL target UE implementation on how to associate SLPP session among UEs to the routing information in the NAS transport with LMF.

Observation 2: With the routing information to identify the SLPP session between UE and LMF within NAS TRANPORT message containing SLPP container, SLPP session ID between SL target UE and LMF is not necessary.

What is more, if SLPP session ID is optionally included for the communication between a UE and the LMF, it is unclear how the both sides to implement the SLPP interaction without clear conditions to include or not. For example, UE vendor may not expect this contained SLPP session ID from LMF, while NW vendor may expect that the knowledge of such session ID from UE feedback. If UE does not provide session ID to LMF, LMF may consider it a malfunction in terms of different implementation. This may cause interoperability issue to SLPP management.

Observation 3: If SLPP session ID is optional provided in the SLPP container of a NAS message between SL target UE and LMF without application conditions, while UE and LMF implement differently on session ID provision, it may cause interoperability issue to SLPP session management.

Based on the above analysis, we think SLPP session ID should be absent in the SLPP message between the target UE and the LMF, including SLPP messages embedded in the Supplementary RSPP message (i.e., for end-to-end communication between LMF and SL reference UEs) and the SLPP messages directly exchanged in between.

Proposal:	RAN2 to Clarify that SLPP session ID is not included in the SLPP message between the target UE and the LMF, including the SLPP message embedded in the Supplementary RSPP message and the SLPP message directly exchanged in between.

- Conclusion

On the basis of the above analysis, we make the following observations and proposal:

Observation 1: According to SA2 specifications, the routing information in NAS messages set as LCS correlation identifier is used to identify the SLPP session to which a SLPP message between target UE and LMF belongs.

Observation 2: With the routing information to identify the SLPP session between UE and LMF within NAS TRANPORT message containing SLPP container, SLPP session ID between SL target UE and LMF is not necessary.

Observation 3: If SLPP session ID is optional provided in the SLPP container of a NAS message between SL target UE and LMF without application conditions, while UE and LMF implement differently on session ID provision, it may cause interoperability issue to SLPP session management.

Proposal:	RAN2 to Clarify that SLPP session ID is not included in the SLPP message between the target UE and the LMF, including the SLPP message embedded in the Supplementary RSPP message and the SLPP message directly exchanged in between.

- Reference

- 3GPP TS23.273 V18.7.0

- 3GPP TS24.501 V18.8.0

- 3GPP TS38.305 V18.3.0

- 3GPP TS38.355 V18.3.0

- 3GPP TS 24.571 V19.0.0

- Text proposal for reference

3GPP TS38.305 v18.3.0:

### 6.2.5	Sidelink Positioning Protocol (SLPP)

The Sidelink Positioning Protocol (SLPP) is exchanged over PC5-U reference point between UEs (e.g., target UE, anchor UE, server UE) to manage the ranging/sidelink positioning sessions among a group of UEs.

SLPP is also exchanged between a target UE and a LMF to support sidelink positioning. In this case, the SLPP messages are carried as transparent PDUs across intermediate network interfaces using the appropriate protocols (e.g., NGAP over the NG-C interface, NAS/RRC over the NR-Uu interface).

The SLPP protocol is intended to enable sidelink positioning and ranging using a multiplicity of different position methods, while isolating the details of any particular positioning method and the specifics of the underlying transport from one another.

The protocol operates on a transaction basis between UEs and between a UE and an LMF, with each transaction taking place as an independent procedure as described in TS 38.355 [47]. More than one such procedure may be in progress at any given moment. A SLPP procedure may involve a request/response pairing of messages or one or more "unsolicited" messages. Each procedure has a single objective (e.g., transfer of assistance data, exchange of SLPP related capabilities, or positioning/ranging of a target device according to some QoS and use of one or more SL positioning methods). Multiple procedures, in series and/or in parallel, can be used to achieve more complex objectives (e.g., positioning of a target device in association with transfer of assistance data and exchange of SLPP related capabilities).

An SLPP Session is used between UEs or between a UE and an LMF to fulfil a ranging/sidelink positioning service request. A SLPP Session comprises one or more SLPP transactions. A UE may simultaneously participate in multiple SLPP sessions. For UE-only scenarios, a SLPP Session ID is used to identify all SLPP transactions belonging to an SLPP session which enables UEs to uniquely distinguish SLPP messages for one session from SLPP messages for other sessions.

NOTE 1:	For LMF involved cases, the SLPP Session ID is assigned by the SL Target UE and used for SLPP messages exchanged between the group of UEs.

NOTE 2:	For LMF involved cases, the SLPP Session ID is not included in SLPP messages transferred between SL Target UE and LMF.

3GPP TS38.355 v18.3.0:

### 4.1.2	SLPP Sessions and Transactions

An SLPP session is used between UEs or a Location Server and a UE in order to obtain location related measurements based on NR PC5 radio signals, a location estimate or to transfer assistance data. A single SLPP session is used to support a single location request (e.g., for a single SL-MT-LR, or SL-MO-LR). Multiple SLPP sessions can be used between the same endpoints to support multiple location requests (as required by TS 23.273 [5]). For UE-only Operation, the instigator of an SLPP session which is the Endpoint who receives the LCS request, initiates an SLPP session by sending an SLPP message containing an assigned session ID (session identifier) to the other endpoint(s). All constituent messages within a session shall contain the same session ID. For LMF involved Operation, the session ID is assigned by target UE and contained in the SLPP messages used for communication between UEs. The session ID is not included in the SLPP message for the communication between a UE and the LMF.