3GPP TSG-RAN WG2 Meeting #128						R2-2409831

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:	8.1.2.2

Source:	Samsung

Title:	Discussion on LCM for UE-sided model for beam management

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss remaining issues/aspects to progress this topic considering RAN1 status.

# RAN1 status on RAN2 LS

In RAN2#127, RAN2 sent an LS to RAN1 to provide agreements and ask specific questions [1]. It is our understanding that RAN1 agreed on answer to Q1 and Q2 but still under discussion on other questions especially Q3 because RAN1 is evaluating three different options for step 3/4/5. That is why RAN1 has not sent an LS in the last RAN1 meeting.

We think RAN2 can review some RAN1 agreements and discuss to progress RAN2 design or to improve RAN2 understanding.

Agreement Answer to Q1 in R1-2407604 as below, In Step 2, RAN1 expects that UE reports its UE-capability information/parameters, i.e., Rel-19 AI/ML-specific FGs (including components and corresponding value ranges). These AI/ML-specific UE capability information/parameters will depend on how FGs are defined including the granularity, that will be discussed in RAN1 later in the WI..

Agreement Incorporating below text into the general part of the LS In RAN1’s discussion of RAN 2 terminologies on beam management, - The concept/terminology “functionality“ of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-specific FGs - The concept/terminology “ functionality“ of Applicable functionalities may refer to CSI-ReportConfig for inference configuration (for option 1) or a set of inference related parameters (for option 2) or information/parameters indicated by UE (for option 3) - The Activated functionalities may be enabled based on CSI framework. Therefore, the meaning and the granularity of “functionality“for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same, depends on certain option in RAN1, and the discussion is still ongoing

Based on the above agreement, RAN2 can understand the more detailed definition of functionality. For further discussion, it would be good to confirm in order to ensure companies have the same understanding.

Proposal 1: RAN2 confirms the following understanding based on RAN1 agreements.

- Supported functionality may refer to AI/ML-specific FGs and it may not be the same as other functionality.

- Activated functionality may refer to CSI Report.

Proposal 2: RAN2 waits further for the definition of applicable functionality i.e. whether applicable functionality refer to CSI Report (same as activated functionality), a set of inference related parameters or information/parameters indicated by the UE.

# Performance monitoring

During the study item, RAN1 identified both Type 1 and Type 2 performance monitoring, where type 1 is NW to control LCM operation of UE-side model and type 2 is UE to control the LCM operation. During the work item, RAN1 agreed to support Type 1 and FFS for Type 2 because Type 2 can be left to UE implementation. For Type 1, RAN1 agreed to support two options (NW-side performance monitoring and UE-assisted performance monitoring) as shown below.

Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), consider the following options with potential down selection for the configuration for monitoring: - Option 1: The resource set(s) for monitoring and report configuration for monitoring are configured (when applicable) within CSI report configuration used for inference - FFS: the resource set(s) for monitoring - UE measures the resource set(s) for monitoring. - FFS how/when to report the monitoring results. - Option 2: Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The dedicated report configuration used for monitoring links to an inference report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - UE measures the resource set(s) for monitoring. - FFS when to report the monitoring results. Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring

Looking at RAN1 agreements (especially highlighted in red above), for all options, the reporting of performance monitoring is configured within CSI report configuration. It can be interpreted that L1 signaling is used to report performance monitoring.

Observation: RAN1 agreed to support L1 signaling to report measurement results for Type 1 performance monitoring.

Given that reporting of performance monitoring is based on L1 signaling, if additional procedure on top of existing CSI reporting framework is needed, RAN1 should discuss and conclude.

Proposal 3: RAN2 wait until RAN1 provides further details on required spec impacts for performance monitoring related.

# Data collection configuration for training

In RAN2 #127bis, RAN2 made the following agreement regarding data collection configuration for training.

- Data collection initiation and configuration for data collection is under network control. FFS how the NW determines whether data collection should be initiated (e.g. via UE requests (UE directly or UE server)

During the discussion, it was not clearly concluded about the possibility of data collection without dedicate NW configuration for training.

Depending on UE implementation especially in the beginning of AI/ML implementation, there is a possibility that the UE vendor may train UE side model via offline training and collect data for training without dedicate network configuration. For this approach, the UE vendor could coordinate with NW vendors to get specific network configuration information (especially NW additional condition) and collect data set. Alternatively, the UE can collect measurement results while it is operated with legacy configuration (i.e. non-AI/ML feature).  In order to fully utilize AI/ML technology, 3GPP should be also open for the case where AI/ML model is trained by offline data collection without dedicate NW configuration. In this case, there could be no standard impact.

Proposal 4: RAN2 confirm that it is also possible that UE sided model can be trained without dedicate NW configuration for data collection and no standard impact is expected.

In the previous RAN2 agreement, it is FFS how the NW determines whether data collection configuration should be initiated. During the discussion, UE request-based data collection configuration was mentioned and added as an example in the agreement.

Technically, we could compare both UE request-based data collection configuration and NW requested data collection configuration.

- UE request-based data collection configuration: UE should be able to know which functionality can be applicable or not based on the current procedure for inference. Therefore, it is straightforward that UE indicates the need of training for certain functionality.

- NW request-based data collection configuration: NW might be able to identify functionality that requires further training based on UE reporting of applicable functionality. However, even in this case, before NW provides data collection configuration, NW needs to know whether UE is capable of training model to support a certain non-applicable functionality. In addition, even when UE does not report as applicable functionality, UE server may have a trained model. In this case, model training is not needed, and instead model transfer from UE server to UE is needed.

Given that UE has more information about whether data collection configuration is needed or not, it is reasonable for NW to determines to provide data collection configuration for training based on UE-request.

Proposal 5: RAN2 agree that NW determines to provide data collection configuration for training via UE request.

The next question is whether which entity (i.e., UE directly vs UE server) requests data collection configuration to NW. The important aspect to consider is that applicability of functionality check is done by UE. So, when UE detects inapplicability, it is reasonable for UE to request data collection configuration to NW in order to train a new model. We also think another scenario where UE server is involved. i.e., when UE detects inapplicability, UE first asks UE server if there is any valid model. If UE receives the response from UE server that there is no valid model, UE can also request data collection configuration to NW. However, even in this scenario, UE should be the one who makes the final decision on the request of data collection configuration. i.e., UE may not request data collection configuration to NW, even after UE server’s response of no valid model due to UE side conditions e.g. memory, processing power, channel situation, etc.

Proposal 6: The request for data collection configuration to NW is decided/sent by UE directly.

# Conclusion

We discussed the following three topics and provide the following proposals.

RAN1 status on RAN2 LS

Proposal 1: RAN2 confirms the following understanding based on RAN1 agreements.

- Supported functionality may refer to AI/ML-specific FGs and it may not be the same as other functionality.

- Activated functionality may refer to CSI Report.

Proposal 2: RAN2 waits further for the definition of applicable functionality i.e. whether applicable functionality refer to CSI Report (same as activated functionality), a set of inference related parameters or information/parameters indicated by the UE.

Performance monitoring

Observation: RAN1 agreed to support L1 signaling to report measurement results for Type 1 performance monitoring.

Proposal 3: RAN2 wait until RAN1 provides further details on required spec impacts for performance monitoring related.

Data collection configuration for training

Proposal 4: RAN2 confirm that it is also possible that UE sided model can be trained without dedicate NW configuration for data collection and no standard impact is expected.

Proposal 5: RAN2 agree that NW determines to provide data collection configuration for training via UE request.

Proposal 6: The request for data collection configuration to NW is decided/sent by UE directly.

# Reference

[1] R2-2407848, LS on applicable functionality reporting for beam management UE-sided model, RAN2#127, August 2024