3GPP TSG-WG2 Meeting #128 	R2-2409833

# Orlando, USA, 18th November - 22nd November

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1981 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the field name of candidate beam list in BFR MAC CE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Core |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-16, NW provides UE with a list of candidate beams for BFR of SCell via candidateBeamRSSCellList in RRCReconfiguration message. When UE detects beam failure for SCell, it sends BFR MAC CE indicating one candidate beam for BFR by setting the index of an SSB/CSI-RS among the SSBs/CSI-RSs in candidateBeamRSSCellList. Note that the candidate beam indication in BFR MAC CE is only for BFR in SCell. For BFR in SpCell, UE should trigger RA procedure for BFR and the candidate beam can be implicitly indicated by the PRACH resource used for the RA. In Rel-17 feMIMO WI, BFR procedure per TRP is introduced considering a cell with two TRPs, which leads to the need of having two separate candidate beam lists for each TRP (i.e., BFD-RS set). For the two candidate lists, RAN2 introduced a new field (candidateBeamRS-List2-r17) for TRP2 and reused the existing field (candidateBeamRSSCellList) for TRP1 by renaming it to candidateBeamRS-List-r16 as in R2-2206881. The reason for renaming is that the field is not only for SCell but also for SpCell from Rel-17 since UE can report the candidate beam per TRP even for BFR in SpCell via a new MAC CE (Enhanced BFR MAC CE). However, in the current Rel-17/18 MAC spec., the name of candidate beam list in the legacy BFR MAC CE is still candidateBeamRSSCellList which does not exist anymore in RRC from Rel-17. Since the BFR MAC CE is used only for one TRP case, the name of candidate beam list field in BFR MAC CE should be corrected to candidateBeamRS-List-r16 among the two candidate beam lists for two TRPs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the wrong field name of candidate beam list in BFR MAC CE. Impact analysis Impacted 5G architecture options: EN-DC, SA, NE-DC, NR-DC Impacted functionality: BFR (Beam Failure Recovery) Inter-operability: If the UE is implemented according to the CR while the network is not, there can be misunderstanding between UE and network on the reference list for candidate beam index in BFR MAC CE. If the network is implemented according to the CR while the UE is not, there can be misunderstanding between UE and network on the reference list for candidate beam index in BFR MAC CE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE cannot properly indicate the candidate beam for BFR in BFR MAC CE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.23 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

6.1.3.23	BFR MAC CEs

The MAC CEs for BFR consists of either:

-	BFR MAC CE; or

-	Truncated BFR MAC CE.

The BFR MAC CE and Truncated BFR MAC CE are identified by a MAC subheader with LCID/eLCID as specified in Table 6.2.1-2 and Table 6.2.1-2b.

The BFR MAC CE and Truncated BFR MAC CE have a variable size. They include a bitmap and in ascending order based on the ServCellIndex, beam failure recovery information i.e. octets containing candidate beam availability indication (AC) for SCells indicated in the bitmap. For BFR MAC CE, a single octet bitmap is used when the highest ServCellIndex of this MAC entity's SCell for which beam failure is detected and the evaluation of the candidate beams according to the requirements as specified in TS 38.133 [11] has been completed is less than 8, otherwise four octets are used. A MAC PDU shall contain at most one BFR MAC CE.

For Truncated BFR MAC CE, a single octet bitmap is used for the following cases, otherwise four octets are used:

-	the highest ServCellIndex of this MAC entity's SCell for which beam failure is detected and the evaluation of the candidate beams according to the requirements as specified in TS 38.133 [11] has been completed is less than 8; or

-	beam failure is detected for SpCell (as specified in Clause 5.17) and the SpCell is to be indicated in a Truncated BFR MAC CE and the UL-SCH resources available for transmission cannot accommodate the Truncated BFR MAC CE with the four octets bitmap plus its subheader as a result of LCP.

The fields in the BFR MAC CEs are defined as follows:

-	SP: This field indicates beam failure detection (as specified in clause 5.17) for the SpCell of this MAC entity. The SP field is set to 1 to indicate that beam failure is detected for SpCell only when BFR MAC CE or Truncated BFR MAC CE is to be included into a MAC PDU as part of Random Access Procedure (as specified in 5.1.3a and 5.1.4), otherwise, it is set to 0;

-	Ci (BFR MAC CE): This field indicates beam failure detection (as specified in clause 5.17) and the presence of an octet containing the AC field for the SCell with ServCellIndex i as specified in TS 38.331 [5]. The Ci field set to 1 indicates that beam failure is detected, the evaluation of the candidate beams according to the requirements as specified in TS 38.133 [11] has been completed, and the octet containing the AC field is present for the SCell with ServCellIndex i. The Ci field set to 0 indicates that the beam failure is either not detected or the beam failure is detected but the evaluation of the candidate beams according to the requirements as specified in TS 38.133 [11] has not been completed, and the octet containing the AC field is not present for the SCell with ServCellIndex i. The octets containing the AC field are present in ascending order based on the ServCellIndex;

-	Ci (Truncated BFR MAC CE): This field indicates beam failure detection (as specified in clause 5.17) for the SCell with ServCellIndex i as specified in TS 38.331 [5]. The Ci field set to 1 indicates that beam failure is detected, the evaluation of the candidate beams according to the requirements as specified in TS 38.133 [11] has been completed, and the octet containing the AC field for the SCell with ServCellIndex i may be present. The Ci field set to 0 indicates that the beam failure is either not detected or the beam failure is detected but the evaluation of the candidate beams according to the requirements as specified in TS 38.133 [11] has not been completed, and the octet containing the AC field is not present for the SCell with ServCellIndex i. The octets containing the AC field, if present, are included in ascending order based on the ServCellIndex. The number of octets containing the AC field included is maximised, while not exceeding the available grant size;

NOTE:	The number of the octets containing the AC field in the Truncated BFR MAC CE can be zero.

-	AC: This field indicates the presence of the Candidate RS ID field in this octet. If at least one of the SSBs with SS-RSRP above rsrp-ThresholdBFR amongst the SSBs in candidateBeamRS-List-r16 or the CSI-RSs with CSI-RSRP above rsrp-ThresholdBFR amongst the CSI-RSs in candidateBeamRS-List-r16 is available, the AC field is set to 1; otherwise, it is set to 0. If the AC field set to 1, the Candidate RS ID field is present. If the AC field set to 0, R bits are present instead;

-	Candidate RS ID: This field is set to the index of an SSB with SS-RSRP above rsrp-ThresholdBFR amongst the SSBs in candidateBeamRS-List-r16 or to the index of a CSI-RS with CSI-RSRP above rsrp-ThresholdBFR amongst the CSI-RSs in candidateBeamRS-List-r16. Index of an SSB or CSI-RS is the index of an entry in candidateBeamRS-List-r16 corresponding to the SSB or CSI-RS. Index 0 corresponds to the first entry in the candidateBeamRS-List-r16, index 1 corresponds to the second entry in the list and so on. The length of this field is 6 bits.

-	R: Reserved bit, set to 0.

Figure 6.1.3.23-1: BFR and Truncated BFR MAC CE with one octet Ci field

Figure 6.1.3.23-2: BFR and Truncated BFR MAC CE with four octets Ci field

Figure 6.1.3.23-2: BFR and Truncated BFR MAC CE with four octets Ci field

End of change