3GPP TSG-RAN WG2 Meeting #128	R2-2409842

Orlando, USA, Nov. 18th –22nd, 2024

Agenda item:	8.6.3

Source: 	Fujitsu

Title: 	Discussions on event triggered L1 measurement reporting

Document for:	Discussion and decision

1. Introduction

The following objective of the work item aims at specifying measurement related enhancements for New WID [1] on NR mobility enhancements:

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

This contribution will provide more considerations on the L1 measurement reporting procedure, including:

- Event evaluation and triggering

- MAC CE for measurement report

2. Discussion

2.1 Event evaluation and triggering

Beam(s) for event evaluation

According to RAN2 agreement, RS associated with indicated beam for the serving cell is used for event evaluation. However, if the indicated beam is associated with PCI of non-serving cell, LTM cell switch based on event-triggered L1 measurement report cannot be supported. In case of inter-cell beam management, the UE is using the beam from the neighbouring cell associated to the serving cell and thus the RS from the neighbouring cell is used for event evaluation. Event e.g. LTM2/3/5 cannot be satisfied since the quality of beam of serving cell used for event evaluation, i.e. the RS from the neighbouring cell, is good.

To support LTM cell switch after ICBM, a set of RS(s) can be configured for the serving cell for L1 measurement. Alternatively, RS associated with activated TCI states of the serving cell can be used for event evaluation.

Proposal 1: RAN2 is asked to support LTM cell switch after ICBM.

Proposal 1bis: For event evaluations, if LTM cell switch after ICBM is supported, RAN2 to discuss if configured RS or RS associated with activated TCI states of the serving cell can be used.

In addition, RAN2 agreed that the same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

To align the measurement RS type, the following options are discussed at last RAN2 meeting:

- Option 1: NW configures the both types of RS (SSB and CSI-RS) for the new beam(s), UE could select the same type of beam when UE evaluate the L1 event measurement.

- Option 2: Only one type of RS is configured for the new beam as a NW restriction, and UE perform the L1 measurement when RS type of the current beam is same with the new beam for event L1 measurement.

- Option 3: NW provides the offset which can be applied one type of RS to evaluate the different type of RS. For example, If the current beam is SSB and the measured beam for the candidate cell is CSI-RS, the UE applies the configured the offset to the measured CSI-RS value and then compare it with the current beam quality i.e. SSB measurement.

- Option 4: Rely on network configuration, i.e. same RS type is configured.

- Option 5: Which type to be used in measurement will be configured by NW. Then the UE only consider the configured RS type in the measurement.

- Option 6: The RS for current beam of serving cell is same as or associated with QCL RS provided in the indicated TCI-state, i.e. using the same type of RS as the candidate beam.

Option 3/4/5 should be excluded. Option 3 is not a solution to align the measurement RS type. Option 4/5 may not work. Because the RS type for serving cell can change dynamically if a new TCI state is indicated by DCI while L1 measurement RS for LTM candidate are configured by RRC signalling.

Based on these, RAN2 can further discuss whether Option 1, Option 2 or Option 6 will be used.

Proposal 2: To align the measurement RS type used for serving and neighbouring cell when event LTM3 and event LTM5 are evaluated, RAN2 to select one from the following options:

- Option 1: NW configures the both types of RS (SSB and CSI-RS) for the new beam(s), UE could select the same type of beam.

- Option 2: Only one type of RS is configured for the new beam as a NW restriction, and UE perform the L1 measurement when RS type of the current beam is same with the new beam for event L1 measurement.

- Option 6: The RS for current beam of serving cell is same as or associated with QCL RS provided in the indicated TCI-state, i.e. using the same type of RS as the candidate beam.

The trigger of L1 measurement reporting

RAN2 has agreed that the L1 measurement report can be triggered when the leaving condition is met, if configured by the network. In event-triggered L3 measurement reporting, a cell list is maintained to evaluate the leaving condition. RAN2 needs to discuss which beam will be used to evaluate the leaving condition of LTM event. For example, beam 1 of candidate LTM cell satisfies the LTM event and MR MAC CE including the information of the beam is transmitted to the network. The issue is whether only beam 1 is used to evaluate the leaving condition of the LTM event, or any beam can be used.

Proposal 3: RAN2 to discuss which beam is used for the leaving condition evaluation, triggered beam, any beam satisfying the event, or any beam.

RAN2 has already agreed that the Timer to Trigger mechanism is applied at least for the entering condition of LTM events. We think that the TTT is applied to the leaving condition as well. This can be configured by the network.

Proposal 4: TTT is applied to the leaving condition. It is configurable.

One remaining issue is TTT operation when current beam changes, e.g. reset or no reset. Based on beam management, new beam can be indicated by the network via DCI, which is in the scope of MIMO. We think that the UE should perform the event evaluation again, i.e. the timer should be stopped, when the current beam changes. If the timer is not stopped, the unnecessary L1 measurement reporting is triggered considering the case that serving cell is getting better and better after current beam change.

Proposal 5: TTT is stopped when current beam changes.

Another issue is the applied level of TTT. The TTT can be applied at beam level or CSI resource set level. If it is applied for CSI resource set, the L1 measurement reporting can be triggered even if the beam satisfying the entering condition of LTM event changes during the TTT. In this case, if the RS set is configured for L1 measurement, the UE performs event evaluation per RS set. Otherwise, the UE performs event evaluation per candidate cell.

Proposal 6: TTT mechanism is applied to each RS set or each candidate cell.

Proposal 6bis: For each RS set or each candidate cell, the MAC entity performs event evaluation to determine if the LTM event is satisfied or not.

Other

For L3 measurement reporting, to address transmission failure of the L3 measurement report, a timer T312 can be configured. The timer T312 can be started if the L3 measurement report is triggered during T310 is running. Failure reporting or connection re-establishment procedure is initiated when the timer expires.

For L1 measurement reporting, RAN2 has agreed that the event triggered L1 measurement reporting should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization

- Select the target beam/cell and trigger LTM cell switch procedure

A similar mechanism can be used for the L1 measurement reporting triggered by the events defined for LTM decision. With this, the UE can take some actions when the transmission of the event triggered L1 measurement report fails.

Proposal 7: If the event is configured for LTM decision, RAN2 considers T312-like mechanism.

2.2 MAC CE for measurement report (MR MAC CE)

Regarding the MR MAC CE, triggered event information, e.g. ReportConfigID will be included. At last RAN2 meeting, it was agreed that the L1 measurement can be triggered when the leaving condition is met. It means that the L1 measurement can be triggered by the entering condition or leaving condition of LTM event. The information of the entering condition or leaving condition is useful for the network and it is better for the network to know this. It can be included in the MR MAC CE. Also, RAN2 has agreed to support event triggered periodic measurement reporting. We think that it is helpful if the network has knowledge that the MR MAC CE is for periodic measurement reporting rather than initial measurement reporting trigged by LTM even.

Proposal 8: For triggered event information, RAN2 to discuss whether to indicate other information in addition to ReportConfigID, e.g. the entering condition or the leaving condition, Initial triggered or event triggered periodic MR.

In addition, beam quantity of up to N beams will be included in the MAC CE. There is one remaining issue, i.e. whether the beam should satisfy event or not. We think that at least the beam which satisfies the LTM event should be included. Corresponding LTM candidate can be considered as one of candidate target cells. For whole picture of the candidate target cell to make LTM decision, it is better to include beam quantity of other beams for the candidate target cell. For these beams, the beam information is not required. In summary, beam information and quantity of the beam satisfying the LTM event, beam quantity of current beam and other beams for candidate target cell can be included in the MR MAC CE.

Proposal 9: The information of following beams can be contained in the MR MAC CE:

- the beam satisfying the LTM event (beam information + beam quantity)

- the beam for the candidate cell to which the beam satisfying the event belongs, no matter if satisfying the LTM event (beam quantity)

- current beam (beam quantity)

It is possible that multiple beams satisfying the LTM events belong to different candidate LTM cell. The question is if the information including beam information and beam quantity of these beams can be included in one MR MAC CE or in different MR MAC CE. It has impacts to the MAC CE design and RAN2 needs to discuss this issue.

Proposal 9bis: RAN2 to discuss if the MR MAC CE can contain more than one beam satisfying the event.

3. Conclusion

In this contribution, we have some discussions on potential measurement enhancements for LTM and made the following proposals:

Beam(s) for event evaluation

Proposal 1: RAN2 is asked to support LTM cell switch after ICBM.

Proposal 1bis: For event evaluations, if LTM cell switch after ICBM is supported, RAN2 to discuss if configured RS or RS associated with activated TCI states of the serving cell can be used.

Proposal 2: To align the measurement RS type used for serving and neighbouring cell when event LTM3 and event LTM5 are evaluated, RAN2 to select one from the following options:

- Option 1: NW configures the both types of RS (SSB and CSI-RS) for the new beam(s), UE could select the same type of beam.

- Option 2: Only one type of RS is configured for the new beam as a NW restriction, and UE perform the L1 measurement when RS type of the current beam is same with the new beam for event L1 measurement.

- Option 6: The RS for current beam of serving cell is same as or associated with QCL RS provided in the indicated TCI-state, i.e. using the same type of RS as the candidate beam.

The trigger of L1 measurement reporting

Proposal 3: RAN2 to discuss which beam is used for the leaving condition evaluation, triggered beam, any beam satisfying the event, or any beam.

Proposal 4: TTT is applied to the leaving condition. It is configurable.

Proposal 5: TTT is stopped when current beam changes.

Proposal 6: TTT mechanism is applied to each RS set or each candidate cell.

Proposal 6bis: For each RS set or each candidate cell, the MAC entity performs event evaluation to determine if the LTM event is satisfied or not.

Proposal 7: If the event is configured for LTM decision, RAN2 considers T312-like mechanism.

Information of MR MAC CE

Proposal 8: For triggered event information, RAN2 to discuss whether to indicate other information in addition to ReportConfigID, e.g. the entering condition or the leaving condition, Initial triggered or event triggered periodic MR.

Proposal 9: The information of following beams can be contained in the MR MAC CE:

- the beam satisfying the LTM event (beam information + beam quantity)

- the beam for the candidate cell to which the beam satisfying the event belongs, no matter if satisfying the LTM event (beam quantity)

- current beam (beam quantity)

Proposal 9bis: RAN2 to discuss if the MR MAC CE can contain more than one beam satisfying the event.

Reference

- RP-242356, “Revised Work Item: NR mobility enhancements Phase 4”, RAN#105, Sep. 9-12, 2024