3GPP TSG-RAN WG2 Meeting #128	                                          R2-2409866

Orlando, USA, 18 – 22, Nov, 2024

Source: 	Xiaomi

Title:  	Discussion on RRM prediction simulation result

Agenda Item:	8.3.2

Document for:	Discussion and Decision

Introduction

RAN2 agreed two goals for RRM prediction, i.e. measurement reduction and handover performance enhancement. Following cases are identified to evaluate the prediction accuracy.

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In this contribution, we provide simulation result of cell level RRM measurement prediction of case 2 3 and 4. Some observations are given.

Discussion

In last meeting, two L1/L3 filtering options are proposed, i.e. sliding or non-sliding. In our simulation, we use the sliding window based L1/L3 filtering option. In this option, the measurement changes in a smoother way, which may be easier for AI to predict. In the L1 filtering, we average the 5 L1 measurements within one measurement period to obtain 1 L1 filtered measurement.

| Parameters | Value |
| --- | --- |
| UE trajectory option (option 1,2,3 in[4]) | 1 |
| UE trajectory boundary processing option (option 1,2,3 in[4]) | Circle-bouncing |
| L1/L3 filtering | Sliding window |
| Whether LOSsoft is modeled or not | No |
| spatial consistency option (A or B) | A |

For cell level measurement prediction model, RAN2 agreed to study the following prediction cases:

Case 1: To predict beam level results, then generate cell level results based on the predicted beam results;

Case 2: To directly predict cell level results based on cell level results.

Case 3: To directly predict cell level results based on beam level results

We did simulation of case 2 to evaluate the prediction accuracy.

FR1 to FR1 inter-frequency prediction

It’s agreed PCC is used to evaluate the correlation between frequencies. It’s up to company to select the number of measurement results to calculate the PCC. In our simulation the PCC is calculated by all the measurement results and PCC is 0.932. In our simulation, 3 cells are deployed on one site at each frequency.

It’s agreed the prediction accuracy performance is comparable between higher-to-lower frequency and lower-to-higher frequency case. We focus on higher-to-lower prediction. Following options for inter-frequency prediction are simulated,

Option 1: single cell-based prediction. Only the collocated serving cell is used as input.

Option 1-1: single cell-based prediction. 3 historic measurement results of the collocated serving cell are used as input.

Option 2: cluster-based prediction 1. The three collocated cells are used as input.

Option 2-1: cluster-based prediction 1-1. The three collocated cells and UE location are used as input.

Option 3: cluster-based prediction 2. The collocated serving cell and six neighbour cells are used as input.

Option 3-1: cluster-based prediction 2.1. The collocated serving cell, six neighbour cells and UE location are used as input.

The prediction accuracy is show as below,

| Option | RSRP diff (dB) |
| --- | --- |
| 1 | 4.54 |
| 1-1 | 4.28 |
| 2 | 4.05 |
| 2-1 | 3.56 |
| 3 | 3.52 |
| 3-1 | 3.50 |

It can be seen the cluster-based option (option 2) can obtain better prediction accuracy performance than single cell-based option (option 1). That’s because the cluster includes more cells and can provide more related information. In option 3, more cells are included in cluster, the prediction accuracy can be further improved. Although the additional neighbour cells are not collocated with the predicted cell, they still provide the prediction gain.

Observation 1: Cluster as input can improve the prediction accuracy than single cell as input.

Observation 2: More cells in cluster as input can improve the prediction accuracy.

Option 1-1 can reach better prediction accuracy, 5%. This indicates the historic measurement results can provide prediction gain.

Observation 3: Addition of historic measurement results as input can provide prediction accuracy gain.

The prediction accuracy can be improved if UE location is added as input compared with option 2-1 and option 2. However, the prediction accuracy performance of Option 3-1 and option 3 is comparable. We understand it’s because the cluster in option 3 comprise more cells than option 2. Therefore, the UE location can be implicitly learned by AI based on the measurement results of multiple cells in cluster. So the addition of UE location can provide significant gain only if the number of cells in cluster is small.

Observation 4: The addition of UE location can provide significant gain only when the number of cells in cluster is small.

Proposal 1: Capture the above observation into TR for case 3.

FR1 to FR1 intra-frequency temporal domain case B

We use the temporal domain observation and prediction pattern depicted in following fig. In this pattern, the measurement in OW and PW are consecutive. The advantage of this pattern is the consecutive measurements in the observation window can provide more information to AI to improve the prediction accuracy.

Fig 1 Temporal domain observation and prediction pattern

The goal of case B is to reduce the measurement. We evaluate different OW/PW length to check how the OW/PW impacts the prediction accuracy. The prediction accuracy simulation results of 60 km/h are shown in following table. Simulation results of other speeds can be found on the FTP. Similar trends can be observed.

| No. | OW (ms) | PW (ms) | MRRT | Avg RSRP difference (dB) |
| --- | --- | --- | --- | --- |
| 1 | 40 | 40 | 50% | 0.649 |
| 2 | 80 | 80 | 50% | 0.706 |
| 3 | 120 | 120 | 50% | 0.906 |
| 4 | 80 | 160 | 66% | 1.119 |
| 5 | 200 | 400 | 66% | 2.088 |

| No. | RSRP difference per step (dB) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.649 |  |  |  |  |  |  |  |  |  |
| 2 | 0.499 | 0.913 |  |  |  |  |  |  |  |  |
| 3 | 0.493 | 0.908 | 1.319 |  |  |  |  |  |  |  |
| 4 | 0.501 | 0.907 | 1.329 | 1.729 |  |  |  |  |  |  |
| 5 | 0.542 | 0.915 | 1.296 | 1.711 | 2.114 | 2.386 | 2.635 | 2.867 | 3.099 | 3.312 |

It’s agreed the longer PW would result in worse prediction accuracy from average RSRP difference point of view. We can further check whether it’s also applied to step based RSRP difference. From scenario 1 to 3 or 4 to 5, the length of PW increases, the prediction accuracy also reduces from last step RSRP difference point of view. The same observation can be applied from last step RSRP difference point of view.

Observation 5: Longer PW length reduces the prediction accuracy from last step point of view.

However, if we focus on specific step other than the last step, e.g. first step, prediction accuracy is not totally aligned with the average RSRP. The prediction accuracy of scenario 2 is better than scenario 1, although the prediction accuracy on average RSRP is worse. We understand it’s because the longer OW in scenario 2 can provide more information to predict step 1 measurement result. However, the prediction accuracy of scenario 3 doesn’t further improve much although the OW is further increased. Similar trends can be observed for other steps, too. It can be observed the increase of OW can improve the prediction accuracy from specific step RSRP difference point of view. However, if OW is larger than certain value, further increase of OW doesn’t provide gain on prediction accuracy. This is similar as case A.

Observation 6: From a specific step point of view, the increase of OW can improve the prediction accuracy. However, if OW is larger than certain value, further increase of OW doesn’t provide gain on prediction accuracy.

The detailed simulation results can be found in the spreadsheets along with this contribution.

Proposal 2: Capture the above observation into TR for case 2.

FR2 to FR2 intra-frequency temporal domain case A

It’s agreed the ratio between observation window and prediction window is at least limited to value range {5,4,3,2,1,1/2,1/3,1/4,1/5}. However, the goal of case A is to improve the mobility performance. The prediction accuracy is more important than power saving. There is no need to use short observation window, as the UE would always perform measurement.

The prediction accuracy simulation results of 90 km/h are shown in following table. Simulation results of other speeds can be found on the FTP. Similar trends can be observed.

| No. | OW/PW | OW (ms) | PW (ms) | Avg RSRP diff (dB) | Per step RSRP diff (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 80 | 80 | 1.422 | 1.422 |  |
| 2 | 1 | 160 | 160 | 1.390 | 0.871 | 1.922 |
| 3 | 2 | 160 | 80 | 0.871 | 0.871 |  |
| 4 | 2 | 320 | 160 | 1.235 | 0.783 | 1.689 |
| 5 | 3 | 240 | 80 | 0.791 | 0.791 |  |
| 6 | 3 | 480 | 160 | 1.205 | 0.766 | 1.643 |

In scenario {1, 4, 6}, the PW comprise two steps. For a specifics step, the increase of OW can improve the prediction accuracy, e.g. from 160ms to 320ms. However, further increase of OW doesn’t provide much gain. This trend is similar from average RSRP difference point of view.

Observation 7: From a specific step point of view, the increase of OW can improve the prediction accuracy. However, if OW is larger than certain value, further increase of OW doesn’t provide gain on prediction accuracy.

If the OW/PW rate is fixed, the increase of OW and PW length would reduce the prediction accuracy. This can be seen by comparison of scenario {1, 2}, {3,4}, {5,6}. However, from first step point of view, this doesn’t apply.

Observation 8: If the OW/PW rate is fixed, increase of OW and PW length would reduce the prediction accuracy from average of view.

The detailed simulation results can be found in the spreadsheets along with this contribution.

Proposal 3: Capture the above observation into TR for case 4.

Conclusions

According to the analysis given above, we have the following observations and proposals:

Case 3 simulation result

Observation 1: Cluster as input can improve the prediction accuracy than single cell as input.

Observation 2: More cells in cluster as input can improve the prediction accuracy.

Observation 3: Addition of historic measurement results as input can provide prediction accuracy gain.

Observation 4: The addition of UE location can provide significant gain only when the number of cells in cluster is small.

Proposal 1: Capture the above observation into TR for case 3.

Case 2 simulation result

Observation 5: Longer PW length reduces the prediction accuracy from last step point of view.

Observation 6: From a specific step point of view, the increase of OW can improve the prediction accuracy. However, if OW is larger than certain value, further increase of OW doesn’t provide gain on prediction accuracy.

Proposal 2: Capture the above observation into TR for case 2.

Case 4 simulation result

Observation 7: From a specific step point of view, the increase of OW can improve the prediction accuracy. However, if OW is larger than certain value, further increase of OW doesn’t provide gain on prediction accuracy.

Observation 8: If the OW/PW rate is fixed, increase of OW and PW length would reduce the prediction accuracy from average of view.

Proposal 3: Capture the above observation into TR for case 4.

Reference

[1] TR 36.839