3GPP TSG-RAN WG2 Meeting #128	                                    R2-2409876

Orlando, USA, Nov.  18th – 22nd , 2024

Agenda Item:	8.9.2

Source: 	OPPO

Title:  	Discussion on Store & Forward satellite operation

Document for:	Discussion and Decision

Introduction

In Rel-19 IoT NTN, Store & Forward satellite operation is one of the features to be supported.

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2, RAN4] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

In this contribution, we discuss some radio interface impact by introducing S&F satellite operation.

Discussion

Below are the agreements made in previous RAN2 meetings

RAN2#127 agreements

Agreements:

1.	RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options)

2.	RAN2 will consider both single satellite pass and multiple satellite pass scenarios

3.	RAN2 will consider both MO and MT data within scope

4.	UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature)

5.	RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected.

7.	RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

RAN2#127bis agreements

Agreements:

1.	The dynamic indication that “the cell is operating in S&F mode” is called “S&F operation” indication (we can come back on the exact name when putting this in the spec, if needed)

2.	RAN2 assumes that if an indication that “the cell is operating in S&F mode” is not provided in a NTN cell, the UE should assume that the NTN cell is operating in real-time mode (i.e. default/normal/”not-S&F” mode). RAN2 assumes there should be no distinction between the case that the UE is served by a NTN cell that do not support S&F capability and the case that the UE is served by a NTN cell that does support S&F capability but indication that “the cell is operating in S&F mode” is not provided.

3.	An S&F explicit capability indication by the serving cell, conceived as a static indication of whether the S&F capability is supported or not by a specific satellite/NTN-cell, in addition to the indication that “the cell is operating in S&F mode”, is not needed

4.	If the “S&F operation” indication is not broadcast, a Rel-19 UE (regardless whether supporting S&F or not) shall follow the legacy barring procedure

5.	When present, the “S&F operation” indication has two possible settings:

‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell)

‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

Additional S&F indication

For an NTN cell with S&F operation, RAN2 has agreed that S&F operation indication can be provided by SIB. And the indication is used to bar Rel-19 UEs supporting S&F mode.

Now that SA2 has supported two architecture options, it would make sense that one NTN cell just implements one architecture, e.g. either split MME or full CN on board. From UE’s perspective, the support for each architecture option could be optional. Therefore, the S&F indication broadcasted in SIB may need further indicate the supported architecture option, so that UE can decide whether to access based on its own capabilities.

S&F operation indication broadcasted in SIB further indicates which architecture option is supported in the current cell.

And another remaining issue is whether to indicate the remaining time of the satellite’s current operating mode. As we already agreed that the current operation mode is dynamically broadcasted to UE, the remaining time of the satellite’s current operation mode will not bring significant gain. UE in idle mode can rely on SIB update to acquire the latest work mode of the cell, and UE in connected mode could rely on NW indication to determine the UE behaviour.

There is no need to indicate remaining time of the satellite’s current operation mode (normal or S&F) via System Information broadcast.

Furthermore, it is agreed that the “S&F operation” indication setting as “0” means the cell is operating in S&F mode for UE s in connected mode, while UE in connected mode will not monitoring the system information update. Thus, it is questionable that how UE in connected mode behaves in the following scenarios:

- Scenario 1: the cell is switching form “S&F mode” to normal mode

- Scenario 2: the cell is switching form normal mode to “S&F mode”

For UEs in the cell that is switching from “S&F mode” to normal mode, no special UE behaviour is needed as the cell in normal mode can provide lower latency than “S&F mode”. While for UEs in the cell that is switching form normal mode to “S&F mode”, the latency requirement of ongoing service may not be fulfilled due to lake of the feeder link connection.

In general, there are three ways of NW handling for the two scenarios above:

- Release UE to idle mode;

- Trigger handover to switch the UE to correct cell;

- Update the radio bearer configuration to adapt the current cell working operation.

We understand either of the handlings is up to NW implementation.

For UE in connected mode, rely on NW implementation for UE handling if NW switch to/from S&F mode from/to normal mode.

Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1	S&F operation indication broadcasted in SIB further indicates which architecture option is supported in the current cell.

Proposal 2	There is no need to indicate remaining time of the satellite’s current operation mode (normal or S&F) via System Information broadcast.

Proposal 3	For UE in connected mode, rely on NW implementation for UE handling if NW switch to/from S&F mode from/to normal mode.

Reference

[1] RP-234036 New WID-R19 Mobility Enhancements-v5.docx