3GPP TSG-RAN WG2 Meeting #128                      R2-2409884

Orlando, USA, Nov.18th – Nov.22nd, 2024

Agenda Item:	8.2.3

Source: 	Transsion Holdings

Title:  	Discussion on Paging for A-IoT

Document for:	Discussion and Decision

- Introduction

In the last RAN2#127b meeting, the A-IoT Paging was discussed and the following aggrements were made[1]:

Agreements on AIoT paging

- The A-IoT paging message can include information to avoid duplicate response from the device to a reader.  This information should be short and simple.  FFS how to indicate.  Wait for further information and requirements from other WGs and make this decision in normative phase.

- Based on this information the device determines whether to skip sending the response to paging.

- From RAN2 perspective it is feasible to support paging multiple device IDs, however depending on TB size it may not be possible to include multiple device IDs.    RAN2 will determine the need for this after considering aspects related to RAN2 multiplexing and other WGs.

- If multiple device IDs in single paging is supported, if A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA

In this contribution, we continue to discuss the Paging for A-IoT.

- Discussion

2.1 A-IoT group or multi-round paging

It has made the agreement that RAN2 will study the following cases for AIoT paging message:

- a message containing an ID of a single A-IoT device.

- a message containing a group ID that maps to multiple A-IoT devices.

- a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message.

- a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion.

Moreover RAN2 has also confirm that slotted-ALOHA is the baseline for A-IoT random access and the A-IoT device will randomly choose a slot based on a Q-like value to access. A appropriate Q-like value is both good for reader’s resouce allocation and device’s access efficiency. To achieve this, it has been agreed that the approximate number of target devices is considered useful to be visible to the reader from CN. With the approximate number of target devices, the reader can set a appropriate Q-like value. However for all devices paging, if the number of devices is huge, the Q-like value has to be set large enough to avoid conflict. However a large Q-like value may cause resouce wasting if many slots are not been seleted by devices and may make device wait long if a device selects a large slot. To improve the access efficiency, the group paging or multi-round paging can be considered. For all device paging, if the number of target devices is huge, the devices can be further grouped and the devices only respond to the paging belong to its own group or round. For example, the reader can include the number of groups N and the current paging group i in the paging message, the device can choose the paging group index based on the number of group N and when the current paging group i in the paging equals its paging group index, it responds to the paging message.

Proposal 1: For all device paging, if the number of target devices is huge, the group paging or multi-round paging can be considered, the devices only respond to the paging belong to its own group or round.

2.2 A-IoT Paging message content

In prevous meeting, it has been agreed that AIoT paging message indicate information from which the device can determine resources to be used for response and a R2D message is used to provide access occasion(s) which can be used for re-access, the R2D message can be AIoT paging message. That means the AIoT paging message can provide resouces for both initial access and re-access. If both initial access resources and re-access resources are provided by AIoT paging message, the devices failed in initial access can try to re-access using the re-access resouces.

Observation 1: The AIoT paging message can provide resources for both initial access and re-access.

Proposal 2: If both initial access resources and re-access resources are provided by AIoT paging message, the devices failed in initial access can try to re-access using the re-access resouces.

For multiple devices paging, as the device ids are already known by the reader, the reader can configure contention free RA, the corresponding access resources can be assigned to each device to increase efficiency and avoid contention, the access resources can include time and frequency configuration. The re-access resources can also be provided for multiple devices paging in contention free RA and the re-access resources can be shared by the devices to improve resource utilization rate.

Proposal 3: The device id list and corresponding access configuration list can be included in the paging message for multiple device paging.

Proposal 4: The re-access resources can be provided for multiple devices paging in contention free RA and the re-access resources can be shared by devices to improve resource utilization rate.

- 3 Conclusions

In this contribution we discussed issues related to A-IoT paging and made the following proposals:

Observation 1: The AIoT paging message can provide resources for both initial access and re-access.

Proposal 1: For all device paging, if the number of target devices is huge, the group paging or multi-round paging can be considered, the devices only respond to the paging belong to its own group or round.

Proposal 2: If both initial access resources and re-access resources are provided by AIoT paging message, the devices failed in initial access can try to re-access using the re-access resouces.

Proposal 3: The device id list and corresponding access configuration list can be included in the paging message for multiple devices paging.

Proposal 4: The re-access resources can be provided for multiple devices paging in contention free RA and the re-access resources can be shared by devices to improve resource utilization rate.

- 4 References

- “Draft_RAN2_127b_Meeting_Report_v1”