3GPP TSG-RAN WG2 Meeting #128		     R2-2409909

Orlando, US: 18th November – 22nd November 2024

Source:	Qualcomm Incorporated

Title:	On Network Side Data Collection

Agenda Item:	8.1.3 NW Side Data Collection

Document for:	Discussion and Decision

# Introduction & Background

The Rel-18 study on AI/ML Air Interface [1] has been concluded. The study has been captured in the Technical Report TR 38.843 [2]. Based on the Rel-18 study outcome, in RAN #102, a new Work Item on artificial intelligence/machine learning for NR air interface has been approved [3].

This contribution paper discusses the data collection for the network side model training for beam management, CSI Feedback, and positioning enhancements use cases.

# Data Collection for Network Side Models

This section discusses the following:

- Data collection for network-side model training for beam management use case,

- Data collection for two-sided model training for CSI feedback use case, and

- Data collection for network-side model training for positioning enhancements use case.

## 2.1   Data Collection for Network-side Models for the Beam Management

In the RAN2#126 meeting [4], RAN2 made the following agreements for the network-side model training for the beam management use case:

Agreements for beam management

1. 	For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature. FFS how to handle case when single RRC message is not sufficient. FFS if there will be any further enhancement needed pending RAN1 agreement.

2.	Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

3.	Enhance the immediate MDT framework to support periodical reporting. FFS whether and what event-based reporting is supported and FFS on network request reporting

In the RAN2#127 meeting [5], RAN2 further agreed on the following for the beam management use case:

Agreements - As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. FFS if multiple configurations can be provided to the UE. FFS if dynamic activation/deactivation is supported. - UE stores the logged training data at the AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size. This is across all use cases - When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging. - Measurements for data collection purposes and logging based can be controlled based on power state of the UE. It is up to UE implementation how the UE determines power state. FFS whether the UE stops autonomously or if it reports to the network . - FFS whether AS buffer event based reporting is supported. FFS if we send availability indication or full report if it is supported - FFS on event based data collection/logging - On-demand request from the network is supported. FFS details on signalling

In the RAN2#127bis meeting [6], RAN2 further agreed on the following for the beam management use case,

Agreements on NW side data collection

- Periodic logging is supported for training data collection procedure in R19

- Event-triggered data logging will be supported.  At least radio condition based event triggered logging will be supported.  FFS the details of radio condition based event.  FFS if other events are supported.

Based on the above agreements, the following issues are FFS

- Training data collection configuration contents,

- Definition of event-triggered data collection,

- Multiple configurations and dynamic activation/deactivation,

- Handling data collection configuration and report during UE mobility.

This section discusses the FFS issues mentioned above.

### 2.1.1	Training Data Collection Configuration Contents

Like any measurement configuration and reporting/logging, the training data configuration should consist of measure targets and logging configuration (e.g., periodicity, event triggers for logging). For training data collection, the UE is expected to log L1-RSRP measurements on SSB/CSI-RS. For the legacy beam management, the resources or resource sets for measurements are configured using the CSI framework. The CSI framework should be used for configuring measurement targets (i.e., CSI resource or resource sets) for training data collections. When configuring/activating the training data collection at the UE, the network should ensure that SSB/CSI-RS configured for measurements across different AI/ML-related tasks (e.g., training data collection configuration, inference, and monitoring) and different non-AI/ML-related tasks (i.e., legacy CSI reporting for different purposes) lies within the maximum number of SSB/CSI-RS measurements supported at the UE.

Proposal 1: The training data collection configuration consists of

- Measurement targets (i.e., CSI resources or resource sets for measurements), and

- Logging configuration (e.g., periodicity, events for logging measurements).

Proposal 2: The CSI framework is used for configuring measurement targets (i.e., CSI resource or resource sets) for training data collections.

Proposal 3: The network should continue to ensure that the maximum number of SSB/CSI-RS configured for measurements lies within the maximum number of SSB/CSI-RS measurements supported at the UE per slot and per FR. That includes SSB/CSI-RS used for

- Different non-AI/ML-related tasks (i.e., legacy CSI reporting for different purposes), and

- Different AI/ML-related tasks (e.g., training data collection configuration, inference, and monitoring), and

- Different AI/ML features (e.g., beam management, CSI prediction, CSI Feedback).

### 2.1.2	Definition of Event-Trigger Data Collection

In the RAN2#127bis meeting [6], RAN2 agreed to support the event-triggered data logging. Furthermore, RAN2 agreed to support at least radio condition-based event trigger logging.

Agreements on NW side data collection

- Event-triggered data logging will be supported.  At least radio condition based event triggered logging will be supported.  FFS the details of radio condition based event.  FFS if other events are supported.

Events for the data collection for the network side model training should be defined such that it achieves the following objectives

- Reduction of redundant data collection at the UE,

- Collect meaningful representative data for model training.

Different event-triggered data collection and logging can be defined to reduce redundancy and collect meaningful representative data for model training. Let us consider the following examples for defining events:

- The network may want to avoid overfitting and, therefore, may configure UE to collect and report/log measurements across different beams (i.e., avoid logging too many measurements on a beam/set of beams). To achieve diversity in the data collection and reporting/logging at the UE, the UE can be provided with the distributions of samples (e.g., beam indices and the number of samples) to be collected and logged when a beam is measured as a top-1 beam. UE stops reporting/logging new measurements when the maximum number of samples configured for a beam is reached (when that beam is the top-1 beam).

- The network may want to capture the channel dynamicity (at the same time avoid redundancy in the data collection). Therefore, the network may configure UE to collect and report/log upon the change of the top-1 beam. UE may be additionally configured with the number of samples to be collected and reported/logged when the event is triggered and its periodicity. If logging is supported, UE may also be configured with the number of past measurement samples to be reported/logged.

Apart from considering the top-1 beam for defining the event for measurement collection and logging, top-K beams, the L1-RSRP of the top-1, or the L1-RSRP of one or more beams among top-K beams can be further considered.

Proposal 4: RAN2 is requested to consider at least the following events for training data collection for network-side model training,

- Based on the number of samples to be collected and reported across different beams, UE triggers the measurement collection and logging if a beam becomes the top-1 beam and the logged number of measurements is less than the configured value.

- Based on the change of the top-1 beam: UE is configured to log the measurement when the top-1 beam changes. UE can additionally be configured with the number of samples to be logged and its periodicity.

### 2.1.3	Multiple Configuration and Dynamic Activation/Deactivation

In the RAN2, the following are FFS issues associated with data collection configuration for network-side model training,

- Whether the UE can be configured with multiple RS configurations for network-side model training,

- Whether dynamic activation/deactivation is supported

Note that the performance of AI/ML functionalities/models depends on the quality/distribution of the training data. For example, with periodic data collection, the network may be able to collect the training data across different beams of the serving cells. However, if a UE is static and configured with periodic data collection, then the collected training data may be redundant. Note that when redundant data is used for training, it may create significant biases. One solution may be that redundancy is removed during the training by the NW-side training server. However, this will require a huge amount of training data to be collected at the UE, as after removing the redundancy the network side may end up with a significantly low amount of meaningful training data. Therefore, the following issues may happen with periodic data collection:

- Huge amount of training data collection at the UEs,

- Significantly high redundancy causing biases during training, if not handled by the training server properly.

Observation 1: Periodic data collection has the following issues,

- Requires a huge amount of training data collection at the UEs, as there may exist significantly high redundancy,

- The network-side training server needs to handle redundancy in the training data, otherwise, it may cause biases in the trained models.

As described in Fig. 2.1.3-1, for the L1-RSRP predictions of top-1/top-K beams, the training data should be independent and identically distributed (i.i.d) across beams. We need a solution that alleviates/minimizes the redundancy in the data collection and captures a good representation across beams, across beam center vs beam center, and others. This can be achieved by introducing randomness in the training data logging/reporting.

Fig. 2.1.3-1: An example of training data collection where the training data is distributed (equally) across the beams, across the beam center, beam edge, etc.

This section further discusses the FFS issues (discussed above) associated with the data collection configuration.

#### 2.1.3.1	 On Supporting Multiple RS Configuration

In the legacy CSI framework, CSI-ResourceConfig indicates the RS configuration, and CSI-ReportConfig is provided for configuring the CSI report. From TS 38.331 [7], we can see that up to 48 CSI-ReportConfigurations can be provided. Similarly, up to 112 RS Configurations can be provided. Therefore, following the legacy CSI report framework, multiple CSI RS configurations and reporting/logging configurations can be provided.

Observation 2: UE can be configured with up to 112 CSI resource configurations in the legacy CSI framework.

Proposal 5: The UE can be configured with multiple CSI resources or resource sets for training data collection.

–	CSI-ResourceConfig

The IE CSI-ResourceConfig defines a group of one or more NZP-CSI-RS-ResourceSet, CSI-IM-ResourceSet and/or CSI-SSB-ResourceSet.

CSI-ResourceConfig information element

-- ASN1START

-- TAG-CSI-RESOURCECONFIG-START

CSI-ResourceConfig ::=      SEQUENCE {

csi-ResourceConfigId        CSI-ResourceConfigId,

csi-RS-ResourceSetList      CHOICE {

nzp-CSI-RS-SSB              SEQUENCE {

nzp-CSI-RS-ResourceSetList  SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL, -- Need R

csi-SSB-ResourceSetList     SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSetsPerConfig)) OF CSI-SSB-ResourceSetId  OPTIONAL  -- Need R

},

csi-IM-ResourceSetList      SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig)) OF CSI-IM-ResourceSetId

},

/* omitted

}

-- TAG-CSI-RESOURCECONFIG-STOP

-- ASN1STOP

–	CSI-ResourceConfigId

The IE CSI-ResourceConfigId is used to identify a CSI-ResourceConfig.

CSI-ResourceConfigId information element

-- ASN1START

-- TAG-CSI-RESOURCECONFIGID-START

CSI-ResourceConfigId ::=            INTEGER (0..maxNrofCSI-ResourceConfigurations-1)

-- TAG-CSI-RESOURCECONFIGID-STOP

-- ASN1STOP

6.4	RRC multiplicity and type constraint values

/*omitted

maxNrofCSI-ResourceConfigurations       INTEGER ::= 112     -- Maximum number of resource configurations

/*omitted

maxNrofNZP-CSI-RS-ResourceSetsPerConfig INTEGER ::= 16      -- Maximum number of resource sets per resource configuration

/*omitted

maxNrofCSI-SSB-ResourceSetsPerConfig    INTEGER ::= 1       -- Maximum number of CSI SSB resource sets per resource configuration

/*omitted

#### 2.1.3.2	On Supporting Dynamic Activation/Deactivation

From TS 38.214 [7], we have the following,

- The reporting configuration for CSI can be aperiodic (using PUSCH), periodic (using PUCCH), or semi-persistent (using PUCCH, and DCI activated PUSCH).

- The CSI-RS Resources can be periodic, semi-persistent, or aperiodic.

Furthermore, Table 5.2.1.4-1 in TS 38.214 illustrates supported combinations of CSI reporting configuration and CSI-RS Resource configuration (also copied below).

| CSI-RS Configuration | Periodic CSI Reporting | Semi-Persistent CSI Reporting | Aperiodic CSI Reporting |
| --- | --- | --- | --- |
| Periodic CSI-RS | No dynamic triggering/activation | For reporting on PUCCH, the UE receives an activation command, as described in clause 6.1.3.16 of [10, TS 38.321]; for reporting on PUSCH, the UE receives triggering on DCI | Triggered by DCI; additionally, subselection indication as described in clause 6.1.3.13 of [10, TS 38.321] possible as defined in Clause 5.2.1.5.1. |
| Semi-Persistent CSI-RS | Not Supported | For reporting on PUCCH, the UE receives an activation command, as described in clause 6.1.3.16 of [10, TS 38.321]; for reporting on PUSCH, the UE receives triggering on DCI | Triggered by DCI; additionally, subselection indication as described in clause 6.1.3.13 of [10, TS 38.321] possible as defined in Clause 5.2.1.5.1. |
| Aperiodic CSI-RS | Not Supported | Not Supported | Triggered by DCI; additionally, subselection indication as described in clause 6.1.3.13 of [10, TS 38.321] possible as defined in Clause 5.2.1.5.1. |

Observation 3: When the Resource Configuration(CSI-RS Configuration = NZP CSI RS) is periodic, the Report Configuration can be set to any type (periodic, semi-persistent, or aperiodic).

Observation 4: When the Resource Configuration(CSI-RS Configuration = NZP CSI RS) is semi-periodic, the Report Configuration can be Semi-persistent or Aperiodic.

Observation 5: When the Resource Configuration (CSI-RS Configuration = NZP CSI RS) is aperiodic, the Report Configuration can only be aperiodic.

As discussed previously, to achieve adequate AI/ML functionality/model performance, the redundancy in the training data should be minimized. This can be achieved by introducing randomness in the training data logging/reporting. Following the above observation 2 – observation 4 from the legacy CSI framework, Fig. 2.2.1.2-1 illustrates a potential solution for redundancy in the training data collection.

Fig. 2.1.3.2-1: Training data collection: UE can be configured with periodic/semi-persistent/aperiodic RS configuration, but the CSI report configuration is semi-persistent/aperiodic.

Proposal 6: Dynamic activation/deactivation of training data collection and logging should be used to randomize the training data collection and logging, following semi-persistent and aperiodic CSI reporting as the reference.

### 2.1.4	Handling Data Collection Configuration and Reporting during UE Mobility

In the RAN2#126 meeting [4], RAN2 agreed that “For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature.” In this section, we discuss several issues on handling data collection configuration and reporting of collected training data during UE mobility.

Observation 6: For the beam management use case, RAN2 agreed that logging of L1-RSRP measurement for gNB-centric / OAM-centric data collection for NW-side model training is an optional feature for the UE.

#### 2.1.4.1 	UE Behavior when the UE has Unretrieved Training Data During Mobility

This section discusses the UE behavior when the UE has unretrieved training data during the mobility. In the legacy immediate MDT, the UE reports the measurements to the gNB that configures the measurements, i.e., no logging and reporting of the measurements later is not supported. The serving gNB determines measurement configuration (for example, based on the configuration received from the OAM) and receives the measurements from the UE which are then forwarded to the TCE. During the handover/reestablishment/resume/reconnection, the immediate MDT configuration is forwarded from the source gNB/AMF to the target gNB if they belong to the MDT PLMN List configured by the OAM. While the reporting of measurements collected is immediate, UE is not required to check the PLMN before reporting. The EU is not aware of the MDT PLMN List. Therefore, no undesired leakage of information/measurements.

In SON and logged MDT reports, the UE stores PLMN identity list (equivalent PLMNs) where the UE can report SON/logged MDT reports. This ensures no undesired leakage of information/measurements.

For the training data collection, UE may need to allocate sufficient resources such as memory and power. Therefore, discarding the unretrieved training data during UE mobility is not desired. We prefer RAN2 to clarify the UE behavior when UE has unretrieved logged training data during handover, RRC reestablishment, handover failures, radio link failures, or RRC state transitions. RAN2 needs to clarify further if there are concerns associated with the leakage of the training data when UE moves from

- One PLMN to another PLMN, i.e., whether training data collected in one PLMN can reported to any PLMN, and

- One infra-vendor to another infra-vendor, i.e., whether training data collected at the NG-RAN from one infra-vendor can reported to NG-RAN from another infra-vendor.

Observation 7: In the following scenarios, the configuring gNB may not be able to retrieve the logged training data

- The configuring gNB was overloaded and could not retrieve the training data before handover, or

- UE has RLF before the training data can be retrieved by configuring gNB1.

Proposal 7: Discarding the unretrieved training data upon handover / RLF / RRC state transition is not desired, as it is a waste of UE power and memory spent towards NW-side data collection.

Proposal 8: RAN2 should clarify if there are concerns associated with the leakage of the training data when UE moves from

- One PLMN to another PLMN, i.e., whether training data collected in one PLMN can reported to any PLMN, and

- One infra-vendor to another infra-vendor, i.e., whether training data collected at the NG-RAN from one infra-vendor can reported to NG-RAN from another infra-vendor.

##### 2.1.4.1.1  On the leakage of information from one PLMN to another PLMN

Let us consider the following scenario (see Fig. 2.1.4.1.1-1) where a cell gNB1 (belonging to vendor 1 and PLMN 1) configures the UE with the training data collection (measurement and reporting configuration), but cannot retrieve the logged training data because of the following reasons

- The gNB1 was overloaded and couldn’t retrieve the training data before the handover, or

- UE has RLF before the training data can be retrieved by gNB1.

2.1.4.1.1-1: OAM-centric data collection, whether the logged training data can be reported to other PLMN or other-infra-vendor cells if they belong to the same/different PLMN.

##### 2.1.4.1.2  On the leakage of information from one infra-vendor to another infra-vendor

In the legacy L3 measurement collection and reporting, data logging is not supported. This implies that the serving gNB configures the UE with measurement/reporting configuration, and receives the measurements. During the handover, the measurement/reporting configuration is handled by the following methods

- UE maintains the configuration if configured with need code M

- UE releases the configuration if configured with need code R

- Serving/target cell can explicitly release the configuration by sending a reconfiguration message

Consider the following scenario (see Fig. 2.1.4.1.2-1) where a single PLMN deploys the cells/gNBs from two infra-vendors. Let us assume that gNB1 from vendor 1 configures the data collection at the UE, but cannot retrieve the logged training data because of the following reasons

- The gNB1 was overloaded and couldn’t retrieve the training data before the handover, or

- UE has RLF before the training data can be retrieved by gNB1.

Fig. 2.1.4.1.2-1: gNB-centric data collection, whether the logged training data can be reported to other infra-vendor cells if they belong to the same/different PLMN.

The UE allocates significant resources, such as memory and power, for NW-side training data collection, therefore reporting unretrieved training data should be supported. Discarding the unretrieved training data upon handover / RLF / RRC state transition is not desired, as it is a waste of UE power and memory spent on NW-side data collection. To allow reporting of unretrieved data, the UE should be provided with area configuration (e.g., list of cells/gNBs/PLMNs) where UE can report the logged training data.

Proposal 9: The UE is provided with an area configuration (e.g., list of cells/gNBs/PLMNs) for the reporting of the unretrieved logged training data.

#### 2.1.4.2 Handing of Training Data Collection Configuration during UE Mobility

##### 2.1.4.2.1 UE Behaviour Related to the Training Data Collection Configuration

In the legacy CSI framework, the CSI-MeasConfig is used to configure CSI-RS (reference signal) belonging to the serving cell. Following the legacy CSI framework, the training data collection configuration for the beam management may be released upon handover/RLF/RRC state transitions from CONNECTED to IDLE/INACTIVE states. Therefore, UE is not configured with area configuration for the measurements.

Proposal 10: UE is not configured with area configuration for the measurement of resources or resource sets for training data collection, as these configurations are cell specific.

##### 2.1.4.2.2 Handing of Training Data Collection Configuration at the network

Legacy immediate MDT can be signaling-based MDT or management-based MDT. Signaling-based immediate MDT is used for collecting data from specific UEs, and the UE selection for immediate MDT is performed at the OAM. For the network-side model training, there is no need to collect training from specific UEs. Therefore, there is no need to support signaling-based immediate MDT for network-side model training.

Observation 8: Signaling-based immediate MDT is used for collecting data from specific UEs, and the UE selection for immediate MDT is performed at the OAM.

Observation 9: For the network-side model training, there is no need to collect training from specific UEs.

Proposal 11: Signaling-based immediate MDT is not supported for NW-side training data collection.

On the other hand, the management-based immediate MDT is used for collecting data in a specific area, and UE selection for immediate MDT is performed at the gNB. For the management-based MDT, the Management Based MDT PLMN List IE is sent HANDOVER REQUEST from the source to target NG-RAN, and from AMF to the target NG-RAN. Therefore, if the serving cell belongs to the Management Based MDT PLMN List IE, then they should, if supported, select UEs for configuring training data collection configuration. Furthermore, note that Management Based MDT PLMN List IE is stored in the UE context for management-based MDT data collection.

Observation 10: For management-based MDT, the data collection is performed at cells/gNBs if they belong to the Management Based MDT PLMN List IE and support MDT data collection.

For the management-based MDT, the OAM may indicate cell(s)/frequency(ies) on which the training data collection should be performed. However, as the CSI resource configuration is serving cell-specific, therefore during handover/Reestablishment/RRC state transition, the new NG-RAN should provide data training collection configuration based on area configuration indicated by OAM.

Proposal 12: For OAM-centric data collection (i.e., for management-based MDT), the OAM may indicate cell(s)/frequency(ies) to the NG-RANs on which the training data collection should be performed.

#### 2.1.4.4 Override Protection of unretrieved Training Data

The UE may allocate sufficient resources and power for NW-side training data collection, the override protection of unretrieved training data should be supported. RAN2 is requested to discuss whether/how override protection of unretrieved training data collection should be supported.

Observation 11: UEs allocate sufficient resources and power for NW-side training data collection, thus override protection of unretrieved training data should be supported.

Proposal 13: RAN2 should further discusses whether/how override protection of unretrieved training data collection should be supported.

## 2.2   Data collection for two-sided model training for CSI feedback use case

In the RAN1#116 meeting to alleviate/resolve the issues related to infra-vendor collaboration of AI/ML-based CSI compression using two-side models, the following were agreed for study [8],

Agreement To alleviate / resolve the issues related to inter-vendor training collaboration of AI/ML-based CSI compression using two-sided model, study the following options: - Option 1: Fully standardized reference model (structure + parameters) - Option 2: Standardized dataset - Option 3: Standardized reference model structure + Parameter exchange between NW-side and UE-side - Option 4: Standardized data / dataset format + Dataset exchange between NW-side and UE-side - Option 5: Standardized model format + Reference model exchange between NW-side and UE-side Note 1: The above options may not be mutually exclusive and may be used together. Note 2: Other options are not precluded. Note 3: The study should consider how different methods of exchanging the parameters / dataset / reference model would affect the feasibility and collaboration complexity of options 3 / 4 / 5 respectively, e.g., over the air-interface, offline delivery, etc. Note 4: “Dataset” refers to a set of data samples of CSI feedback and associated target CSI.

Agreement For the study of inter-vendor collaboration issues for AI/ML-based CSI compression using a two-sided model, consider at least the following aspects when comparing different options: - Inter-vendor collaboration complexity, e.g., whether bilateral collaboration is required between vendors. - Performance. - Interoperability and RAN4 / testing related aspects. - Feasibility.

In RAN1#116bis meeting [9], RAN1 further made the following agreements

Agreement - For Option 3, further define the two sub-options: - 3a: Parameters received at the UE or UE-side goes through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., potential re-training, re-development of a different model, and/or offline testing. - 3b: Parameters received at the UE are directly used for inference at the UE without offline engineering, potentially with on-device operations. - For Option 5, further define the two sub-options: - 5a: Model received at the UE or UE-side goes through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., potential re-training, re-development of a different model, and/or offline testing. - 5b: Model received at the UE are directly used for inference at the UE without offline engineering, potentially with on-device operations. - For Option 4, it is clarified that: - Dataset received at the UE or UE-side goes through offline engineering at the UE- side (e.g., UE-side OTT server), e.g., model training or offline testing. - Note: The descriptions under each option are only for the purpose of simplified discussion and do not mean deprioritizing any other flavours (such as an exchange originating from the UE-side and ending at the NW-side) from potential specification.

Agreement (amended as shown in red in Wednesday session) - For Option 3/4/5, focus further discussion on the following assumptions: - Option 3a/5a - The model(5a)/parameter(3a) exchange originates from the NW-side and ends at the UE-side. - Model(5a)/parameters(3a) exchanged from the NW-side to UE-side is either CSI generation or reconstruction part or both. - Option 3a-1/5a-1: Model/Parameters exchanged from the NW-side to UE-side is CSI generation part. - Option 3a-2/5a-2: Model/Parameters exchanged from the NW-side to UE-side is CSI reconstruction part. - Option 3a-3/5a-3: Model/Parameters exchanged from the NW-side to UE-side are both CSI generation part and CSI reconstruction part. - Some additional information, if necessary, may be shared from the NW-side to help UE-side offline engineering and provide performance guidance. - Performance target - Dataset or information related to collecting dataset - Study different methods of exchanging, e.g., over the air-interface, offline delivery, etc. - Option 3b - The method of exchanging is over the air-interface via model transfer/delivery Case z4. - The parameter exchange is from NW to UE. - Parameters exchanged from the NW-side to UE-side is CSI generation part. - Option 5b - The method of exchanging is over the air-interface via model transfer/delivery Case z4, assuming that the model structure is aligned based on offline inter-vendor collaboration. - The model exchange is from NW to UE. - Model exchanged from the NW-side to UE-side is CSI generation part. - Option 4: - The dataset exchange originates from the NW-side and ends at the UE-side. - Option 4-1: Dataset exchanged from the NW-side to UE-side consists of (target CSI, CSI feedback). - Option 4-2: Dataset exchanged from the NW-side to UE-side consists of (CSI feedback, reconstructed target CSI). - Option 4-3: Dataset exchanged from the NW-side to UE-side consists of (target CSI, CSI feedback, reconstructed target CSI). - Some additional information, if necessary, may be shared from the NW-side to help UE-side offline engineering and provide performance guidance. - Performance target - Study different methods of exchanging, e.g., over the air-interface, offline delivery, etc. - Note: For each option/sub-option of interest, companies to bring discussion on how inter-vendor collaboration complexity, interoperability, and feasibility may be addressed. Companies to strive to provide solution(s) that can address all the following aspects: inter-vendor collaboration complexity, performance, interoperability, and feasibility. - Note: The descriptions under each option are only for the purpose of simplified discussion and do not mean deprioritizing any other flavours (such as an exchange originating from the UE-side and ending at the NW-side) from potential specification.

Conclusion: - Conclude, from RAN1 perspective, that Option 1, if feasible for specification, eliminate the inter-vendor collaboration complexity (e.g., whether bilateral collaboration is required between vendors). - It is RAN1’s understanding that Option 1 corresponds to RAN4 options, e.g., RAN4-Option3, or RAN4-Option4. Further study and final conclusion on interoperability and RAN4 testing of the RAN4-Option3 and RAN4-Option4 is up to RAN4. Observation - Option 1 and 2 may have limited performance in the field compared to Options 3, 4, and 5, further study is needed - Option 1 and 2 may require high specification effort from RAN1 perspective. Conclusion - Deprioritize Option 2 for inter-vendor training collaboration. - Note: This de-prioritization shall not affect the ongoing discussion in RAN4 on RAN4-Option3 and RAN4-Option4.

In RAN1#117 meeting [15], RAN1 further made the following agreements:

Conclusion Standardized signalling, if feasible and specified, can be used for parameter / model exchange in option 3a/5a and 3b to alleviate/resolve the inter-vendor training collaboration complexity. - Standardized signalling may be reused for exchanging CSI generation part, CSI reconstruction part, or both, etc, when necessary and feasible. - Standarized signalling may be over-the-air, or other approaches. Standardized signalling, if feasible and specified, can be used for dataset exchange in option 4 to alleviate/resolve the inter-vendor training collaboration complexity. - Standardized signalling may be reused for dataset exchanging, when necessary and feasible. - Standarized signalling may be over-the-air, or other approaches. Note: feasibility will be discussed separately. Agreement - For option 3a/3b/4/5a and their sub-options, at least the following potential specification impacts have been identified. Further study the necessity, feasibility, their specification impact. - Exchange - Parameter / model exchange methods, format/contents, and related spec impacts (3a/3b/5a) - Dataset exchange methods, format/type/contents of data/dataset, and related spec impacts (4) - Additional information, if necessary, that may be shared from the NW-side to help UE-side offline engineering and provide performance guidance (3a/5a/4) - Performance target (3a/5a/4) - Dataset or information related to collecting dataset (3a/5a) - Any other additional information - Model pairing (3a/3b/4/5a) - UE capability (3a/3b/4/5a) - Model related aspects, such as scalability (e.g., payload sizes, antenna ports, bandwidth), rank and layer handling (3a/3b/4/5a) - Quantization of feedback (3a/3b/4/5a) - Model structure details (3a/3b) Note: Option 3a/4/5a and option 3b serve two different deployment time scales, UE capabilities, device-side optimizations, and training methods, and therefore may be complementary to each other, with potential specification of both. - Specification of option 1, if needed from RAN1, can reuse specification of option 3a/3b, with the additional specification of parameters.

In RAN1#118 meeting [16], RAN1 further made the following agreements:

Observation Option 5 alone doesn’t fully address inter-vendor collaboration complexity, but Option 5 has potential performance benefit and flexibility compared to Option 3. Agreement Continue the study of following directions - on-device operation and UE side offline engineering - Direction A: Sharing parameters/reference model/dataset that enables UE-side offline engineering (Inter vendor collaboration option 3a/5a/4) - Potential down-selection into one or more among sub-options 3a/5a-1, 3a/5a-2, 3a/5a-3, 4-1, 4-2, and 4-3 considering their feasibility and performance, including at least the following issues - [Issue 1] What additional information should be shared from NW-side to UE-side to enable UE-side encoder training, validation, and testing? - [Issue 2] Is there concern for NW’s proprietary information disclosure, and if so, how to address it? - [Issue 3] Is there an overhead concern, and if so, how to address it? - [Issue 4] Is there performance impact due to mismatch between NW side data distribution and UE side data distribution, and if so, how to address it? - Direction B: Sharing NW side encoder parameter to UE side for UE side inference directly with on-device operation (Inter vendor collaboration option 3b), including at least the following issues - [Issue 3] Is there an overhead concern, and if so, how to address it? - [Issue 5] Whether it is feasible to use a common encoder across UEs, and whether it is feasible for NW-side to train multiple encoders for different UEs? - [Issue 6] Is there performance impact due to mismatch between NW side data distribution and UE side inference data distribution, and if so, how to address it? - [Issue 7] Is there concern for NW’s and UE’s proprietary information disclosure, and if so, how to address it? - Direction C: Fully standardized reference model(s) and parameters with specified CSI generation part and/or CSI reconstruction part (Inter vendor collaboration option 1), including at least the following issues - [Issue 8] Whether to consider 3GPP’s statistical channel model or field data for reference model(s) training. In case of the latter, how does RAN1 collect field data and agree on them? - [Issue 9] Is there performance impact due to mismatch between the distribution of the dataset used for reference model(s) training, UE-side data distribution, and NW-side data distribution, and if so, how to address it? - [Issue 10] What additional information should be specified to enable UE-side encoder training, validation, and testing, and NW-side decoder training, validation, and testing? - Note: - 1-1: Only reference encoder is specified, and NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately compatible to the reference encoder. - 1-2: Only reference decoder is specified, and NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately compatible to the reference decoder. - 1-3: Both reference encoder and reference decoder are specified, and NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately that are compatible to the reference decoder/encoder. - Note: UE-side data and NW-side data in “UE-side data distribution” and “NW-side data distribution” are field data. - Note: Some issues identified in one direction may/may not be applicable for other Direction. - Note: potential down selection among the 3 directions is not precluded - Study of data distribution mismatch to consider the use of synthetic data and/or field data.

In RAN1#118bis meeting [17], RAN1 further made the following agreements:

Agreement For Directions of addressing inter-vendor collaboration complexity for two-sided CSI compression, - For Direction A: - Further study the following options with potential down-selection - 4-1: dataset is {target CSI, CSI feedback}, - 3a-1: encoder parameter exchange, with {target CSI} - Note: Additional information beyond what’s listed above may need to be exchanged if necessary.

From RAN1 agreements in meeting RAN1#116 [8] and RAN1#116bis [9], we can see the dataset, parameter, and reference model exchange can be supported using

- Over-the-Air Signaling,

- Offline delivery/transfer.

Observation 12: Based on RAN1 agreements [2][3], the dataset, parameter, and reference model exchange can be supported using

- Over-the-Air Signaling,

- Other standard-based solutions for offline delivery/transfer.

As RAN1 concluded that standardized signalling, if feasible and specified, can be used for parameter / dataset / model exchange in option 3a/4/5a and 3b to alleviate/resolve the inter-vendor training collaboration complexity.

Conclusion Standardized signalling, if feasible and specified, can be used for parameter / model exchange in option 3a/5a and 3b to alleviate/resolve the inter-vendor training collaboration complexity. - Standardized signalling may be reused for exchanging CSI generation part, CSI reconstruction part, or both, etc, when necessary and feasible. - Standarized signalling may be over-the-air, or other approaches. Standardized signalling, if feasible and specified, can be used for dataset exchange in option 4 to alleviate/resolve the inter-vendor training collaboration complexity. - Standardized signalling may be reused for dataset exchanging, when necessary and feasible. - Standarized signalling may be over-the-air, or other approaches. Note: feasibility will be discussed separately.

As alleviating/resolving issues related to inter-vendor training collaboration is one of the major Rel-19 study objectives for two-sided CSI compression,

SID objective (RP-242399) - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing necessary specification impact analysis, as well as, other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843.

RAN2 should at least discuss and conclude the existence of feasible solutions for dataset / parameter, reference model exchange in order to fulfill the study objectives.

Observation 13: Alleviating/resolving issues related to inter-vendor training collaboration is one of the major Rel-19 study objectives for two-sided CSI compression.

This section further discusses, dataset, parameter, and reference model exchange using the over-the-air signaling and offline delivery/transfer.

#### 2.2.1 Dataset, Parameter, and Reference Model Exchange Over the Air

Note that training at the UE-side is offline training, therefore the training dataset, parameter, or reference model must be sent to the UE-side training server. Fig. 2.2.1-1 illustrates the dataset, parameter, or reference model delivery/transfer over the air. In Fig. 2.2.1-1, the RAN sends the required dataset, parameter, or reference model, after that, the UE sends the dataset, parameter, or reference model (sent by the RAN) to the UE side training server. This method of exchanging the dataset, parameter, or reference model for training is not suitable, as the significantly large amount of training data is duplicated from multiple UEs, and at the same time it causes the Uu overhead twice: (i) for delivering/transferring the dataset, parameter, or reference model from the RAN to the UE, and (ii) for delivering/transferring the dataset, parameter, or reference model from the UE to the UE side server.

Fig. 2.2.1-1: Dataset, parameter, or reference model delivery/transfer over the air

As discussed above, the UE part of the two-side model will not be trained at the UE, rather it will be trained at a UE side server.

Observation 14: The UE part of the two-side model is trained at a UE side server not at the UE, i.e., training is not performed by the UE rather it is performed by the UE vendors.

Observation 15: Over-the-air exchange/transfer of the training dataset, parameter, or reference model for supporting UE-side model training causes the following issues

- Significant duplication of data, as the same training dataset, parameter, or reference model may need to be sent to all UEs CONNECTED to a cell/gNB,

- Double the Uu overhead: (i) for delivering/transferring the dataset, parameter, or reference model from the RAN to the UE, and (ii) for delivering/transferring the dataset, parameter, or reference model from the UE to the UE side server.

- Training may not be frequently performed at both the UE side or the network side for two-sided, but the exchange of the training dataset, parameter, or reference model may need to be supported continuously. There is no method by which gNB can ensure that all UE vendors have received the required training dataset, parameter, or reference model, and it can stop transferring/delivering the training dataset, parameter, or reference model.

- Even if the network-part model of the two-side model is well generalized across multiple cells/gNBs, it needs to transfer/deliver the same training dataset, parameter, or reference model from multiple cells/gNBs.

Considering the issues associated with the over-the-air transfer of the training dataset, parameter, or reference model to support UE-side model training, transferring the training dataset, parameter, or reference model is a not scalable solution. RAN2 is requested to develop a solution that minimizes/alleviates the Uu overhead associated with transferring the training dataset, parameter, or reference model to the UE side. One possible solution is that serving cell(s) may configure UEs to provide the information for delivering/transferring the training dataset, parameter, or reference model to the UE side. The serving cell(s) can provide OAM or a Network Function (NF) with this information for delivering/transferring the training dataset, parameter, or reference mode.

Proposal 14: Over-the-air delivery/transfer of the training dataset, parameter, or reference model to support UE-side model training is not a suitable or scalable solution. Please refer to the issues mentioned in observation 15.

#### 2.2.2 Other Standard-based Solutions for Dataset, Parameter, and Reference Model Delivery/Transfer

In the SA WGs, we have precedence where different types of data are shared with application function (within/outside the 3GPP network) and or edge cloud, for example,

- Target UE location to AF [TS 23.273],

- Analytics exposure from NWDAF to AF, e.g., network status reporting [TS 23.288],

- Traffic volume, UL/DL data rates, PDU session status, and others [TS 23.501].

Observation 16: Within the SA WGs, we have precedence where different types of data are shared with application function (within/outside the 3GPP network) and or edge cloud, for example,

- Target UE location to AF [TS 23.273],

- Analytics exposure from NWDAF to AF, e.g., network status reporting [TS 23.288],

- Traffic volume, UL/DL data rates, PDU session status, and others [TS 23.501].

To minimize the inter-vendor collaboration while reducing the Uu overhead, RAN2 is requested to discuss other standard-based solutions (other than transferring over the Uu) for transferring the training dataset, parameter, or reference model from the network entity (e.g., RAN/OAM/NF) to the UE-side training server (within/outside the 3GPP network).

Proposal 15: RAN2 concludes that a standard-based solutions (other than delivering/transferring over the Uu) for delivering/transferring the training dataset, parameter, or reference model from the network entity (e.g., RAN/OAM/NF) to the UE-side training server (within or outside the 3GPP network) area is feasible and the 3GPP can develop such a solution in the Rel-20 normative work.

## 2.3   Data Collection for Network-side Model for Positioning Enhancements

### 2.3.1	UE-assisted positioning with LMF-side model (Case 2b)

For (2nd priority) Case 2b, the UE reports DL-PRS-based measurements to an LMF which may be any of the existing measurement types (e.g., RSTD, UE Rx-Tx Time Difference, etc.) or any new measurement types defined by RAN1 (e.g., channel impulse response measurements, etc.). For Case 2b data collection, SA2 agreed the general procedure in Figure 2.3.1-1 [11], [12].

Fig. 2.3.1-1: Data collection by LMF to train the AI/ML based positioning model using UE measurements (Case 2b) (according to [11], [12])

At Step 1, the LMF subscribes to an AMF to retrieve the list of SUPIs located in an area of interest using the Namf_EventExposure_Subscriber_Request operation. The AMF then responds with a Namf_EventExposure_Subscriber_Response operation including a list of SUPIs in the area of interest. After the LMF verified the user consent for data collection for model training for each SUPI, the LMF requests necessary data from the UE to train the LMF-side models at Step 4.

To obtain the input data for model training from the UE, any of the existing positioning procedures can be used [11],[13]. For example, the LMF may use a UE-based/UE-assisted LPP positioning procedure at Step 4. If periodic or event triggered measurement reporting is desired, the LMF may instigate a deferred MT-LR procedure as specified in TS 23.273 [11], using a desired event type (e.g., periodic location, motion event, area event, etc.). Additional trigger criteria can be added to existing messages/procedures, if needed. The deferred MT-LR procedures already support e.g., handover, handles the Correlation ID, supports changes of LMF, supports UE/LMF triggered cancellation procedures, etc. In addition, the Event Reports (with embedded LPP Provide Location Information messages) can also be sent over user plane.

Proposal 16: For LMF-side data collection in Positioning Enhancements Case 2b, existing LCS and UE positioning procedures (e.g., NI-LR, MT-LR and/or deferred MT-LR) are used to obtain the desired measurements from a UE/PRU.

For the label generation for both, model training data generation and model performance metric calculation in label-based model monitoring, the UE/PRU 'ground-truth' location would be needed at the LMF. However, obtaining a UE/PRU location by an LMF is an existing functionality. A LMF could use any of the existing UE-based, UE-assisted, or network-based positioning methods to obtain the UE/PRU location (dependent on UE and network capability). The UE/PRU location determination must be independent on the data collected for model training. For example, the LMF may use a RAT-independent location method (e.g., GNSS) to obtain the label, and using DL-TDOA, DL-AoD, or Multi-RTT positioning procedures to collect the desired data for model training (the details on the measurements depend on RAN1 progress). For PRU's, the PRU location may be a surveyed location provided to an LMF during PRU registration [11].

Proposal 17: For Positioning Enhancements Case 2b ground-truth label generation (for training and label-based monitoring), the entity which obtains the UE/PRU location is an LMF and existing LCS [11] and UE positioning [13] procedures are used to obtain the UE/PRU location estimate by an LMF.

### 2.3.2	Network-based positioning with LMF-side model (Case 3b)

For case 3b, the NG-RAN node reports SRS-based measurements to an LMF which may be any of the existing measurement types (e.g., RTOA, gNB Rx-Tx Time Difference, etc.) or any new measurement types defined by RAN1 (e.g., channel impulse response measurements, etc.). For Case 3b data collection, SA2 agreed the general procedure in Figure 2.3.2-1 [14].

Fig. 2.3.2-1: Data collection by LMF to train the AI/ML based positioning model using NG-RAN measurements (according to [14])

Steps 2 – 5 in Figure 2.3.2-1 are the same as in Case 2b (Figure 2.3.1-1). At Step 6, the LMF configures a desired SRS in the UE/PRU using existing NRPPa and RRC procedures and requests the desired measurements for data collection from NG-RAN using the existing NRPPa measurement procedures [13].

For Step 7 in Figure 2.3.2-1, the same as discussed in section 2.3.1 above applies: A LMF could use any of the existing UE-based, UE-assisted, or network-based positioning methods to obtain the UE/PRU 'ground-truth' location at Step 7 in Figure 2.3.2-1.

The details of the data collection for Case 3b (Step 6) are up to RAN3. However, Step 7 in Figure 5 involves the UE/PRU and may be in RAN2's scope. Therefore:

Proposal 18: For Positioning Enhancements Case 3b ground-truth label generation (for training and label-based monitoring), the entity which obtains the UE/PRU location is an LMF and existing LCS [11] and UE positioning [13] procedures are used to obtain the UE/PRU location estimate by an LMF.

### 2.3.3	Network-based positioning with gNB-side model (Case 3a)

For Case 3a, the AI/ML model is deployed at the NG-RAN node (gNB/TRP). The NG-RAN node reports AI/ML enhanced "SRS-based measurements", which may be AI/ML enhanced legacy measurements (e.g., RTOA, gNB Rx-Tx Time Difference, etc.) or new measurements.

For training data generation, the measurements and related attributes (e.g., timestamp) are generated by the TRP/NG-RAN node. Therefore, the training data are directly available at the NG-RAN node (gNB/TRP) and RAN3 agreed at RAN3#125bis that the model training is performed by the gNB:

Consider aggregated gNB case first, then split architecture. The existing LCS framework is re-used for AI/ML positioning. Case 3a agreements: - The AI/ML model (inference and training functions) is at the gNB. - LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN. FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design. - How the LMF obtains the ground truth label and related data information is outside of RAN3 scope. - NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1). The details of the measurements are still pending and subject to further discussion in RAN1. - WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

An example is illustrated in Figure 2.3.3-1.

Fig. 2.3.3-1: Data collection by NG-RAN to train the AI/ML based positioning model using NG-RAN measurements (Case 3a)

The desired SRS for model training can be configured in the UE/PRU directly by the gNB. The gNB then measures the UE transmission and determines any desired data for model training at Step 2 in Figure 2.3.3-1. The label for the training data can be provided by an LMF upon request at Step 3. Per RAN3 agreement shown above, how an LMF obtains the ground-truth label is out of RAN3 scope:

- How the LMF obtains the ground truth label and related data information is outside of RAN3 scope.

How an LMF obtains the ground truth label would be up to implementation/deployment. The LMF could use any of the existing LCS [11] and UE positioning [13] procedures to obtain the ground-truth label at Step 4, which the LMF provides at Step 5 to the NG-RAN. The user consent for data collection can be checked by the LMF like in Figure 2.3.1-1 or 2.3.2-1. In addition, since existing positioning procedures are used at Step 4, existing privacy procedures for UE-positioning should also apply.

Similar to the discussion for Case 3b in section 2.3.2 above, the details of the data collection for Case 3a are up to RAN3. However, Step 4 in Fig. 2.3.3-1 involves the UE/PRU and may be in RAN2's scope. Therefore:

Proposal 19: For Positioning Enhancements Case 3a ground-truth label generation (for training and label-based monitoring), the entity which obtains the UE/PRU location is an LMF and existing LCS [11] and UE positioning [13] procedures are used to obtain the UE/PRU location estimate by an LMF.

# Conclusion

Type of Data Required for NW-side model and UE-side models

Proposal 1: The training data collection configuration consists of

- Measurement targets (i.e., CSI resources or resource sets for measurements), and

- Logging configuration (e.g., periodicity, events for logging measurements).

Proposal 2: The CSI framework is used for configuring measurement targets (i.e., CSI resource or resource sets) for training data collections.

Proposal 3: The network should continue to ensure that the maximum number of SSB/CSI-RS configured for measurements lies within the maximum number of SSB/CSI-RS measurements supported at the UE per slot and per FR. That includes SSB/CSI-RS used for

- Different non-AI/ML-related tasks (i.e., legacy CSI reporting for different purposes), and

- Different AI/ML-related tasks (e.g., training data collection configuration, inference, and monitoring), and

- Different AI/ML features (e.g., beam management, CSI prediction, CSI Feedback).

Proposal 4: RAN2 is requested to consider at least the following events for training data collection for network-side model training,

- Based on the number of samples to be collected and reported across different beams, UE triggers the measurement collection and logging if a beam becomes the top-1 beam and the logged number of measurements is less than the configured value.

- Based on the change of the top-1 beam: UE is configured to log the measurement when the top-1 beam changes. UE can additionally be configured with the number of samples to be logged and its periodicity.

Observation 1: Periodic data collection has the following issues,

- Requires a huge amount of training data collection at the UEs, as there may exist significantly high redundancy,

- The network-side training server needs to handle redundancy in the training data, otherwise, it may cause biases in the trained models.

Observation 2: UE can be configured with up to 112 CSI resource configurations in the legacy CSI framework.

Proposal 5: The UE can be configured with multiple CSI resources or resource sets for training data collection.

Observation 3: When the Resource Configuration(CSI-RS Configuration = NZP CSI RS) is periodic, the Report Configuration can be set to any type (periodic, semi-persistent, or aperiodic).

Observation 4: When the Resource Configuration(CSI-RS Configuration = NZP CSI RS) is semi-periodic, the Report Configuration can be Semi-persistent or Aperiodic.

Observation 5: When the Resource Configuration (CSI-RS Configuration = NZP CSI RS) is aperiodic, the Report Configuration can only be aperiodic.

Proposal 6: Dynamic activation/deactivation of training data collection and logging should be used to randomize the training data collection and logging, following semi-persistent and aperiodic CSI reporting as the reference.

Observation 6: For the beam management use case, RAN2 agreed that logging of L1-RSRP measurement for gNB-centric / OAM-centric data collection for NW-side model training is an optional feature for the UE.

Observation 7: In the following scenarios, the configuring gNB may not be able to retrieve the logged training data

- The configuring gNB was overloaded and could not retrieve the training data before handover, or

- UE has RLF before the training data can be retrieved by configuring gNB1.

Proposal 7: Discarding the unretrieved training data upon handover / RLF / RRC state transition is not desired, as it is a waste of UE power and memory spent towards NW-side data collection.

Proposal 8: RAN2 should clarify if there are concerns associated with the leakage of the training data when UE moves from

- One PLMN to another PLMN, i.e., whether training data collected in one PLMN can reported to any PLMN, and

- One infra-vendor to another infra-vendor, i.e., whether training data collected at the NG-RAN from one infra-vendor can reported to NG-RAN from another infra-vendor.

Proposal 9: The UE is provided with an area configuration (e.g., list of cells/gNBs/PLMNs) for the reporting of the unretrieved logged training data.

Proposal 10: UE is not configured with area configuration for the measurement of resources or resource sets for training data collection, as these configurations are cell specific.

Observation 8: Signaling-based immediate MDT is used for collecting data from specific UEs, and the UE selection for immediate MDT is performed at the OAM.

Observation 9: For the network-side model training, there is no need to collect training from specific UEs.

Proposal 11: Signaling-based immediate MDT is not supported for NW-side training data collection.

Observation 10: For management-based MDT, the data collection is performed at cells/gNBs if they belong to the Management Based MDT PLMN List IE and support MDT data collection.

Proposal 12: For OAM-centric data collection (i.e., for management-based MDT), the OAM may indicate cell(s)/frequency(ies) to the NG-RANs on which the training data collection should be performed.

Observation 11: UEs allocate sufficient resources and power for NW-side training data collection, thus override protection of unretrieved training data should be supported.

Proposal 13: RAN2 should further discusses whether/how override protection of unretrieved training data collection should be supported.

Data collection for two-sided model training for CSI feedback use case

Observation 12: Based on RAN1 agreements [2][3], the dataset, parameter, and reference model exchange can be supported using

- Over-the-Air Signaling,

- Other standard-based solutions for offline delivery/transfer.

Observation 13: Alleviating/resolving issues related to inter-vendor training collaboration is one of the major Rel-19 study objectives for two-sided CSI compression.

Observation 14: The UE part of the two-side model is trained at a UE side server not at the UE, i.e., training is not performed by the UE rather it is performed by the UE vendors.

Observation 15: Over-the-air exchange/transfer of the training dataset, parameter, or reference model for supporting UE-side model training causes the following issues

- Significant duplication of data, as the same training dataset, parameter, or reference model may need to be sent to all UEs CONNECTED to a cell/gNB,

- Double the Uu overhead: (i) for delivering/transferring the dataset, parameter, or reference model from the RAN to the UE, and (ii) for delivering/transferring the dataset, parameter, or reference model from the UE to the UE side server.

- Training may not be frequently performed at both the UE side or the network side for two-sided, but the exchange of the training dataset, parameter, or reference model may need to be supported continuously. There is no method by which gNB can ensure that all UE vendors have received the required training dataset, parameter, or reference model, and it can stop transferring/delivering the training dataset, parameter, or reference model.

- Even if the network-part model of the two-side model is well generalized across multiple cells/gNBs, it needs to transfer/deliver the same training dataset, parameter, or reference model from multiple cells/gNBs.

Proposal 14: Over-the-air delivery/transfer of the training dataset, parameter, or reference model to support UE-side model training is not a suitable or scalable solution. Please refer to the issues mentioned in observation 15.

Observation 16: Within the SA WGs, we have precedence where different types of data are shared with application function (within/outside the 3GPP network) and or edge cloud, for example,

- Target UE location to AF [TS 23.273],

- Analytics exposure from NWDAF to AF, e.g., network status reporting [TS 23.288],

- Traffic volume, UL/DL data rates, PDU session status, and others [TS 23.501].

Proposal 15: RAN2 concludes that a standard-based solutions (other than delivering/transferring over the Uu) for delivering/transferring the training dataset, parameter, or reference model from the network entity (e.g., RAN/OAM/NF) to the UE-side training server (within or outside the 3GPP network) area is feasible and the 3GPP can develop such a solution in the Rel-20 normative work.

Data collection and reporting for positioning enhancements use case:

Proposal 16: For LMF-side data collection in Positioning Enhancements Case 2b, existing LCS and UE positioning procedures (e.g., NI-LR, MT-LR and/or deferred MT-LR) are used to obtain the desired measurements from a UE/PRU.

Proposal 17: For Positioning Enhancements Case 2b ground-truth label generation (for training and label-based monitoring), the entity which obtains the UE/PRU location is an LMF and existing LCS [11] and UE positioning [13] procedures are used to obtain the UE/PRU location estimate by an LMF.

Proposal 18: For Positioning Enhancements Case 3b ground-truth label generation (for training and label-based monitoring), the entity which obtains the UE/PRU location is an LMF and existing LCS [11] and UE positioning [13] procedures are used to obtain the UE/PRU location estimate by an LMF.

Proposal 19: For Positioning Enhancements Case 3a ground-truth label generation (for training and label-based monitoring), the entity which obtains the UE/PRU location is an LMF and existing LCS [11] and UE positioning [13] procedures are used to obtain the UE/PRU location estimate by an LMF.

# References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#94-e, Dec. 2021

- TR 38.843 v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18),” Dec. 2023

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #102, Dec. 2023

- Meeting Report, “RAN2#126 meeting report”

- Meeting Report, “RAN2#127 meeting report”

- Meeting Report, “RAN2#127bis meeting report”

- TS 38.331, RRC Specification

- Meeting Report, “RAN1#116 meeting report”

- Meeting Report, “RAN1#116bis meeting report”

- TS 38.214

- 3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

- S2-2411256, CR to 23.273 "Update on Data Collection by LMF procedure", SA2#165.

- 3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

- S2-2411277, CR to 23.273 "KI#1 - Addressing Editor´s Note: Procedure for Data Collection for AI/ML based positioning from NG-RAN", SA2#165.

- Meeting Report, “RAN1#117 meeting report”

- Meeting Report, “RAN1#118 meeting report”

- Meeting Report, “RAN1#118bis meeting report”