3GPP TSG-RAN WG2 Meeting #128			         R2-2409927

Orlando, USA, Nov 18th – 22th, 2024

Agenda Item:	8.5.3

Source: 	LG Electronics Inc.

Title:         	On-demand transmission of SIB1

Document for: 	Discussion and Decision

Introduction

In this contribution, we will discuss all remaining issues to complete the study on on-demand SIB1, such as need of WUS configuration acquisition from NES cell, need of SIB1 validity, and whether/how to support case 3 defined in RAN1.

Discussion

- SIB1 request condition in Case 2

To maximize the NES gain through the on-demand SIB1 transmission, RAN2 agreed that the UE can request SIB1 only for NES cell that meets the cell reselection criteria. Based on this baseline agreed during SI phase, detailed triggering conditions for WUS transmission need to be discussed and decided for normative work.

RAN2#127

6. If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

The cell reselection criteria consist of several conditions and rules, and depend on the frequency priority of the neighbor cell. First, the cell reselection criteria for a cell of a higher priority are as follows:

For a cell of a higher priority, the cell reselection criteria are: - If more than 1 second has elapsed since the UE camped on the current serving cell, and - If threshServingLowQ is broadcast in system information, and if a cell of a higher priority frequency fulfils Squal > ThreshX, HighQ during a time interval TreselectionRAT, or - If threshServingLowQ is not broadcast in system information, and if a cell of a higher priority frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT,

The cell reselection criteria include the 1-second rule to prevent UE from performing frequent cell reselection. The cell reselection criteria are reused as they are for the condition of the SIB1 request, UE can request SIB1 of NES Cell starting from 1 second after it has camped on the current cell. However, if so, the cell reselection to the NES Cell will be delayed by the sum of the time taken for MSG1 transmission, MSG2 reception, and the time for SIB1 acquisition, compared to the cell reselection to legacy cell. Even if SIB1 of an NES Cell is acquired within 1 second of the last cell reselection, the UE will not perform cell reselection to the NES Cell until 1 second has passed since the last cell reselection, according to the exiting 1-second rule in the cell reselection criteria. Therefore, UE should be able to transmit UL WUS to the NES Cell, although 1 second has not elapsed since the UE camped on the current serving cell.

Observation 1	If the 1-second rule in the cell reselection criteria is applied to the triggering condition of UL WUS transmission, it will lead to unnecessary delays in the cell reselection to OD-SIB1 cell.

Therefore, we propose to exclude the 1-second rule from the WUS triggering condition and to define it for OD-SIB1 cell of a higher priority as follows:

Proposal 1	The existing 1-second rule in the cell reselection criteria is not applied to the triggering condition of UL WUS transmission.

Proposal 2	For OD-SIB1 cell of a higher priority UE requests SIB1, if following conditions are met:

- If threshServingLowQ is broadcast in system information, and if an OD-SIB1 cell of a higher priority frequency fulfils Squal > ThreshX, HighQ during a time interval TreselectionRAT, or

- If threshServingLowQ is not broadcast in system information, and if an OD-SIB1 cell of a higher priority frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT.

For a cell of a lower priority, the cell reselection criteria are: - If more than 1 second has elapsed since the UE camped on the current serving cell, and - If threshServingLowQ is broadcast in system information, and if The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority NR or E-UTRAN RAT/ frequency fulfils Squal > ThreshX, LowQ during a time interval TreselectionRAT, or - If threshServingLowQ is broadcast in system information, and if The serving cell fulfils Srxlev < ThreshServing, LowP and a cell of a lower priority RAT/ frequency fulfils Srxlev > ThreshX, LowP during a time interval TreselectionRAT.

The cell reselection criteria for a cell of a lower priority also includes the 1-second rule, and the same conclusion on it can be applied to the triggering condition of WUS transmission for OD-SIB1 cell of a lower priority.

Proposal 3	For OD-SIB1 cell of a lower priority UE request SIB1, if following conditions are met:

- If threshServingLowQ is broadcast in system information, and if the serving cell fulfils Squal < ThreshServing, LowQ and an OD-SIB1 cell of a lower priority frequency fulfils Squal > ThreshX, LowQ during a time interval TreselectionRAT.

- If threshServingLowQ is not broadcast in system information, and if the serving cell fulfils Srxlev < ThreshServing, LowP and an OD-SIB1 cell of a lower priority frequency fulfils Srxlev > ThreshX, LowP during a time interval TreselectionRAT.

In addition to the inter-frequency cell reselection criteria, there are two rules for inter-frequency cell reselection:

- Cell reselection to a higher priority RAT/frequency shall take precedence over a lower priority RAT/frequency if multiple cells of different priorities fulfil the cell reselection criteria. - If more than one cell meets the above criteria, the UE shall reselect a cell as follows: - If the highest-priority frequency is an NR frequency, the highest ranked cell among the cells on the highest priority frequency(ies) meeting the criteria according to clause 5.2.4.6

Above two rules also need to be applied to the triggering condition of WUS transmission to minimize the awakening of the OD-SIB1 cell.

Proposal 4	SIB1 request to a higher priority frequency shall take precedence over a lower priority frequency if multiple OD-SIB1 cells of different priorities fulfil the SIB1 request criteria.

Proposal 5	If more than one OD-SIB1 cell meets the triggering condition of WUS transmission, the UE requests SIB1 of the highest ranked OD-SIB1 cell.

The cell reselection to a cell on an intra-frequency or equal priority inter-frequency is based on cell-ranking as follows:

If rangeToBestCell is not configured, and if the highest ranked cell is a NES Cell, the UE requests SIB1 of the NES cell. If rangeToBestCell is configured, and if the cell with the highest number of beams above the threshold (i.e. absThreshSS-BlocksConsolidation) among the cells whose R value is within rangeToBestCell of the R value of the highest ranked cell is the NES Cell, the UE request SIB1 of the NES cell. If there are multiple such NES cells, the UE requests SIB1 of the highest ranked NES cell among them.

The SIB1 request from an NES Cell on an equal priority NR frequency also shall be based on ranking as follows:

Proposal 6	For intra-frequency or equal priority inter-frequency OD-SIB1 cell the UE requests SIB1, if following condition is met:

- If rangeToBestCell is not configured, and if the highest ranked cell is an OD-SIB1 cell, the UE requests SIB1 of the OD-SIB1 cell.

- If rangeToBestCell is configured, and if the cell with the highest number of beams above the threshold among the cells whose R value is within rangeToBestCell of the R value of the highest ranked cell is a OD-SIB1 cell, the UE requests SIB1 of the OD-SIB1 cell. If there are multiple such OD-SIB1 cells, the UE requests SIB1 of the highest ranked OD-SIB1 cell among them.

- Broadcast status of OD-SIB1

When a neighbor NES Cell satisfies the SIB1 request condition, if SIB1 of a neighbor NES Cell is already being broadcast and the UE is aware of this, it can save its power by acquiring the SIB1 from the NES Cell without transmitting WUS. For the same reason, network informs UEs of si-BroadcastStatus for each SI message in the existing SI request procedure. In the last meeting, RAN2 agreed to adopt similar approach to on-demand SIB1.

Agreement in RAN2#127bis

8.	If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

For OD-SIB1 of neighbor NES Cell, the si-BroadcastStatus also could be provided by Cell A along with the WUS configuration. When an UE capable of OD-SIB1 camp on a cell supporting OD-SIB1 transmission, the UE would considers the SIB X including the WUS configuration as a required SI and request its transmission, if it is not being broadcast. That is, the UE would typically acquire the WUS configuration in advance, before the condition for WUS transmission is met. However, the broadcast status of the OD-SIB1 may have changed after the UE acquired the WUS configuration. Therefore, the UE should request first the transmission of SIB including the WUS configuration to acquire up-to-date si-BroadcastStatus of SIB1.

Figure 1

In the exiting OD-SI procedure, the checking si-BroadcastStatus is free, because the si-BroadcastStatus is included in SIB1 and it is always on. However, even SIB1 is no longer always-on, and UE needs to perform random access to acquire SIB1 or SIB X (including WUS configuration). Considering the purpose of checking si-BroadcastStatus is to avoid unnecessary RA procedure, performing RA procedure to acquire up-to-date si-BroadcastStatus should be prohibited.

Although both SIB1 and SIB X can be transmitted in on-demand manner, the probability of always broadcasting SIB1 is higher than that of always broadcasting SIB X. If SIB1 including the si-BroadcastStatus of the neighbor NES Cell is always broadcast, the UE can avoid unnecessary SIB1 request from NES Cell based on the si-BroadcastStatus without performing additional RA procedure. Therefore, we propose followings:

Proposal 7	si-BroadcastStatus of each neighbor NES Cell is indicated in SIB1 of Cell A.

Proposal 8	For OD-SIB1 acquisition from neighbor NES Cell, if SIB1 of Cell A is being broadcast, the UE decide whether to transmit WUS to the NES Cell based on the si-BroadcastStatus in SIB1 of Cell A, which is associated with the NES Cell.

Proposal 9	For OD-SIB1 acquisition from neighbor NES Cell, if SIB1 of Cell A is not being broadcast, the UE is not required to check the si-BroadcastStatus in SIB1 of Cell A, which is associated with the NES Cell.

Since the broadcast status of OD-SIB1 can be dynamically changed, if the change of si-BroadcastStatus associated with OD-SIB1 leads to the SI change indication, it will cause frequent false alarm to UEs in RRC_IDLE/INACTIVE. As the legacy si-BroadcastStatus for SI messages does not lead to the SI change indication, the change of the broadcast status of OD-SIB1 also should not result in the SI change indication.

Proposal 10	Change of si-BroadcastStatus of OD-SIB1 does not result in system information change indication.

If UE is required to check whether serving cell’s OD-SIB1 is being broadcast based on the si-BroadcastStatus, the only option is to include that information in the MIB. However, there are hardly any bits left available in the MIB, this seems like overkill. Although such information is not indicated explicitly, UE is able to check whether OD-SIB1 is being broadcast by monitoring it blindly. Therefore, we propose not to introduce si-BroadcastStatus for OD- SIB1 of serving NES cell, and rely on the blind detection of SIB1 transmission.

Proposal 11	Do not support si-BroadcastStatus for OD-SIB1 of serving NES Cell.

- RO selection for WUS transmission

If the SIB1 request condition is met, the UE transmits MSG1 to the neighbor OD-SIB1 cell to request SIB1 transmission. According to the RAN2 agreements, the UE monitors MSG2 from the neighbor OD-SIB1 cell after transmitting MSG1. Until now, there has been no cases where UEs in RRC_IDLE/INACTIVE perform the random access procedure on the neighbor cell. UE in RRC_IDLE/INACTIVE should monitor periodically the serving cell for paging reception. If UE transmits MSG1 to a neighbor OD-SIB1 cell or monitors RAR window during the paging occasion, the UE could miss paging. Receiving paging is much more important than acquiring SIB1 of neighbor cell. The MSG1 transmission and MSG2 reception for SIB1 request should not interfere with the paging reception from serving cell.

Observation 2	The paging reception from the serving cell should be prioritized over the WUS transmission and the response reception on the neighbor OD-SIB1 cell.

Therefore, UE should select a PRACH occasion which does not overlap with the paging occasion, and the RAR window associated with the selected PRACH occasion also should not overlap with the paging occasion.

Proposal 12	When determining the PRACH occasions of the neighbor NES cell for WUS transmission, the UE takes into account the possible overlapping between its paging occasion and PRACH occasion/RAR window.

- SIB1 reception failure

If UE is unable to acquire SIB1 from a cell, the cell is considered as barred. The conditions under which the UE determines that the SIB1 reception fails are not specified. As it is up to UE implementation, there is no limit on the number of attempts or time restrictions for SIB1 reception in the existing cell re-selection procedure. That is, if UE fails to receive the first expected SIB1 transmission, it may either retry to acquire SIB1 or immediately consider the cell as barred. Although the SIB1 is transmitted in on-demand manner, there is no reason to specify detailed conditions for SIB1 acquisition failure.

The Random access is only a part of the process for receiving OD-SIB1. The reception of OD-SIB1 consists of three steps as follows:

- Step 1: WUS configuration acquisition

- Step 2: SIB1 request via Random Access

- Step 3: Monitoring PDCCH during SIB1-window

SIB1 reception failure can occur at any of the steps. The UE may fail to acquire the WUS configuration, or it may fail the Random Access even after acquiring the WUS configuration. Additionally, the UE may still fail to acquire SIB1 even after the successful Random Access, e.g. within the specified time window. If we really need to specify detailed conditions for OD-SIB1 reception failure, we should specify separate failure declaration conditions for SIB1 reception failures that may occur at each step, not just those occurring at a specific step. We think this is overkill and prefer to leave the condition for SIB1 acquisition failure up to UE implementation as in legacy cell re-selection procedure.

Proposal 13	It is up to UE implementation when to consider the OD-SIB1 reception fails, as in the legacy SIB1 acquisition for cell re-selection.

- OD-SIB1 of serving NES Cell

RAN2 discussed whether the WUS configuration needs to be transmitted from the NES cell and concluded that it needs to be transmitted for UEs camping on the NES cell.

RAN2#127

Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

However, RAN2 has not discussed the condition for SIB1 request of the serving NES Cell, i.e. when UE needs to request SIB1 after camping on the NES Cell. According to TS 38.331, UE is required to re-acquire SIB1 of the serving cell in the following cases:

- Case 1. If UE receives a Short Message with systemInfoModification bit

- Case 2. If UE receives a Short Message with etwsAndCmasIndication bit

- Case 3. When UE wants to acquire a SIB for which it does not have a valid version

Regardless of the RRC state that the UE is in, if etwsAndCmasIndication bit or systemInfoModification bit is set in the received Short Message, the UE shall re-acquire SIB1 immediately or from the start of the next modification period. Since RAN2 agreed on-demand SIB1 does not affect UEs in RRC_CONNECTED, OD-SIB1 Cell should start SIB1 transmission upon transmission of Short Message with systemInfoModification or etwsAndCmasIndication. If not, UEs in RRC_CONNECTED could fail to acquire SIB1.

RAN2#127

RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).

Observation 3	If the OD-SIB1 cell transmits SIB1 based on the UE’s request after sending a Short Message with systemInfoModification or etwsAndCmasIndication, UEs in RRC_CONNECTED might not be able to receive SIB1.

Therefore, after transmitting the Short Message with systemInfoModification or etwsAndCmasIndication, the OD-SIB1 cell should start transmitting SIB1, even if it has not received UL WUS from UEs. That is, UEs in RRC_IDLE/INACTIVE don’t need to transmit UL WUS when the SIB1 acquisition is triggered by reception of Short Message with systemInfoModification or etwsAndCmasIndication.

Proposal 14	UE in RRC_IDLE/INACTIVE is not required to transmit WUS when the SIB1 acquisition is triggered by reception of Short Message with systemInfoModification or etwsAndCmasIndication. (The OD-SIB1 Cell should start SIB1 transmission upon transmission of Short Message with systemInfoModification or etwsAndCmasIndication, even if it has not received UL WUS from UEs.)

When the UE wants to acquire a SIB for which it does not have a valid version, if it is in RRC_IDLE/INACTIVE or if it is in RRC_CONNECTED with an active BWP configured with common search space with the field searchSpaceOtherSystemInformation, it determines whether to perform SI request or acquire the SI message containing the required SIB without SI request, based on the si-BroadcastStatus associated with the required SIB. Since the change of si-BroadcastStatus does not result in system information change notifications in Short Message, even though the UE already have a valid version of SIB1, it should re-acquire SIB1 to check the up-to-date si-BroadcastStatus of the required SIB. However, unlike case 1 and 2, the network does not know when the UE will want to receive an SIB that it does not have, so it is impossible for UEs in RRC_CONNECTED to acquire SIB1 from serving OD-SIB1 cell in case 3.

Observation 4	UEs in RRC_CONNECTED could not check up-to-date si-BroadcastStatus by re-acquiring SIB1 from OD-SIB1 cell, when it wants to acquire a SIB for which it does not have a valid version.

When UE in RRC_IDLE/INACTIVE wants to acquire a SIB for which it does not have a valid version, it can transmit the WUS to request SIB1 for the serving cell. However, considering that the reason for introducing the si-BroadcastStatus was to prevent UE performing unnecessary random access procedure, it is undesirable to make UE perform random access to acquire the si-BroadcastStatus. If required SIB is not being broadcast, the UE camping on the OD-SIB1 cell will perform random access twice to acquire it, i.e. one for SIB1 request and another for SI request.

Observation 5	If required SIB is not being broadcast, UE camping on an OD-SIB1 cell will perform random access twice to acquire it, i.e. one for SIB1 request and another for SI request.

Therefore, if a UE is connected to or camps on the OD-SIB1 cell, the UE should be allowed to perform SI request without checking the si-BroadcastStatus of the required SIB, i.e. without re-acquiring SIB1.

Proposal 15	UE which camps on or is connected to the OD-SIB1 cell is allowed to perform SI request without checking the si-BroadcastStatus of the required SIB, i.e. without re-acquiring SIB1.

If the proposal 14 and 15 are agreed, for UEs camping on the OD-SIB1 cell there is no case where the WUS needs to be transmitted to the serving OD-SIB1 cell. Then, we don’t need to consider the SIB1 request for serving OD-SIB1 cell.

Proposal 16	Do not consider the scenario where UE requests OD-SIB1 from serving NES cell.

Conclusion

Regarding WUS triggering condition:

Proposal 1	The existing 1-second rule in the cell reselection criteria is not applied to the triggering condition of UL WUS transmission.

Proposal 2	For OD-SIB1 cell of a higher priority UE requests SIB1, if following conditions are met:

- If threshServingLowQ is broadcast in system information, and if an OD-SIB1 cell of a higher priority frequency fulfils Squal > ThreshX, HighQ during a time interval TreselectionRAT, or

- If threshServingLowQ is not broadcast in system information, and if an OD-SIB1 cell of a higher priority frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT.

Proposal 3	For OD-SIB1 cell of a lower priority UE request SIB1, if following conditions are met:

- If threshServingLowQ is broadcast in system information, and if the serving cell fulfils Squal < ThreshServing, LowQ and an OD-SIB1 cell of a lower priority frequency fulfils Squal > ThreshX, LowQ during a time interval TreselectionRAT.

- If threshServingLowQ is not broadcast in system information, and if the serving cell fulfils Srxlev < ThreshServing, LowP and an OD-SIB1 cell of a lower priority frequency fulfils Srxlev > ThreshX, LowP during a time interval TreselectionRAT.

Proposal 4	SIB1 request to a higher priority frequency shall take precedence over a lower priority frequency if multiple OD-SIB1 cells of different priorities fulfil the cell reselection criteria.

Proposal 5	If more than one OD-SIB1 cell meets the triggering condition of WUS transmission, the UE requests SIB1 of the highest ranked OD-SIB1 cell.

Proposal 6	For intra-frequency or equal priority inter-frequency OD-SIB1 cell the UE requests SIB1, if following condition is met:

- If rangeToBestCell is not configured, and if the highest ranked cell is an OD-SIB1 cell, the UE requests SIB1 of the OD-SIB1 cell.

- If rangeToBestCell is configured, and if the cell with the highest number of beams above the threshold among the cells whose R value is within rangeToBestCell of the R value of the highest ranked cell is a OD-SIB1 cell, the UE requests SIB1 of the OD-SIB1 cell. If there are multiple such OD-SIB1 cells, the UE requests SIB1 of the highest ranked OD-SIB1 cell among them.

Regarding Broadcast status of OD-SIB1:

Proposal 7	si-BroadcastStatus of each neighbor NES Cell is indicated in SIB1 of Cell A.

Proposal 8	For OD-SIB1 acquisition from neighbor NES Cell, if SIB1 of Cell A is being broadcast, the UE decide whether to transmit WUS to the NES Cell based on the si-BroadcastStatus in SIB1 of Cell A, which is associated with the NES Cell.

Proposal 9	For OD-SIB1 acquisition from neighbor NES Cell, if SIB1 of Cell A is not being broadcast, the UE is not required to check the si-BroadcastStatus in SIB1 of Cell A, which is associated with the NES Cell.

Proposal 10	Change of si-BroadcastStatus of OD-SIB1 does not result in system information change indication.

Proposal 11	Do not support si-BroadcastStatus for OD-SIB1 of serving NES Cell.

RO selection for WUS transmission:

Proposal 12	When determining the PRACH occasions of the neighbor NES cell for WUS transmission, the UE takes into account the possible overlapping between its paging occasion and PRACH occasion/RAR window.

SIB1 reception failure:

Proposal 13	It is up to UE implementation when to consider the OD-SIB1 reception fails, as in the legacy SIB1 acquisition for cell re-selection.

OD-SIB1 of serving NES Cell

Proposal 14	UE in RRC_IDLE/INACTIVE is not required to transmit WUS when the SIB1 acquisition is triggered by reception of Short Message with systemInfoModification or etwsAndCmasIndication. (The OD-SIB1 Cell should start SIB1 transmission upon transmission of Short Message with systemInfoModification or etwsAndCmasIndication, even if it has not received UL WUS from UEs.)

Proposal 15	UE which camps on or is connected to the OD-SIB1 cell is allowed to perform SI request without checking the si-BroadcastStatus of the required SIB, i.e. without re-acquiring SIB1.

Proposal 16	Do not consider the scenario where UE requests OD-SIB1 from serving NES cell.