3GPP TSG-RAN WG2 Meeting #128	R2-2409957

Orlando, USA, 18 – 22 November 2024

Agenda item:	8.7.4.2

Source:	Apple

Title:	Views on DSR Enhancements

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion / Decision

# Introduction

Based on the study in Rel-19 XR Phase 3, the following objective has been added to the work item [1]:

-	Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]:

-	Specify additional Logical Channel priority handling using delay/deadline information of packets;

-	Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

For DSR enhancements, we have made the following agreements in previous meetings [2,3]:

RAN2 #127 Agreements: Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG. For enhanced DSR: - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR RAN2 #127bis Agreements: We do not change the definition of delay-critical data For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s) Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc. For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - • Buffer size of data volume in each portion - • Shortest remaining time among PDCP SDUs buffered in each portion. A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

This paper presents some of our views on DSR enhancement.

# Discussions

## Issues with Triggering Threshold and Reporting Thresholds

In previous meetings, we have debated about whether the DSR triggering threshold can be configurable, or if it should be simply set as the largest reporting threshold. A few companies have pointed out that, if non-delay-critical data is also included in the DSR, there may be some issues such as ambiguity of DSR cancellation.

To resolve this issue, we can first check how the DSR cancellation procedure works in Rel-18:

TS 38.321 V18.2.0: A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-critical PDCP SDU (as defined in TS 38.323 [4]) associated with the logical channel which triggered the DSR. A MAC PDU shall contain at most one DSR MAC CE. A MAC PDU shall not contain a DSR MAC CE if it includes all PDCP SDUs associated with all the pending DSRs. After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR.

Based on the excerpt of TS 38.321 above, we have noted two things:

- A PDCP SDU is said to be “associated with a DSR” only if it is a delay-critical PDCP SDU (i.e. only if its remaining time is smaller than the DSR triggering threshold)

- Cancellation of DSR is solely depending on if any “PDCP SDU associated with the DSR” (which by definition has to be delay-critical) is still in the buffer.

Thus, regardless of which reporting threshold is used as the triggering threshold, the procedure for DSR cancellation does not change. Even if the triggering threshold is below the largest reporting threshold, the DSR is cancelled when all PDCP SDUs with remaining time smaller than the triggering threshold are gone. Whether a non-delay-critical PDCP SDU is to be included in buffer size calculation for DSR or not, does not really matter.

Observation 1: Whether a DSR should be triggered or cancelled depends on if there is any buffered PDCP SDU with remaining time smaller than the DSR triggering threshold. It does not depend on the remaining time of PDCP SDU that is to be included for DSR buffer size calculation.

With this in mind, we do not see how DSR cancellation procedure will be impacted, if Rel-19 DSR allows the UE to report volume of data with remaining time that is larger than the DSR triggering threshold. Based on such observation, we believe the network can flexibly configure one of the reporting thresholds as the triggering threshold, and there is no need to impose any constraint about the threshold configuration.

Proposal 1: The network can select any of the multiple reporting thresholds configured for an LCG as the DSR triggering threshold.

## Non-Delay-Critical Data Before Delay-Critical Data

In the previous meetings, the issue relating to non-delay-critical data queued before delay-critical data has been raised. The concern is, if the gNB allocates UL-SCH resource based on DSR, the UE may first allocate non-delay-critical data that is in the front of the queue, before allocating delay-critical data that is targeted by the UL-SCH resource. As a result, the delay-critical data still cannot be transmitted on time. In light of such potential issue, there are some proposals about including non-delay-critical data ahead of delay-critical data in buffer size calculation, to ensure that the gNB can allocate sufficient resource.

In general, we think this is a corner case, as typically the queue is based on a first-in first-out model, so the packets that are queued in front are naturally more urgent. Furthermore, in most scenarios we believe we can rely on UE implementation to resolve this issue (e.g. sorting the packets in the queue based on their level of urgency). Hence, we do not think such corner case can justify the change in the data volume calculation procedures.

Proposal 2: There is no need to include non-delay critical data ahead of delay-critical data in the buffer size calculation for DSR.

## DSR MAC CE Format Selection

Another issue that has been discussed during RAN2 #127bis is, whether the UE should fallback and select Rel-18 DSR MAC CE format, when all the pending DSRs are triggered by LCHs of LCGs configured with only one remaining time threshold. In our views, if the resultant signaling overheads for both Rel-18 and Rel-19 DSR MAC CEs are the same, there is no need to mandate the UE behaviors in terms of DSR MAC CE format selection. Depending on the UE implementation, there may be different levels of complexity between the procedures to generate these MAC CE formats. Hence, we think the UE should have such flexibility to choose the MAC CE format, so the specification should allow the UE to select either Rel-18 or Rel-19 DSR format in such cases.

Nonetheless, if companies prefer to mandate a DSR MAC CE format for such cases, we think Rel-18 DSR MAC CE should be the baseline that the UE should select. Potentially, the computational resources required by the UE to derive Rel-18 DSR MAC CE format is lower than its counterpart for Rel-19.

Proposal 3: If all pending DSRs are triggered by the LCGs configured with only one remaining time threshold, the specification should allow the UE to select either Rel-18 DSR MAC CE or Rel-19 DSR MAC CE if they have the same signaling overheads. There is no need to mandate the UE behaviors for DSR MAC CE format selection in such cases.

## PDU Set Considerations for Data Volume Calculations

In Rel-18 DSR, if pdu-SetDiscard is configured, any PDCP SDU that belongs to the same PDU Set should also be considered as delay-critical. Furthermore, it should also be included in the delay-critical data volume for reporting, even though the remaining time for some of these PDCP SDUs may still larger than the threshold. This is described in the definition of Delay-Critical PDCP SDUs in TS 38.323:

Delay-critical PDCP SDU: if pdu-SetDiscard is not configured, a PDCP SDU for which the remaining time till discardTimer expiry is less than the remainingTimeThreshold. If pdu-SetDiscard is configured, a PDCP SDU belonging to a PDU Set of which at least one PDCP SDU has the remaining time till discardTimer expiry less than the remainingTimeThreshold.

Since the buffer of a LCG is divided into multiple portions based on the multiple remaining time thresholds in Rel-19, RAN2 needs to discuss how the integrated handling of PDU Sets can be extended in such cases. When calculating the buffer size of portions corresponding to different remaining time ranges, we think all PDCP SDUs of a PDU Set should be included in the buffer size for the portion corresponding to the smallest remaining time among the PDCP SDUs of this PDU Set. Therefore, when the most urgent PDCP SDU of a PDU Set is being included in the buffer size calculation for a specific portion, all other PDCP SDUs of the same PDU Set should also be included in the same portion, if pdu-SetDiscard is configured.

Proposal 4: If pdu-SetDiscard is configured, all PDCP SDUs of a PDU Set should be included in the data volume calculation for the same buffer portion that corresponds to the smallest remaining time among the PDCP SDUs of this PDU Set.

Moreover, if a PDCP SDU has been included in the data volume calculation for a buffer portion (for example, in the pdu-SetDiscard cases elaborated in Proposal 4), it should not be included again in the data volume calculation for another buffer portion, regardless of its remaining time and the remaining time range that these buffer portions correspond to. Apparently, this is needed as we should avoid “double counting” issues of data volume calculation that can result in inaccurate buffer size information of DSR.

Proposal 5: A PDCP SDU that has been included in the data volume calculation for one buffer portion should not be counted again in the data volume calculation for another buffer portion for DSR, regardless of its remaining time.

# Conclusions

In this paper, we have the following observation and proposals for Rel-19 DSR enhancements:

Observation 1: Whether a DSR should be triggered or cancelled depends on if there is any buffered PDCP SDU with remaining time smaller than the DSR triggering threshold. It does not depend on the remaining time of PDCP SDU that is to be included for DSR buffer size calculation.

Proposal 1: The network can select any of the multiple reporting thresholds configured for an LCG as the DSR triggering threshold.

Proposal 2: There is no need to include non-delay critical data ahead of delay-critical data in the buffer size calculation for DSR.

Proposal 3: If all pending DSRs are triggered by the LCGs configured with only one remaining time threshold, the specification should allow the UE to select either Rel-18 DSR MAC CE or Rel-19 DSR MAC CE if they have the same signaling overheads. There is no need to mandate the UE behaviors for DSR MAC CE format selection in such cases.

Proposal 4: If pdu-SetDiscard is configured, all PDCP SDUs of a PDU Set should be included in the data volume calculation for the same buffer portion that corresponds to the smallest remaining time among the PDCP SDUs of this PDU Set.

Proposal 5: A PDCP SDU that has been included in the data volume calculation for one buffer portion should not be counted again in the data volume calculation for another buffer portion for DSR, regardless of its remaining time.

# References

- RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur), Sept. 2024, Melbourne, Australia.

- 3GPP RAN2 #127 Chair’s Notes, Maastricht, Netherlands, August 2024.

- 3GPP RAN2 #127bis Chair’s Notes, Hefei, China, October 2024.