# 3GPP TSG RAN WG2 Meeting #128      	                           							R2-2409963
Orlando, USA, November 18-22, 2024

Agenda item:	8.2.2

Source:	Apple

Title:	Functional Aspects of Ambient IoT

WID/SID:	FS_Ambient_IoT_solutions – Release 19

Document for:	Discussion and Decision

# 1	Introduction

In prior RAN2 meetings [1][2][[3][4],  the following function aspects on A-IoT have been agreed:

RAN2#125bis agreements: 1 RRC connection management is not supported. FFS how the resource configuration is provided to the device (if needed based on RAN1 progress) 2 RRM L3 measurement reporting is not supported by Ambient IoT devices. 3 RAN2 assumes, AIoT devices are not required to support ASN.1 encoding/decoding. 4 Periodical System information and MIB are not supported by AIoT devices. This doesn’t preclude any RAN1 defined broadcast signals. 5 RAN2 assumes that RRC layer is not necessary between the reader and the device. RAN2 will continue to study the functionalities required and later discuss whether we will have: 1) a new AS protocol on top of A-IoT MAC layer; or 2) A-IoT MAC 6. SDAP is not supported for UP protocol stack. 7. PDCP layer is not needed. FFS how to handle AS security (if needed pending SA3 discussion) and any other really needed functionalities. 8. RLC layer is not needed. FFS how to handle segmentation (if needed and depending on RAN1 design and upper layer packet size). RAN2 considers segmentation and reassembly would add complexity, however further discussions are needed. 9. No HARQ and RLC AM 10 FFS about the level of visibility required by the reader and what information is necessary for AS layer operations. 11 RAN2 assumes that no per-packet QoS and no per-QoS flow is supported at AS level (for both UL/DL). FFS how to handle the general QoS requirements from SA2 RAN2#126 agreements: 1. Multiple “AIoT logical channels” for upper layer data are not supported. FFS if AIoT logical channel concept is used depending on final modeling issue. 2 legacy NR BSR/SR is not needed for A-IoT communication. 3 FFS whether further indication of device message size/status is needed 4 AS-layer (above PHY layer) RLC-like retransmission/repetition is not supported. This doesn’t preclude the reader and device sending the payload again as new transmission from MAC perspective. FFS how we handle segmentation case (if needed) RAN2#127 agreements: - Send an LS to RAN1 regarding TBS size. Ask SA2 on the max and typical upper layer packet size. - RAN2 will study the need and use of energy status report to be delivered from the device to the reader in case the device does not have energy for the follow up messages. FFS details on signaling - RAN2 will study the need and use of a simple message “size” reporting to the reader - At least the following information are considered useful to be visible to the reader from CN - The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful - targeted for one or more than one devices; - approximate number of target devices (if available). FFS on mandatory/optional - RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the AIOT interface as upper layer data. RAN2#127bis agreements: Agreements on AS ID - RAN2 assumes that if “AS ID” is defined it is used at least for purpose of D2R scheduling and R2D reception. Up to RAN1 to decide whether a “AS ID” is defined. - RAN2 assumes this “AS ID” should be a short AS layer ID, rather than the full upper layer device ID. FFS on the length. FFS if AS ID can be based on partial upper layer device ID. - From RAN2 perspective, following options are possible for “AS ID”: - Option 1: a random ID if used in Msg1 can be reused; - Option 2: reader assigns this “AS ID”. FFS via which R2D message. - RAN2 will aim to define one common design for all RA procedures (if technically possible) Agreements on energy bit indication - Capture in the TR the option: the device may include energy status indication in D2R messages (e.g. MSG1,MSG3 and Command Response message), if the device can. 1 bit indication can be captured in the TR. We will capture the use case in the TR. FFS whether it is reader controlled. Agreements on message size - From RAN2 perspective, it is beneficial for the reader to know an estimate of expected D2R message size. Two options can be captured: 1) from the CN and 2) from the device (simple message size indication). Capture the advantages/disadvantages. - The D2R message size would be beneficial but it is not essential. - Ask SA2 if it is possible to provide the expected (e.g. approximate, estimate, exact, max) future response D2R message size. Is it always available, sometimes, or never. Agreements on segmentations 1. RAN2 will study segmentation and RAN2 will need to give some guidance once there is more clarity on solution, on the minimum TB size. 2. Study the functionality of segmentation in the MAC. We focus on D2R and can consider the support for R2D later depending on SA2 and RAN1. The solution should aim to be applicable to both direction (i.e. unified solution), if we decide to support on both directions. 3. Do not support sequence number for A-IoT segmentation functionalities. 4. Do not support segment number and the number of segments for A-IoT segmentation functionalities. 5. Capture the following option in TR: An indication is used to indicate to reader whether the data is segmented and whether it is last segment. FFS the details on how this is designed and number of bits (one or two). 6. RAN2 assumes that for the device we will not support AS layer buffering for A-IoT segmentation functionalities, i.e., all buffered segment(s) are stored in upper layer(s). Agreements on DoA - From RAN2 perspective, at least the A-IoT paging is an aspect/part of the current design which is not sufficient for the DO-A use case. - From RAN2 perspective, DO-A use case cannot be supported with the current design discussed in the study item.

In this contribution we provide our views on the outstanding stage-2 issues and functional aspects of Ambient IoT design. The main focus is to address the FFS and issues that need to be addressed in the SI, with the understanding that many details can be finalized in the WI phase.

# 2   	Discussion

### 2.1 Ambient IoT Protocol Stack

For Ambient IoT user plane support, only PHY layer and MAC layer are needed. As agreed in RAN2#125bis meeting [1] that:

RAN2 assumes that RRC layer is not necessary between the reader and the device. RAN2 will continue to study the functionalities required and later discuss whether we will have: 1) a new AS protocol on top of A-IoT MAC layer; or 2) A-IoT MAC

For Control Plane design, there could be a dedicated A-IoT CP protocol (e.g. for paging and initial access) on top of MAC layer, but it can also be implemented as MAC Control PDUs. Thus, the presence of “AIoT CP Protocol” is not really needed. For the sake of time efficiency in Rel-19 study, we propose to remove this layer and RAN2 can focus on MAC layer design only for both UP and CP altogether.

Proposal 1	No need for a dedicated AS control plane protocol (i.e. control plane functions are part of the MAC protocol).

On top of data transmission protocol in MAC layer, there could be an upper layer designated as “AIoT layer” (e.g., AIoT-NAS for CP, FFS UP) to communicate with Core Network for both UP and CP, which is to be studied by SA2 and CT1, but out of RAN2 scope, as those data contents may be transparent to the gNB reader.

We think RAN can consider the protocol stack in Figure 2 as a baseline. Of course, depending on the inputs from other 3GPP WGs (RAN1, SA2 or SA3), certain changes would still be considered.

Figure 2. Protocol Stack for A-IoT Topology 1

Proposal 2	RAN2 consider use the above protocol stack as baseline for Ambient IoT Topology 1.

### 2.2 Multiplexing in A-IoT MAC PDUs

RAN2 has agreed that “Multiple AIoT logical channels” for upper layer data are not supported.” But it is still regarded as an open modeling issue whether multiplexing is allowed in MAC layer. This is because MAC PDU design may or may not support different types of “upper layer” traffic. Although the upper layer traffic may be diverse with multiple different use cases (inventory, command, etc.) to support,  we think some discussion from MAC layer perspective can be beneficial to clarify this design aspect.

There is some misunderstanding that this multiplexing support is just equivalent to providing different A-IoT MAC message formats or message variants. But even if there are different message formats for DL paging message or DL command message, it is still questionable whether a single MAC transmission (e.g., a TB) will include those multiple message formats or not, and whether a single MAC transmission has to be to the same device destination or can support different device destinations.

For DL traffic, we can have a high-level classification of  all R2D traffic as follows :

- R2D data that should trigger response

  - R2D upper-layer data triggering response with CBRA (e.g, Paging)

  - Msg 2 in 3-step CBRA

  - R2D upper-layer data triggering response w/o CBRA (e.g., CFRA)

- R2D data (no ACK/response needed)

As we can see, there are at most only a few different message types to be potentially supported in A-IoT MAC layer design for DL traffic. Moreover, for each “destination ID” (device ID, group ID, etc), there should be only one valid type of R2D data at one time. For example, there is no such a case that an A-IoT reader sends “ R2D data triggering response” and “R2D data (no ACK/response needed)” simultaneously to the same device, because aggregating those two upper-layer R2D data shall be merged by CN as a single “super-command” in need of device response, and such “aggregation” operation is transparent to the reader and A-IoT air interface, and should be out of RAN2 scope.  Also, even if Msg2 is defined as a MAC CE in A-IoT DL, the reader will not multiplex Msg2 MAC CE to a device together with command message to the same device.

Therefore, we think RAN2 can discuss a variety of message format design options to accommodate a single type of upper layer data and/or a single MAC CE towards a single “device ID/group ID”.  This can be further extended to the logical channel-based multiplexing in the future releases, when different types of upper layer A-IoT data towards the same device may occur. Nonetheless,  the “multiplexing” issue in DL is an extra requirement, which requires that a DL MAC PDU is allowed to contain different R2D data or different MAC CE belongs to different A-IoT device destinations. But since different devices may be located at different distances from the reader and may encounter different path loss, it will be quite challenging to guarantee the delivery of those R2D contents towards different devices in a single TB transmission. Given the assumption that R2D segmentation is not supported and HARQ-retransmission and RLC retransmission are also not supported, not supporting multiplexing in DL is a reasonable choice in Rel-19 work.

Proposal 3	Multiplexing R2D data towards different device IDs in the same  DL A-IoT MAC PDU is not supported.

For UL traffic, we can also have three D2R traffic defined as the following:

- CBRA Msg 1

- CBRA Msg 3

- Other D2R response for DO-DTT (included response transmitted via CFRA)

Note that if CBRA Msg1 contains random ID, then it is supposed to be terminated in the reader and not reach the CN. But for all the other cases, the upper layer D2R data (including device ID) encapsulated in D2R transmissions from a certain A-IoT device will be terminated at CN. Logically, the A-IoT UL MAC PDU can be very simple, it will be always one of the following two cases:

- Msg1 (Random ID)

- Device ID and/or any upper layer “D2R response” data

Logically, the above two cannot be transmitted at the same time by an A-IoT device. The only exception is the Msg1 in 2-step RACH which includes both Random ID, device ID and D2R data, which is agreed in RAN2#127 meeting [3]. But that is supported as a special message format design which is not regarded as aggregating multiple D2R MAC SDUs. Therefore, there is no multiplexing needed in UL MAC PDU. Note that we do not think the “energy status bit” or “more data” bit from UE as part of UL multiplexing problem, as that could be a fixed part in MAC header format.

Proposal 4	Not support Multiplexing in UL A-IoT MAC PDU.

### 2.3 Visibility to AS layer of reader

RAN2 have the following agreement regarding the visibility issue:

At least the following information are considered useful to be visible to the reader from CN

  - The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful

  - targeted for one or more than one devices;

  - approximate number of target devices (if available). FFS on mandatory/optional

First, we believe device id(s) or device group id(s) are always provided to AS layer for each A-IoT procedure. This is obviously needed for “command” procedure, as a command usually targets a specific device or a group of devices. Also, this may also be beneficial for “inventory” procedure, as it is reasonable to assume that in many scenarios the application would need to inventory specific devices and/or groups of devices. The alternative of always performing “inventory” for all the devices in coverage of a reader and then filtering out the devices the application is not interested in is clearly a waste of resources and time.

Based on this,  we think  “targeted for one or more than one devices” is somehow overlaps with “device ID/group ID” information, as obviously whether the inventory/command targets one or more devices can be deduced from the list of device/group id(s) made visible to the reader. Thus, we think RAN2 may need to clarify this understanding that this agreement does not necessarily mean adding a new indication in CN-reader signalling or cross-layer indication for this.

Proposal 5 	RAN2 clarify that “whether it targets one more than one device” can be realized by providing “group ID/device ID” to the AS layer of reader w/o new explicit indication.

Although the agreement above says “at least the following information…” , we don’t think any further discussion of additional information beyond those is needed in the study phase. Naturally, some additional information (e.g. expected D2R data size) can be beneficial in certain cases, but it is just an optimization – no additional information is strictly needed. In fact, some of the already agreed information can also be considered an optimization. For instance, the “approximate number of devices” may or may not be provided by CN. If not provided by CN, the A-IoT reader can allocate resource based on its own estimation of the number of devices and performs the procedure(s).

To help SA2 understand the core necessary information which needs to be provided by CN, we think RAN2 need to point out which information is mandatory, and which is optional. In general, we think A-IoT optimizations should be considered in the next release, and the first release should be dedicated to the required functionality.

Proposal 6	Only the following information are mandatory to be visible to AS layer: 1) The service type of A-IoT (e.g. inventory, command), 2) targeted for one or more than one device.

### 2.4 Energy Status Report and D2R Message Size Indication

On message size indication, in RAN2#127-bis, we’ve agreed send the LS to SA2 asking about the option to indicate message size from CN. Naturally, if this information can be provided from CN, it is preferrable over an indication over the A-IoT air interface. Furthermore, both options have been captured in the TR and therefore there is no need to discuss this issue until we receive a reply from SA2.

On energy status report, we’ve agreed to support a 1-bit indication. However, there were opinions raise about whether this is reader controlled and since there is a corresponding FFS, this issue needs to be resolved.

First, we think that not all the devices will support the said energy status indication. Therefore, in general, the system should be designed in such a way, that everything works with or without the indication. In other words, the indication is just an optimization that some devices may use.

Second, as dynamic device capabilities are hardly feasible for A-IoT device, and flexible MAC headers (assuming the indication will be in MAC header) are too complex to support, we think there should just be a dedicated bit in a fixed header location for energy status indication.

For example, a one-bit flag in D2R MAC header can be used to indicate the device “has energy to continue” when the flag is set to zero. If this flag is set as “1” by the device, then this means the device alerts the reader that the device would have energy issue. If a device does not support “energy status reporting”, then this flag bit will remain as default value “0”, and the reader can assume there is no special handing needed for this device in regards of energy status.

Proposal 7	Support a dedicate 1-bit flag at a fixed position in A-IoT D2R MAC header for energy status indication.

With this, i.e. if proposal 6 is agreed, it should be obvious that from the reader perspective (if the reader doesn’t want to use the energy status report, even if it is provided by the device) there is no difference whether the bit is set or not – the reader can simply ignore it.

Furthermore, if we are to introduce a mechanism to control devices to send or not to send the energy status indication, such control mechanism (besides being unnecessary) is likely to be more complex than the indication itself.

Proposal 8	No need to introduce reader control of whether energy status bit is used or not by a device.

## 2.5 Segmentation

On segmentation, RAN2#127bis have agreed to support a simple segmentation indication used to indicate to reader whether the data is segmented and whether it is last segment., with the FFS on whether it is defined as 1 bit or 2 bits.

Our understanding is that if we go for a 1-bit indication, it can be designed as follows:

- 0: this is a segment other than the last segment

- 1: this is the last segment or the whole message

Alternatively, with 2-bit indication we are looking at an RLC-like scheme such as:

- 00: contains all bytes of a message

- 01: contains the first segment

- 10: contains the last segment

- 11: contains neither the first nor last segment

We do acknowledge that there is a reason RLC design in Uu interface includes 2 bits. That being said, for A-IoT we are not looking at the same level of robustness as in NR and device complexity is a much bigger concern. For example, the reader can discern whether 1-bit indication of “0” is the last segment of a segmented message, or just a whole message w/o segmentation, when the reader tries to re-assemble the message. There is no need to further differentiate those two cases in A-IoT air interface. Furthermore, regardless of the design (1-bit or 2-bit), due to nature of A-IoT communications there will be frequent losses and the reader (and/or upper layers) should be able to handle them.

With this in mind, we think that 1-bit indication for segmentation is sufficient.

Proposal 9 		1-bit flag is sufficient for A-IoT segmentation indication in D2R MAC PDU.

Another remaining question for segmentation is that whether RAN2 need support retransmission of failed segments in PDRCH. Note that even though RAN2 has agreed to not introduce sequence number or segment number in MAC layer, it is still feasible to have some sort of ARQ design for segmentation by providing ACK/NACK indication in DL (PRDCH) to trigger a retransmission.  However, given the complexity of ARQ scheme and  the fact RAN2 has already agreed in principle in prior meeting that no RLC-like retransmission is supported, we suggest to have a simple segmentation scheme w/o per-segment retransmissions. Basically, in this scheme:

- Device only continues with the next-segment transmission upon receiving reader’s scheduling message

- Reader re-initiates a new R2D trigger (e.g., command) if fails to receive an expected segment. Do not support automatic retransmission of D2R segment(s).

With this approach, there is no failure/success indication of segment needed and per-segment retransmission is not supported.

Proposal 10	 	Not support ACK/NACK-based segment retransmission.

# 3	Conclusions and Proposals

In this paper, we have discussed the functional aspects of A-IoT.  We have the following proposals:

Proposal 1	No need for a dedicated AS control plane protocol (i.e. control plane functions are part of the MAC protocol).

Proposal 2	RAN2 consider use the above protocol stack as baseline for Ambient IoT Topology 1.

Proposal 3	Multiplexing R2D data towards different device IDs in the same  DL A-IoT MAC PDU is not supported.

Proposal 4	Not support Multiplexing in UL A-IoT MAC PDU.

Proposal 5 	RAN2 clarify that “whether it targets one more than one device” can be realized by providing “group ID/device ID” to the AS layer of reader w/o new explicit indication.

Proposal 6	Only the following information are mandatory to be visible to AS layer: 1) The service type of A-IoT (e.g. inventory, command), 2) targeted for one or more than one device.

Proposal 7	Support a dedicate 1-bit flag at a fixed position in A-IoT D2R MAC header for energy status indication.

Proposal 8	No need to introduce reader control of whether energy status bit is used or not by a device.

Proposal 9 		1-bit flag is sufficient for A-IoT segmentation indication in D2R MAC PDU.

Proposal 10 		Not support ACK/NACK-based segment retransmission.

# 4	References

[1] 		RAN2#125bis meeting minutes

[2] 		RAN2#126 meeting minutes

[3] 		Chairman’s Notes, RAN2#127

[4]		Chairman’s Notes, RAN2#127bis