# 3GPP TSG RAN WG2 Meeting #128      	                           R2-2409969
Orlando, USA, November 18-22, 2024

Agenda item:	8.13.3

Source:	Apple

Title:	Discussion on SRAP for Multi-hop Layer-2 U2N Relay

WID/SID:	NR_SL_relay_enh2– Release 19

Document for:	Discussion and Decision

# 1 Introduction

In this paper, we discuss the SRAP design for Layer-2 Multi-hop UE-to-NW relay.

# 2 Discussion

In RAN2#127[1], it was agreed that the protocol stack used in legacy U2N Relay (i.e., end-to-end for Uu PDCP and above and hop-by-hop for SRAP and below) is reused for Rel-19 Multi-hop U2N relay. Therefore, we think SRAP protocol is also to be used as the protocol to carry end-to-end information and used by UEs to determine how to forward an end-to-end packet between a remote UE and gNB.

For the new “intermediate relay UE” introduced in Rel-19 work, we think it would just have a single PC5 SRAP entity which handles ingress and egess PC5 Relay RLC Channels. There is no Uu SRAP entity used for intermediate relay UE operation.

Proposal 1	 Each Layer-2 Intermediate Relay UE has a single PC5 SRAP entity.

Regarding the contents of SRAP header, it is very clear that the end-to-end traffic is still between a remote UE and gNB, as similar to the legacy case, so we think what we have in the existing SRAP header, which is remote UE ID and bearer ID are still needed.

Proposal 2	 Remote UE ID and BEARER ID are included in SRAP header for multi-hop L2 U2N relay.

Then, in legacy L2 U2N Relay design, only one set of PC5 Relay RLC channel configurations are provided to a L2 U2N remote UE or L2 U2N relay UE in legacy single-hop relay design [2]. Then, for L2 remote UE, it is supposed to create PC5 Relay RLC channels based on the RRC configuration on the PC5 link between the remote UE the relay UE. For relay UE, it should create PC5 Relay RLC channels based on the configuration on each of the PC5 link to each of the connected remote UE(s). This works in legacy single-hop case because either remote UE or relay UE in Rel-17/Rel-18 design only has one direction to deliver the SRAP traffic, either towards the remote UE or towards the relay UE (and gNB). But In Rel-19 topology, each intermediate relay UE has two directions to deliver traffic, then it is unclear which one of the following two options would apply:

- Option 1: gNB provide two set of PC5 Relay RLC channel configurations to each intermediate relay UE, one for the direction to NW, one for the direction to remote UE;

- Option 2: gNB still provide a single set of PC5 Relay RLC channel configurations to each intermediate relay UE, but the UE need to establish (at least) two identical PC5 Relay RLC channels in both directions (one towards a downstream UE and one towards an upstream UE) for the end-to-end delivery.

Both of the above two options are feasible. For Option 2, it seems that the intermediate UE are obliged to establish symmetric PC5 Relay RLC channels in both UL and DL directions. However, in SRAP mapping of end-to-end traffic, it is very possible that UL and DL mapping may use different number of egress PC5 Relay RLC channels, and are not symmetric. This means some PC5 Relay RLC channels configured are not needed, at least in one of the directions. However, even if a PC5 Relay RLC channel is established, it can still be released by UE if not used. So, Option 2 can still work with probably less signalling overhead, no matter whether the PC5 Relay RLC channel configurations is provided by dedicated RRC signalling or SIB/Pre-configuration. So, we think Option 2 can be adopted.

Proposal 3	For SRAP mapping in intermediate relay, a single set of PC5 Relay RLC channel configurations is used for both directions, whereas different SRAP mapping could be used in each direction.

Regarding relay UE’s handling of SRAP PDU to determine forwarding, the legacy U2N relay UE design is very simple:

- If an SRAP PDU received from Uu interface (Uu SRAP), then it is supposed to be mapping to an egress PC5 Relay RLC channel based on SRAP mapping

- If an SRAP PDU is received from PC5 interface (PC5 SRAP), then it is supposed to be mapping to an egress Uu relay RLC channel based on SRAP mapping.

However, for intermediate relay UE in multi-hop scenario, there is some new complexity in decision-making of traffic forwarding. Since there is only one PC5 SRAP entity associated with multiple PC5 links, the intermediate relay UE needs to determine which PC5 link should be used for forwarding the SRAP PDU based on SRAP mapping. If the SRAP header itself does not have an indication of “direction”, the relay UE need to dig into the source layer-2 address to understand which PC5 link the packet comes from and determine that the Src L2 ID of the received packet is in its upstream or downstream. This means the intermediate relay UE has to maintain additional context of L2 IDs of each PC5 link and maintain another mapping table per each remote UE for those L2 IDs. This would be inconvenient for Sidelink UE as L2 ID(s) may need change from time to time for a PC5 link. We think a simple enhancement to streamline the SRAP forwarding operation is to add a “directional” indication bit in SRAP header, this single-bit indicator can tell the relay whether this is UL traffic or DL traffic, so the relay UE can decide this in SRAP layer solely based on the SRAP mapping and SRAP header, w/o checking additional lower layer information.

Proposal 4	A “direction” bit to discern UL/DL is included in SRAP header.

Finally, we think although the SRAP control PDU is not used in legacy U2N relay design, it could be a useful tool in Multi-hop relay scenario to piggyback some information among UEs along the path with an end-to-end message. For example, when the RRCSetupRequest is transmitted from the remote UE to gNB along the multi-hop path, the remote UE’s L2 ID is an important information to help the nodes along the path to understand the relationship of the path and the remote UE identity, so that the relay UE(s) can match the later-assigned local ID to this remote UE. This is something cannot be done in discovery procedure because only upper layer “User Info”, not L2 ID are used in relay discovery procedures.

Proposal 5	Support SRAP Control PDU including remote UE L2 ID to be transported together with the first RRC message .

# 3 Conclusion

In this contribution, we discuss the SRAP design for Multi-hop SL relay, and we have the following proposals:

Proposal 1	Each Layer-2 Intermediate Relay UE has a single PC5 SRAP entity.

Proposal 2	Remote UE ID and BEARER ID are included in SRAP header for multi-hop L2 U2N relay.

Proposal 3	For SRAP mapping in intermediate relay, a single set of PC5 Relay RLC channel configurations is used for both directions, whereas different SRAP mapping could be used in each direction.

Proposal 4	A “direction” bit to discern UL/DL is included in SRAP header.

Proposal 5	Support SRAP Control PDU including remote UE L2 ID to be transported together with the first RRC message .

# 4 References

[1] Chairman’s Notes, RAN2#127

[2] TS 38.331 v18.3.0