3GPP TSG-RAN WG2 #128                                             	    R2-2409987

Orlando, USA, November 18th –22nd, 2024

Agenda item:	   8.6.3

Source: 		Qualcomm Incorporated

Title: 	L1 event-triggered measurement reporting for LTM

Document for:	Discussion

# 1	Introduction

RAN2#127b agreed the following [1]:

Agreements on L1 event triggered MR - MR can be sent when the leaving condition is met, based on NW configuration. - Event triggered periodic MR can be supported, based on NW configuration. - For measurement resource configuration, R18 LTM CSI resource configuration is reused if possible. If CSI-RS resource only IE needs to be defined, we can revisit it in the stage 3. - For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused if possible. We can revisit it in the stage 3 if needed. - For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible. We can revisit it in the stage 3 if needed. - The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1. - TTT operates only based on a timer (like TTT used in L3 event triggered MR). - Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5). - Basic information included in MR MAC CE: - Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id) - Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams - Triggered event information (e.g., ReportConfigID) MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not). N is configurable by NW. - Additional information included in MR MAC CE: - The information and quantity of current beam, based on NW configuration. - The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE. - NW can configure a dedicated SR configuration for MR MAC CE transmission.

RAN2#127 agreed the following [2]:

- Event LTM1 is not defined. - Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. - Any beam in candidate RS configuration can be used for LTM event evaluation. - Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case. - R19 LTM event triggered measurement configuration can be taken as baseline: - LTM measurement resource configuration is provided in LTM-config. - Event triggered report config is provided in serving cell config. - MAC layer handles the event evaluation and measurement report triggering. - Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

This paper discusses further aspects related to measurement enhancements for LTM.

# 2	LTM CSI resource configuration

## 2.1	Measurement RS type alignment between current beam and candidate beam(s)

RAN2#127b discussed how to align the measurement RS types between the current beam (i.e., a beam corresponding to the indicated TCI state) and the candidate beam.

RAN1 has agreed in RAN1 #116b & #117 for Event 2 (equivalent to Event LTM3) that the RS for the current beam is implicitly derived to be either the QCL RS in the indicated TCI state or the SSB which is QCLed with the QCL RS in the indicated TCI state. And for new beams, it’s been agreed in RAN1 #118 that the RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration.

Similarly for LTM, at least, UE shall be configured to measure the RSs with the same RS type as candidate beams. The detailed signalling is FFS. This will avoid any ambiguity or overhead in measuring candidate beams when the current beam’s RS type changes according to the indicated TCI state. And for the RS configuration for the current beam measurement, NW shall make sure to configure the proper RS resource set with the same RS type as the candidate beam RSs’.

Observation 1: While the RS type for candidate beams can be explicitly configured as part of RS configuration, the RS type for the current beam can change depending on RS configuration for indicated TCI state.

- Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5)

Proposal 1: Measurement RS type relies on the network configuration, i.e., NW shall make sure a single RS type is measured for candidate beams (detailed signaling is FFS), and the proper RS resource set with the same RS type shall be configured for the current beam measurement.

# 3	LTM event evaluation

## 3.1	Event dependencies

RAN2#127 agreed that any beam in the candidate RS configuration can be used for event evaluation. This implies that the UE may concurrently evaluate an event using multiple candidate beams. There are several scenarios to consider:

Case 1: Only one candidate beam meets the event condition.
This scenario is straightforward, as a single measurement report (MR) is triggered.

Case 2: More than one beam of the candidate cell meets the event condition around the same time (i.e., before issuing an MR based on any of the triggering beams).
In this case, triggering multiple measurement reports simultaneously is redundant, as all information can be included in a single MR.

Observation 2: For a given candidate cell, multiple candidate beams may meet an event trigger condition before an MR has been issued based on the triggered event.

Proposal 2: If multiple beams of the same candidate cell meet an event triggering condition and no MR has been successfully transmitted yet, only one MR should be triggered.

Case 3: One candidate beam triggers an MR, which is successfully transmitted. Shortly after, another candidate beam of the same candidate cell meets the event condition.
In this scenario, there are two options:

- Option 1: Multiple MRs may independently be triggered.

- Option 2: A new MR can only be triggered after a “sufficient” amount of time has passed since the previous MR was triggered. This requires introducing a prohibit timer that starts after an event condition is met, during which no new MRs can be triggered.

We observe that Option 1 is a special case of Option 2 by basically configuring a prohibit timer value of zero. Option 2 should be supported.

Observation 3a: For a given candidate cell, one candidate beam may meet an event trigger condition after another candidate beam has “recently” triggered a MR based on the same event.

Observation 3b: The following options are possible:

- Option 1: A new MR is independently triggered when the new candidate beam meets the event trigger condition.

- Option 2: A new MR is triggered only after the expiry of a prohibit timer, which starts once an MR has been triggered for the same candidate cell.

Observation 3c: Option 1 is a special case of Option 2 by setting the prohibit timer to zero.

Proposal 3a: Introduce a prohibit timer with the following behavior:

- The timer starts after a MR is triggered based on fulfilling an event condition for a candidate cell

- While the timer is running, no new MR can be triggered for the same candidate cell

RAN2 should further discuss the granularity of the prohibit timer.

Proposal 3b: RAN2 to discuss whether the granularity of configuration of a prohibit timer:

- Option (a): per event per candidate cell

- Option (b): per candidate cell

- Option (c): any or both Option (a) and Option (b) based on configuration

## 3.2	Multi-beam event evaluation

RAN2#127 agreed that a beam-level measurement result, as opposed to a cell-level measurement result, is used for LTM event evaluation. RAN2 should further clarify that a single-beam level measurement result is used for event evaluation. Using more than one beam for event evaluation is more complex. Additionally, multi-beam evaluation may not be necessary for reducing the overhead of measurement reporting, as this can be controlled using a prohibit timer as discussed above.

Observation 4: Defining event evaluation based on multiple beams is more complex and may not be needed, as MR overhead can be controlled with a prohibit timer.

Proposal 4: RAN2 confirms that the measurements of a single beam in candidate RS configuration is used at a given time for LTM event evaluation.

## 3.3	Time-to-trigger (TTT) management

RAN2#127b agreed that TTT operates in the same way as in L3 MR, i.e. when an event condition is met for TTT amount of time window consistently, the measurement report is triggered. When TTT is applied to an event based on a beam-level measurement result, there could be two scenarios to consider – one is the current beam change and the other is the candidate beam handling.

### 3.3.1	TTT for the current beam change

Based on previous RAN1 and RAN2 agreements, the current beam is not explicitly tied with a particular RS, but “virtually” mapped to either the QCL RS of or the SSB QCLed with the indicated TCI state. Due to this “floating” nature of the current beam and the possibly dynamic current beam switches, the current beam change doesn’t need to reset TTT. As long as the event condition is met after the current beam change, TTT should keep running.

Observation 5: The current beam is not tied with a particular RS (implicit mapping) and it can change dynamically based on the indicated TCI state.

Proposal 5: The current beam change doesn’t reset TTT unless the event condition is not met upon the beam change.

### 3.3.2	TTT for the candidate beam handling

The main purpose of performing a beam-level measurement and event evaluation is to find the best combination of both a candidate cell and its beam for the potential LTM cell switch. Applying TTT across candidate beams under a candidate cell could help trigger an L1 event-triggered measurement report earlier, but the best candidate beam selection based on the measurement report may not be reliable enough for LTM cell switch, which could lead to possibly lower channel quality after LTM or even LTM failure.

Observation 6: Finding a best candidate beam for a candidate cell is the key for LTM cell switch performance. TTT handling across candidate beams under a candidate cell may not provide a reliable candidate beam report.

Proposal 6: TTT operation is handled per-beam for candidate beams.

# 4	MAC-CE-based measurement report

## 4.1	Configuration aspects

RAN#127 agreed that the event-triggered report configuration should be provided in the serving cell configuration. This agreement can be evaluated by considering the scenario where the UE is configured with CA. In this case, different UE behaviours can be considered:

- Option A: UE has one report configuration per serving cell group:

  - Option A1: UE may only send the MAC-CE report via the PCell.

  - Option A2: UE may send the MAC-CE report via any serving carrier.

- Option B: UE may have one report configuration per carrier, and UE issues MAC-CE reports on a carrier using the respective report configuration.

Observation 7a: For a UE configured with CA, the following options are possible:

- Option A: UE has one report configuration per serving cell group

- Option B: UE may have one report configuration per serving carrier, and UE sends a MAC-CE report on any carrier using the respective report configuration of that carrier.

Noting that it is the same serving gNB-DU that would receive the MAC-CE reports regardless of which carrier is used to carry the report, so one report configuration is sufficient. This implies Option A should be agreed upon. This further implies that the event-triggered report configuration does not need to be inside the serving cell configuration but rather may be within the CellGroupConfig (reverting the prior agreement).

Observation 7b: All MAC-CE reports issued on any carrier of the same serving cell group are received by the same gNB-DU, so multiple report configurations are not needed.

Proposal 7a: UE receives one event-triggered report configuration for the serving cell group.

Proposal 7b: Event-triggered report configuration is within CellGroupConfig but outside the serving cell configuration (reverting prior RAN2 agreement).

## 4.2	Report content

We propose the MAC CE report to include the following:

Proposal 8: MAC-CE-based report to include:

- Triggered event ID

- Config ID(s) of candidate cell group, for event types LTM3/4/5

- (List of) Triggering candidate beam ID(s)

- Measurement metric corresponding to triggering candidate beam ID(s)

- Measurements of other non-triggering candidate beams

  - Can be present if the number of event triggering candidate beams is less than N

- Current beam quantity (if configured)

Regarding the measurement metric representation, the differential format approach (similar to LTM L1-RSRP report via UCI) can be adopted to reduce the overall report message size.

Proposal 9: RAN2 to support a differential L1-RSRP format for measurement metric representation with the followings considered.

- Absolute L1-RSRP value for the largest measured RSRP among reported ones

- Differential L1-RSRP value for the rest measured RSRPs (up to N-1 beams)

- FFS: range and step size of differential L1-RSRP

# Conclusion

This contribution discussed aspects related to measurement enhancements for LTM. The following observations and proposals have been made:

Measurement RS type alignment between current beam and candidate beam(s)

Observation 1: While the RS type for candidate beams can be explicitly configured as part of RS configuration, the RS type for the current beam can change depending on RS configuration for indicated TCI state.

- Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5)

Proposal 1: Measurement RS type relies on the network configuration, i.e., NW shall make sure a single RS type is measured for candidate beams (detailed signaling is FFS), and the proper RS resource set with the same RS type shall be configured for the current beam measurement.

Event dependencies

Observation 2: For a given candidate cell, multiple candidate beams may meet an event trigger condition before an MR has been issued based on the triggered event.

Proposal 2: If multiple beams of the same candidate cell meet an event triggering condition and no MR has been successfully transmitted yet, only one MR should be triggered.

Observation 3a: For a given candidate cell, one candidate beam may meet an event trigger condition after another candidate beam has “recently” triggered a MR based on the same event.

Observation 3b: The following options are possible:

- Option 1: A new MR is independently triggered when the new candidate beam meets the event trigger condition.

- Option 2: A new MR is triggered only after the expiry of a prohibit timer, which starts once an MR has been triggered for the same candidate cell.

Observation 3c: Option 1 is a special case of Option 2 by setting the prohibit timer to zero.

Proposal 3a: Introduce a prohibit timer with the following behavior:

- The timer starts after a MR is triggered based on fulfilling an event condition for a candidate cell

- While the timer is running, no new MR can be triggered for the same candidate cell

Proposal 3b: RAN2 to discuss whether the granularity of configuration of a prohibit timer:

- Option (a): per event per candidate cell

- Option (b): per candidate cell

- Option (c): any or both Option (a) and Option (b) based on configuration

Multi-beam event evaluation

Observation 4: Defining event evaluation based on multiple beams is more complex and may not be needed, as MR overhead can be controlled with a prohibit timer.

Proposal 4: RAN2 confirms that the measurements of a single beam in candidate RS configuration is used at a given time for LTM event evaluation.

TTT for the current beam change

Observation 5: The current beam is not tied with a particular RS (implicit mapping) and it can change dynamically based on the indicated TCI state.

Proposal 5: The current beam change doesn’t reset TTT unless the event condition is not met upon the beam change.

TTT for the candidate beam handling

Observation 6: Finding a best candidate beam for a candidate cell is the key for LTM cell switch performance. TTT handling across candidate beams under a candidate cell may not provide a reliable candidate beam report.

Proposal 6: TTT operation is handled per-beam for candidate beams.

Configuration aspects

Observation 7a: For a UE configured with CA, the following options are possible:

- Option A: UE has one report configuration per serving cell group

- Option B: UE may have one report configuration per serving carrier, and UE sends a MAC-CE report on any carrier using the respective report configuration of that carrier.

Observation 7b: All MAC-CE reports issued on any carrier of the same serving cell group are received by the same gNB-DU, so multiple report configurations are not needed.

Proposal 7a: UE receives one event-triggered report configuration for the serving cell group.

Proposal 7b: Event-triggered report configuration is within CellGroupConfig but outside the serving cell configuration (reverting prior RAN2 agreement).

Report content

Proposal 8: MAC-CE-based report to include:

- Triggered event ID

- Config ID(s) of candidate cell group, for event types LTM3/4/5

- (List of) Triggering candidate beam ID(s)

- Measurement metric corresponding to triggering candidate beam ID(s)

- Measurements of other non-triggering candidate beams

  - Can be present if the number of event triggering candidate beams is less than N

- Current beam quantity (if configured)

Proposal 9: RAN2 to support a differential L1-RSRP format for measurement metric representation with the followings considered.

- Absolute L1-RSRP value for the largest measured RSRP among reported ones

- Differential L1-RSRP value for the rest measured RSRPs (up to N-1 beams)

- FFS: range and step size of differential L1-RSRP

# References

- Chairman Notes; TSG RAN WG2 Meeting #127bis

- Chairman Notes; TSG RAN WG2 Meeting #127