3GPP TSG-RAN WG2 Meeting #128	 R2-2410042

Orlando, US, November 18-22, 2024

Source:	KDDI Corporation

Title:	Discussion on Random access procedure for On-demand SIB1

Tdoc Type:	Discussion

Agenda Item:	8.5.3	On-demand SIB1

Document for:  Discussion

Release:       Rel-19

1	Introduction

In RAN2#126[1], on-demand SIB1 was discussed and below agreements were made.

Agreements on on-demand SIB1: RAR monitoring for MSG1 based OD-SIB1 REQ 13. RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

In this contribution, we further discuss RAR procedure for MSG1 based OD-SIB1 REQ. In addition, UE is expected to receive the RAR responding to the preamble transmission for MSG1 based OD-SIB1 REQ but initial transmission failure has not been discussed so we would like to have further discussion.

2	Discussion

2.1 WUS configuration for RAR

In Ondemand SI, UE receives PDCCH indicating the scheduling information for RAR "As specified in TS 38.321, 'else if a valid (as specified in TS 38.213 [6]) downlink assignment has been received on the PDCCH for the RA-RNTI and the received TB is successfully decoded’, and follows the received information to receive RAR according to TS 38.331. Therefore, applying the specifications of the Ondemand SI RAR procedure for Ondemand SIB1 is straightforward, as the UE receives the RAR following the transmission of the RACH preamble.

Proposal1: RAN2 agree that Ondemand SIB1 uses RAR procedure for Ondemand SI.

From Section 5.2.2.3.3 of TS 38.331: 1> else: 2> if the UE is neither a RedCap nor an eRedCap UE and if SIB1 includes si-SchedulingInfo containing si-RequestConfigMSG1-Repetition and criteria to select normal uplink and to apply MSG1 repetition as defined in TS 38.321[3], clause 5.1.1e for the concerned si-RequestConfigMSG1-Repetition are met; or 2> if the UE is an (e)RedCap UE and if initialUplinkBWP-RedCap is not configured in UplinkConfigCommonSIB and if SIB1 includes si-SchedulingInfo containing si-RequestConfigMSG1-Repetition and criteria to select normal uplink and to apply MSG1 repetition as defined in TS 38.321[3], clause 5.1.1e for the concerned si-RequestConfigMSG1-Repetition are met: 3> trigger the lower layer to initiate the Random Access procedure on normal uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) associated with the applicable MSG1 repetition number in corresponding to the SI message(s) that the UE requires to operate within the cell, and for which si-BroadcastStatus is set to notBroadcasting; 3> if acknowledgement for SI request is received from lower layers: 4> acquire the requested SI message(s) as defined in clause 5.2.2.3.2, immediately;

From Section 5.1.4 of TS 38.321: 1> else if a valid (as specified in TS 38.213 [6]) downlink assignment has been received on the PDCCH for the RA-RNTI and the received TB is successfully decoded: 2> if the Random Access Response contains a MAC subPDU with Backoff Indicator: 3> set the PREAMBLE_BACKOFF to value of the BI field of the MAC subPDU using Table 7.2-1, multiplied with SCALING_FACTOR_BI. 2> else: 3> set the PREAMBLE_BACKOFF to 0 ms. 2> if the Random Access Response contains a MAC subPDU with Random Access Preamble identifier corresponding to the transmitted PREAMBLE_INDEX (see clause 5.1.3): 3> consider this Random Access Response reception successful. 2> if the Random Access Response reception is considered successful: 3> if the Random Access Response includes a MAC subPDU with RAPID only: 4> consider this Random Access procedure successfully completed; 4> indicate the reception of an acknowledgement for SI request to upper layers. 1> if ra-ResponseWindow configured in RACH-ConfigCommon expires, and if the Random Access Response containing Random Access Preamble identifiers that matches the transmitted PREAMBLE_INDEX has not been received: 2> consider the Random Access Response reception not successful; 2> increment PREAMBLE_TRANSMISSION_COUNTER by 1; 2> if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1: 3> if the Random Access Preamble is transmitted on the SpCell: 4> indicate a Random Access problem to upper layers; 4> if this Random Access procedure was triggered for SI request: 5> consider the Random Access procedure unsuccessfully completed.

Next, UE receives configuration to receive RAR that SIB1 broadcasts in advance, and then uses those parameters to proceed with RAR reception in Ondemand-SI. However, in On-demand SIB1, SIB1 that NES cell broadcasts is received after WUS transmission, so it is necessary to organize when to receive configuration. As a solution, it is necessary to include the information required for RAR reception in the WUS configuration, allowing UE to obtain configuration in advance. Specifically, for DCI reception, ra-SearchSpace included in PDCCH-ConfigCommon for the Type1-PDCCH CSS set is required [2]. Furthermore, UE can expect to receive the RAR within the ra-responsewindow by receiving the ra-ResponseWindow included in RACH-ConfigCommon for RAR reception.

From Section 10.1 of TS 38.213: a Type1-PDCCH CSS set configured by ra-SearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a RA-RNTI, a MsgB-RNTI, or a TC-RNTI on the primary cell

Proposal2: RAN2 agree that WUS configuration includes PDCCH configuration to receive Type-1 PDCCH Common Search Space for DCI format 1_0 with CRC scrambled by RA-RNTI, RAR scheduling on PDSCH.

Proposal3: RAN2 agree that UE receives RAR within ra-responsewindow as configured in the WUS configuration.

Figure1. Ondemand-SIB1 with RAR

2.2 RACH preamble retransmission

It is necessary to discuss whether to retransmit WUS (RACH preamble) if WUS initial transmission fails. First, it is reasonable to assume that the trigger for UE to determine WUS transmission failure is when it fails to receive the RAR within the ra-responsewindow, because we can use the legacy specification.

Additionally, UE that are located far from the base station or affected by interference may not be able to receive Msg1. This issue is typically resolved by retransmitting the preamble and increasing the transmission power during retransmission, which has the advantage of improving communication quality. Therefore, since the retransmission of Msg1 is standardized in On-demand SI, it is straightforward to support retransmission in On-demand SIB1 as well.

Furthermore, it is necessary to introduce retransmission of RACH preamble and PowerRampingStep that increases the uplink transmission power during retransmissions.

Proposal4: RAN2 agree that UE will retransmit RACH preamble when UE fails to receive RAR from NES Cell within the raResponseWindow.

Proposal5: RAN2 agree to introduce the use of powerRampingStep to increase the uplink transmission power for WUS signal retransmissions.

2.3 SIB1 monitoring window

In RAN2 #127 meeting, it was discussed that the decision regarding how long UE should wait to receive SIB1 could be left to the specific implementation of UE. Consequently, it was decided to wait for RAN1 progress this issue.

P6 in R2-2408461 (Google) Proposal 5: If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE considers the cell as being barred. Wait for more RAN1 progress.

However, it would be beneficial to clarify the necessity of SIB1 monitoring window from the perspective of RAN2.

The purpose of SIB1 monitoring window is not only to determine whether UE has acquired SIB1 but also to be applied in deciding whether to retransmit WUS.

Firstly, in the case of On-demand SIB1 with RAR, the system considers On-demand SIB1 based on the reception of RAR, and the decision for WUS retransmission is sufficiently made by RAR reception. However, in scenarios without RAR, there is no acknowledgment from the network, so a substitute method is needed to decide on the retransmission of WUS. One solution to address this issue is to introduce SIB1 monitoring window, which would enable the decision on the necessity of WUS retransmission.

For these reasons, the necessity of the SIB1 monitoring window varies depending on the RAR scenario, and it is essential for RAN2 to clarify the scope of discussion regarding SIB1 monitoring window in Rel-19.

Proposal6: RAN2 discuss SIB1 monitoring window according to the following options.

Option1 : If considering On-demand SIB1 scenario without RAR in Rel-19, RAN2 agree to introduce SIB1 monitoring window.

Option2 : If not considering On-demand SIB1 scenario without RAR in Rel-19, RAN2 agree not to  introduce SIB1 monitoring window.

3	Conclusion

We make the following observations and proposals:

Proposal1: RAN2 agree that Ondemand SIB1 uses RAR procedure for Ondemand SI.

Proposal2: RAN2 agree that WUS configuration includes PDCCH configuration to receive Type-1 PDCCH Common Search Space for DCI format 1_0 with CRC scrambled by RA-RNTI, RAR scheduling on PDSCH.

Proposal3: RAN2 agree that UE receives RAR within ra-responsewindow as configured in the WUS configuration.

Proposal4: RAN2 agree that UE will retransmit RACH preamble when UE fails to receive RAR from NES Cell within the raResponseWindow.

Proposal5: RAN2 agree to introduce the use of powerRampingStep to increase the uplink transmission power for WUS signal retransmissions.

Proposal6: RAN2 discuss SIB1 monitoring window according to the following options.

Option1 : If considering On-demand SIB1 scenario without RAR in Rel-19, RAN2 agree to introduce SIB1 monitoring window.

Option2 : If not considering On-demand SIB1 scenario without RAR in Rel-19, RAN2 agree not to  introduce SIB1 monitoring window.

3	　Reference

[1]　RAN2#126 chairnote

[2]　3GPP TS 38.213 V18.1.0