3GPP TSG-RAN WG2 #128	R2-2410061

Orlando, USA, 18 – 22 November 2024

Agenda item:		8.5.2	On-demand SSB SCell operation

Title:		Further discussion on On-demand SSB for SCell

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In RAN2#127 and #127bis, RAN2 had some agreements on the On-demand SSB operator for SCell [1][2].

Agreements on OD-SSB @ #127

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

Agreements on OD-SSB SCell @ #127bis

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

In this contribution, we discuss some details of network signalling and RRM measurements.

# 2. Discussion

2.1	MAC signalling for OD-SSB (de)activation

For the OD-SSB activation/deactivation, RAN2 agreed to introduce the new MAC CE which can indicate for multiple SCells simultaneously. The new MAC CE can be used for both the scenario 2 (OD-SSB activation before SCell activation) and 2A (OD-SSB activation at the same time as SCell activation).

We assume that from the network perspective, these two scenarios can co-exist. For example, the network configures two SCells with deactivated state initially for a UE. At some point in time, the network sends the new MAC CE indicating OD-SSB activation on the SCell#1 and #2 with the legacy SCell activation/deactivation MAC CE indicating activation of SCell#1. Later, the network sends the legacy SCell activation/deactivation MAC CE indicating activation of SCell#2. For the SCell#1, it corresponds to the scenario 2A, while for the SCell#2 it corresponds to the scenario 2. We assume this handling is allowed and it is up to the network implementation.

Proposal 1: RAN2 to confirm that it is up to network implementation how the new MAC CE and the legacy SCell activation/deactivation MAC CE are used in combination.

Fig.1 Example combination of new MAC CE and legacy MAC CE

2.2	RRM measurements

In RAN2#127bis, RRM measurements for the case #2 (Always-on SSB is periodically transmitted on the cell) was discussed with two options on the table [3] but postponed.

- Option1: UE uses on-demand SSB for RRM measurements when they are transmitted, otherwise UE uses periodic SSB

- Option2: UE uses on-demand SSB for RRM measurements when they are transmitted in addition to periodic SSB for RRM measurements.

On this issue, there were some questions, for example whether the OD-SSB and always-on SSB are on the same frequency or different frequency. Depending on the exact scenarios or network configurations, the option to be selected can be different, or there may be benefits in both options. For example, in the Option 1 the UE power consumption can be saved compared with the Option 2, if both SSBs are transmitted on the same frequency. If both SSBs are transmitted on the different frequencies, the Option 2 may be necessary depending on the reason why both are on the different frequencies. So, firstly clarification on expected scenarios would be necessary.

For RRM measurements on OD-SSB, RAN4 has also discussed the possible scenarios for the Case #2 and made some agreements including [4]:

Agreement: - In OD-SSB Case 2, RAN4 to first discuss the deactivated SCell requirement for Scenario 1. RAN4 to further discuss whether to define the requirement for the scenario 2 and 3 if scenario 2 and 3 are supported in RAN1 based on RAN1’s agreement. - Scenario 1: OD-SSB and always-on SSB are with same SSB carrier frequency, same offset but different periodicities. - Scenario 2: OD-SSB and always-on SSB are with same SSB carrier frequency, different offsets and periodicities. - Scenario 3: OD-SSB and always-on SSB are within different SSB carrier frequencies. - Send the above information to RAN1, further discuss the text including the questions to RAN1 in the LS. - RAN4 will inform RAN1 if any further agreement.

Then, RAN4 sent the LS to RAN1 (Cc to RAN2) in [5]. The LS includes the questions below:

- Q1: What is the relation in terms of periodicity between always-on SSB and OD-SSB?

- Q2: What is the relation in terms of time location between always-on SSB and OD-SSB?

- Q3: What is the relation in terms of frequency location between the always-on SSB and OD-SSB?

- Q4: What is the spatial relation between the always-on SSB and OD-SSB?

These questions, especially Q2 and Q3, from RAN4 are related to RAN2 discussions on RRM measurements. From RAN2 perspective, it is better to wait for RAN1 response to this LS.

Proposal 2: For the issue of RRM measurements on OD-SSB, RAN2 to wait for RAN1 response to RAN4 LS (R4-2416913) related to expected scenarios.

# 3. Conclusion

In this contribution we discussed some details of network signalling and RRM measurements and made the following proposals.

Proposal 1: RAN2 to confirm that it is up to network implementation how the new MAC CE and the legacy SCell activation/deactivation MAC CE are used in combination.

Proposal 2: For the issue of RRM measurements on OD-SSB, RAN2 to wait for RAN1 response to RAN4 LS (R4-2416913) related to expected scenarios.

# References

[1] R2-2407571, Report from session on V2X/SL, R19 NES and MOB, Vice Chairman (Samsung)

[2] R2-2409211, Report from session on V2X/SL, R18/19 MOB, and R19 NES, Vice Chairman (Samsung)

[3] R2-2408295, On-demand SSB SCell Operation, Samsung Electronics Co., Ltd

[4] R4-2416916, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson

[5] R2-2409521=R4-2416913, LS on SSB relation in On-demand SSB and SSB adaptation on SCell, RAN4 (To RAN1, Cc RAN2)

# Annex for reference [4]

### Sub-topic 1-3: OD-SSB based deactivated SCell L3 measurement (Case 2)

Issue 1-3-1: Always-on SSB and OD-SSB time/frequency domain relation

- Background

From moderator’s understanding, the following scenarios for always-on SSB and OD-SSB relation are possible based on RAN1’s current agreements.

  - Scenario 1: OD-SSB and always-on SSB are with same frequency, same offset but different periodicities.

  - Scenario 2: OD-SSB and always-on SSB are with same frequency, different offsets and periodicities.

  - Scenario 3: OD-SSB and always-on SSB are within different frequencies.

Agreement:

- In OD-SSB Case 2, RAN4 to first discuss the deactivated SCell requirement for Scenario 1. RAN4 to further discuss whether to define the requirement for the scenario 2 and 3 if scenario 2 and 3 are supported in RAN1 based on RAN1’s agreement.

  - Scenario 1: OD-SSB and always-on SSB are with same SSB carrier frequency, same offset but different periodicities.

  - Scenario 2: OD-SSB and always-on SSB are with same SSB carrier frequency, different offsets and periodicities.

  - Scenario 3: OD-SSB and always-on SSB are within different SSB carrier frequencies.

- Send the above information to RAN1, further discuss the text including the questions to RAN1 in the LS.

- RAN4 will inform RAN1 if any further agreement.

Issue 1-3-2: Deactivated SCell measurement before OD-SSB transmission activation

Agreement:

- The UE is assumed to perform deactivated SCell measurement based on legacy SSB (measCycleSCell) before UE receiving OD-SSB transmission indication.

Issue 1-3-3: Deactivated SCell measurement after OD-SSB transmission deactivation

Agreement:

- The UE is assumed to perform deactivated SCell measurement based on legacy SSB (measCycleSCell) after the OD-SSB transmission deactivation.

Issue 1-3-4: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2)

Agreements:

OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2)

  - Option 1: RAN4 requirement is defined based on OD-SSB periodicity.(CATT, Samsung, CMCC, ZTE)

  - Option 2: RAN4 requirement is defined based on legacy measCycleSCell (Apple, Xiaomi, vivo, OPPO)

  - Option 3:  RAN4 requirement is defined based on combining the always-on SSB and OD-SSB. (Ericsson, LG, ZTE)

  - Option 4: RAN4 for further discussion (Nokia, LG, CMCC, ZTE, Qualcomm)

  - Option 5: RAN4 requirement is defined based on the indicated measurement periodicity (either OD-SSB periodicity or measCycleSCell for always-on SSB) (vivo)