3GPP TSG-RAN WG2 Meeting #128												R2-2410088

Orlando, USA, November 18th – 22nd, 2024

Agenda Item:	8.11.2

Source: 	Sharp

Title:  	RA Aspects for SBFD

Document for: 	Discussion

# Introduction

In RAN2#127bis meeting, RAN2 made the following agreements:

RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact. From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework. Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network. For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first. The following two RACH configuration options are considered for SBFD based random access: - Option 1: Use one single RACH configuration based on the existing parameters of the single RACH configuration. Can extend the existing parameters if needed. - Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration For RACH configuration Option 2, RAN2 needs to specify RRC signalling for the new SBFD based RACH configuration with a new set of parameters. The RACH configuration for SBFD is transmitted via SIB1. FFS dedicated RRC signalling detail. FFS whether NW can provide both configurations.

This document discusses open issues on SBFD random access.

# Discussion

## Early Indication

The first issue is whether an early indication of SBFD capability during the random access using legacy RO is needed. We see that an early indicating of SBFD capability during the random access is beneficial. One of the most important benefits of SBFD is latency reduction compared to legacy TDD. An early indication enables the benefit of latency reduction during the random access.

Regarding how to support the early indication, RAN2 concluded that there is no need to introduce SBFD as a feature combination. It means that Msg1-based early indication is not supported. Instead, we can consider Msg3-based early indication which still has a benefit to start transmission and reception early on SBFD symbols.

Msg3-based early indication in legacy ROs during RA procedure is supported to indicate UE’s SBFD capability.

## RO Selection Condition for Initial Transmission

The next issue is RO selection condition for initial transmission. For RO selection among additional RO in SBFD symbols and legacy RO, an issue is not to cause too strong cross link interference. If the UE selects an additional RO without consideration of the interference or coverage, it will lead unnecessarily many RA attempts with power ramp-up.

We think that potential cross link interference aggressor UEs should not utilize additional ROs in SBFD symbols. To achieve this, we can reuse well-known RSRP-based criteria for RO selection. More specifically, if the measured RSRP is higher than a new RSRP threshold for RO selection, which is provided upon PRACH configuration, the UE can select additional ROs. Otherwise, the UE selects legacy ROs. With this approach, SBFD-aware UE will not select additional ROs if the UE is expected to use high transmission power for PRACH by checking the RSRP threshold for RO selection.

RAN2#127bis agreement mentions additional indication for initial condition. Additional indications included in SIB1 may require SIB modification procedure, which could be additional complexity of UE behavior. If the network does not want to allow any UE to use SBFD RO, it is possible that SBRD RO is not configured or the RSRP threshold is set to a very high value. In short, the RSRP-based RO selection is sufficient.

An SBFD-aware UE selects either additional RO or legacy RO based on comparing measured RSRP with a RSRP threshold for RO selection in case of initial RA attempt. No additional indication is necessary.

## RO Selection Condition for Retransmission

Due to the mobility or other obstacle, UE’s RSRP at the preamble retransmission could be different from that at the initiation of random access. So, if the UE keeps using the same RO-type, e.g. additional RO, it may cause significant cross-link interference or unnecessary retransmissions of PRACH preamble. We think the same RO-type selection rule based on RSRP should be also applied for retransmission. In other words, the UE should always check the RO-type selection condition for both initial transmission and retransmissions.

An SBFD-aware UE selects either additional RO or legacy RO based on comparing measured RSRP with a RSRP threshold for RO selection in case of RA re-attempts.

## RO-Type Change from Legacy RO to SBFD RO

In RAN2#127bis meeting, RAN2 agreed to support RO fallback from SBFD RO to legacy RO. The other direction of RO-type change is FFS.

Regardless of the direction, the RO-type change procedure is beneficial when one RO is experiencing congestion that many UEs are attempting the preamble transmissions. For example, additional ROs in SBFD symbols can be less crowded as legacy UEs are not aware of them. In this case, a RO-type change procedure from legacy RO to additional RO can be also useful from offloading perspective.

We do not see the fallback procedure requires a big complexity. The procedure will be almost symmetric for both directions. It is desirable that the network tries to maximize the system performance. Hence, we see that RO-type changes for both directions can be supported.

RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold.

## Simultaneous Configurations of RACH Configuration Options

The last open issue is whether both RACH configuration options can be provided simultaneously by the network. It might be possible that some UEs support only Option 1 whereas the other UEs support only Option 2. The network does not know whether and how many UEs supports each option, until the UE establishes the RRC connection. If the network is capable, we can allow the network to use two options simultaneously. If the network does not want to use two options, it is possible that only one option is used.

Both RACH configurations options can be used simultaneously by the network at a given time.

# Conclusions

Based on the discussion given above, RAN2 is requested to discuss and agree the following proposals:

- Msg3-based early indication in legacy ROs during RA procedure is supported to indicate UE’s SBFD capability.

An SBFD-aware UE selects either additional RO or legacy RO based on comparing measured RSRP with a RSRP threshold for RO selection in case of initial RA attempt. No additional indication is necessary.

An SBFD-aware UE selects either additional RO or legacy RO based on comparing measured RSRP with a RSRP threshold for RO selection in case of RA re-attempts.

RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold.

Both RACH configurations options can be used simultaneously by the network at a given time.