3GPP TSG-RAN WG2 Meeting #128	           R2-2410100

Orlando, US, Nov 18th – Nov 22nd, 2024

Agenda item:	8.5.3

Source:	China Telecom

Title:	Discussion on OD-SIB1

Document for:	Discussion

- Introduction

During RAN#105 meeting, the WID is revised [1]. The study objective of on-demand SIB1 turned into normative work, the updated objective is as the following:

Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

RAN2 has made good progress in previous meetings. In this paper, we will further discuss on OD-SIB1.

- Discussion

After a NES UE request the NES cell to send on-demand SIB1, it is not clear whether the NES cell broadcast SIB1 periodically or not. RAN1#116bis meeting mentioned two options as the following, but no further discussion since that.

- RAN1#116bis Companies to consider the following for future meetings - Option 1: SIB1 monitoring occasions within a time window - FFS: The starting time and duration of the time window - FFS: Interval between two SIB1 monitoring occasions in the time window - FFS: How gNB informs UE the details related to the time window - Option 2: Periodic SIB1 monitoring occasions until gNB turns off the SIB1 transmission - FFS: The staring time of the SIB1 monitoring occasions - FFS: How gNB informs UE the SIB1 transmission is turned off - FFS: How gNB informs the UE the details related to periodicity - Other options are not precluded - FFS: Further details on SIB1 monitoring occasions

On the other hand, at least during RAN2#127bis meeting, when RAN2 discussed cell barring topic, some companies mentioned the case when SIB1 is periodically broadcast or the case when the SIB1 is not periodically broadcast, if NES UE request the NES cell to send on-demand SIB1. Whether SIB1 is periodically broadcast or not after NES UE request on-demand SIB1 has impact on the design/understanding of UE behavior regarding monitoring SIB1 and legacy UE camping on NES cell. Thus we suggest to discuss or decide whether to support one or two of the options, to make progress and align companies’ understanding.

Proposal 1: RAN2 discuss whether SIB1 is periodically broadcast or not if NES UE request on-demand SIB1, to align companies’ understanding.

In our understanding, RAN2’s previous agreements work well for the case that SIB1 is not periodically broadcast if NES UE request on-demand SIB1. If the case that SIB1 is periodically broadcast is supported, we see some potential additional specification work. For example, if the NES cell periodically broadcast SIB1 until the network turns off the SIB1 transmission, the network need to indicate/configure the UE SIB1 transmission related information. When the NES cell periodically broadcast SIB1, whether the ‘NES cell’ will be treated as normal cell or not. If the NES cell is treated as normal cell when SIB1 is periodically broadcast triggered by UL WUS, whether the NES cell need to inform Cell A/neighbor cell to change the NES cell list or not.

Proposal 2: If SIB1 periodically broadcast triggered by on-demand SIB1 request is supported, RAN2 discuss whether the NES cell is treated as a normal cell or not when SIB1 transmission is turned on.

- Conclusions

Based on the discussion above, we have the following proposals.

Proposal 1: RAN2 discuss whether SIB1 is periodically broadcast or not if NES UE request on-demand SIB1, to align companies’ understanding.

Proposal 2: If SIB1 periodically broadcast triggered by on-demand SIB1 request is supported, RAN2 discuss whether the NES cell is treated as a normal cell or not when SIB1 transmission is turned on.

4	References

- [1] RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson.