3GPP TSG-RAN WG2 Meeting #128	R2-2410117

Orlando, USA, Nov.18th – 22nd, 2024

Agenda Item:	7.2.3

Source:	MediaTek Inc.

Title:	Correction on LTM UE capability based on the LS from RAN1

Document for:	Discussion, Decision

Introduction

Maximum number of (joint or separate) TCI across cells

In the last meeting, RAN2 discuss the different understanding of maxNumberJointTCI-AcrossCells in FG45-3a [2]:

Proposal 1: RAN2 to clarify the understanding of maxNumberJointTCI-AcrossCells in FG45-3a:

- Understanding 1: The volume of this component includes the legacy TCI state indicated by FG23-1-1 UnifiedJointTCI-r17, which represent a total maximum number of activated TCI states that UE can support.

- Understanding 2: The volume of this component only includes LTM TCI states indicated in LTM-TCI-info. The value of UnifiedJointTCI-r17 is not taken into account.

Proposal 2: No matter which understanding, add a note in TS 38.306 for clarification.

Different understanding may further lead to corrections on the value of the component, which was discussed in [3]. Since the similar issue was raise in RAN1, RAN2 agreed to wait for RAN1 decision.

Noted. We will wait for RAN1 decision.

In this meeting, an LS from RAN1 (R1-2409043) is received with the updated RAN1 UE feature list with the related changes. Corrections are need to UE capability specification.

Intra/inter frequency discrepancy for RAN1 features

In RAN2 #127 meeting, RAN2 introduced an additional bit (per UE) ltm-InterFreq-r18 to indicate the support of inter-frequency LTM. RAN2 further ask RAN1 if RAN1 feature 45-3a/4a, 45-5/5a/6 should also be indicated separately for intra and inter-frequency LTM cell switch.

Additionally, RAN2 would like to check with RAN1: Given the RAN2 agreement that RAN2 has defined separate capabilities for intra and inter frequency LTM cell switch (agreement 1 above), RAN2 would like to check with RAN1 if the UE support of RAN1 features 45-3a/4a, 45-5/5a/6 should also be indicated separately for intra and inter-frequency LTM cell switch.

In this meeting, an LS from RAN1 (R1-2409045) is received. Corrections can be further made to UE capability specification for clarification.

Discussion

The updated Feature list from RAN1

In this meeting, an LS from RAN1 is received with the updated feature list table, which is included in the appendix. The changes can be summarized as follows:

- Confirming on understanding 1 and make further clarification to the description.

- The values of related components are updated from {1, 2, 3, 4, 8, 16 (,32)} to continuous value {1 to 32}.

Based on the updated feature list, corrections are needed to the UE capability spec TS 38.306 and TS 38.331.

Correction to TS 38.306

The updated description in the RAN1 feature list should also be reflected in TS 38.306 components description as follows:

- TP1

---Start of TP ---

| ltm-MAC-CE-JointTCI-r18 Indicates whether the UE supports MAC-CE activated joint LTM TCI states. This capability comprises the following parameters: - qcl-Resource-r18 indicates the supported QCL source RS for MAC-CE activated DL/UL LTM TCI states configuration. - maxNumberJointTCI-PerCell-r18 indicates the maximum number of MAC-CE activated joint LTM TCI states per candidate cell. - maxNumberJointTCI-AcrossCells-r18 indicates the maximum number of MAC-CE activated joint LTM TCI states across candidate cells and serving cell TCI states across serving cells in the band. A UE supporting this feature shall also indicate support of ltm-BeamIndicationJointTCI-r18. NOTE: The maximum number of MAC-CE activated joint TCI states across all servings cells is limited by of unifiedJointTCI-r17. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| ltm-MAC-CE-SeparateTCI-r18 Indicates whether the UE supports MAC-CE activated DL/UL LTM TCI states. This capability comprises the following parameters: - qcl-Resource-r18 indicates the supported QCL source RS for MAC-CE activated DL/UL LTM TCI states configuration. - maxNumberDL-TCI-PerCell-r18 indicates the maximum number of MAC-CE activated DL TCI states per candidate cell. - maxNumberUL-TCI-PerCell-r18 indicates the maximum number of MAC-CE activated UL TCI states per candidate cell. - maxNumberDL-TCI-AcrossCells-r18 indicates the maximum number of MAC-CE activated LTM DL TCI states across all candidate cells and serving cell DL TCI states across all serving cells in the band. - maxNumberUL-TCI-AcrossCells-r18 indicates the maximum number of MAC-CE activated LTM UL TCI states across all candidate cells and serving cell UL TCI states across all serving cells in the band. A UE supporting this feature shall also indicate support of ltm-BeamIndicationSeparateTCI-r18. The maximum number of MAC-CE activated DL/UL TCI states across all servings cells is limited by unifiedSeparateTCI-r17. | Band | No | N/A | N/A |

---End of TP ---

One thing to be highlight is that or the component 4 (DL TCI state) of ltm-MAC-CE-SeparateTCI-r18 in the updated feature list from RAN1, the “in the band” description is missing. We believe this is only a typo error that should be corrected, as the feature is anyway reported per band. Therefore, the same “in the band” description is added to component 4, consistent with component 5.

Proposal 1: RAN2 is asked to adopt the changes to TS 38.306 in TP1.

Correction to TS 38.331

According to the updated RAN1 feature list, the values of component maxNumberJointTCI-AcrossCells-r18 in FG45-3a should be changed from {1, 2, 3, 4, 8, 16, 32} to a continuous range of {1 to 32}. The values of component maxNumberDL-TCI-AcrossCells-r18 and maxNumberUL-TCI-AcrossCells-r18 in FG45-4a should be changed and increased from {1, 2, 3, 4, 8, 16} to a continuous range of {1 to 32}.

Since ASN.1 is frozen, the correction should be made using BC ways. We have prepared several options of the TP for discussion.

- TP 2 option1

Dummy the entire UE capabilities for FG45-3a and FG45-4a, and add the new capabilities.

---Start of TP ---

BandNR ::=                          SEQUENCE {

<Skip unchanged IEs>

[[

<Skip unchanged IEs>

dummy1                                       SEQUENCE {

qcl-Resource-r18                                                ENUMERATED {ssb, trs, both},

maxNumberJointTCI-PerCell-r18                                   INTEGER (1..16),

maxNumberJointTCI-AcrossCells-r18                               ENUMERATED {n1,n2,n3,n4,n8,n16,n32}

}                                                                                                                          OPTIONAL,

<Skip unchanged IEs>

dummy2                                      SEQUENCE {

qcl-Resource-r18                                                ENUMERATED {ssb, trs, both},

maxNumberDL-TCI-PerCell-r18                                     INTEGER (1..8),

maxNumberUL-TCI-PerCell-r18                                     INTEGER (1..8),

maxNumberDL-TCI-AcrossCells-r18                                 ENUMERATED {n1,n2,n4,n8,n16},

maxNumberUL-TCI-AcrossCells-r18                                 ENUMERATED {n1,n2,n4,n8,n16}

} 																									OPTIONAL,

<Skip unchanged IEs>

]],

[[

-- R1 45-3a: MAC-CE activated joint LTM TCI states

ltm-MAC-CE-JointTCI-r18                                         SEQUENCE {

qcl-Resource-r18                                                ENUMERATED {ssb, trs, both},

maxNumberJointTCI-PerCell-r18                                   INTEGER (1..16),

maxNumberJointTCI-AcrossCells-r18                               INTEGER (1..32)

} 																									OPTIONAL,

-- R1 45-4a: MAC-CE activated DL/UL LTM TCI states

ltm-MAC-CE-SeparateTCI-r18                                      SEQUENCE {

qcl-Resource-r18                                                ENUMERATED {ssb, trs, both},

maxNumberDL-TCI-PerCell-r18                                     INTEGER (1..8),

maxNumberUL-TCI-PerCell-r18                                     INTEGER (1..8),

maxNumberDL-TCI-AcrossCells-r18                                 INTEGER (1..32),

maxNumberUL-TCI-AcrossCells-r18                                 INTEGER (1..32)

} 																									OPTIONAL,

]]

---End of TP ---

- TP 2 option2

Dummy only the changed components of the capabilities and add the new IEs with updated components.

---Start of TP ---

BandNR ::=                          SEQUENCE {

<Skip unchanged IEs>

[[

<Skip unchanged IEs>

-- R1 45-3a: MAC-CE activated joint LTM TCI states

ltm-MAC-CE-JointTCI-r18                                         SEQUENCE {

qcl-Resource-r18                                                ENUMERATED {ssb, trs, both},

maxNumberJointTCI-PerCell-r18                                INTEGER (1..16),

dummy1                               							ENUMERATED {n1,n2,n3,n4,n8,n16,n32}

}                                                                                                                          OPTIONAL,

<Skip unchanged IEs>

-- R1 45-4a: MAC-CE activated DL/UL LTM TCI states

ltm-MAC-CE-SeparateTCI-r18                                      SEQUENCE {

qcl-Resource-r18                                                ENUMERATED {ssb, trs, both},

maxNumberDL-TCI-PerCell-r18                                     INTEGER (1..8),

maxNumberUL-TCI-PerCell-r18                                     INTEGER (1..8),

dummy2                                 ENUMERATED {n1,n2,n4,n8,n16},

dummy3                                 ENUMERATED {n1,n2,n4,n8,n16}

}

<Skip unchanged IEs>

]]                                                                                                                        OPTIONAL,

[[

-- R1 45-3a: MAC-CE activated joint LTM TCI states

ltm-MAC-CE-JointTCI-v18xx                                         SEQUENCE {

maxNumberJointTCI-AcrossCells-r18                               INTEGER (1..32)

}                                                                                                                          OPTIONAL,

-- R1 45-4a: MAC-CE activated DL/UL LTM TCI states

ltm-MAC-CE-SeparateTCI-v18xx                                     SEQUENCE {

maxNumberDL-TCI-AcrossCells-r18                               INTEGER (1..32)

maxNumberUL-TCI-AcrossCells-r18                               INTEGER (1..32)

}                                                                                                                          OPTIONAL,

]]

---End of TP ---

Proposal 2: RAN2 to discuss the two options for the changes to TS 38.331 in TP2.

The LS from RAN1

In the LS R1-2409045, RAN1 provides the following agreement:

Agreement: - Send a reply LS to RAN2 to inform the following RAN1’s understanding. - FG45-3a/4a, 45-5/5a/6 do not need to be indicated separately for intra and inter-frequency LTM cell switch. - If UE supports ltm-InterFreq-r18, the support of above FGs implies the support for inter-frequency scenarios in addition to intra-frequency scenarios. If UE only supports ltm-MCG-IntraFreq-r18 or ltm-SCG-IntraFreq-r18, the support of above FGs implies the support for intra-frequency scenarios. Same understanding is applicable to FG45-3/4 as well

From LS, the following UE capabilities for early synchronization do not need to be indicate separately for intra/inter differences, as ltm-InterFreq-r18 can cover the support of these capabilities in inter-frequency scenarios:

- ltm-MAC-CE-JointTCI-r18(FG 45-3a)

- ltm-MAC-CE-SeparateTCI-r18(FG 45-4a)

- rach-EarlyTA-Measurement-r18(FG 45-5)

- rach-EarlyTA-BandList-r18 (FG 45-5a)

- ue-TA-Measurement-r18(FG 45-6)

In addition to the features asked by RAN2, RAN1 also noted that the same understanding applies to the following capabilities.

- ltm-BeamIndicationJointTCI-r18 (FG45-3)

- ltm-BeamIndicationSeparateTCI-r18 (FG45-4)

For rach-EarlyTA-BandList-r18, it indicates the support of the band pair for simultaneous transmission to handle the overlap between UL transmission on serving cell(s) and PRACH on candidate cell(s). The granularity is already per FS and the supported band pair is explicitly indicated by the feature set. It should not be impacted by ltm-InterFreq-r18.

For the rest of these capabilities, clarifications needed to be updated in the description in TS38.306. Due to the extensive changes, we have placed them in the appendix 5.2.

Proposal 3: RAN2 is asked to adopt the changes to TS 38.306 in TP3 in the appendix.

Conclusion

In this contribution, the following observations and proposals are made:

Proposal 1: RAN2 is asked to adopt the changes to TS 38.306 in TP1.

Proposal 2: RAN2 to discuss the two options for the changes to TS 38.331 in TP2.

Proposal 3: RAN2 is asked to adopt the changes to TS 38.306 in TP3 in the appendix.

References

- R1-2409043 Updated RAN1 UE features list for Rel-18 NR after RAN1#118bis

- R1-2409045 LS on RAN1 UE features list for Rel-18 NR after RAN1_118bis

- R2_127b_R18 SL_MOB_R19 NES_MOB_Session Notes_EoM

- R2-2408258  Discussion and corrections on RRC and UE capability for LTM	MediaTek Inc.	discussion	Rel-18	NR_Mob_enh2-Core

- 3GPP TS 38.306 V18.3.0

- 3GPP TS 38.331 V18.3.0

Appendix

Updated RAN1 UE features list for Rel-18 NR after RAN1#118bis

| 45. NR_Mob_enh2 | 45-3a | MAC-CE activated joint LTM TCI states | 1. Supported QCL source RS for MAC-CE activated joint LTM TCI states 2. Maximum number of MAC-CE activated joint LTM TCI states per candidate cell 3. Maximum number of MAC-CE activated joint LTM TCI states across candidate cells and serving cell TCI states across serving cells in the band | 45-3 | Yes | No | UE does not support MAC-CE activated joint LTM TCI states | Per band | No | No | n/a | Component 1 candidate values: {SSB, TRS, both} Component 2 candidate values for K: {1,2,3,4,…,15,16} Component 3 candidate values: {1,2,3,4,…30,31,32} Note: The maximum number of MAC-CE activated joint TCI states across all servings cells is limited by component 5 in FG 23-1-1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5. NR_Mob_enh2 | 45-4a | MAC-CE activated DL/UL LTM TCI states | 1. Supported QCL source RS for MAC-CE activated DL/UL LTM TCI states 2. Maximum number K1 of MAC-CE activated DL TCI states per candidate cell 3. Maximum number K2 of MAC-CE activated UL TCI states per candidate cell 4. Maximum number of MAC-CE activated LTM DL TCI states across all candidate cells and serving cell DL TCI across all serving cells 5. Maximum number of MAC-CE activated LTM UL TCI states across all candidate cells and serving cell UL TCI across all serving cells in the band | 45-4 | Yes | No | UE does not support MAC-CE activated DL/UL TCI states | Per band | No | No | n/a | Component 1 candidate values: {SSB, TRS, both} Component 2 candidate values: {1, 2,3,4,5,6,7,8} Component 3 candidate values: {1, 2,3,4,5,6,7,8} Component 4 candidate values: {1,2,3,…,30,31,32} Component 5 candidate values: {1,2,3,…,30,31,32} Note: The maximum number of MAC-CE activated DL/UL TCI states across all servings cells is limited by component 7 and 8 in FG 23-10-1 | Optional with capability signalling |

TP3 for TS 38.306

---Start of TP ---

| ltm-BeamIndicationJointTCI-r18 Indicates whether the UE supports unified TCI with joint DL/UL LTM TCI-state indication for LTM procedure, indicating and activating a single joint LTM TCI state in a cell switch command. This capability comprises the following parameters: - maxNumberJointTCI-PerCell-r18 indicates the maximum number of configured joint LTM TCI state(s) per candidate cell - qcl-Resource-r18 indicates of the supported QCL source RS in the LTM TCI-state- configuration. - maxNumberJointTCI-AcrossCells-r18 indicates index N of the maximum number of configured separate DL LTM TCI state(s) across candidate cells. The maximum number of configured joint LTM TCI state(s) across candidate cells is N*8, where N={1..128}. - maxNumberCells-r18 indicates the maximum number of configured cells for joint LTM TCI state(s). A UE supporting this feature shall also indicate support of unifiedJointTCI-r17 and at least one of ltm-MCG-IntraFreq-r18 or ltm-SCG-IntraFreq-r18. If the UE supports this feature and also ltm-InterFreq-r18, it implies the support of this feature is also applicable in the inter-frequency scenarios in addition to intra-frequency scenarios. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

| ltm-BeamIndicationSeparateTCI-r18 Indicates whether the UE supports unified TCI with separate DL/UL TCI-state indication for LTM procedure and indicating/activating a pair of UL/DL TCI-state in a cell switch command. This capability comprises the following parameters: - maxNumberDL-TCI-PerCell-r18 indicates the maximum number of configured DL TCI state(s) per candidate cell. - maxNumberUL-TCI-PerCell-r18 indicates the maximum number of configured UL TCI state(s) per candidate cell. - qcl-Resource-r18 indicates the supported QCL source RS in the LTM TCI-state configuration. - maxNumberDL-TCI-AcrossCells-r18 indicates value N of the maximum number of configured separate DL LTM TCI state(s) across candidate cells. The maximum number of configured separate DL LTM TCI state(s) across candidate cells is N*8, where N={1..128}. - maxNumberUL-TCI-AcrossCells-r18 indicates value N of the maximum number of configured separate UL LTM TCI state(s) across candidate cells. The maximum number of configured separate UL LTM TCI state(s) across candidate cells is N*8, where N={1..64}. - maxNumberCells-r18indicates the maximum number of configured cells for separate DL/UL LTM TCI states A UE supporting this feature shall also indicate support of unifiedSeparateTCI-r17 and at least one of ltm-MCG-IntraFreq-r18 or ltm-SCG-IntraFreq-r18. If the UE supports this feature and also ltm-InterFreq-r18, it implies the support of this feature is also applicable in the inter-frequency scenarios in addition to intra-frequency scenarios. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

| ltm-MAC-CE-JointTCI-r18 Indicates whether the UE supports MAC-CE activated joint LTM TCI states. This capability comprises the following parameters: - qcl-Resource-r18 indicates the supported QCL source RS for MAC-CE activated DL/UL LTM TCI states configuration. - maxNumberJointTCI-PerCell-r18 indicates the maximum number of MAC-CE activated joint LTM TCI states per candidate cell - maxNumberJointTCI-AcrossCells-r18 indicates the maximum number of MAC-CE activated joint LTM TCI states across candidate cells and serving cells A UE supporting this feature shall also indicate support of ltm-BeamIndicationJointTCI-r18. If the UE supports this feature and also ltm-InterFreq-r18, it implies the support of this feature is also applicable in the inter-frequency scenarios in addition to intra-frequency scenarios. NOTE: The maximum number of MAC-CE activated joint TCI states across all servings cells is limited by of unifiedJointTCI-r17. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

| ltm-MAC-CE-SeparateTCI-r18 Indicates whether the UE supports MAC-CE activated DL/UL LTM TCI states. This capability comprises the following parameters: - qcl-Resource-r18 indicates the supported QCL source RS for MAC-CE activated DL/UL LTM TCI states configuration. - maxNumberDL-TCI-PerCell-r18 indicates the maximum number of MAC-CE activated DL TCI states per candidate cell - maxNumberUL-TCI-PerCell-r18 indicates the maximum number of MAC-CE activated UL TCI states per candidate cell. - maxNumberDL-TCI-AcrossCells-r18 indicates the maximum number of MAC-CE activated DL TCI states across all candidate cells and serving cells - maxNumberUL-TCI-AcrossCells-r18 indicates the maximum number of MAC-CE activated UL TCI states across all candidate cells and serving cells A UE supporting this feature shall also indicate support of ltm-BeamIndicationSeparateTCI-r18. If the UE supports this feature and also ltm-InterFreq-r18, it implies the support of this feature is also applicable in the inter-frequency scenarios in addition to intra-frequency scenarios. The maximum number of MAC-CE activated DL/UL TCI states across all servings cells is limited by unifiedSeparateTCI-r17. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

| rach-EarlyTA-Measurement-r18 Indicates the maximum number of candidate cells for TA acquisition based on PDCCH ordered CFRA procedure before receiving cell switch command MAC-CE. Power ramping for PRACH retransmission based on PDCCH order indication. UE also supports dropping the serving cell UL to handle the overlap between UL transmission on serving cell(s) and PRACH on candidate cell(s). A UE supporting this feature shall also indicate support of ta-IndicationCellSwitch-r18 and at least one of ltm-MCG-IntraFreq-r18 or ltm-SCG-IntraFreq-r18. If the UE supports this feature and also ltm-InterFreq-r18, it implies the support of this feature is also applicable in the inter-frequency scenarios in addition to intra-frequency scenarios. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

| ue-TA-Measurement-r18 Indicates whether the UE supports UE-based TA measurement by indicating the maximum number of candidate cells that the UE maintains the TA for. A UE supporting this feature shall also indicate the support of at least one of ltm-MCG-IntraFreq-r18 or ltm-SCG-IntraFreq-r18. If the UE supports this feature and also ltm-InterFreq-r18, it implies the support of this feature is also applicable in the inter-frequency scenarios in addition to intra-frequency scenarios. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

---End of TP ---