3GPP TSG-RAN WG2 #128 	R2-2410152

Orlando, USA, 18th – 22nd November 2024

Agenda Item:	8.2.4

Source:	Ericsson

Title:	UL multiple access for Ambient IoT

Document for:	Discussion/Decision

# 1	Introduction

Random access (RA) is one model of UL multiple access that needs to be studied for Ambient IoT (A-IoT). RAN2 has agreed to study 4-step like RA and 2-step like RA. In addition, contention-based access procedure and contention-free access procedure are also to be discussed. We further discuss details for different procedures in this paper.

# 2	Discussion

## 2.1	RAN2 agreements

In RAN2#127bis, the following agreements were made related to access procedure in random access AI 8.2.5.

Agreements

- In case of D2R data transmission failure, device follows the reader instruction:

	it is supported to re-access in another opportunity controlled/provided by the reader (i.e. retry the random access); and

	reader can repeat the R2D “upper layer command” to trigger the device to re-send the same D2R “response” (i.e., device just follows the received R2D to transmit D2R).

2	Support optional explicit R2D failure/success feedback indication for at least MSG3 for re-access purpose.  FFS for following D2R data.

3	For 2step CBRA, if mgs2 is not received by the device, the device is not expected to autonomously re-access.  The re-access is always controlled by reader.

4	For 2step and 3step CBRA, The A-IoT device performs re-access in another opportunity provided by the reader (i.e. retry the random access) at least in case of contention resolution failure.

5	R2D message is used to provide access occasion(s) which can be used for re-access purpose. One option to be captured in the TR is that this R2D message is AIoT paging message.   FFS additional indications to differentiate if needed in paging message.    FFS if other message can be used.

6	From RAN2 perspective, at least for TDMA the device can randomly select one access occasion for A-IoT Msg1 within the access occasions provided/assigned by the reader, as the baseline for CBRA.  FFS if this is applicable to FDMA.  Further enhancement option(s) can be considered after more RAN1 progress on TDMA/FDMA.

In addition, we use following terms.

CR ID: Contention resolution ID

AS ID: Access Stratum ID

## 2.2	Paging and Access Round

The random-access procedure for inventory or command use case is initiated by a paging message. During this random access, there is probability that some devices may fail to access, and thus re-access is needed. Currently, there are couple of options where re-access can be trigger by a paging message or an access trigger message. But before getting into details regarding these two options, let us define following terminologies.

Access occasion: It is defined as resource opportunity for transmitting Msg1 and possibly successive messages if reader successfully echoes back the Msg1 to the device in the same occasion.

Access round: A round consists of N random-access occasions allocated by the reader to group of devices where a device selects one of the occasions randomly for its access.

Paging round: Depending on the option, a paging round can be defined as an equivalent to single access round or consisting of multiple access rounds.

The transmissions in contention based random-access procedure may be prone to collisions when there is high number of devices contending the channel. In addition, in general, the channel error probability for A-IoT transmissions would be higher than of typical NR transmission due to device’s limited capabilities and most likely its inability to do channel measurements. As a consequence, A-IoT transmissions can be unreliable and thus, a suitable feedback mechanism can be useful to indicate device’s UL message success or failure. However, the feedback has a cost on device energy and processing resources which are already scarce and limited.

Subject to feedback design, reliability requirements, there can be multiple ways re-access for the failed devices can be done. For instance, re-access can be done in same or next inventory/access round, therefore, first, it is important if we agree on terminologies related to paging, access, inventory round and the interactions which RAN2 has not defined properly. See below figure which depicts an interaction between paging message, access round, occasions, etc.

Figure 1: A paging message triggers an access round composed of N occasions serving group of devices. Each occasion can be used for CBRA based access. If the paging targets a single device and only single occasion is allocated in the access round, then the allocation behaviour reduces to CFRA.

An access round can be defined as a time period over which devices can randomly select resources for its UL transmissions from a resource pool in a contention based manner.

The access round with more than one occasion is allocated to serve group of devices in a CBRA manner.

The access round with one occasion is allocated to serve single device in a CFRA manner.

During the RACH email discussion i.e., [POST127][033][AIoT] Random Access (Huawei), there are on-going discussions on the terminologies, i.e., the need and definition of terminologies including access round, paging round. In our view, it is necessary for RAN2 to first discuss and agree on these terminologies.

The are two options discussing the interaction between paging and access round.

Option A: Paging round is equivalent to access round.

In this option, the access round is same as paging round. The paging message triggers the paging/access round consisting of N random-access occasions. In order to do re-access of failed devices, the reader can send new message to perform re-access with new resource allocation, e.g., more or less random-access occasions, updated MCS, etc. Below figure portrays a paging/access round consisting of N random-access occasions.

Figure 2: A paging round is equivalent to an access round.

Option B: In this option, a paging round consists of multiple access rounds. Apart from 1st round, the remaining access rounds are primarily of re-access for the devices which unable to access in first round within a paging round. Given, multiple rounds are assumed within a paging round, essentially there can be two trigger message types:

- Paging trigger: Initiates first round or initial access round.

- Access trigger: Initiates successive rounds within a paging round. It differs from paging trigger as it may have reduced information. For instance, access trigger may not have information pertinent to transaction ID or devices IDs, etc. depending on what scenarios we foresee and define this trigger message accordingly with reduced overhead. If access trigger is defined like paging trigger, then this option reduces to Option A.

Figure 3: A paging round consists of multiple access rounds.

In the below table, we capture pros and cons of both options.

Table 1

Pros and cons of both options with paging round consisting of single or multiple access rounds.

|  | Option A: Paging round equal to access round | Option B: Paging round consists of multiple access rounds |
| --- | --- | --- |
| Benefits | - Reader and device complexity is lower as there is single message type is needed (i.e., paging trigger). - The device which has missed initial paging message (unable to decode due to channel error or lack of energy) may not particpate in correponding access round, however, it can particpate in next round if network delivers paging message again for re-access or new access for devices for the same request (e.g., same transaction ID). | - Signaling overhead may be lower slightly when it comes to re-acess given the device will be sent access trigger message which may contain only relevant or reduced infiormation, say updated resource allocation, such as new number of occasions in the round. - Given, the acces trigger may have relatively smaller size, the energy cost for processing such message will be lower. |
| Drawbacks | - Signaling overhead may increase when it comes to re-acess given the device will be sent paging message containing full or redundant information (as the failed user may already know some of the information when it had attempted failed initial access in previous round. - In contrast to acces trigger in Option B, the failed device may need to decode whole paging message everytime for re-access, and thus may have slightly increased energy cost. | - Reader and device complexity will be slightly higher as there will be multiple messages or triggers, for instance, paging trigger, access trigger, etc. - The access trigger messge may have reduced information which means both reader and device (with failed attempt) needs to remember paging message/trigger which adds cost to memory or buffering reuirement of old/paging information which is extremely critical for devices of Type 1 or passive nature. - The device which has missed initial paging message (unable to decode due to channel error or lack of energy) may not particpate in correponding access round, however, it may happen it still may not able particpate in next round initiated by access trigger if the message is defined in such as manner that some of the information in paging messag is excluded from access trigger message. In this round, only device can particpate which have successfully decoded paging message inittially but failed with uplink access, e.g., failure in Msg1 transmission or Msg3 transmission. |

RAN2 to capture both options in TR with paging round consists of single or multiple access rounds along with their pros and cons as the below text

  - Option 1 - one paging round is equivalent to one access round

    - the reader and device complexity is lower as there will be single format for round trigger, i.e., paging message

    - if a device has missed a paging round, it can have re-access opportunity in the next paging round subject to reader implementation without the device requiring to retain any information from any previous round.

  - Option 2 – one paging round is equivalent to multiple access rounds

    - Multiple formats of round trigger messages are defined (i.e., paging and access trigger) where access trigger may have lower overhead with some of the redundant information removed compared with paging message.

    - energy or processing cost at the device side may be lower associated with monitoring and decoding of access trigger due to its relative lower overhead.

  - FFS on whether both or either access round and paging round will be adopted. Stick to paging round if only one is adopted.

Adopt the TP on access occasion, access round and paging round options in clause 5.1.

Further, when a reader triggers an access for failed devices corresponding to a given service, there is possibility that the reader can initiate or trigger an access round for the devices associated one or more service requests. RAN2 needs to enquire SA2 on whether this can be supported.

RAN2 needs inputs from SA2 on whether the reader can serve devices in the same access round associated with multiple service requests.

## 2.3	AS Feedback for subsequent D2R data

RAN2 has made the below agreement for Msg3 feedback in AS.

- Support optional explicit R2D failure/success feedback indication for at least MSG3 for re-access purpose.  FFS for following D2R data.

Regarding the FFS for following D2R data, we see no reason to treat D2R data differently from Msg3. RAN2 shall aim for a unified solution for both Msg3 and subsequent D2R data if any. This can simplify design efforts for RAN2.

It is beneficial to simplify design efforts for RAN2 to adopt a unified solution for both Msg3 and subsequent D2R data regarding AS feedback.

Therefore, we make the below proposal.

Same as Msg3, support optional explicit R2D failure/success feedback indication for following D2R data after Msg3.

## 2.4	Re-access

In last meeting, it was agreed that R2D message is used to provide access occasion(s) which can be used for re-access purpose, where one option to be captured in the TR is that this R2D message is AIoT paging message.

In addition to that, we believe, another R2D message is Msg2 which can provide re-access for failed devices. The re-access can occur in the same occasion (which can be termed as Msg1 retransmission) or in the remaning occasions of the same access round with a possbility of including some back-off occasions based time window. Both of these options have pros and cons, and have applicability in specific scenarios. The reransmission of Msg1 indicated by Msg2 is more practical if Msg1 failure is due to channel errors. The network can reserve some codewords for RN16 (if needed) which can be signalled in Msg2 as form of explicit means to indicate Msg1 retransmission. The other solution can be via implicit means, where the absence of device’s chosen RN16 in Msg2 in an indicative of Msg1 retransmission. However, certain rules are needed to differentiate Msg1 retransmission and Msg3 retransmission using Msg2 as a trigger.

The retransmission opportunity for device with failed Msg1 transmission in the same access occasion is suitable for the device with Msg1 transmission failure caused by channel error.

The re-access opportunity for device with failed Msg1 transmission in one of remaining access occasion in the same access round is suitable for the device with Msg1 transmission failure caused by collision error.

In addition to A-IoT paging, RAN2 to agree that Msg2 is used to provide access occasion(s) which can be used for re-access purpose. The access occasion can be the same where Msg1 transmission has failed, or separate occasion belonging to the same access round.

On a question whether the rest competing devices also need to be signalled with an explicit acknowledgement message, we think this is not needed, since the acknowledgement message to the winners have already indicated that other competing devices (with different temporary IDs) fail to their accesses. However, in the same message, additional information can be indicated for the failed devices.

As there can be more than one competing device, where one or more devices have won the contention (depending on resource usage policies) which can be indicated in Msg2, and additionally address the devices which have failed the contention without any explicit addressing. The reasons gNB cannot identify some devices can be numerous. For example, collisions or it can be as well due to channel failure, and gNB able correctly identify the device resulting in failing to positively acknowledge the device. Hence, in such scenarios, the gNB can respond with information in Msg2 that can help device re-access the inventory round. For instance, the Msg2 can target unidentified failed devices with backoff indicator (BI) e.g., information related to allowed access in same or different or adjusted round, see following proposals discussion, or exercise halting of immediate re-access for probable failed devices in order to decrease collision probability, etc.

For devices with unsuccessful random access, the response message (Msg2) can be considered to indicate additional information related to back-off and re-access parameters.

## FDMA and Msg2

RAN2 has agreed to support random selection for TDMA based UL multiple access, and FFS on FDMA. In addition, RAN1 has made further progress on FDMA at RAN1#118bis. Specifically for large frequency shift, RAN1 has made the below agreement

Agreement

TR captures following descriptions for large frequency shift for device 2a.

- Large frequency shift may be technically feasible in terms of power consumption, and device complexity.

- Remaining technical feasibility of large frequency shift in terms of image suppression, harmonics suppression may be relevant to device emission requirement, spectrum usage. Additional complexity and power consumption may be needed to achieve image suppression and harmonics suppression.

- The necessity or effectiveness of large frequency shift may depend on spectrum for R2D/CW2D/D2R, full duplex capability of reader, whether device 1 and 2a are supported by a single reader, spectrum regulation/usage, frequency error, coverage, etc.

The below observations can be made

- It is feasible to support small frequency shift at least for device type 2a and 2b

- The feasibility of large frequency shift needs further studies.

It is clearly that RAN2 would need further RAN1 progress on FDMA in order to discuss the below issues

Issue 1: how to select FDMA access occasions in UL?

For issue 1, it may be infeasible to always perform random selection among FDM access occasions, especially when there are multiple frequency domain access occasions associated with different frequency shift ranges. RAN2 has wait for RAN1 further progress on the feasibility of supporting large frequency shifts by devices.

Issue 2: how to echo Msg1 and allocate resources for Msg3 via Msg2?

This creates an issue how to address Msg2 or provide feedback to these multiple devices with Msg1 transmission in a FDMA manner.

One solution can be based on RAR functionality where Msg2 can be addressed to multiple devices with their RN16s as their sub-headers and corresponding resource allocation information for their Msg3 and Msg4 transmissions as MAC sub-payloads. The major issue is the message size would increase as it would include all the successfully transmitted RN16s which a device may not be interested in the message.

Other solution can be based on L1 mapping where a preceding DL control information is sent before individual Msg2 transmissions. Each Msg2 is addressed to single device and the preceding DL control information provides mapping information of Msg2 resource allocation, see below figure. The individual message size is decreased compare RAR inspired option, however, there is a need of additional DL control information prior to Msg2 transmissions which provides the resource mapping.

Capture below RAN2 issues which are depending on RAN1 further progress on FDMA in the TR

  - Issue 1: how to select FDMA access occasions in UL.

  - Issue 2: how to echo Msg1 and allocate resources for Msg3 via Msg2.

Regarding 2step CBRA, RAN2 has made the below agreement on Msg2.

For 2step CBRA, if mgs2 is not received by the device, the device is not expected to autonomously re-access.  The re-access is always controlled by reader

We think it is necessary for RAN2 to further clarify this agreement, due to below aspects

1) This agreement doesn’t resolve the issue on whether Msg2 is mandatory or optional

2) Device behaviours if Msg2 is absent is still unclear.

At least for 2), we think RAN2 needs to further clarify the device behaviours. We see there may be two possible options for defining device behaviours.

Option 1: the device interprets that its Msg1 transmission was successful if Msg2 is absent.

Option 2: the device waits for further messages from the reader indicating whether Msg1 was successful or failed.

To discuss device behaviours if Msg2 is absent in case of 2-step CBRA and capture in the TR.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	An access round can be defined as a time period over which devices can randomly select resources for its UL transmissions from a resource pool in a contention based manner.

Observation 2	The access round with more than one occasion is allocated to serve group of devices in a CBRA manner.

Observation 3	The access round with one occasion is allocated to serve single device in a CFRA manner.

Observation 4	It is beneficial to simplify design efforts for RAN2 to adopt a unified solution for both Msg3 and subsequent D2R data regarding AS feedback.

Observation 5	The retransmission opportunity for device with failed Msg1 transmission in the same access occasion is suitable for the device with Msg1 transmission failure caused by channel error.

Observation 6	The re-access opportunity for device with failed Msg1 transmission in one of remaining access occasion in the same access round is suitable for the device with Msg1 transmission failure caused by collision error.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to capture both options in TR with paging round consists of single or multiple access rounds along with their pros and cons as the below text

a.	Option 1 - one paging round is equivalent to one access round

i.	the reader and device complexity is lower as there will be single format for round trigger, i.e., paging message

ii.	if a device has missed a paging round, it can have re-access opportunity in the next paging round subject to reader implementation without the device requiring to retain any information from any previous round.

b.	Option 2 – one paging round is equivalent to multiple access rounds

i.	Multiple formats of round trigger messages are defined (i.e., paging and access trigger) where access trigger may have lower overhead with some of the redundant information removed compared with paging message.

ii.	energy or processing cost at the device side may be lower associated with monitoring and decoding of access trigger due to its relative lower overhead.

c.	FFS on whether both or either access round and paging round will be adopted. Stick to paging round if only one is adopted.

Proposal 2	Adopt the TP on access occasion, access round and paging round options in clause 5.1.

Proposal 3	RAN2 needs inputs from SA2 on whether the reader can serve devices in the same access round associated with multiple service requests.

Proposal 4	Same as Msg3, support optional explicit R2D failure/success feedback indication for following D2R data after Msg3.

Proposal 5	In addition to A-IoT paging, RAN2 to agree that Msg2 is used to provide access occasion(s) which can be used for re-access purpose. The access occasion can be the same where Msg1 transmission has failed, or separate occasion belonging to the same access round.

Proposal 6	For devices with unsuccessful random access, the response message (Msg2) can be considered to indicate additional information related to back-off and re-access parameters.

Proposal 7	Capture below RAN2 issues which are depending on RAN1 further progress on FDMA in the TR

a.	Issue 1: how to select FDMA access occasions in UL.

b.	Issue 2: how to echo Msg1 and allocate resources for Msg3 via Msg2.

Proposal 8	To discuss device behaviours if Msg2 is absent in case of 2-step CBRA and capture in the TR.

# 4	References

RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, CMCC, Huawei, T-Mobile USA, Maastricht, Netherlands, August 8-19, 2024

R2-2409812, “TP for TR 38.769 update”, Huawei, CMCC, T-Mobile USA, RAN2#128.

5	Text Proposal

In this clause, the proposed texts are captured in yellow based on the TP [2].

5.1 TP on paging, access round and access occasion

### 6.3.4	A-IoT random access procedure

A-IoT random access procedure is used for the Ambient IoT device(s) to access the network for data transmission.

The A-IoT random access procedure is triggered by the reader, including triggering the access for a single A-IoT device, group of A-IoT devices, or all A-IoT devices under the coverage of the reader.

The slotted-ALOHA is the baseline for A-IoT random access procedure.

Figure 6.3.4-1 General framework of slotted-ALOHA for A-IoT random access procedure

Access occasion: An opportunity of time-frequency resource for A-IoT device(s) to perform access (e.g., transmitting the A-IoT Msg1 by the device). A set of access occasion(s) for different A-IoT device(s) is scheduled via the R2D message (referring to the “R2D transmission triggering random access” in sub-clause 6.1.4) by the reader. The subsequent messages, such as Msg2, Msg3, etc. are assumed to be part of same access occasion where Msg1 is successfully transmitted.

Access round: A round consists of set of access occasions allocated by the reader to devices where a device selects one of the occasions randomly for its access from an access round.

Paging round: A paging round can be considered equivalent to single access round triggered by a paging trigger message, or consisting of multiple access rounds where first round triggered by paging trigger message and subsequent round(s) triggered by an access trigger message. Both options have pros which are stated as following.

As the first option, i.e., one paging round is equivalent to one access round, the reader and device complexity is lower as there will be single defined trigger message, i.e., paging message. Further, if a device has missed a paging round for its initial transmission, it can have re-access opportunity in the next paging round subject to reader implementation without the device requiring to retain any information from any previous round.

As the second option, i.e., one paging round equivalent to multiple access rounds, an additional format of round trigger message is required compared to the first option, which may have lower overhead with some of the redundant information removed compared to the paging message. The energy or processing cost at the device side may be lower associated with monitoring and decoding of this additional round trigger due to its relative lower overhead.

When the A-IoT device is selected to respond in accordance to the clause 6.3.3, the A-IoT device performs the following procedure:

-	Step 1: Random access type (i.e., contention-free or contention-based) and access occasion/resource determination:

-	The A-IoT device determines the random access type from the A-IoT paging message, in accordance to clause 6.3.3. The reader can configure either contention-free access or contention-based random access (and the corresponding configurations). It can be further discussed on whether it is explicitly or implicitly.

-	If the random access is contention-free access:

-	Selects the indicated D2R occasion/resource;

-	Skips the contention resolution in Step 2 and performs the data transmission in accordance to clause 6.3.5.

-	If the random access is contention-based random access:

-	Performs access occasion/resource selection: as the baseline for CBRA, at least for TDMA case, the device can randomly select one access occasion for A-IoT Msg1 within the access occasions provided/assigned by the reader. It can be further discussed if this is applicable to FDMA case. Further enhancement option(s) can be also considered after more physical layer detailed design on TDMA and FDMA;

-	Performs the Step 2 for contention resolution.

-	Step 2: Contention resolution of contention-based random access:

-	There are two candidate solutions being studied for the contention resolution, as below (it can be further discussed on the down selection and/or the unified design):

-	Solution 1: A-IoT Msg1 without data

-	A-IoT Msg1: When the A-IoT device identifies the start of its own access occasion, it sends one 16-bit random ID generated by the A-IoT device to the reader.

-	A-IoT Msg2: The reader responds with the successfully received random ID.

If the A-IoT device receives the A-IoT Msg2 including a random ID, which is the same as the previously transmitted one in A-IoT Msg1, it considers the contention resolution as successful.

-	Solution 2: A-IoT Msg1 with data

-	A-IoT Msg1: When the A-IoT device identifies the start of its own access occasion, it sends the A-IoT Msg1 including the upper layer data, which can be the device ID and/or any other upper layer data, in addition to one 16-bit random ID generated by the A-IoT device to the reader.

-	A-IoT Msg2: The reader may respond with the successfully received random ID.

If the A-IoT device receives the A-IoT Msg2 including a random ID, which is the same as the previously transmitted one in A-IoT Msg1, it considers the contention resolution as successful. If the A-IoT Msg2 is not received by the device, the re-access is not autonomously performed while the re-access is always controlled by the reader.

NOTE 1:	The random ID is randomly generated by the A-IoT device.

NOTE 2:	The A-IoT Msg2 is used for contention resolution, since it is assumed that the size of random ID in A-IoT Msg1 should be sufficient for contention resolution purpose. It is assumed that, with the sufficient range for random ID, the probability of multiple A-IoT devices selecting the same access occasion and the same random ID for A-IoT Msg1 transmission will be sufficiently low.

NOTE 3:	It is up to the reader implementation whether A-IoT Msg2 is sent in random access Solution 2.

NOTE 4:	Further information may be included in A-IoT Msg2 by considering the discussion in sub-clause 6.1.

After the A-IoT device considers the contention resolution as successful if the contention-based random access is used, or if the contention-free access is used, it may perform the upper layer data transmission with the reader, which can be the device ID and/or any other upper layer data, if any, in accordance to clause 6.3.5.

It is supported for the A-IoT device to re-access in another opportunity controlled/provided by the reader (i.e., to retry the random access above), in case of D2R data transmission failure and contention resolution failure of contention-based random access.

The A-IoT device is not expected to autonomously re-access. The re-access is always controlled by reader. It is supported for reader to use the optional explicit R2D failure/success feedback indication to determine the re-access of device:

-	This indication can be used at least to determine the re-access for addressing the transmisison failure of the first D2R message, which contains the device ID and/or any other upper layer data (i.e. Msg3);

-	It can be further discussed on whether this indication can be used for the following D2R data, to determine the re-access for addressing the transmisison failure.

The R2D message is used by the reader to provide access occasion(s), which can be used for re-access purpose. A-IoT paging message is one of the options for this R2D message (e.g., see the subsequent A-IoT paging in Figure 6.3.4-1). It can be further discussed if other R2D message can be used (e.g., some R2D message between A-IoT paging). It can be further discussed if additional information is needed in this R2D message to differentiate the re-access purpose.

Editor’ s Note x: FFS on whether both or either of access round and paging round will be adopted. Stick to paging round if only one is adopted.

Editor’ s Note xx: FFS on how to select access occasions for FDM based UL multiple access.

Editor’ s Note xxx: FFS on how to echo Msg1 and allocate resources for Msg3 via Msg2 for FDM based UL multiple access.

Editor’ s Note xxxx: FFS on device behaviours if Msg2 is absent in case of 2-step CBRA.

Editor’ s Note xxxxx: whether the reader can serve devices in the same access round associated with multiple service requests is pending on SA2 inputs.