3GPP TSG-RAN WG2 Meeting #128																					R2-2410165

Orlando, USA, Nov.18th–22nd, 2024

Agenda item:		8.5.4

Title: 	Consideration on common signal/channel transmissions

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In this paper, we share some understanding on the further details of paging adaptation option-b, and RAN2 consideration on PRACH adaptation in time domain.

- Discussion

  - Paging occasion adaptation

The following agreements have been made in the recent meeting on paging adaptation [1]:

Agreements on paging adaptation

Select option-b as baseline for R19 NES paging enhancement.

R2 should aim at signalling overhead minimization (e.g., Ns=8 and FFS for other larger values)

Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration.

Legacy UE is not barred.Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.

The ASN.1 for the current PCCH-Config is as follows:

PCCH-Config ::=             SEQUENCE {

defaultPagingCycle                  PagingCycle,

nAndPagingFrameOffset               CHOICE {

oneT                                NULL,

halfT                               INTEGER (0..1),

quarterT                            INTEGER (0..3),

oneEighthT                          INTEGER (0..7),

oneSixteenthT                       INTEGER (0..15)

},

ns                                  ENUMERATED {four, two, one},

firstPDCCH-MonitoringOccasionOfPO   CHOICE {

sCS15KHZoneT                                                                SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                                  SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                                 SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT             SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS480KHZquarterT-SCS120KHZoneSixteenthT                                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919)

}      OPTIONAL,           -- Need R

...,

[[

nrofPDCCH-MonitoringOccasionPerSSB-InPO-r16        INTEGER (2..4)               OPTIONAL  -- Cond SharedSpectrum2

]],

[[

ranPagingInIdlePO-r17                              ENUMERATED {true}            OPTIONAL,  -- Need R

firstPDCCH-MonitoringOccasionOfPO-v1710  CHOICE {

sCS480KHZoneEighthT        SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT     SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..71679)

}                                                                                  OPTIONAL  -- Need R

]]

}

We understand the following parameters will be impacted: nAndPagingFrameOffset, ns, firstPDCCH-MonitoringOccasionOfPO.

As we aim to minimize the signaling overhead, we would like to only introduce the following values:

| nAndPagingFrameOffset | oneThirtysecondT INTEGER (0..31), oneSixtyfourthT INTEGER (0..63), |
| --- | --- |
| ns | sixteen, eight |

Proposal 1: Introduce the following value for nAndPagingFrameOffset, and n:

| nAndPagingFrameOffset | oneThirtysecondT INTEGER (0..31), oneSixtyfourthT INTEGER (0..63), |
| --- | --- |
| ns | sixteen, eight |

The value range of firstPDCCH-MonitoringOccasionOfPO depends on SCS and N (number of PFs in paging cycle T):

The existing values range and potential new values with newly introduced N are summarized as follows:

| N | SCS/KHz | The maximum number of PDCCH monitoring occasions for a PF | N | SCS/KHz | The maximum number of PDCCH monitoring occasions for a PF | N | SCS/KHz | The maximum number of PDCCH monitoring occasions for a PF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T | 15 | 140 | T/2 | 15 | 280 | T/4 | 15 | 560 |
|  | 30 | 280 |  | 30 | 560 |  | 30 | 1120 |
|  | 60 | 560 |  | 60 | 1120 |  | 60 | 2240 |
|  | 120 | 1120 |  | 120 | 2240 |  | 120 | 4480 |
|  | 480 | 4480 |  | 480 | 8960 |  | 480 | 17920 |
| T/8 | 15 | 1120 | T/16 | 15 | 2240 | T/32 | 15 | 4480 |
|  | 30 | 2240 |  | 30 | 4480 |  | 30 | 8960 |
|  | 60 | 4480 |  | 60 | 8960 |  | 60 | 17920 |
|  | 120 | 8960 |  | 120 | 17920 |  | 120 | 35840 |
|  | 480 | 35840 |  | 480 | 71680 |  | 480 | 143360 |
| T/64 | 15 | 8960 | T/128 | 15 | 17920 | T/256 | 15 | 35840 |
|  | 30 | 17920 |  | 30 | 35840 |  | 30 | 71680 |
|  | 60 | 35840 |  | 60 | 71680 |  | 60 | 143360 |
|  | 120 | 71680 |  | 120 | 143360 |  | 120 | 286720 |
|  | 480 | 286720 |  | 480 | 573440 |  | 480 | 1146880 |

If we introduce N with new value T/32, the following new parameters are needed for firstPDCCH-MonitoringOccasionOfPO:

sCS15KHZoneThirtysecondT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS30KHZoneThirtysecondT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS60KHZoneThirtysecondT											SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17920),

sCS120KHZoneThirtysecondT											 SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..35839),

sCS480KHZoneThirtysecondT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..143359)

If we introduce N with new value T/32 and T/64, the following new parameters are needed for firstPDCCH-MonitoringOccasionOfPO:

sCS15KHZoneThirtysecondT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS30KHZoneThirtysecondT-SCS15KHZoneSixtyfourthT                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS60KHZoneThirtysecondT-SCS30KHZoneSixtyfourthT                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17920),

sCS120KHZoneThirtysecondT-SCS60KHZoneSixtyfourthT                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..35839),

sCS120KHZoneSixtyfourthT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..71679),

sCS480KHZoneThirtysecondT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..143359),

sCS480KHZoneSixtyfourthT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..286719)

Proposal 2: Introduce the following new values for firstPDCCH-MonitoringOccasionOfPO if N with new value T/32 and T/64 are introduced.

sCS15KHZoneThirtysecondT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS30KHZoneThirtysecondT-SCS15KHZoneSixtyfourthT                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS60KHZoneThirtysecondT-SCS30KHZoneSixtyfourthT                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17920),

sCS120KHZoneThirtysecondT-SCS60KHZoneSixtyfourthT                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..35839),

sCS120KHZoneSixtyfourthT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..71679),

sCS480KHZoneThirtysecondT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..143359),

sCS480KHZoneSixtyfourthT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..286719)

Also, with the new value 8 and 16 for ns introduced, the number of maximum PO per PF should also be introduced with new value: 16.

Proposal 3: Introduce new value, i.e. 16 for maxPO-perPF.

  - RACH adaptation in time domain

Regarding RACH adaptation in time domain, RAN1 [2] has agreed to introduce additional PRACH resources for NES-capable UEs in additional to PRACH resources for legacy UEs and use DCI to indicate the adaptation for additional PRACH resources for UE in idle/inactive and connected.

From RAN2’s perspective, we understand the following issues need to be considered:

- Issue 1: CBRA, CFRA or both?

- Issue 2: 2-step RACH, 4-step RACH or both?

- Issue 3: Whether to support RACH partitioning? Or exclude some features?

In our understanding, the additional PRACH resources is mainly used to provide more RACH resources when traffic arrives so that the transmission delay can be reduced. Thus, we understand the additional PRACH resources should offer similar functions as the baseline RACH resources so that all kinds of traffic with differentiated requirements can be supported.

Thus, we understand both CBRA and CFRA, 2-step RACH and 4-step RACH can be supported in the additional PRACH resource and it is up to network to configure.

Proposal 4a: Both CBRA and CFRA are supported for PRACH adaption in time domain.

Proposal 4b: Both 2-step RACH and 4-step RACH are supported for PRACH adaption in time domain.

Regarding the RACH partitioning, the following features are considered: RedCap, SDT, slicing and CE. We understand all these features can be supported by an NES cell thus would like to support all of them.

FeatureCombination-r17 ::= SEQUENCE {

redCap-r17                 ENUMERATED {true}                                    OPTIONAL,  -- Need R

smallData-r17              ENUMERATED {true}                                    OPTIONAL,  -- Need R

nsag-r17                   NSAG-List-r17                                        OPTIONAL,  -- Need R

msg3-Repetitions-r17       ENUMERATED {true}                                    OPTIONAL,  -- Need R

msg1-Repetitions-r18       ENUMERATED {true}                                    OPTIONAL,  -- Need R

eRedCap-r18                ENUMERATED {true}                                    OPTIONAL,  -- Need R

spare2                     ENUMERATED {true}                                    OPTIONAL,  -- Need R

spare1                     ENUMERATED {true}                                    OPTIONAL   -- Need R

}

NSAG-List-r17 ::= SEQUENCE (SIZE (1.. maxSliceInfo-r17)) OF NSAG-ID-r17

Proposal 5: RACH partitioning with all the features, i.e. RedCap, SDT, Slicing, and CE, and feature combinations, are supported for PRACH adaption in time domain.

- Conclusion and proposals

Based on the analysis in previous sections, the following proposals are given:

Proposal 1: Introduce the following value for nAndPagingFrameOffset, and n:

| nAndPagingFrameOffset | oneThirtysecondT INTEGER (0..31), oneSixtyfourthT INTEGER (0..63), |
| --- | --- |
| ns | sixteen, eight |

Proposal 2: Introduce the following new values for firstPDCCH-MonitoringOccasionOfPO if N with new value T/32 and T/64 are introduced.

sCS15KHZoneThirtysecondT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS30KHZoneThirtysecondT-SCS15KHZoneSixtyfourthT                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS60KHZoneThirtysecondT-SCS30KHZoneSixtyfourthT                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17920),

sCS120KHZoneThirtysecondT-SCS60KHZoneSixtyfourthT                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..35839),

sCS120KHZoneSixtyfourthT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..71679),

sCS480KHZoneThirtysecondT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..143359),

sCS480KHZoneSixtyfourthT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..286719)

Proposal 3: Introduce new value, i.e. 16 for maxPO-perPF.

Proposal 4a: Both CBRA and CFRA are supported for PRACH adaption in time domain.

Proposal 4b: Both 2-step RACH and 4-step RACH are supported for PRACH adaption in time domain.

Proposal 5: RACH partitioning with all the features, i.e. RedCap, SDT, Slicing, and CE, and feature combinations, are supported for PRACH adaption in time domain.

- Reference

- Draft_RAN2_127b_Meeting_Report_v3

- Draft_Minutes_report_RAN1#118b_v020