3GPP TSG RAN WG2 Meeting #128                                                          R2-2410170

# Orlando, US, November 18-22, 2024

Agenda item:		8.5.3

Source:			III

Title:		On-demand SIB1 for Idle/Inactive mode UEs

Document for:	 	Discussion

- Introduction

In the latest RAN#105, the issue regarding on-demand SIB1 for idle/inactive mode UEs was approved to enter normative phase from study phase with focus on case 2 only, whose up-to-date objectives [1] are listed as follows.

The objectives of the work item are the following: - Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

In the previous RAN2#127-bis, some relevant agreements [2] were made, including:

Agreements on OD-SIB1: 1. We will inherit all agreements made during SI phase to WI phase. 2. A NES cell can include neighbouring NES cell’s WUS configuration. 3. NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration). 4. In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1. 5. The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred. 6. The legacy UE behaviour can be reused upon on-demand SIB1 acquisition failure, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell. 7. A cell for which SIB1 request configuration is available, can periodically broadcast SIB1. 8. If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell. 9. Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced. 10. NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities). 11. RAN2 will not start the discussion on the issue when the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, unless we’re asked to do that by other WG, e.g. RAN3.

This contribution provides some input on NES cell’s WUS configuration and SIB1 checking before on-demand requests for further discussions.

- Discussion

NES cell’s WUS configuration

It is agreed that Case 2 (Option 1+B+X) is the only normative work for on-demand SIB1 in Rel-19 [1,3], that is:

- Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell

In this regard, the WUS configuration of any NES cells is normally retrieved from a Cell A. If an NES cell is to provide the WUS configuration of its neighboring NES cells, it will seemingly make no difference between a Cell A and an NES cell, which goes against the purpose of energy saving on NES cells.

Meanwhile, it is also agreed that for a Cell A which is periodically transmitting its own SIB1, the WUS configuration of any NES cells is regularly provided via a new SIB in Cell A. However, for an NES cell which may transmit SIB1 transmission in response to UL WUS from a UE, it can only do so during the period of its SIB1 transmission. Beyond that, the WUS configuration of its neighboring NES cells may not be possibly retrieved.

Moreover, if an NES cell happens to be in the coverage of more than one Cell A, the provision of neighboring NES cell’s WUS configuration will become more complicated, because extra filtering on the NES cell will be needed to determine which neighboring NES cells are located in the coverage of the same Cell A.

Proposal 1. RAN2 focuses on the provision of NES cell’s WUS configuration by Cell A, and deprioritizes that by neighboring NES cells.

SIB1 checking before on-demand requests

Checking if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell does have the benefit of reducing redundant on-demand SIB1 requests from other NES UEs. However, such checking is strongly related to the issues below:

In RAN1 #118, the following is agreed for time domain on-demand SIB1 transmission behavior: Agreement At least for Case-2: For further study of type 0 PDCCH monitoring occasions for on demand SIB1, on the starting time and duration of the time window of type 0 PDCCH monitoring occasions, RAN1 to down select from the following two options: - Option 1: starting time and duration are indicated in RAR of the UL-WUS transmission - Option 2: starting time and duration are indicated in the UL WUS configuration Agreement At least for Case-2: For further study of type 0 PDCCH monitoring occasions for on demand SIB1, on reference time point to determine the window starting time, RAN1 to down select from the following two options: - Option 1: The reference time point is defined based on the RAR reception time of the UL-WUS transmission - FFS: Definition of RAR reception time - Option 2: The reference time point is defined based on the UL-WUS transmission time - Option 3: The reference time point is defined based on the RAR window of the UL-WUS transmission Agreement For Case-2: For type 0 PDCCH monitoring occasions for on demand SIB1, on how the search space zero configuration is provided, RAN1 to down select from the following options: - Option 1: searchSpaceZero for on-demand SIB1 is provided from MIB on NES cell - Option 2: searchSpaceZero for on-demand SIB1 is provided from UL WUS configuration - Option 3: searchSpaceZero for on-demand SIB1 is provided from the RAR of UL WUS. Combination of multiple options is not precluded.

As these issues are still being discussed in RAN1 [4], and therefore RAN2 may start discussions on the SIB1 checking after some progresses have been made in RAN1.

Proposal 2. Considering the strong relationship with issues being discussed in RAN1, RAN2 may start discussions on the SIB1 checking after some progresses have been made in RAN1.

- Conclusion

In this contribution, we have the following proposals:

Proposal 1. RAN2 focuses on the provision of NES cell’s WUS configuration by Cell A, and deprioritizes that by its neighboring NES cells.

Proposal 2. Considering the strong relationship with issues being discussed in RAN1, RAN2 may start discussions on the SIB1 checking after some progresses have been made in RAN1.

References

[1] RP-242354 Revised WID: Enhancements of network energy savings for NR.

[2] Draft RAN2 127b Meeting Report v2.

[3] RP-242384 Way forward on NES checkpoint.

[4] R1-2409016 FL summary 5 for on-demand SIB1.