3GPP TSG-RAN WG2 Meeting #128	R2-2410185

Orlando, USA, Nov.  18th – 22nd, 2024

Agenda Item:	8.13.4

Source:	ASUSTeK

Title:	Discussion on measurement report for multi-hop U2N Relay

Document for:	Discussion and Decision

1	Introduction

The following intra-gNB service continuity scenarios are specified in the new WID on NR sidelink multi-hop relay [1]:

First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

[POST127][401][Relay] Multi-hop relay discovery and (re)selection [2] raises the following proposals on Relay discovery message forwarding:

Proposal 3: The discovery announcement message can be forwarded at the intermediate Relay UE when the SD-RSRP or SL-RSRP between the last/parent intermediate Relay UE and itself is above a configured threshold.

Proposal 4: The following model B discovery forwarding condition can be baseline for further discussion:

- For Model B, the intermediate Relay UE forwards the solicitation message only if the PC5 RSRP between the Remote UE (or intermediate Relay UE) and the intermediate Relay UE is above a threshold.

…

To support path switching to/from a multi-hop U2N relay path, the Remote UE may need to provide measurement report(s) of the candidate/serving relay path(s) so that the gNB can make the path switching decision.

This document discusses potential information to be included in the measurement report of a concerned relay path for multi-hop U2N Relay.

2	Discussion

2.1	Information included in the measurement report for single-hop U2N Relay

According to TS 38.300 [3], the following information may be included in the measurement report for single-hop U2N Relay: L2 U2N Relay UE's source L2 ID, serving cell ID (i.e. NCGI/NCI), and sidelink measurement quantity result (i.e. SL-RSRP/SD-RSRP).

Observation 1: The measurement report for single-hop U2N Relay may include L2 U2N Relay UE's source L2 ID, serving cell ID (i.e., NCGI/NCI), and sidelink measurement quantity result (i.e. SL-RSRP/SD-RSRP).

2.2	Whether information included in the measurement report of single-hop U2N Relay can be reused for multi-hop U2N Relay

The first issue to discuss is whether the information included in the measurement report of single-hop U2N Relay can be reused for multi-hop U2N Relay. Let’s examine one-by-one.

- L2 U2N Relay UE's Serving cell ID (i.e. NCGI):

Since NCGI is included in the Announcement message (Model A) or the Response message (Model B) sent by the Last Relay UE according to TS 23.304 [4], the NCGI is available and can be reused in the measurement report for multi-hop U2N Relay.

- L2 U2N Relay UE's source L2 ID:

In single-hop U2N Relay, the L2 U2N Relay UE's source L2 ID is used by the gNB to indicate the target U2N Relay UE for the Remote UE. In our opinion, the source L2 ID of the Last Relay UE is also needed for multi-hop U2N Relay. However, the Remote UE does not connect directly with the Last U2N Relay UE in multi-hop U2N Relay and thus the Remote UE cannot know the source L2 ID of the Last Relay UE. A way for the Remote UE to obtain the L2 U2N Relay UE's source L2 ID should be considered.

Proposal 1:  Like measurement report in single-hop U2N Relay, the Remote UE includes the serving cell ID and source L2 ID of the Last U2N Relay UE in the measurement report for supporting path switching in multi-hop U2N Relay.  FFS: how the Remote UE obtains the source L2 ID of the Last U2N Relay UE.

- Sidelink measurement quantity result:

Since there are multiple Relay UEs in multi-hop U2N Relay, there may be two potential options for the sidelink measurement quantity result:

- Option 1: Sidelink measurement quantity results of all neighboring UEs on the concerned relay path are reported

- Option 2: Only sidelink measurement quantity result between the Remote UE and the First Relay UE is reported.

As proposed in the email discussion on multi-hop relay discovery [2], the intermediate Relay UE forwards the Relay discovery message only if the PC5 RSRP between the Relay UEs is above a configured threshold. If this proposal is agreeable, the sidelink qualities between Relay UEs on the concerned relay path can be ensured. Thus, we think it should be sufficient and simpler for the Remote UE to report only the sidelink measurement quantity result between the Remote UE and the First Relay UE.

Observation 2: Since the intermediate Relay UE forwards the Relay discovery message only if the PC5 RSRP between the Relay UEs is above a configured threshold, the sidelink qualities between neighboring Relay UEs on the concerned relay path can be ensured.

Proposal 2:  The Remote UE reports only the sidelink measurement quantity result between the Remote UE and the First Relay UE for supporting multi-hop U2N Relay.

2.2	Any other potential information to be included in the measurement report for multi-hop U2N Relay

According to subclause 6.3.2.5.3 in TS 23.304 [4], the Remote UE may perform relay path selection based on e.g., the PC5 signal strength and the number of hops to the 5G ProSe UE-to-Network Relay. Basically, less hop count could induce less signalling/traffic transfer latency. Thus, we think the hop count of the concerned relay path should be useful for gNB path switching decision.

Observation 3: The hop count of the concerned relay path is useful for gNB path switching decision.

Proposal 3:  The Remote UE includes the hop count of the concerned relay path in the measurement report for supporting multi-hop U2N Relay.

3 	Conclusion

In this contribution, we raise the following observations and proposals:

Observation 1: The measurement report of single-hop U2N Relay may include L2 U2N Relay UE's source L2 ID, serving cell ID (i.e., NCGI/NCI), and sidelink measurement quantity result (i.e. SL-RSRP/SD-RSRP).

Observation 2: Since the intermediate Relay UE forwards the Relay discovery message only if the PC5 RSRP between the Relay UEs is above a configured threshold, the sidelink qualities between neighboring Relay UEs on the concerned relay path can be ensured.

Observation 3: The hop count of the concerned relay path is useful for gNB path switching decision.

Proposal 1:  Like measurement report in single-hop U2N Relay, the Remote UE includes the serving cell ID and source L2 ID of the Last U2N Relay UE in the measurement report for supporting path switching in multi-hop U2N Relay.  FFS: how the Remote UE obtains the source L2 ID of the Last U2N Relay UE.

Proposal 2:  The Remote UE reports only the sidelink measurement quantity result between the Remote UE and the First Relay UE for supporting multi-hop U2N Relay.

Proposal 3:  The Remote UE includes the hop count of the concerned relay path in the measurement report for supporting multi-hop U2N Relay.

- References

[1] RP-241609, New WID on NR sidelink multi-hop relay

[2] R2-240xxxx, Report of [POST127][401][Relay] Multi-hop relay discovery and (re)selection

[3] TS 38.300-i30, NR; NR and NG-RAN Overall Description; Stage 2 (Release 18)

[4] TS 23.304-j10, Proximity based Services (ProSe) in the 5G System (5GS) (Release 19)