3GPP TSG-RAN WG2 Meeting #128	R2-2410228

Orlando, Florida, USA, November 18-22 2024

Source: 	ITRI

Title:  	Discussion on reference configuration for inter-CU LTM

Document for:	Discussion and decision

Agenda Item:	8.6.2

Introduction

In RAN plenary #105, a revised Work Item on NR mobility enhancements Phase 4 was approved in RP-242356 [1]. Objective #1 of this Work Item is to specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM).

As described in the Work Item, the Rel-18 introduced LTM can offer improvements in handover latency and interruption time compared to Layer 3 based mobility. However, its operation is only supported for mobility between cells of the same gNB (same CU). Depending on the deployment of the network this may significantly limit the opportunities to use LTM. To gain the benefits of LTM for a far greater number of handovers, Objective #1 of Rel-19 NR mobility enhancements is to enable LTM operation between cells of different gNBs (i.e. inter-CU). The content of Objective #1 is listed below.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

During RAN2#127 meeting held in August, an agreement regarding reference configuration was made [2]. The agreement as shown below states that inter-CU LTM will re-use the reference configuration from Rel-18 LTM and no additional reference configurations are supported. That is, inter-CU LTM supports a single reference configuration only.

RAN2#127 (August, 2024) Agreement [2]:

- Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations (no multiple reference configurations) are supported.

During RAN2#127bit meeting held in October, a multi-company co-signed contribution [3] was treated. The contribution proposed to support multiple reference configurations in inter-CU LTM as well. However, it was not agreed.

RAN2#127bit (October, 2024) [4]:

R2-2409031 Multiple reference configuration for inter-CU LTM KDDI Corporation, LG Electronics, Nokia, ZTE, Rakuten, KT, CMCC, ETRI, ITL, ITRI, Fujitsu, SK telecom, Uplus discussion

- Noted.

Based on the agreements above, the situation that inter-CU LTM re-uses the reference configuration from Rel-18 LTM, i.e. a single reference configuration only, remains unchanged. However, different from Rel-18 intra-CU LTM, there are multiple CUs involved in the inter-CU LTM. How to decide the single reference configuration for multiple involved CUs needs to be study.

In this contribution, we discuss the strategy and baseline procedure on how the single reference configuration is decided for inter-CU LTM.

Discussion

If one of the involved CUs decides the reference configuration based on its own candidate cell(s), it may not be the optimum strategy as the CU may only contain partial candidate cell(s) in the inter-CU LTM, i.e. all other candidate cell(s) may belong to other involved CU(s). Therefore, inter-CU coordination for the reference configuration decision is needed.

In our understanding, the LTM candidate configuration(s) and thus the single reference configuration should be prepared in the LTM preparation phase. To avoid incurring additional Xn messages, re-using the handover request message and handover request acknowledgement message for reference configuration coordination is the simplest way. It also has the least specification impact.

Therefore, the reference configuration coordination among the involved CUs is initiated by the source CU via sending handover request message to each candidate CU. This makes sense as the source CU is basically the CU that initiates contact with each candidate CU during LTM preparation. Source CU can invite each candidate CU to join the reference configuration preparation and request the candidate configuration(s) that the candidate CU intends to configure via the handover request message.

Each candidate CU can accept or reject to join the reference configuration preparation. If a candidate CU accepts it, the candidate CU acknowledges the source CU with its intended candidate configuration(s) in the handover request acknowledgement message. Otherwise, the candidate CU rejects it via the handover request acknowledgement message, which means the candidate CU uses its own candidate configuration(s) rather than delta configuration(s) on top of the single reference configuration.

Proposal 1: The reference configuration coordination can be done via handover request message and handover request acknowledgement message to avoid additional Xn messages.

Proposal 1a: Source CU initiates the reference configuration coordination by sending handover request message to each candidate CU e.g. to invite the candidate CU to join the reference configuration preparation and request the candidate configuration(s) that the candidate CU intends to configure.

Proposal 1b: Each candidate CU acknowledges the source CU, e.g. with its intended candidate configuration(s) or its rejection of the invitation to join the reference configuration preparation in the acknowledgement, via handover request acknowledgement message.

The reference configuration can be decided by the source CU since the source CU is the CU that has all intended candidate configuration(s) of the candidate CU(s) (that agree to join the reference configuration preparation). For example, the source CU decides the reference configuration based on the candidate CU(s)’s intended candidate configuration(s). In this way, the reference configuration is decided by considering not only one CU’s need but multiple CU(s)’s.

Proposal 2: Source CU decides the reference configuration e.g. based on the candidate CU(s)’s intended candidate configuration(s).

In case RAN2 reaches agreements on this issue, synchronize RAN2’s perspective with RAN3’s by LS if needed.

Proposal 3: In case RAN2 reaches agreements on this issue, synchronize RAN2’s perspective with RAN3’s by LS.

Conclusion

In this contribution, the following proposals are made:

Proposal 1: The reference configuration coordination can be done via handover request message and handover request acknowledgement message to avoid additional Xn messages.

Proposal 1a: Source CU initiates the reference configuration coordination by sending handover request message to each candidate CU e.g. to invite the candidate CU to join the reference configuration preparation and request the candidate configuration(s) that the candidate CU intends to configure.

Proposal 1b: Each candidate CU acknowledges the source CU, e.g. with its intended candidate configuration(s) or its rejection of the invitation to join the reference configuration preparation in the acknowledgement, via handover request acknowledgement message.

Proposal 2: Source CU decides the reference configuration e.g. based on the candidate CU(s)’s intended candidate configuration(s).

Proposal 3: In case RAN2 reaches agreements on this issue, synchronize RAN2’s perspective with RAN3’s by LS.

Reference

- RP-242356 Revised Work Item: NR mobility enhancements Phase 4, RAN#105, September 2024.

- R2-24079xx Draft Report of 3GPP TSG RAN WG2 meeting #127, Maastricht, Netherlands, 19 - 23 August, 2024.

- R2-2409031 Multiple reference configuration for inter-CU LTM, KDDI Corporation, LG Electronics, Nokia, ZTE, Rakuten, KT, CMCC, ETRI, ITL, ITRI, Fujitsu, SK telecom, Uplus.

- R2-24095xx Draft Report of 3GPP TSG RAN WG2 meeting #127bis, Hefei, China, 14 - 18 October, 2024.