3GPP TSG RAN WG2#128	R2-2410231

Orlando, USA, Nov.  18th – 22nd , 2024

Agenda item:	8.8.4

Source: 	ITRI

Title: 	Discussions on configuration of intended service areas

Document for:	Discussion and Decision

1	Introduction

RAN2 agreed that the intended service area ID is included into MCCH. The remaining issues including how to configure the intended service areas and the related IDs, and the mechanism to allow UE located beyond intended service areas of the interested MBS session skipping MCCH re-acquisition.

Following is the related RAN2 agreements [1]:

RAN2#127bis Agreements: 1. For each MBS service we include one or more intended service area IDs into MCCH. FFS whether the list of the intended service areas (and related IDs) is also included in MCCH or if it is provided in a new or existing SIB. We will consider possible enhancements (including enhancements left up to UE implementation) to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service.

In this contribution we analyze the signaling overhead and propose to include the configuration of intended service areas in SIB. The UE behaviors of (re)acquiring configuration of intended service areas and MCCH are also discussed.

2	Discussion

2.1 	Configuration of intended service areas

The issues of carrying the list of intended service areas in a SIB or in the MCCH including signalling overhead, the bits required for the configuration, and not to impact non-MBS UE when the configuration changes.

- Signalling overhead

According to [2], If the configuration of intended service areas is included in a SIB, the periodicity of a SI message ranges from 80 ms to 5120 ms.

SchedulingInfo ::=                  SEQUENCE {

si-BroadcastStatus                  ENUMERATED {broadcasting, notBroadcasting},

si-Periodicity                      ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512},

sib-MappingInfo                     SIB-Mapping

}

If the configuration is included in MCCH (e.g., included in the MBSBroadcastConfiguration message), the periodicity of MCCH ranges from 10 ms to 2560 ms.

MCCH-RepetitionPeriodAndOffset-r17 ::= CHOICE {

rf1-r17                                INTEGER(0),

rf2-r17                                INTEGER(0..1),

rf4-r17                                INTEGER(0..3),

rf8-r17                                INTEGER(0..7),

rf16-r17                               INTEGER(0..15),

rf32-r17                               INTEGER(0..31),

rf64-r17                               INTEGER(0..63),

rf128-r17                              INTEGER(0..127),

rf256-r17                              INTEGER(0..255)

}

From the viewpoint of transmission periods, the signalling overhead of including the configuration in a SIB may be less than including the configuration in MCCH.

Observation 1: Comparing the transmission periods of SI message and MCCH, the signalling overhead of including the configuration of intended service areas in a SIB may be less than including the configuration in MCCH.

- Resource required to carry the configuration

Each entity of the list of intended service areas is either represented by a (set of) referenceLocation and radius or by a (set of) polygon(s). The referenceLocation and each point of a polygon (a geographic shape) are represented in the format of Ellipsoid-Point which requires 48 bits to contain the information. The radius requires 16 bits.

There may be multiple intended service areas in the coverage of a NTN cell and SIB20 may only have few available bits for additional information. It is preferable to include the configuration of intended service areas in a new SIB or in MCCH (e.g., in MBSBroadcastConfiguration message).

Observation 2: Considering the required resource, the configuration of intended service areas could be included in a new SIB rather than in SIB20.

- Not to impact non-MBS UE when the configuration changes

If the list of intended service area is included in a SIB, the change of intended service areas may result in notification of System Information modification. Both MBS UE and non-MBS UE would re-acquire System Information.

However, according to [2], the UE capable of MBS broadcast which is receiving or interested to receive MBS broadcast service(s) via a broadcast MRB shall ensure having a valid version of SIB20, regardless of the RRC state the UE is in. Similarly, the MBS capable UE also needs to ensure having a valid version of the SIB with the list of intended service areas and related IDs.

Thus, it is feasible to specify that the information update of intended service areas and related IDs would not cause System Information modification procedure to prevent impacting non-MBS UE. Changes of the SIB could be indicated by a valueTag associated with the SIB. While MBS capable UE in any RRC state interested in MBS broadcast services shall ensure having a valid version of the SIB, the change of the list of intended service areas would be obtained by UE interested in MBS broadcast services.

Observation 3: it is possible that only UE interested in MBS broadcast services would be impacted when the configuration of intended service areas changes by specifying the change of the list of intended service areas would not cause System Information modification procedure.

Proposal 1: It is proposed to include the list of intended service areas and the related IDs in a new SIB.

Proposal 2: It is feasible to specify that the update of intended service areas and the related IDs shall not result in System Information modification procedure.

2.2 	Procedure of MCCH (re)acquiring

UE acquires MCCH to obtain the information of the MBS sessions broadcast in the serving cell and the configuration for MTCH reception. The MCCH may be updated e.g., to indicate the mapping of MBS session and the intended service areas, to change the configuration for MTCH reception, or to modify the mapping between MBS services and the neighbouring cells.

When a UE is not in any intended service areas of the interested MBS service(s), the UE needs not to re-acquire MCCH for the up-to-date configuration for associated MTCH reception.

Observation 4: When a UE is not in any intended service areas of the interested MBS service(s), the UE needs not to re-acquire MCCH for the up-to-date configuration for MTCH reception.

Considering that the mapping of MBS sessions and the intended service areas changes, or the information of the neighbouring cells and the provided MBS services changes. If the location of the UE happens to be within the update intended service areas associated with the MBS session, the UE may not establish MRB promptly due to lacking of up-to-date configuration carried by MCCH. Or the UE may reselect a cell without providing the interested MBS services due to the neighbouring cells and the provided MBS services changes.

Observation 5: UE may miss MBS services due to outdated MCCH information.

To prevent UE from missing MBS services due to outdated information of MCCH, a UE capable of MBS broadcast service may re-acquire MCCH upon events, e.g., MBS service announcement, re-acquiring SIB20 or requiring the SIB carrying the list of intended service areas, or when (re)acquire GNSS information of the UE location.

Observation 6: To prevent UE from missing MBS services due to skipping re-acquisition of MCCH, a UE capable of MBS broadcast service may re-acquire MCCH according to specific events, e.g., MBS service announcement, re-acquiring SIB20 or requiring the SIB carrying the list of intended service areas, or when (re)acquire GNSS information of the UE location.

Proposal 3: If the list of intended service areas and the related IDs are included in a SIB, a UE capable of MBS broadcast service is allowed to skip MCCH re-acquisition when the UE is not within intended service area of any interested broadcast service.

3	Conclusion

For providing the list of intended service areas and related IDs, we have the following observations:

Observation 1: Comparing the transmission periods of SI message and MCCH, the signalling overhead of including the configuration of intended service areas in a SIB may be less than including the configuration in MCCH.

Observation 2: Considering the required resource, the configuration of intended service areas could be included in a new SIB rather than in SIB20.

Observation 3: it is possible that only UE interested in MBS broadcast services would be impacted when the configuration of intended service areas changes by specifying the change of the list of intended service areas would not cause System Information modification procedure.

Observation 4: When a UE is not in any intended service areas of the interested MBS service(s), the UE needs not to re-acquire MCCH for the up-to-date configuration for MTCH reception.

Observation 5: UE may miss MBS services due to outdated MCCH information.

Observation 6: To prevent UE from missing MBS services due to skipping re-acquisition of MCCH, a UE capable of MBS broadcast service may re-acquire MCCH according to specific events, e.g., MBS service announcement, re-acquiring SIB20 or requiring the SIB carrying the list of intended service areas, or when (re)acquire GNSS information of the UE location.

The following proposals are provided accordingly:

Proposal 1: It is proposed to include the list of intended service areas and the related IDs in a new SIB.

Proposal 2: It is feasible to specify that the update of intended service areas and the related IDs shall not result in System Information modification procedure.

Proposal 3: If the list of intended service areas and the related IDs are included in a SIB, a UE capable of MBS broadcast service is allowed to skip MCCH re-acquisition when the UE is not within intended service area of any interested broadcast service

4	References

[1] R2-24095xx	Report of 3GPP TSG RAN WG2 meeting #127bis, Hefei, China, 2024.

[2] 3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".