3GPP TSG-RAN WG2 Meeting #128	R2-2410257

Orlando, USA, 18 – 22 November 2024

Agenda item:	8.5.4

Source:	Nokia, Nokia Shanghai Bell

Title:	On Adaptation of common signal/channel transmissions for NES

WID/SID:	Netw_Energy_NR_enh

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we discuss aspects related to the potential adaptations of the common signal/channel transmissions.

# 2	Adaptation of SSB in time domain

During the previous meetings, there were no RAN2 agreements on this topic.

Pending agreements from the RAN1 meetings are

Agreement​– [RAN1#118] For adaptation mechanism(s) of SSB in time-domain,​ - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported ​ - FFS: Adaptation for SSB that is not CD-SSB is supported (A2)​ - FFS: Adaptation for SSB not on sync raster is supported (A3)​ - For Rel-19 NES-capable UE’s SCell​ - Adaptation of SSB configured for the SCell is supported for the following cases​ - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell​ - Adaptation for SSB that is not CD-SSB on sync raster (B2’)​ - Adaptation for SSB that is not CD-SSB not on sync raster (B3’)​ Agreement – [RAN1#117] For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities FFS: Details including applicable scenarios

As per the current RAN1 agreements, SSB adaptation in the time domain adaptation includes changing the SSB burst periodicity using one or more SSB burst periodicity value(s) and at least for non-cell defining SSBs (on and off the sync raster) of the SCell of a Rel.19 NES-capable UE. Adaptation of CD-SSB in the SCell and adaptation of NCD-SSB or SSB not on the sync raster in the PCell of a Rel.19 NES-capable UE are still FFS.

Such adaptation can be performed based on parameters dedicated for SSB adaptation configured via Serving Cell Configuration, e.g.,

- new periodicity value,

- time duration of performing the adaptation,

- time instance of initiating the adaptation

The adaptation can be indicated dynamically based on RRC signaling or MAC-CE or dynamic control information.

In our view, the described formulation for enabling SSB adaptation in the time domain is quite similar to the AI 8.5.2 that focuses on “On-demand SSB SCell operation”. For this reason, we believe that at least RRC signaling and MAC-CE aspects to indicate SSB adaptation in time domain can be treated in conjunction with the respective designs discussed/agreed in AI 8.5.2.

During the RAN4 #112bis meeting, an LS was sent from RAN4 to RAN2 asking about the SMTC impact due to SSB adaptation, as observed below

RAN4#112bis extensively discussed impact on RRM requirements from SSB adaptation. The following agreements were reached after discussion: Agreements: - RAN4 to discuss whether there are any impacts on at least the following RRM requirements due to SSB adaption: - L1-RSRP for serving cell - L1-SINR - BFD - CBD - SCell activation - TCI state switching - NR intra-frequency L3 measurement - NR inter-frequency L3 measurement However, L3 measurement requirement in RAN4 is defined based on SMTC configuration rather than SSB configuration in legacy. RAN4 would like to ask RAN2 that whether there is any SMTC impact due to SSB adaptation. 2. Actions: To RAN2. ACTION: RAN4 kindly asks RAN2 to answer the SMTC impact question due to SSB adaptation above.

Dynamic adaptation of this SSB transmission, either on the serving cell or a neighboring cell, may have an impact on L3 measurements performed by the UE, e.g., with the UE performing excess measurements when there is no SSB burst transmission, as in Figure 1 or e.g., with the UE missing transmission occasions of SSB bursts, as in Figure 2.

Figure 1: SSB burst periodicity is adapted to a longer value and the UE attempts unnecessary measurements.

Figure 2: SSB burst periodicity is adapted to a shorter value and the UE misses useful measurement occasions.

A way to address these issues would be to align, the SSB adaptation in time domain, e.g., burst periodicity, with a dedicated SMTC to be applied for measurements of adapted SSBs.

Such adaptation can be performed based on parameters dedicated for SSB adaptation configured via Serving Cell Configuration, in addition to, e.g., a new periodicity value (and potentially other parameters as described earlier).

This adaptation can be indicated dynamically together with the SSB adaptation based on RRC signaling or MAC-CE or dynamic control information.

For adaptation of non-cell defining SSBs in an SCell, we believe that the configuration of such an SMTC can be aligned with AI 8.5.2 that focuses on “On-demand SSB SCell operation” (as previously indicated for the SSB adaptation parameters).

Proposal 1: When designing on-demand SSB and SSB adaptation activation MAC CE/RRC signaling they should be done jointly in RAN2

# 3		PRACH Adaptation in time domain

As per the latest RAN1#117 meeting outcome, it was agreed to support at least the PRACH adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any). It was also noted that NES-capable UEs can use both additional PRACH resources and PRACH resources configured for legacy UEs.

In our view, the agreed PRACH adaptation based on additional PRACH resources should be applied for both 4-step RACH and 2-step RACH (if configured in the cell). Particularly for 2-step RACH, further clarifications and considerations would be needed. For instance, it is not clear whether additional PUSCH resources for NES-capable shall be considered for MsgA payload transmission or the same PUSCH as configured for legacy UEs shall be considered.

Proposal 2: Clarify whether further considerations are needed for the 2-step PRACH adaptation e.g., the resource used by the UE to transmit MsgA.

There are two types of PRACH that can be configured:

- PRACH associated with CBRA configured by cell-specific signaling.

- PRACH associated with CFRA configured by UE-dedicated signaling.

The agreed PRACH adaptation shall at least target CBRA procedures. PRACH adaptation associated with CFRA procedures shall be studied as well, if time allows.

Proposal 3: PRACH adaptation solution(s) are at least applied to CBRA. PRACH adaptation applied to CFRA shall also be investigated, if time allows, however no CFRA-specific PRACH adaptation solutions should be considered.

The likelihood that cells are designed to only serve R19 UEs is low; thus, the PRACH adaptation should be applicable to cells that support both legacy and Rel-19 (NES-capable) UEs. Moreover, in order to avoid any impact on legacy UEs as noted in the WID, and to enable the initial access for all UEs before being able to identify whether they are legacy or NES-capable UEs, the cell should support both legacy and NES-capable UEs.

Proposal 4: PRACH adaptation should be applicable to cells that support both legacy and Rel-19 UEs.

The configuration of a PRACH configuration index for the additional PRACH resources different from the PRACH configuration index for the legacy resource offers more flexibility to the NW and can help accommodating scheduler needs. However, the possible overlaps between sub-sets of ROs would need to be further studied in this case, which is under discussion in RAN1.

The configuration of the PRACH configuration index for the additional PRACH resources as the legacy resource allows the network energy consumption for PRACH reception to occur mainly (or only) during system frames where legacy ROs are located and, at the same time, ensures that PRACH resources are not overlapping in time and/or frequency.

Additional mechanisms are also being discussed in RAN1 for determining the additional PRACH resources in time domain including time offset, RO muting/masking but we should wait for RAN1 progress on this before discussing RAN2 implications.

Proposal 5: RAN2 to wait for RAN1 to conclude on PRACH adaptation mechanisms before discussing RAN2 implications.

One potential implication on RAN2 could be on the Conditional handover (CHO) procedure as the NW does not know the time instant when UE initiates CHO and the UE can’t be aware in advance about the potential activation/deactivation of additional PRACH resources at one or more potential target cell(s). So RAN2 should consider whether it is feasible to support additional PRACH resources for CHO and if they want to be supported then how it can be achieved..

Proposal 6: Study whether further enhancements are needed for the conditional handover procedure when the additional PRACH resources are activated at one or more candidate target cells.

Another implication on RAN2 will be the beam failure recovery configuration as for beam failure recovery RAN2 needs to discuss further how additional PRACH resources can be used for beam failure recovery.

Proposal 7: RAN2 to consider how to enable usage of additional PRACH resources for beam failure recovery procedure.

From our perspective, it is natural to expect that additional PRACH resources are configured semi-statically in Rel-19. Whether adaptation of the resources will be performed only via DCI format 1_0, as per RAN1#118bis agreements, or also via semi-static signalling will need to be further discussed. Indeed, system Information (SI), e.g. SIB1, could also be considered for indicating PRACH adaptation to UEs that have missed the DCI-based indication of additional PRACH resources for NES-capable UEs. The SIB1 would then include the information about the configuration of additional PRACH resources and the activation status. Such procedure would not require any SI change.

Proposal 8: Higher-layer signalling based on SI, e.g. SIB1, for PRACH adaptation could be considered, in addition to the DCI-based adaptation.

Proposal 9: RAN2 needs to wait for more RAN1 inputs before we can progress detailed signaling of PRACH adaptation

# Adaptation of paging occasions

In RAN2 #127bis the following was agreed:

Agreements on paging adaptation - Select option-b as baseline for R19 NES paging enhancement. - R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values) - Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration. - Legacy UE is not barred. - Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.

A key open aspect is how to handle the potential PRACH load increase, especially for Ns=8 or larger, when the POs are clustered in time.

## PRACH Adaptation in time domain in conjunction with adaptation of paging occasions

The PRACH load level will inherently increase if the paging occasions are confined in time domain. This is because the UEs will have to be paged during such confined time period, thereby causing the UEs to respond to the paging also in a confined time interval. So it should be possible to temporarily increase the amount of PRACH resources in order to accommodate more clustered paging responses from UEs. The legacy approach of configuring a high amount/frequently occurring PRACH resources will impact the network’s resource availability and RO monitoring energy consumption. Thus, one approach which can be considered by RAN2 is to temporarily make additional PRACH resources available, when one or more UEs are paged. Such additional PRACH resources would not be available, when no UEs are paged, and thus the PRACH resource can be repurposed and the RO monitoring omitted.

Proposal 10: PRACH adaptation in time domain is supported to temporarily increase the PRACH capacity to be aligned with clustered paging and spread the PRACH load when paging occasion adaptation is used.

# Conclusion

Proposal 1: When designing on-demand SSB and SSB adaptation activation MAC CE/RRC signaling they should be done jointly in RAN2

Proposal 2: Clarify whether further considerations are needed for the 2-step PRACH adaptation e.g., the resource used by the UE to transmit MsgA.

Proposal 3: PRACH adaptation solution(s) are at least applied to CBRA. PRACH adaptation applied to CFRA shall also be investigated, if time allows, however no CFRA-specific PRACH adaptation solutions should be considered.

Proposal 4: PRACH adaptation should be applicable to cells that support both legacy and Rel-19 UEs.

Proposal 5: RAN2 to wait for RAN1 to conclude on PRACH adaptation mechanisms before discussing RAN2 implications.

Proposal 6: Study whether further enhancements are needed for the conditional handover procedure when the additional PRACH resources are activated at one or more candidate target cells.

Proposal 7: RAN2 to consider how to enable usage of additional PRACH resources for beam failure recovery procedure.

Proposal 8: Higher-layer signalling based on SI, e.g. SIB1, for PRACH adaptation could be considered, in addition to the DCI-based adaptation.

Proposal 9: RAN2 needs to wait for more RAN1 inputs before we can progress detailed signaling of PRACH adaptation

Proposal 10: PRACH adaptation in time domain is supported to temporarily increase the PRACH capacity to be aligned with clustered paging and spread the PRACH load when paging occasion adaptation is used.