3GPP TSG-RAN WG2 Meeting #127bis							       R2-2410353

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.2.2

Source: 	NEC

Title:	Ambient-IoT Functionality Aspects

Document for:	Discussion/Decision

- Introduction

The Rel-19 SID for Ambient IoT (also referred to as A-IoT) includes following RAN2-led objectives [1].

- RAN2-led: - Study and decide which functions are needed for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission, and study those functions. For example: - Paging - Random access - Data transmission, including necessary radio resource control aspects, respecting the limitation in the General Scope - Interactions with upper layers For functionalities not listed above, they are studied only if found essential.

This contribution discusses the functionality aspects based on the RAN2-led objectives and the latest RAN1 and RAN2 agreements.

- Discussion

  - User Plane Functions

    - Padding

In A-IoT, the D2R transmission is triggered by the reader while there isn’t any buffer status at that time. Thus, it may happened that the grant is larger than the higher layer data available to transmit. In NR, padding is used to solve this problem by reaching the exact grant size in UL. Since there is no need to keep byte alignment for A-IoT, the device can terminate the transmission early according to the exact data size. How to solve it is also related to the following agreement of RAN1#116bis [2] meeting on acquiring the end of PDRCH transmission:

RAN1#116bis Agreements: For the reader to acquire the end of PDRCH transmission, study at least following options: • Option 1: D2R postamble immediately follows the PDRCH • Option 2: Based on control information

Either one of the above options can be used for the indication of the early PDRCH termination. So we suggest not to adopt padding function for MAC layer and the exact method for the indication of the early PDRCH termination is up to RAN1.

RAN2 doesn’t support padding function. And the method for the indication of the early PDRCH termination is up to RAN1.

    - Multiplexing

The transmission of signaling and the transmission of data are usually separate. Thus, there is no need to consider the multiplexing of multiple layer2 (e.g., logical) channels. Moreover, there is no need to consider the LCP of multiple layer2 channels. In previous RAN2 meeting, only one logical channel is agreed for A-IoT transmission.

However, in RAN2#127bis [3] meeting, there are agreements on energy bits, segmentation indication shown as below:

Capture in the TR the option: the device may include energy status indication in D2R messages (e.g. MSG1,MSG3 and Command Response message), if the device can.  1 bit indication can be captured in the TR.    We will capture the use case in the TR.   FFS whether it is reader controlled.

Capture the following option in TR: An indication is used to indicate to reader whether the data is segmented and whether it is last segment.    FFS the details on how this is designed and number of bits (one or two).

These indications may be reported to the reader via MAC layer. Therefore, these indications may be sent together with the higher layer data. So we propose:

RAN2 to considerate the multiplexing of channel for higher layer data and control information.

  - Control Plane Functions

    - AS ID

RAN2#127bis meeting discussed AS ID aspects and reached following agreements:

Agreements on AS ID

RAN2 assumes that if “AS ID” is defined it is used at least for purpose of D2R scheduling and R2D reception.  Up to RAN1 to decide whether a “AS ID” is defined.

RAN2 assumes this “AS ID” should be a short AS layer ID, rather than the full upper layer device ID.  FFS on the length.   FFS if AS ID can be based on partial upper layer device ID.

From RAN2 perspective, following options are possible for “AS ID”:

Option 1: a random ID if used in Msg1 can be reused;

Option 2: reader assigns this “AS ID”. FFS via which R2D message.

RAN2 will aim to define one common design for all RA procedures (if technically possible)

Regarding the listed 2 options, firstly we discuss their potential issues.

For Option 1, there might be following two potential issues:

- potential random ID collision risk:

Actually, in our understanding, the potential random ID collision risk would be quite low since RAN2#126 agreed that “RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose”. If the design target is to realize an almost collision-free “AS ID”, one potential solution is to combine Option 1 and 2, i.e., defining AS ID with the random ID used in Msg.1 as the baseline, additionally only when the random ID collision occurs, the reader assigns an “AS ID” which is collision free from random IDs the reader has received/echoed so far. In such a case, the reader assigned “AS ID” has the same length as random ID, i.e., 16 bits. Another potential solution is to enhance Option 1 with additionally using random access occasion ID as “AS ID”, only when random ID collision occurs.

- how to define a common design for CBRA and CFRA:

At RAN2#127 meeting, RAN2 achieved following agreements:

- Agreements

- For 3-step CBRA Support fixed random ID size is 16 bit. The ID is randomly generated.

- In contention-free access, the A-IoT device directly sends the upper layer data (e.g. device ID) in its very first D2R message after being triggered (i.e. skip contention resolution Msg1/2).   FFS if a short AS ID is also included in the message and what type of ID for scheduling purposes.

One possible interpretation seems that CFRA doesn’t support sending random ID in it very first D2R message after being triggered. From contention resolution point of view, it is correct. However, we still have an FFS highlighted in yellow, that is from D2R scheduling and R2D reception perspective. If Option 1 is adopted as baseline, we think it is nature to include the short “AS ID” (i.e., 16 bit random ID) in this very first D2R message. Since readers may have different intentions on scheduling D2R transmission via “AS ID”, for CFBA, it would be beneficial to allow reader to indicate whether to include the short “AS ID” (i.e., 16 bit random ID) in the very first D2R message after being triggered.

For Option 2, we have one FFS, i.e., via which R2D message to inform devices the reader assigned “AS ID”.

Based on the following overall AS procedures between A-IoT device and reader, illustrated in the TP, there are two alternatives:

Alt.1 reader provides the assigned “AS ID” via R2D message in Step B (i.e., Msg.2)

Alt.2 reader provides the assigned “AS ID” via R2D message in Step C (e.g., R2D data transmission in C1)

Fig.1 Overall AS procedures between A-IoT device and reader

Both alternatives are workable, down selection between them depends on target of supporting D2R scheduling and R2D reception. If D2R scheduling and R2D reception is expected to be supported from the beginning of Step C, then Alt.1 is better than Alt.2. Anyway, such a down selection can be done in normative phase.

Based on above analyses, we have following observations and proposals:

- When the random ID in Msg.1 is reused as “AS ID”, the potential “AS ID” collision risk would be quite low.

If the design target is to realize an almost collision-free “AS ID”, following two potential solutions can be further studied:
Option 1-a. Combine Option 1 and 2, i.e., as base line, reuse random ID in Msg.1 as “AS ID”. Only when “AS ID” collision occurs, the reader assigns another “AS ID” which is collision free from random IDs the reader has received/echoed so far.
Option 1-b. Enhance Option 1 with additionally using random access occasion ID as “AS ID”, i.e., as base line, reuse random ID in Msg.1 as “AS ID”. Only when “AS ID” collision occurs, the reader additionally uses random access occasion ID as “AS ID”.

There are two alternatives for providing the reader assigned “AS ID” to devices:
Alt.1 reader provides the assigned “AS ID” via R2D message in Step B (i.e., Msg.2)

Alt.2 reader provides the assigned “AS ID” via R2D message in Step C (e.g., R2D data transmission in C1)

Capture following two potential solution to the TR for designing an almost collision-free “AS ID”:
Option 1-a. Combine Option 1 and 2, i.e., as base line, reuse random ID in Msg.1 as “AS ID”. Only when “AS ID” collision occurs, the reader assigns another “AS ID” which is collision free from random IDs the reader has received/echoed so far.
Option 1-b. Enhance Option 1 with additionally using random access occasion ID as “AS ID”, i.e., as base line, reuse random ID in Msg.1 as “AS ID”. Only when “AS ID” collision occurs, the reader additionally uses random access occasion ID as “AS ID”.

Capture following two alternatives for providing the reader assigned “AS ID” to devices:
Alt.1 reader provides the assigned “AS ID” via R2D message in Step B (i.e., Msg.2).
Alt.2 reader provides the assigned “AS ID” via R2D message in Step C (e.g., R2D data transmission in C1).

    - Repetition

Transmission via the A-IoT air interface between the reader and the A-IoT device may encounter signal fading or blockage, just like traditional Uu interface. This may lead to D2R data transmission failure. In RAN2#127bis [3] meeting, there is an agreement on repetition of R2D transmission (e.g., upper layer command) by the reader to trigger re-sending the same D2R response shown as below:

1 In case of D2R data transmission failure, device follows the reader instruction:

	it is supported to re-access in another opportunity controlled/provided by the reader (i.e. retry the random access); and

	reader can repeat the R2D “upper layer command” to trigger the device to re-send the same D2R “response” (i.e., device just follows the received R2D to transmit D2R).

That is, the reader may send inventory/command several times. To control the number of repetitions, the following methods can be considered:

- The specific number of repetition is visible to the reader from CN to control the repetition.

- The survival time of a single inventory/command request is visible to the reader. And the specific repetition times depend on the reader implementation.

The control information for repetition (e.g. specific number of repetitions or survival time of a single inventory/command request) is useful to be visible to the reader from CN.

    - Message size indication

In RAN2#127bis [3] meeting, there is an agreement on message size reporting shown as below:

- From RAN2 perspective, it is beneficial for the reader to know an estimate of expected D2R message size.  Two options can be captured: 1) from the CN and 2) from the device (simple message size indication).    Capture the advantages/disadvantages.

As we agreed it is beneficial for the reader to know an estimate of expected D2R message size, the main difference between two options from our perspective would be on different efficiency. Device based option is more dynamic reported than CN based option, therefore more real and accurate message size can be provided in each D2R transmission. With this real-time message size indication, reader can do the best scheduling to guarantee high efficiency of D2R message transmission. Apparently, the disadvantage of device based message size indication is more transmission overhead in PRDCH and obviously additional spec impact on indicator signalling design. However, for AIoT system it is worthy to have such real-time/frequent report since higher scheduling efficiency is of much importance for AIoT device. Lower efficiency D2R scheduling may result in additional D2R transmission with more power consumption. If device run out of power, energy harvesting and delay transmission would be envisioned which needs another round of random access procedure and large delay for this round of inventory/command task. From perspectives of both latency and energy consumption of AIoT devices, more efficient scheduling with relatively higher transmission overhead is preferred.

- D2R message size reported by device can provide more efficient scheduling of D2R transmissions.

- D2R message size reported by device can potentially provide D2R transmission with low latency and low power consumption.

- D2R message size reported by device has larger spec impact and signaling overhead on extra design of report format.

In terms of device based D2R message size report, simple message size indication with less spec impact and simple signalling design as well as overhead is preferred. Targeting this, a ratio based message size reporting mechanism is proposed, where a ratio defined as remaining message size over previous scheduled size at device side is designed. By introducing this metric, efficient data scheduling for AIoT system can be supported as shown in Fig. 1.

Fig. 1 Ratio based message size reporting mechanism for AIoT communication

To be specific,

Step 1. A first PRDCH with TBS1 indication of a subsequent PDRCH is transmitted from reader to device.

Step 2. After device receives the PRDCH and obtains TBS1, a ratio r = remaining message size / TBS1 = (total D2R message size - TBS1) / TBS1 will be calculated by device. Next, device reports this ratio together with the scheduled D2R transmission in corresponding PDRCH.

Step 3. When reader receives PDRCH successfully, reader calculates the remaining D2R message size by multiply the reported r with TBS1. Based on the calculated remaining message size, reader could indicate TBS2 in another PRDCH transmission for another PDRCH scheduling. This procedure will repeat until the completion of all D2R message transmission.

- Ratio based message size report is a simple and efficient mechanism for AIoT message scheduling.

Ratio based message size reporting can be supported for efficient scheduling in AIoT.

    - Device variable control

Considering the limited size and complexity required by practical applications for battery-less devices with no energy storage capability or devices with limited energy storage that do not need to be replaced or recharged manually, the persistence of device variables may consider both the energy status of the A-IoT device and the type of the device variables.

From the persistence behavior perspective, three types of device variable control can be considered:

Type 1 Infinite persistent: The variables required to be kept even the A-IoT device is de-energized. For example, the information related to registration with the NW, which is stored in the device memory.

Type 2 Limited persistent: The variables may be refreshed under some circumstances. For example, device maintain those variables under specific temperature range and the energy storage meet the persistence requirements, otherwise, the device will lose the stored value.

Type 3 Non-persistent: A-IoT device shall refresh non-persistent variables when powered, meaning that every time an A-IoT device loses power its non-persistent variables will be lost.

Considering three types of Device variable control behaviors:

Type 1 Infinite persistent: The variables need to be stored even the A-IoT device is de-energized, such as information related to registration with the NW, etc.

Type 2 Limited persistent: The variables may be refreshed under some circumstances. For example, device maintain those variables under specific temperature range and the energy storage meet the persistence requirements, otherwise, the device will lose the stored value.

Type 3 Non-persistent: A-IoT device shall refresh non-persistent variables when powered, meaning that every time an A-IoT device loses power its non-persistent variables will be lost.

- Conclusion

Based on the discussion in this document, we have the following observations and proposals:

Observation 1:	When the random ID in Msg.1 is reused as “AS ID”, the potential “AS ID” collision risk would be quite low.

Observation 2:	If the design target is to realize an almost collision-free “AS ID”, following two potential solutions can be further studied: 
Option 1-a. Combine Option 1 and 2, i.e., as base line, reuse random ID in Msg.1 as “AS ID”. Only when “AS ID” collision occurs, the reader assigns another “AS ID” which is collision free from random IDs the reader has received/echoed so far. 
Option 1-b. Enhance Option 1 with additionally using random access occasion ID as “AS ID”, i.e., as base line, reuse random ID in Msg.1 as “AS ID”. Only when “AS ID” collision occurs, the reader additionally uses random access occasion ID as “AS ID”.

Observation 3:	There are two alternatives for providing the reader assigned “AS ID” to devices: Alt.1 reader provides the assigned “AS ID” via R2D message in Step B (i.e., Msg.2)

Observation 4:	D2R message size reported by device can provide more efficient scheduling of D2R transmissions.

Observation 5:	D2R message size reported by device can potentially provide D2R transmission with low latency and low power consumption.

Observation 6:	D2R message size reported by device has larger spec impact and signaling overhead on extra design of report format.

Observation 7:	Ratio based message size report is a simple and efficient mechanism for AIoT message scheduling.

Observation 8:	Considering three types of Device variable control behaviors:

−	Type 1 Infinite persistent: The variables need to be stored even the A-IoT device is de-energized, such as information related to registration with the NW, etc.

−	Type 2 Limited persistent: The variables may be refreshed under some circumstances. For example, device maintain those variables under specific temperature range and the energy storage meet the persistence requirements, otherwise, the device will lose the stored value.

−	Type 3 Non-persistent: A-IoT device shall refresh non-persistent variables when powered, meaning that every time an A-IoT device loses power its non-persistent variables will be lost.

Proposal 1:	RAN2 doesn’t support padding function. And the method for the indication of the early PDRCH termination is up to RAN1.

Proposal 2:	RAN2 to considerate the multiplexing of channel for higher layer data and control information.

Proposal 3:	Capture following two potential solution to the TR for designing an almost collision-free “AS ID”: 
Option 1-a. Combine Option 1 and 2, i.e., as base line, reuse random ID in Msg.1 as “AS ID”. Only when “AS ID” collision occurs, the reader assigns another “AS ID” which is collision free from random IDs the reader has received/echoed so far. 
Option 1-b. Enhance Option 1 with additionally using random access occasion ID as “AS ID”, i.e., as base line, reuse random ID in Msg.1 as “AS ID”. Only when “AS ID” collision occurs, the reader additionally uses random access occasion ID as “AS ID”.

Proposal 4:	Capture following two alternatives for providing the reader assigned “AS ID” to devices: 
Alt.1 reader provides the assigned “AS ID” via R2D message in Step B (i.e., Msg.2). 
Alt.2 reader provides the assigned “AS ID” via R2D message in Step C (e.g., R2D data transmission in C1).

Proposal 5:	The control information for repetition (e.g. specific number of repetitions or survival time of a single inventory/command request) is useful to be visible to the reader from CN.

Proposal 6:	Ratio based message size reporting can be supported for efficient scheduling in AIoT.

- Reference

- RP-240826, Rel-19 SID of Ambient IoT.

- Chairman Notes, RAN1 #116bis meeting.

- Chairman Notes, RAN2 #127bis meeting.