3GPP TSG RAN WG2 Meeting #127bis                          	   R2-2410428

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.5.4

Source:	Qualcomm Incorporated

Title:	Discussion on Adaptation of Common Signal/Channel Transmissions

WID/SID:	Netw_Energy_NR_enh-Core – Release 19

Document for:	Discussion

# Introduction

The objectives for adaptation of common signal and channel transmissions are outlined with the follows [1].

The objectives of the work item are the following: - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g., adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

In this contribution, we discuss scenarios or options for adaptation of SSB in time domain, adaptation of PRACH in time domain and adaptation of paging occasions, and the potential impact to RAN2’s specification work.

# Discussion

## Adaptation of SSB in time domain

For adaptation of SSB in time domain, the following agreements were made at RAN1 #116 for further study [2].

Agreement For adaptation of SSB in time-domain, consider the following adaptation mechanisms for further study. - Adaptation of SSB burst periodicity. - Adaptation based on two SSB configurations where up to two configurations can be active. - Adaptation based on skipping/transmitting some SSB bursts non-uniformly with single SSB configuration. - Adapting the transmitted number of SSBs within a SSB burst - Cell DTX for SSB adaptation - Whether to support new SSB burst periodicity value(s) - Whether to support new SSB burst(s) (i.e., how SSB transmission is made within a burst) - New compact SSB burst(s) - Adapting the position of SSBs within a SSB burst - Other mechanisms/combinations are not precluded. Agreement For the adaptation mechanisms of SSB in time-domain, study further applicable scenarios and associated legacy UE impact/handling (if any) based on the following: - Applicability to UE in idle/inactive and/or connected mode. - Applicability to PCell and/or SCell(s) Agreement For the adaptation mechanisms of SSB in time-domain, study further following mechanisms: - Adaptation mechanism indicated or configured by gNB without UE trigger. - Adaptation triggered by UE (if any) FFS: Details of associated signaling/indication/configuration

Additionally, the following agreements were made at RAN1 #116bis [3].

Agreement For indication of adaptation of SSB in time-domain, Support at least SSB adaptation provided by gNB without UE trigger. Agreement Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s): - For cell with both legacy UEs and Rel-19 NES-capable UEs - Rel-19 NES-capable UE’s PCell (Connected mode) - Study from the following options: - Option A1: adaptation for CD-SSB - Option A2: adaptation for SSB that is not CD-SSB - Option A3: adaptation for SSB not on sync raster - Rel-19 NES-capable UE’s SCell - Study from the following options: - Option B1: adaptation for CD-SSB - Option B2: adaptation for SSB that is not CD-SSB - Option B3: adaptation for SSB not on sync raster - FFS: Rel-19 NES-capable UE in idle/inactive mode - Note: Impact to idle/inactive UEs shall be minimized

Furthermore, the following agreement was made in RAN1#118 meeting [5].

Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

At RAN1 #118bis, no agreement was made for supporting adaptation of SSB for idle mode UEs [5].

Conclusion There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19

Observation 1. There is no consensus yet on the support of adaptation of SSB for idle mode UEs.

Proposal 1. For SSB adaptation impact to UEs’ operations at RRC Idle/Inactive state, RAN2 can wait for RAN1's further conclusion on adaptation options with cell-defining SSB on sync raster for SCell.

For SSB adaptation in time, the following agreement was made at RAN1 #117 [4].

Agreement For adaptation of SSB in time-domain, Option 1 is supported. - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g., two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB

The SSB adaptation in time with burst periodicity (i.e., Option 1) is similar to the on-demand SSB transmission cases. For example, the UE may be configured with at one or more on-demand SSB burst periodicities which can be activated with an indication of SSB adaptation transmission, e.g., the indication indicates not only the availability of SSB transmission but also the adaptation of the SSB burst periodicity.

It was agreed at RAN1#117 to support MAC CE based signaling to indicate on-demand SSB transmission for on-demand SSB operation in SCells. Given the commonality between on-demand SSB transmission and SSB transmission with burst periodicity adaptation, a unified MAC CE may be used to indicate both on-demand SSB transmission and SSB transmission with burst periodicity adaptation.

Observation 2. The indication for SSB adaptation in time with burst periodicity (i.e., Option 1) is similar to the indication for on-demand SSB transmission Case #2.

Proposal 2. RAN2 considers reusing the design for OD-SSB Case #2 to support the adaptation of SSB periodicity for SCell.

## Adaptation of PRACH in time domain

For adaptation of PRACH in time domain, the following agreement wwas made at RAN1 #116 for further study [2].

Agreement For adaptation of PRACH in time-domain, consider the following adaptation mechanisms for further study. - Adaptation based on configuration of additional[/different] PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any) - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs - For the additional PRACH resources, - Adaptation of PRACH resource periodicity/PRACH occasion - Adaptation at PRACH configuration/association period/association pattern period level and SSB to RO mapping cycle - Adaptation based on extending cell DRX operation for PRACH. - Concentrating ROs in time domain - Other options are not precluded

Additionally, the following agreements were made at RAN1 #116bis [3].

Agreement For adaptation of PRACH in time-domain, support at least the following: - Adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any) - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs - Configuration of additional PRACH resources is provided by semi-static signalling - FFS: details including whether there is overlap of additional PRACH resources and PRACH resources for legacy UEs - FFS: adaptation mechanism for additional PRACH resources - Note: No change to the existing PRACH configuration tables in 38.211 Agreement For adaptation of PRACH in time-domain, support the following: - SSB-RO mapping for the additional PRACH resources is separate from the SSB-RO mapping of the PRACH resources for legacy UEs (if any) - FFS: whether/how to handle SSB-RO mapping if the additional PRACH resources overlap in both time and frequency with the PRACH resources for legacy UEs - Note: SSB-RO mapping of the PRACH resources for legacy UEs is not impacted if Rel-19 UE uses these PRACH resources - FFS: SSB-RO mapping for the additional PRACH resources Agreement Support adaptation mechanisms of PRACH in time-domain for following: - UE in idle/inactive mode - UE in connected mode

Furthermore, following agreements were made at RAN1 #117 for the configuration of additional PRACH resources [4]:

Agreement For adaptation of PRACH in time-domain, support at least the following case(s) - Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 2: time-domain overlap but no overlap in frequency domain between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 3: additional PRACH resources for NES-capable UEs and legacy PRACH resources overlap neither in time nor frequency domains - FFS: whether additional conditions are needed to support the above cases - FFS: Additional case whether full/partial overlap in both time and frequency is allowed - Above does not preclude discussion for the case where the configuration for additional PRACH resources contains legacy PRACH resources Agreement For adaptation of PRACH in time-domain, the additional PRACH resources are configured based on at least: - a PRACH configuration index - FFS: whether the PRACH configuration index is same and/or different from the PRACH configuration index for the legacy PRACH resources Study further the following - When the PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resource, - Additional parameter(s) for determining the additional PRACH resources e.g. - Scaled/adjusted PRACH configuration period - Additional timing offset - Adjusting the parameters (e.g., (x, y) value and slot number) of the PRACH configuration - Muting/masking ROs - When the PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resource - Additional mechanisms (if any) for determining the additional PRACH resources e.g. - Muting/masking ROs (e.g. for the case when the PRACH configuration index for the additional PRACH resources contains legacy resources) - Additional parameters to facilitate condensed/cluster RACH resources in time-domain (including whether needed)

For configuration of additional PRACH resources, the following agreement was made in RAN1#118 [5].

Agreement For adaptation of PRACH in time-domain, select at least one from the following alternatives for configuration of the additional PRACH resources - Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources and - Discuss further additional mechanism(s) for determining the additional PRACH resources, e.g. - Opt 1-1: Scaled/adjusted PRACH configuration period - Opt 1-2: Adjusting the parameters (e.g., (x, y) value and slot number) of the PRACH configuration - Opt 1-3: Muting/masking ROs - Opt 1-4: additional timing offset(s) - Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources, - Discuss further additional mechanism(s) for determining the additional PRACH resources, e.g. - Opt 2-1: Muting/masking ROs (e.g. for the case when the PRACH configuration index for the additional PRACH resources contains legacy resources) - Opt 2-2: Additional timing offset(s) - FFS: Additional parameters to facilitate condensed/cluster RACH resources in time-domain (including whether needed) - FFS: Additional frequency domain parameter(s) (e.g., freq. starting offset)

At RAN1 #118bis, more agreements were made for supporting adaptation of PRACH in time-domain [5].

Agreement For adaptation of PRACH in time-domain, the same PRACH preamble format is used for the additional RACH resources and legacy PRACH resources. Agreement For adaptation of PRACH in time-domain, support both of the following - Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources - Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources - FFS: Additional details Working Assumption For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode. - P-RNTI is used Agreement For adaptation of PRACH in time-domain, the frequency domain resources for the additional PRACH resources and legacy PRACH resources can be same or different - FFS: applicable case(s) (i.e., case(s) from the RAN1#117 agreement). - Discuss further following options for signaling - Option 1: at least the following parameter(s) can be configured separately for the additional PRACH resources. - msg1-FrequencyStart at least for 4-step RACH - FFS: other applicable legacy frequency domain parameter(s) - Option 2: Offset to legacy frequency domain parameter(s) are configured for the additional PRACH resources - Note: Offset to legacy frequency domain parameter(s) is a new parameter - FFS: applicable legacy frequency domain parameter(s) Agreement For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain, - For Alt 1 (same PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s), - Opt 1-2a: up to N1 additional value(s) of (x, y) - FFS: value of N1(>=1) - Opt 1-2b: up to N2 additional value(s) of y - FFS: value of N2 (>=1) - Opt 1-3: Muting/masking ROs - Opt 1-4: - up to N4_1 additional timing offset(s) at frame-level - up to N4_2 additional timing offset(s) at slot-level - FFS: values of N4_1(>=1) and N4_2(>=1) - Opt 1-5: No additional mechanism is selected. - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211) - For Alt 2 (different PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s), - Opt 1-2a: up to N1 additional value(s) of (x, y) - FFS: value of N1(>=1) - Opt 1-2b: up to N2 additional value(s) of y - FFS: value of N2 (>=1) - Opt 1-3: Muting/masking ROs - Opt 1-4: - up to N4_1 additional timing offset(s) at frame-level - up to N4_2 additional timing offset(s) at slot-level - FFS: values of N4_1(>=1) and N4_2(>=1) - Opt 1-5: No additional mechanism is selected. - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211)

Proposal 3. For PRACH adaptation in time domain, RAN2 waits for RAN1’s further decisions.

## Adaptation of paging occasions

At RAN2 #127, the following agreement was made for paging adaptation [7].

Agreements on paging adaptation 1. Option-a) is about the bundling of PF for R19 NES UEs. 2. R2 observe that the option-a) and option-b) can be designed to configure the POs at same time position.

Additionally, the following agreements were made for paging adaptation in time at RAN2 #127bis [9].

Agreements 1. Select option-b as baseline for R19 NES paging enhancement. 2. R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values) 3. Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration. 4. Legacy UE is not barred. 5. Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.

As shown in Figure 3, with option b) distributed Paging Frames (e.g., removing the PF1), more PDCCH monitoring occasions are available for increased number of POs.

Figure 1: Adaptation with further distributed Paging Frames (with extra POs)

Proposal 4. To support coexist with legacy UEs, the new value of N (e.g., N = T/32) and new value of Ns (e.g., Ns = 8) are configured separately for Rel-19 UEs.

# Conclusion

In this contribution, we discussed scenarios or options for adaptation of SSB in time domain, adaptation of PRACH in time domain and adaptation of paging occasions and concluded with the following observations and proposals.

Adaptation of SSB in time domain

Observation 1. There is no consensus yet on the support of adaptation of SSB for idle mode UEs.

Proposal 1. For SSB adaptation impact to UEs’ operations at RRC Idle/Inactive state, RAN2 can wait for RAN1's further conclusion on adaptation options with cell-defining SSB on sync raster for SCell.

Observation 2. The indication for SSB adaptation in time with burst periodicity (i.e., Option 1) is similar to the indication for on-demand SSB transmission Case #2.

Proposal 2. RAN2 considers reusing the design for OD-SSB Case #2 to support the adaptation of SSB periodicity for SCell.

Adaptation of PRACH in time domain

Proposal 3. For PRACH adaptation in time domain, RAN2 waits for RAN1’s further decisions.

Adaptation of paging occasions

Proposal 4. To support coexist with legacy UEs, the new value of N (e.g., N = T/32) and new value of Ns (e.g., Ns = 8) are configured separately for Rel-19 UEs.

# References

[1] RP-242354 “Revised WID: Enhancements of network energy savings for NR”, RAN Meeting #105, Melbourne, Australia, Sep 9-12, 2024.

[2] “Chair notes RAN1#116 eom0” RAN1 #116	 Athens, Greece, February 26th – March 1st, 2024

[3] “Chair notes RAN1#116bis eom0” RAN1 #116bis	 Changsha, China, April 15th – 19th, 2024

[4] “Chair notes RAN1#117 eom0” RAN1 #117 Fukuoka, Japan, May 20th – 24th, 2024, 2024

[5] “Chair notes RAN1#118 eom0” RAN1 #118, Maastricht, NL, August 19th – 23rd, 2024

[6] “R2_125bis_SL_R19 NES_MOB_Session Notes (Kyeongin Jeong) _EoM”, RAN2 #125bis, Changsha, Hunan Province, China, April 15th – 19th, 2024.

[7] R2-2407571 “Report from session on V2X/SL, R18/19 MOB, and R19 NES” RAN2 #127, Maastricht, NL, August 19th – 23rd, 2024.

[8] “Chair notes RAN1#118bis eom0” RAN1 #118bis, Hefei, China, October 14th – 18th, 2024

[9] “Report from session on V2X/SL, R18/19 MOB, and R19 NES” RAN2 #127bis, Hefei, China, October 14th – 18th, 2024.