3GPP TSG-RAN WG2 Meeting #128	R2-2410442

Orlando, USA, 18th – 22nd of November 2024

Agenda item:	7.2.3

Source:	Nokia

Title:	Miscellaneous Rel-18 LTM Aspects and Corrections

WID/SID:	NR_Mob_enh2-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In this paper we are addressing several remaining issues for Release 18 LTM.

# 2	Discussion

## 2.1 	On the order of actions in Stage-2 LTM message sequence

As concluded in RAN4 and captured in [1], the requirements for PDCCH-ordered RACH apply only for the UEs that have provided L1- or L3-RSRP report (for FR1) or L1-RSRP (for FR2) :

### 6.2.2C PDCCH ordered Random Access for LTM #### 6.2.2C.1 Introduction The requirements in this clause apply for a UE capable of [FG 45-5: RACH-based early TA acquisition] configured to trigger RACH procedure toward target LTM candidate cell before receiving cell switch command MAC-CE. The requirements in this clause apply if PDCCH order-based RACH on candidate cell is triggered based on L1-RSRP report or L3-RSRP beam-level report in FR1, or based on L1-RSRP report in FR2.

Observation 1: As per RAN4 specification, PDCCH-ordered RACH requirements apply only for the UE that has provided L1- or L3-RSRP report (for FR1) or L1-RSRP report (for FR2).

Similar concerns are applicable to TCI state activation. As per RAN4 and their [1], TCI state activation can be done only for a known TCI state (i.e. UE has recently reported L1-RSRP for the RS associated to the TCI state) or, in case of FR1, for an unknown TCI state if the UE has reported L3 measurement result with the associated SSB index of the TCI state.

Observation 2: As per RAN4 requirements, TCI state activation can be done only for the known TCI state or, in case of FR1, for unknown TCI state if the UE has reported L3 measurement result with the associated SSB index of the TCI state.

This implies Figure 9.2.3.5.2-1 in [2] and/or the corresponding description might require an update as these were drafted prior to the aforementioned RAN4 requirements have been made.

Observation 3: Figure 9.2.3.5.2-1 and the corresponding description in TS 38.300 were drawn prior to the RAN4 LTM requirements.

There are two options how to resolve the issue: either the figure can be updated or one of the steps in the associated description should be extended with the information on the required L1-RSRP or L3-RSRP reporting.

The topic has been discussed at RAN2#127bis, both online and via offline thread (report available in [4]). During the discussions few points were raised on how to best address this issue (e.g. do nothing, modify the figure or remove the details and leave just a reference to 38.133). We think it might be acceptable to just point to 38.133, even if that means not all details are given in one single place (i.e. Stage-2 description). Thus, we suggest to agree the simple changes in Annex A.

Proposal 1: RAN2 is asked to adopt the changes to TS 38.300, depicted in Annex A.

## 2.2 	LTM PDCP data recovery

For the LTM execution procedure, if the “ltm-NoResetID” is not the same as the “VarLTM-ServingCellNoResetID”.  UE resets the RLC bearers for the cell group that triggered the cell change. For the MCG LTM, only the MCG RLC bearers will be reset. However, UE triggers PDCP data recovery for all the DRBs that is part of the current UE configuration, as such this would include trigger PDCP recovery for all bearers including MN terminated SCG bearers or SN terminated MCG bearers.

1> if the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID: 2> for each logicalChannelIdentity and logicalChannelIdentityExt that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered: 3> if servedRadioBearer is set to drb-Identity: 4> after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config; 2> for each bh-LogicalChannelIdentity that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered: 3> after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config; 2> for each drb-Identity value that is part of the current UE configuration: 3> if this DRB is an AM DRB: 4> after the end of this procedure, trigger the PDCP entity of this DRB to perform data recovery as specified in TS 38.323 [5], after applying the LTM configuration in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config;

Observation 4: UE triggers PDCP data recovery for all the DRBs that are part of the current UE configuration. This would include triggering PDCP recovery for all bearers including MN terminated SCG bearers and SN terminated MCG bearers.

Triggering PDCP data recovery for MN terminated SCG bearers is redundant. This is already captured in S-CPAC specification by triggering the PDCP data recovery only for the bearers that are associated with a re-established RLC entity. Similar behaviour should be adopted for LTM procedure.

##### 5.3.5.13.8 Subsequent CPAC execution … 4> if there is an associated SCG RLC bearer in the selected subsequent CPAC candidate configuration that is part of the current UE configuration: 5> re-establish the SCG RLC entity as specified in TS 38.322 [4]; 4> if the RLC entity of the associated RLC bearer(s) is re-established; or 4> if an associated RLC bearer is released in the selected subsequent CPAC candidate configuration: 5> if the bearer is an AM DRB: 6> trigger the PDCP entity of the bearer to perform PDCP data recovery as specified in TS 38.323 [5];

Proposal 2: Adopt the TP in Annex B.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: As per RAN4 specification, PDCCH-ordered RACH requirements apply only for the UE that has provided L1- or L3-RSRP report (for FR1) or L1-RSRP report (for FR2).

Observation 2: As per RAN4 requirements, TCI state activation can be done only for the known TCI state or, in case of FR1, for unknown TCI state if the UE has reported L3 measurement result with the associated SSB index of the TCI state.

Observation 3: Figure 9.2.3.5.2-1 and the corresponding description in TS 38.300 were drawn prior to the RAN4 LTM requirements.

Proposal 1: RAN2 is asked to adopt the changes to TS 38.300, depicted in Annex A.

Observation 4: UE triggers PDCP data recovery for all the DRBs that are part of the current UE configuration. This would include triggering PDCP recovery for all bearers including MN terminated SCG bearers and SN terminated MCG bearers.

Proposal 2: Adopt the TP in Annex B.

# References

- 3GPP TS 38.133 NR; Requirements for support of radio resource management, V18.6.0 (2024-07)

- 3GPP TS 38.300 NR; NR and NG-RAN Overall Description; Stage 2, V18.3.0 (2024-09)

- 3GPP TS 38.331 Radio Resource Control (RRC) protocol specification, V18.3.0 (2024-09)

- R2-2409356 Report from [AT127b][104][MOB] (Nokia) 3GPP TSG-RAN WG2 Meeting #127bis Hefei, China, 14th – 18th of October 2024

# Annex A – TP for the order of actions in Stage-2 LTM message sequence

##### 9.2.3.5.2	C-Plane Handling

Cell switch command is conveyed in a MAC CE, which contains the necessary information to perform the LTM cell switch.

The overall procedure for LTM is shown in Figure 9.2.3.5.2-1 below. Subsequent LTM is done by repeating the early synchronization, LTM cell switch execution, and LTM cell switch completion steps without releasing other LTM candidate configurations after each LTM cell switch completion. The general procedure over the air interface is applicable to SCG LTM. Further details of SCG LTM can be found in TS 37.340 [21].

Figure 9.2.3.5.2-1. Signalling procedure for LTM

The procedure for LTM is as follows:

1.	The UE sends a MeasurementReport message to the gNB. The gNB decides to configure LTM and initiates LTM preparation.

2.	The gNB transmits an RRCReconfiguration message to the UE including the LTM candidate configurations.

3.	The UE stores the LTM candidate configurations and transmits an RRCReconfigurationComplete message to the gNB.

4a.	The UE performs DL synchronization with the LTM candidate cell(s) before receiving the cell switch command. The UE may activate and deactivate TCI states of LTM candidate cell(s), as triggered by the gNB and defined in TS 38.133[13].

4b.	The UE may perform UL synchronization with LTM candidate cell(s) before receiving the cell switch command, by using UE-based TA measurement, if configured, and/or by transmitting a preamble towards the candidate cell, as triggered by the gNB. When UE-based TA measurement is configured, UE acquires the TA value(s) of the candidate cell(s) by measurement. UE performs early TA acquisition with the candidate cell(s) as requested by the network before receiving the cell switch command as specified in clause 9.2.6 and TS 38.133[13]. This is done via CFRA triggered by a PDCCH order from the source cell, following which the UE sends preamble towards the indicated candidate cell. In order to minimize the data interruption of the source cell due to CFRA towards the candidate cell(s), the UE does not receive random access response from the network for the purpose of TA value acquisition and the TA value of the candidate cell is indicated in the cell switch command. The UE does not maintain the TA timer for the candidate cell and relies on network implementation to guarantee the TA validity.

5.	The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the gNB. L1 measurement should be performed as long as RRC reconfiguration (step 2) is applicable.

6.	The gNB decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

7.	The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

8.	The UE completes the LTM cell switch procedure by sending RRCReconfigurationComplete message to target cell. If the UE has performed a RA procedure in step 7 the UE considers that LTM cell switch execution is successfully completed when the random access procedure is successfully completed. For RACH-less LTM, the UE considers that LTM cell switch execution is successfully completed when the UE determines that the network has successfully received its first UL data.

The steps 4-8 can be performed multiple times for subsequent LTM cell switch executions using the LTM candidate configuration(s) provided in step 2.

The procedure over the air interface described in Figure 9.2.3.5.2-1 is applicable to both intra-gNB-DU LTM and inter-gNB-DU LTM. The overall LTM procedures over F1-C interface are captured in TS 38.401[4].

# Annex B – TP for PDCP data recovery

##### 5.3.5.18.6	LTM cell switch execution

Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, the UE shall:

…

Unchanged parts skipped.

1>	if the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 does not contain the field ltm-NoResetID and if the UE does not have any value stored of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID; or

1>	if the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID:

2>	for each logicalChannelIdentity and logicalChannelIdentityExt that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

3>	if servedRadioBearer is set to drb-Identity:

4>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config;

2>	for each bh-LogicalChannelIdentity that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

3>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config;

3>	for each drb-Identity value that is part of the same RLC entity that is re-established:

4>	if this DRB is an AM DRB:

5>	at the end of this procedure, trigger the PDCP entity of this DRB to perform data recovery as specified in TS 38.323 [5], after applying the LTM configuration in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config;

2>	if the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID:

3>	replace the value of ltm-ServingCellNoResetID in VarLTM-ServingCellNoResetID with the value of ltm-NoResetID in the LTM-Candidate in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3;

…