3GPP TSG-RAN WG2 Meeting #128	R2-2410444

Orlando, US, Nov 18th – 22nd, 2024

Source: 	ZTE Corporation

Title:            Discussion on L1 event triggered measurement reporting

Agenda item:	8.6.3

Document for:	Discussion and decision

Introduction

At last meeting, we discussed event-triggered L1 measurement reporting for LTM and made the following agreement:

Agreements on L1 event triggered MR

- MR can be sent when the leaving condition is met, based on NW configuration.

- Event triggered periodic MR can be supported, based on NW configuration.

- For measurement resource configuration, R18 LTM CSI resource configuration is reused if possible. If CSI-RS resource only IE needs to be defined, we can revisit it in the stage 3.

- For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused if possible. We can revisit it in the stage 3 if needed.

- For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible. We can revisit it in the stage 3 if needed.

- The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1.

- TTT operates only based on a timer (like TTT used in L3 event triggered MR).

- Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5).

- Basic information included in MR MAC CE:

- Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id)

- Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams

- Triggered event information (e.g., ReportConfigID)

MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not).

N is configurable by NW.

- Additional information included in MR MAC CE:

- The information and quantity of current beam, based on NW configuration.

11.  The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE.

12.	NW can configure a dedicated SR configuration for MR MAC CE transmission.

In this contribution, we discussed some remaining issues for L1 event triggered measurement reporting.

Discussions

Event evaluation

RS type

In RAN2#127bis, RAN2 confirms the working assumption that same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. However, how to ensure the same RS type for serving and neighbour cells is still FFS because no consensus was made in last RAN2 meeting.

In RAN2#127 meeting, it was agreed that the beam of the serving cell is the beam corresponding to the indicated TCI state, referred to as the "current beam", which is used for event evaluation in L1 measurement reporting. From network implementation point of view, for beam management, most likely finer beam (e.g. CSI-RS) will be used, which means the beam indicated by TCI state will be CSI-RS. However, for neighbor cell L1 measurements, the network may want to use SSB because it is simply and it does not require additional UE capability. In this case, to ensure same RS type is used for serving and neighbour cell in even evaluation, we propose to use the beam associated with QCL RS provided in the indicated TCI state for the serving cell evaluation, i.e. the same RS type as the candidate beam. But some companies think the UE

Meanwhile, we notice that RAN1 already made conclusion at RAN1 last meeting, that QCLed beam can be used for serving cell.

RAN1#118bis-Agreement

The serving cell RS for event evaluation is at least derived from QCL RS or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell

- QCL RS above is the RS w.r.t. QCL-TypeD when the indicated joint/DL TCI state is configured with two QCL RSs.

- FFS: Details on determination of QCL RS or SSB QCLed with QCL RS.

Note: This does not imply the support of mTRP scenarios.

RAN1 already agreed in last meeting that the beam QCLed with the RS beam indicated by TCI state can be used for serving cell evaluation.

From RAN2 perspective, we suggest to confirm this approach, for the details, RAN2 can wait for RAN1.

- If measured RS type for candidate cell is different from the RS type that indicated by serving cell’s TCI-state, the UE uses the RS beam that QCLed with the RS beam indicated TCI-state for serving cell (in event LTM3 and event LTM5 evaluation), details up to RAN1.

RS type configuration

In previous RAN2 meeting, we agreed that both SSB and CSI-RS are supported as the measurement resource for event-triggered measurement report. And RAN2 also agreed to reuse Rel-18 LTM-CSI-ResourceConfig-r18 as a baseline. So, it’s possible to configure the SSB resources and CSI-RS resources in one measurement resource configuration, but the NW may want to enable the event-triggered measurement report for only one type of measurement resource. Thus, it would be more flexible to allow the NW indicate which RS is used to provide the measurement result for the LTM event evaluation in the report configuration. This is similar to legacy L3 measurement reporting configuration.

- Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation, e.g. introduce rsType in the LTM event report configuration.

TTT evaluation

In RAN2#127bis, RAN2 agreed that TTT is evaluated only based on timer (no counter will be introduced). However, how TTT is evaluated is FFS due to no consensus. So far, there are three options on the table:

- Option 1: TTT is evaluated per-beam, i.e. the UE maintains separate TTT timer for each beam of the candidate cell. For a given target beam, the corresponding TTT shall reset if the beam fulfills leaving condition;

- Option 2: TTT is evaluated per-cell, i.e. the TTT shall keep running as long as at least one beam of the candidate cell meets the entering condition of the LTM event;

- Option 3: TTT is evaluated per CSI-RS resource set, i.e. the TTT shall keep running as long as at least one beam of configured in the same CSI-RS resource set meets the entering condition of the LTM event.

Among the three options, we have strong concern on Option 1, from NW perspective, after configuring multiple LTM candidate cells, the first thing that network care is which cell should be select, irrespective of which beam should be selected. Some companies think if the TTT is evaluated per-cell, when MR is triggered, the best beam may not be stable enough because the beam quality may satisfy the threshold for less than duration TTT. However, we don’t think this is a big problem for handover, although the best beam changes, as long as any one beam fulfills the threshold, it means the cell is good enough compared with other candidate cells.

For option 1, considering that the L1 measurement result of a specific beam may fluctuate larger than L3 measurement result, e.g. due to the lack of filtering, the TTT criteria may be not easily satisfied continuously. Thus, the triggering of the L1 report and LTM cell switch may be delayed due to the TTT reset, e.g. caused by the beam level measurement result fluctuation.

Considering that the L1 measurement result of a specific beam may fluctuate larger than L3 measurement result, it may be not easy to meet the TTT criteria continuously for a specific beam. If the TTT is evaluated per beam, the triggering of the L1 measurement report and LTM cell switch may be delayed due to the TTT reset.

An example of the TTT evaluation (per beam vs. per cell) is shown as below:

Figure 1. An example of the TTT evaluation (per beam vs. per cell)

As shown in the Figure 1, two candidate beams of a cell meet the threshold of the LTM event in different time points. If the TTT is evaluated per beam, the TTT shall be reset if the beam becomes worse than the threshold, and restart when the beam becomes better than the threshold again. The L1 reporting shall be triggered only when a beam continuously meets the threshold. But if the TTT is evaluated per cell, the TTT shall keep running as long as there is a beam of the cell can meet the threshold, which follows the similar logic as the TTT for L3 measurement evaluation. So, option 2 is preferred.

For option 3, we understand it provides more flexibility. For example, for a given candidate cells, the NW can configure two separate CSI-RS resource set that contains different beams, so the event can be evaluated for a specific set of beams. However, for mobility purpose, we haven’t seen clear use case for it. On the other hand, if the network configures the beams of two candidate cells within one CSI-RS resource set, it is unclear how TTT will be evaluated and how MR is triggered. For simplicity, we think option 2 is enough for Rel-19.

- For the LTM event evaluation, TTT is evaluated per cell, i.e. not per beam or per resource set. The TTT shall keep running as long as at least one beam of the cell meets the entering condition of the LTM event.

Another issue is whether the serving cell beam changes will reset TTT evaluation. Some companies commented that if serving cell beam changes, it means the serving quality becomes better and TTT evaluation should be restarted. However, in our view, even if the serving beam changes, as long as the serving and candidate still meet the thresholds (for event LTM 3 and LTM 5), the TTT should not stop, otherwise, the MR will also be delayed.

- TTT does not reset if the serving cell beam which used for event evaluation is changed.

Cell specific offset

Besides, it’s agreed that beam specific offset can be applied for LTM event evaluation at RAN2#126 meeting [2], but it’s FFS whether cell specific offset is applied as well.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

In the current RRM event evaluation, both cell specific offset and MO specific offset can be applied to adjust the threshold for serving cell and/or neighbour cell. Correspondingly, both cell specific offset and beam specific offset can be considered for LTM event evaluation. Especially, considering that beams from multiple cells may be grouped in one measurement resource set for the LTM event, the cell specific offset is beneficial for the more flexible adjustment of the event threshold applicable to different cells.

- The cell specific offset can be applied for LTM event evaluation.

Content of measurement report

Beam info of candidate cell

At last meeting, RAN2 agreed that the MR MAC CE can include up to N beams and N is configured by the NW, however, it is FFS whether the reported beams should fulfill the event threshold. In our view, the reported beam information can be used to decide the active TCI-state or for triggering early RACH...it can also be used to update the candidate configuration (RACH config).

From network perspective, if TTT is evaluated per-cell, after receiving the MR MAC CE, the NW knows which candidate cell is good enough, regarding the beam information, the best beam matters more, and we don’t see clear benefit to include the beam that does not fulfill the threshold. So, if the UE only reports the beams which met the event entering condition, it can further reduce the MR size.

- In MR MAC CE, the UE mandatorily includes the best beam of candidate cell, in addition to the best beam, the UE includes up to N-1 beams of candidate cell which fulfills the event entering condition (does not need to meet TTT).

Beam info of serving cell

In RAN2 previous meeting, companies discussed whether to always report serving cell measurement results in event-triggered report. For this issue, we notice that RAN1 has made below agreements in last RAN1 meeting:

RAN1#118bis-Agreement

The agreement “Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting” made in RAN#118 is further clarified for L1-RSRP as follows:

- UCI format defined in Table 6.3.1.1.2-8C of TS38.212 can be used by replacing SSBRI with CRI.

- Whether the L1-RSRP(s) of serving cell is always included is configurable (in line with Rel-18)

- The quantization method defined in clause 5.2.1.4.3 of TS38.214 and bit width defined in Table 6.3.1.1.2-6 of TS38.212 can be used

- No L1 specified filtering for time and spatial domain is introduced

- No enhancement on how to report L cells x M beams

- Periodic reporting on PUCCH is supported

  - FFS: semi-persistent reporting on PUCCH/PUSCH, and aperiodic reporting on PUSCH

Based on above agreements, RAN1 already agreed that NW can configure UE whether to always report serving cell measurement results, same as in Rel-18.

RAN1 already agreed in last meeting that the including of serving cell measurement results supported and it is configurable.

From RAN2 perspective, we suggest to confirm this approach in RAN2 and in our understanding, the configuration should be per reporting config.

- Network can configure (per reporting config) whether the UE should always include serving cell measurement results in MR MAC CE.

Besides, RAN2 can further discuss how many beams of serving cell can be reported? and which beam(s) should be reported? Firstly, as we proposed in Proposal 1, for serving cell measurement, the beam used for event evaluation should be have the same RS type as for the beams of candidate cell. So, in the MR, it makes sense to include serving beam that are used for event evaluation, this may not be the one that indicated by TCI-state, but QCLed.

On the other hand, as we proposed in Proposal 4, we think the change of serving beam should not reset TTT, considering the UE only use one serving beam for event evaluation at a time. We think it is sufficient to report only the last beam info which used for event evaluation.

- In MR MAC CE, if configured, the UE includes the measurement results of latest serving cell beam which is used for event evaluation. (i.e. it can be the beam indicated by TCI-state or the beam QCLed with the beam indicated by TCI-state depending on configured rsType).

In legacy L3 measurement report, to reduce the size of MR, network can configure whether only beam index is reported or both beam index and beam results are reported. We think the similar principle can also be applied to L1 event reporting.

On the other hand, as we proposed in Proposal 4, we think the change of serving beam should not reset TTT, considering the UE only use one serving beam for event evaluation at a time. We think it is sufficient to report only the last beam info which used for event evaluation.

- Network can configure (per reporting config) whether only beam level measurement results should be reported, if not configured, the UE only reports beam index(es).

Conclusion and Proposals

In this contribution, we discussed the event-triggered L1 measurement reporting with the following observations and proposals:

Observation 1:	RAN1 already agreed in last meeting that the beam QCLed with the RS beam indicated by TCI state can be used for serving cell evaluation.

Observation 2:	Considering that the L1 measurement result of a specific beam may fluctuate larger than L3 measurement result, it may be not easy to meet the TTT criteria continuously for a specific beam. If the TTT is evaluated per beam, the triggering of the L1 measurement report and LTM cell switch may be delayed due to the TTT reset.

Observation 3:	RAN1 already agreed in last meeting that the including of serving cell measurement results supported and it is configurable.

Event evaluation

Proposal 1:	If measured RS type for candidate cell is different from the RS type that indicated by serving cell’s TCI-state, the UE uses the RS beam that QCLed with the RS beam indicated TCI-state for serving cell (in event LTM3 and event LTM5 evaluation), details up to RAN1.

Proposal 2:	Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation, e.g. introduce rsType in the LTM event report configuration.

Proposal 3:	For the LTM event evaluation, TTT is evaluated per cell, i.e. not per beam or per resource set. The TTT shall keep running as long as at least one beam of the cell meets the entering condition of the LTM event.

Proposal 4:	TTT does not reset if the serving cell beam which used for event evaluation is changed.

Proposal 5:	The cell specific offset can be applied for LTM event evaluation.

Content of measurement report

Proposal 6:	In MR MAC CE, the UE mandatorily includes the best beam of candidate cell, in addition to the best beam, the UE includes up to N-1 beams of candidate cell which fulfills the event entering condition (does not need to meet TTT).

Proposal 7:	Network can configure (per reporting config) whether the UE should always include serving cell measurement results in MR MAC CE.

Proposal 8:	In MR MAC CE, if configured, the UE includes the measurement results of latest serving cell beam which is used for event evaluation. (i.e. it can be the beam indicated by TCI-state or the beam QCLed with the beam indicated by TCI-state depending on configured rsType).

Proposal 9:	Network can configure (per reporting config) whether only beam level measurement results should be reported, if not configured, the UE only reports beam index(es).