3GPP TSG-RAN WG2 Meeting #128		R2-2410490

Orlando, USA, 18th – 22nd Nov, 2024

Agenda Item: 	8.7.6

Source:		Sony

Title:	Recommended bit rate based XR rate control

Document for:	Discussion & Decision

# Introduction

RAN2_127bis agreed for uplink congestion signaling:

- FFS if the indication is per DRB or per QoS flow. Companies should analyse the impact on QoS enforcement, interworking with L4S etc.

In this contribution we think that there is a benefit of supporting per QoS flow. However, it depends if it is feasible to implement it per QoS flow. Otherwise, the indication should be specified per DRB.

# Discussions

RAN aware XR rate control can help the XR application to adjust its codec bit rate according to radio conditions and ensure user experience. RAN2 agreed in the past that it is feasible for RAN to estimate the congestion information per QoS flow and per DRB in both UL and DL direction. For UL, RAN can do it without any UE impacts. If a RAN node is congested, then it can signal the congestion indication to the XR application which in turn will adjust the XR data rate for DL direction. At the same time, RAN should be able to reduce the data rate for XR application on the uplink direction.

For rate control, we think the use of Recommended bit rate MAC-CE can be enhanced and used in this case. Existing RRC messages can be used for this purpose.

Proposal 1: Both UL/DL bit rate recommendation and UL/DL bit rate recommendation query RRC messages are supported for XR UL rate control.

On the question of per QoS flow/per DRB, current MAC-CE is per LCH ID. Some companies argued that per DRB indication wont allow QoS enforcement and it is not at the same level as L4S, which is per QoS flow. We think that gNB will receive the indication that rate control needs to be performed for certain QoS flow(s). gNB has the option of indicating both per QoS flow or per DRB to the UE and both options are feasible. If the indication is per QoS flow and more than one service’s QoS flows are mapped to a single DRB then the UE application layer may need to lower the data rate provided this indication of per QoS flow is passed to the UE application layer. However, if such flexibility does not exist in the UE implementation between modem and application chipset then this indication of per QoS flow is not useful. In the absence of this information, rate control is applied over radio and application layer will eventually learn that the rate control has been applied and may adjust its codec rate.

Proposal 2: On per QoS flow/per DRB, we see the benefit of per QOS flow indication. But it should be supported only if implementable, otherwise per DRB is defined.

# Summary

In this contribution, we have the following proposals for RAN2 to agree:

Proposal 1: Both UL/DL bit rate recommendation and UL/DL bit rate recommendation query RRC messages are supported for XR UL rate control.

Proposal 2: On per QoS flow/per DRB, we see the benefit of per QOS flow indication. But it should be supported only if implementable, otherwise per DRB is defined.

# References

- RP-241771, “Revised WID on XR (eXtended Reality) for NR Phase 3,” RAN#105.

- TR 23.700-60, SA2, Study on XR (Extended Reality) and media services