3GPP TSG-RAN WG2 #128	R2-2410542

Orlando, USA, November 18th – 22nd, 2024

Agenda item:	8.18

Source: 	Ericsson

Title:  	Quality Indication in Msg3 for SDT

Document for:	Discussion and Decision

# Introduction

In the WID [1] for MT-SDT it is stated that the objective is

“Specify the support for paging-triggered SDT (MT-SDT) [RAN2, RAN3]

- MT-SDT triggering mechanism for UEs in RRC_INACTIVE, supporting RACH procedure based and CG-SDT procedure based UL response;

MT-SDT procedure for initial DL data reception and subsequent UL/DL data transmissions in RRC_INACTIVE.”

This contribution primarily discusses the scheduling restrictions owing to bandwidth part constraints in MT-SDT. We further propose some solutions to enhance the transmission capability for MT-SDT.

# Discussion

## BWP Restriction in MT-SDT

Rel-17 agreements in the MO-SDT WI restricts small data transmissions over initial bandwidth part (BWP); the reason being to render SDT procedure as close to legacy initial access procedure as possible.

For UL transmissions initiated as part of MO-SDT, frequency resources across the entire initial UL BWP are available. However, for DL transmission, initial BWP operation restricts the frequency domain resource allocation to the size of CORESET 0. In specification, this is detailed shown below.

For first DL MT-SDT data, the frequency domain resource assignment is provided in TS 38.212 [3] as follows.

3GPP TS 38.212 18.4.0

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by TC-RNTI: - Identifier for DCI formats - 1 bit - The value of this bit field is always set to 1, indicating a DL DCI format - Frequency domain resource assignment - bits - is the size of CORESET 0

The same restriction is applicable for subsequent transmissions as well. In TS 38.212, this is provided as follows.

3GPP TS 38.212 18.4.0 – Paragraph 7.3.1.2.1

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: - Identifier for DCI formats - 1 bits - The value of this bit field is always set to 1, indicating a DL DCI format - Frequency domain resource assignment - bits where is given by Clause 7.3.1.0

3GPP TS 38.212 18.4.0 – Paragraph 7.3.1.0

Determine DCI format 1_0 monitored in a common search space according to clause 7.3.1.2.1 where is given by - the size of CORESET 0 if CORESET 0 is configured for the cell; and - the size of initial DL bandwidth part if CORESET 0 is not configured for the cell.

According to TS 38.331 [2], the UE applies this restriction until after the reception of RRCSetup/RRCResume/RRCReestablishment when it switches to Connected mode.

Therefore, we make the following observation.

MT-SDT operation over initial BWP restricts DL MT-SDT transmissions to the bandwidth of CORESET 0 in RRC_INACTIVE state.

For 30kHz subcarrier spacing, this means the scheduler can allocate a maximum of up to 48 PRBs within CORESET 0 [4]. If no DL quality indication is introduced in MT-SDT, a conservative MCS should be used to schedule MT-SDT PDSCH and PDCCH, e.g with low MCS or even MCS index 0 and high CCE aggregation level. Note that at MCS 0, the TB allocation can be as small as 1320 bits for 48 PRBs. This results in a larger number of subsequent transmissions.

MT-SDT BWP restriction with no DL quality feedback results in smaller, more conservative TB allocations in the downlink at low MCS values. For larger data volumes, smaller TBS translate to a higher number of subsequent transmissions.

Since higher number of subsequent transmissions considerably reduce the energy efficiency of SDT, RAN2 should discuss solutions to overcome the BWP restriction in MT-SDT and some options are:

Option 1: Remove the CORESET 0 restriction for DL BWP for SDT from/after first DL MT-SDT data

This option involves considerable changes to RAN 1 specifications: especially in TS 38.212 and TS 38.214, and some minor modifications to TS 38.331. This implies that a new WI would be necessary. In addition, this option would require BWP switching and it is not clear how this could be performed prior to reception of Msg4.

Option 2: Introduce Channel Quality Indication (CQI) in Message 3 or MsgA

An indicator of the channel quality from the UE allows the scheduler to adapt to better modulation and coding rates at good radio channel conditions. Even with a restriction to 48 PRBs, this allows for considerably larger TB sizes.

This would require MT-SDT data volumes to be substantially lower than that for MO-SDT (MO-SDT allowed DVT to be as high as 96000 bytes) to be efficient compared to going to RRC_CONNECTED mode. We think that the restriction reduces the usefulness of the MT-SDT procedure since the gNB will move the UE to connected mode at smaller data volumes than would be needed.

RAN2 addresses the BWP restriction in MT-SDT by introducing a Channel Quality Indication (CQI) in Msg3 or MsgA.

# Conclusion

In the previous sections we made the following observations:

Observation 1	MT-SDT operation over initial BWP restricts DL MT-SDT transmissions to the bandwidth of CORESET 0 in RRC_INACTIVE state.

Observation 2	MT-SDT BWP restriction with no DL quality feedback results in smaller, more conservative TB allocations in the downlink at low MCS values. For larger data volumes, smaller TBS translate to a higher number of subsequent transmissions.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 addresses the BWP restriction in MT-SDT by introducing a Channel Quality Indication (CQI) in Msg3 or MsgA.

# References

3GPP RP-222993. “New WI: Mobile Terminated-Small Data Transmission (MT-SDT) for NR”

3GPP 38.331, 18.3.0, NR; Radio Resource Control (RRC); Protocol specification

- [3]	3GPP 38.212, 18.4.0, NR; Multiplexing and channel coding

- [4]	3GPP 38.214, NR; Physical layer procedures for data