3GPP RAN WG2 Meeting #128	R2-2410549

Orlando, USA, November 18th – 22nd 2024

Agenda Item:	8.8.2

Source:	InterDigital

Title:	Downlink coverage enhancement for non-terrestrial networks

Document for:	Discussion, Decision

# Introduction

An objective for Rel-19 NR non-terrestrial networks (NTN) is to study DL coverage impacts and modifications to support dynamic power sharing among satellite beams. Enhancements can be considered at both the link level (e.g., improving the link margin of selected physical channels) and system level (e.g., supporting dynamic and flexible power sharing between beams). Detailed objectives include the following [1]:

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

Work in RAN1 (e.g., RAN1#116 [2] and RAN1#116b [4] and RAN1#117 [6]) has focused on defining additional reference satellite parameters, evaluation metrics, and assumptions.

This contribution discusses access control and system-level aspects of dynamic power sharing, specifically Cell barring, reselection to a cell oeparting with DL coverage enhancements (CE), and other aspects related to deriving the state of dynamic power sharing such as additional assistance information.

# Discussion

TR 38.821 [3] defines satellite parameters including EIRP (Equivalent Isotropic Radiated Power) per beam. Recent discussion has revealed the parameter set doesn’t account for power limitations onboard a satellite, and for satellites with many beams (e.g., 1200) power must be split among different beams or turned off entirely. This results in lower EIRP density per beam and worse DL coverage than originally expected. Rel-19 NTN is therefore introducing DL coverage enhancements to address such a scenario.

## Cell barring

The following was agreed in RAN2#127bis [8] regarding access control for UEs supporting Rel-19 NTN downlink coverage enhancements:

- If it turns out that there is a need to bar UEs not supporting DL-CE, then we need to introduce a barring mechanism to control access of UEs supporting Rel19 NTN DL-CE. FFS on the details. (This also implies that UEs supporting Rel19 NTN DL-CE will not consider the existing NTN barring bit)

One option is to re-purpose an existing barring bit other than cellBarredNTN for Rel-19 NTN DL coverage enhancements. For example, there is already specified support for cell barring due to cell DTX/DRX in the NES context via cellBarredNES. If a SIB1 containing cellBarredNES originates from an NTN cell, the UE may consider the cell DTX/DRX is related to NTN DL coverage enhancements instead of NES. This solution is, however, complex and may limit future combination of NES and NTN features.

Alternatively, a new barring bit (e.g., cellbarredDLcov) can be introduced for UEs supporting Rel-19 NTN DL coverage enhancements. A new barring bit is clear and in line with how cell DTX/DRX has been handled in NES, however, may require additional (but minimal) specification effort and complexity. Considering the simplicity and future-proof nature of a new barring bit  it is proposed a new barring bit is introduced to control access of UEs supporting Rel19 NTN DL-CE (if it turns out that there is a need to bar UEs not supporting DL-CE).

Proposal 1a:	A new barring bit is introduced to control access of UEs supporting Rel-19 NTN DL-CE.

With the introduction of a new barring bit to control access of UEs supporting Rel-19 NTN DL-CE, RAN2 may also confirm that such UEs will not consider the existing NTN barring bit, as per previous agreement.

Proposal 1b:	Confirm that UEs supporting Rel-19 NTN DL-CE will not consider the existing NTN barring bit.

## Down-prioritising/prevent re-selection to a cell operating with DL-CE

In RAN2#127bis, RAN2 made the following agreements regarding prevention of DL-CE not capable UEs from accessing cells operating with DL-CE [8].

Agreements:

1.	If it turns out that there is a need to bar UEs not supporting DL-CE, Rel-19 UEs not supporting DL-CE can be barred from accessing a cell operating with DL-CE using the existing NTN bar bit, in the same way as pre-Rel19 NTN UEs (this is an extension of the previous agreement to include also Rel-19 UE not supporting DL-CE)

2.	If it turns out that there is a need to bar UEs not supporting DL-CE, then we need to introduce a barring mechanism to control access of UEs supporting Rel19 NTN DL-CE. FFS on the details. (This also implies that UEs supporting Rel19 NTN DL-CE will not consider the existing NTN barring bit)

3.	(also depending on the details of the RAN1 solution) we can further consider methods to allow UEs not supporting DL CE to down-prioritize or prevent re-selection to cells operating with DL CE.

To prevent UEs not supporting DL-CE from accessing cells operating with DL-CE, only existing IEs can be used for the legacy UEs while new IEs can be used for Rel-19 UEs.

Observation 1:  Only existing IEs can be used to handle legacy UEs to prevent from accessing to the cells operating with DL-CE.

There are at least 2 potential solutions for the down-prioritisation/prevention of reselection to the cell operating with DL-CE: 1) IFRI (intra-frequency reselection IE) based; and 2) Absolute priority based

1. IFRI based prevention of reselection to cells operating with DL-CE

MIB ::=                             SEQUENCE {

systemFrameNumber                   	BIT STRING (SIZE (6)),

subCarrierSpacingCommon             	ENUMERATED {scs15or60, scs30or120},

ssb-SubcarrierOffset                	INTEGER (0..15),

dmrs-TypeA-Position                 	ENUMERATED {pos2, pos3},

pdcch-ConfigSIB1                    	PDCCH-ConfigSIB1,

cellBarred                          		ENUMERATED {barred, notBarred},

intraFreqReselection                	ENUMERATED {allowed, notAllowed},

spare                               		BIT STRING (SIZE (1))

}

intraFreqReselection

Controls cell selection/reselection to intra-frequency cells when the highest ranked cell is barred, or treated as barred by the UE, as specified in TS 38.304 [20]. This field is ignored by IAB-MT and NCR-MT.

IFRI was introduced for NR in Rel-15 and the IE is present in MIB:

This IE is set to “notAllowed” when the operator doesn’t expect any UEs to reselect to any cells on the same frequency when the current cell is barred. This is useful for the case that an operator deploys cells used only for SCells with CA operation on a specific frequency and no UEs should be camped on the SCells.

This mechanism would work perfectly fine for the DL-CE case if operators would deploy the cells operating with DL-CE on a specific frequency like legacy. However, there is a problem with this solution. There is no such IE defined when the NTN bar bit was introduced and so the legacy UEs won’t be able to be controlled by the IFRI for NTN as it’s defined in Rel-19 and only Rel-19 UEs would be able to follow the rule (i.e. against Observation 1 requirement).

2. Absolute priority based down-prioritisation of cells operating with DL-CE

Absolute priority reselection was introduced for NR in Rel-15. The absolute priority values are signalled by SIB4 and SIB5 and all the legacy UEs can follow the absolute priority information given by the existing signalling (i.e. SIB4 and SIB5).

So we can down-prioritise the frequency, where cells operating with DL-CE are deployed by setting the lowest absolute priority value for the DL-CE frequency in SIB4. It satisfies the requirement given by Observation 1 above.

In addition to that, the down-prioritisation shouldn’t be applied for the UE supporting DL-CE as those UEs are able to camp on the cell operating with DL-CE. Thus, we need to apply different absolute priority values for the UEs supporting DL-CE so that the network can let the UEs supporting DL-CE cell reselecting to the cells operating with DL-CE by applying reasonable absolute priority values for the frequency, where the cells operating with DL-CE are deployed. Thus, we propose;

Proposal 2:	Define absolute priority information for DL-CE capable UEs.

## Dynamic power sharing

Other DL coverage enhancements under consideration for Rel-19 NTN include system level enhancements to allow dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. Dynamically changing satellite beam power, pattern, or size affects downlink coverage. Coverage variation impacts not only channel reliability and throughput but system-level performance, reducing the effectiveness of various procedures like measurement reporting, mobility, and cell (re)selection.

Observation 2:	Changes to beam power sharing/pattern/size can impact many aspects of the system (e.g., application of DL coverage enhancements, measurement reporting, CHO execution etc.)

Adjusting reactively (e.g., based on measurement reporting) to a drop in DL coverage risks sub-optimal configuration or even connection loss if issues are not detected/corrected early enough. This is especially important in NTN, where large RTT can introduce significant delays in addressing coverage issues.

Modifying beam power, size or patterns are fully under network control, and thus predictable. A network could notify affected UEs of upcoming power sharing, along with other information like magnitude of power reduction, changes to beam size/pattern, when the changes will apply/end etc. Considering the deterministic nature of satellite movement, this information could also be provided for one or more upcoming neighbour cells/satellites.

Observation 3:	Changes to beam power sharing/pattern/size are based on NW decision and thus predictable. Notifying UEs of upcoming changes can reduce undesirable UE behaviour.

Providing network assistance information on the current or planned power sharing state can inform the UE of how power sharing will impact DL coverage over time, reducing the risk of RLF and service disruption.

Proposal 3:	UE can be notified of changes to beam power sharing/pattern/size for both serving and neighbouring cell(s).

Predicting when a UE will be impacted by a power sharing state change is straightforward in (quasi)earth fixed cells since the impacted area is fixed relative to earth, and any changes will apply simultaneously to all UEs served within that area (e.g., all UEs served by a cell can determine that any power sharing for the current cell will end at t-Service along with cell coverage).

From RAN2#126 it is assumed that downlink coverage enhancements are supported for both earth-moving and quasi-earth fixed cells. Predicting when the UE will be impacted by a planned changes to beam power sharing/pattern/size is more complicated for earth moving cells, since when and for how long a UE is impacted depends on the UE’s relative location within the cell/beam, the satellite movement, and cell/beam characteristics like footprint size.

Observation 4:	Predicting when a UE will be impacted by changes to beam power sharing/pattern/size in earth-moving cell(s) requires satellite assistance information and UE calculation.

Previous releases use satellite assistance information (e.g., ephemeris information, reference locations, footprint information, epoch time etc.) to support various mobility and measurements enhancements in earth moving cells. Procedures developed enable a UE to approximate when current cell coverage ends, and neighbouring cell coverage begins. Similar solutions may be used to approximate when a UE will be impacted by future changes to beam power sharing/pattern/size.

Proposal 4:	Satellite assistance information (e.g., within SIB19) can be used to predict future changes to beam power sharing/pattern/size.

Application of DL-CE to both serving and neighbouring cell(s) could impact the robustness of the mobility procedure. Taking beam power sharing as an example, if an A3 event is triggered while the neighbour cell is at full power but about to be reduced soon due to power sharing, then a measurement report may be triggered based on outdated (or soon irrelevant) measurements. A similar issue could occur if the serving cell is currently under power sharing but about to improve relative to the neighbouring cell.

Although mobility problems caused by incorrectly triggering a measurement report may be somewhat mitigated by the network (since it may consider any planned power sharing prior to sending a handover command), a more critical example of this issue can occur during the evaluation and execution of CHO. For example, a UE unaware of future power sharing for a CHO candidate may risk executing the CHO to a cell which is about to experience DL coverage reduction. Similarly, a power sharing change may improve the coverage of a former cell after the CHO was executed, possibly causing the UE to ping pong.

Observation 5:	Future change(s) to beam power sharing/pattern/size in both source cell and target cell could impact measurement reporting and evaluation and execution of CHO.

The UE can avoid handing over to a cell which may soon no longer be suitable by considering future power sharing of a neighbouring cell or CHO candidate, avoiding possible RLF or service interruption due to an incorrect mobility event. This can be supported by providing power sharing assistance information for both the source and candidate cells, as previously discussed.

Proposal 5:	UE can consider a future change to beam power sharing/pattern/size during mobility and cell (re)selection.

# Conclusion

In this contribution the following observations and proposals are made regarding DL coverage enhancements:

Observation 1:  Only existing IEs can be used to handle legacy UEs to prevent from accessing to the cells operating with DL-CE.

Observation 2:	Changes to beam power sharing/pattern/size can impact many aspects of the system (e.g., application of DL coverage enhancements, measurement reporting, CHO execution etc.)

Observation 3:	Changes to beam power sharing/pattern/size are based on NW decision and thus predictable. Notifying UEs of upcoming changes can reduce undesirable UE behaviour.

Observation 4:	Predicting when a UE will be impacted by changes to beam power sharing/pattern/size in earth-moving cell(s) requires satellite assistance information and UE calculation.

Observation 5:	Future change(s) to beam power sharing/pattern/size in both source cell and target cell could impact measurement reporting and evaluation and execution of CHO.

Proposal 1a:	A new barring bit is introduced to control access of UEs supporting Rel-19 NTN DL-CE.

Proposal 1b:	Confirm that UEs supporting Rel-19 NTN DL-CE will not consider the existing NTN barring bit.

Proposal 2:	Define absolute priority information for DL-CE capable UEs.

Proposal 3:	UE can be notified of changes to beam power sharing/pattern/size for both serving and neighbouring cell(s).

Proposal 4:	Satellite assistance information (e.g., within SIB19) can be used to predict future changes to beam power sharing/pattern/size.

Proposal 5:	UE can consider a future change to beam power sharing/pattern/size during mobility and cell (re)selection.

# References

RP-241667 – Non-Terrestrial Networks (NTN) for NR Phase 3 – Thales, CATT

RAN1#116 chair notes

3GPP TR 38.821

RAN1#116b chair notes

RAN2#125bis chair notes

RAN1#117 chair notes

R2-2405767 – LS on DL coverage enhancements – CMCC

RAN2#127bis chair notes